Original Research Article

Effect of Mechanical Vapour Ejector System on thermal performance of Multipurpose Processing Vat during the manufacture of Khoa

.

ABSTRACT

The present investigation was conducted to study the effect of Mechanical Vapour Ejector System (MVES) on thermal performance of Multipurpose Processing Vat (MPV) during the manufacture of Khoa. The Multipurpose Processing Vat (MPV) is a steam-jacketed, conical frustum scraped surface kettle used to produce Khoa, Burfi, Kalakand, Rabri, Basundi, Halwa, etc. The investigation was aimed to intensify the process of heat transfer and improve the thermal performance of multipurpose processing vat used for manufacturing various traditional dairy products. In the study, the air flow rate of MVES, steam pressure and speed of scrapper were varied in different combinations, while the feed quantity, initial and final total solids of milk were kept constant for assessment of thermal performance parameters during khoa manufacturing. The effect of MVES was examined on influencing parameters of thermal performance of MPV. The significant (p<0.05) increase in rate of evaporation and overall heat transfer co-efficient different stages of manufacturing khoa was found increase with air flow rate, scrapper speed and steam pressure. Whereas, steam consumption and power requirement were significantly (p<0.05) reduced compared to MPV without MVES in manufacture of khoa. Additionally, the physico-chemical parameters and sensory characteristics of khoa were evaluated and compared for both systems, with the results showing significant improvement in sensory attributes.

Keywords: Multipurpose Processing Vat, mechanical vapour ejector system, thermal performance, overall heat transfer co-efficient

1. INTRODUCTION

Traditional milk products are an essential part of Indian cuisine and make a significant contribution to the socio-economic life of Indian society. In India, the market potential as well as the growth rate of traditional milk products are unparalleled and poised for a significant expansion with advancements in mass production technology. Further, India is the largest producer of milk globally, with approximately 50-55% of its total milk production being used to manufacture a wide range of traditional dairy products [1]. India’s milk production continued to increase about 6 per cent per annum, while the per capita availability registered a growth of above 4.5 per cent during the past five years. The milk production of India has reached to 230.58 million tonnes per annum in the year 2022-23 [2], accounting for more than 24 per cent of the world’s total milk production.

In India, about 46 per cent of the total milk produced, is consumed as liquid milk and remaining milk is converted into variety of traditional milk products such as paneer and its products, chhana, Khoa, and Khoa based sweets and desserts [3,4]. Further, in India, a diverse array of traditional milk products is manufactured, including Khoa and Khoa-based sweets, chhana, paneer, shrikhand, rabri, basundi, kheer, halwa, kunda, payasam, thabdi, thabdi peda, along with fermented products and various region-specific ethnic delicacies. Each of these products boasts distinctive flavors, textures, and appearances [5]. The production of traditional milk products is primarily limited to small-scale operations within the unorganized sector [6,7]. In India, traditional dairy products account significant importance and make up over 90% of all dairy products consumed in the country [8].

Khoa is a traditional milk product made through heat desiccation, holds a central role among traditional Indian dairy products, serving as the foundation for numerous sweets. Approximately 7% of milk produced in India is converted into Khoa [9]. It is prepared by partial desiccation of milk through heating under controlled conditions [10]. In India, approximately 6 lakh tonnes of khoa is produced each year, accounting for 7% of the country's total milk production [11].

The khoa is traditionally manufactured by process of heat desiccation in open pan with constant stirring. However, these methods of khoa making requires a lot of labour as well as energy and therefore energy saving requires to pay more attention. Further, the traditional khoa manufacturing methods have several drawbacks, such as limited capacity due to batch processing, inconsistent product quality for commercial use, and inefficient energy utilization. These methods also result in larger equipment due to low heat transfer, a high reliance on manual labor, and poor hygiene and sanitation standards. In view of the above facts, the present investigation was aimed to assess the effect Mechanical Vapour Ejector System (MVES) on thermal performance of Multipurpose Processing Vat (MPV) during the manufacture of Khoa.

2. material and methods

Pasteurized packed mix milk having 4.5% fat and 8.5% SNF was procured and collected in clean and dry steel cans and kept at refrigeration temperature. The milk samples for chemical analysis were prepared as per the method described in the BIS Handbook [12].

2.1 Experimental Set Up

The open type Scraped Surface Heat Exchangers (SSHEs) are widely used in the dairy industries for carrying out heat desiccation operation. They are ideally well-matched for products that are viscous, sticky and that contain particulate matter. Particularly, the kettle with bottom jacket are well known for their application in the manufacture of Khoa. The Multipurpose Processing Vat (MPV) for traditional milk products is based on steam jacketed conical frustum open type kettle having volumetric capacity of 40 liter. The kettle is considered as Multipurpose Processing Vat as it serves the multiple purposes of manufacture of Khoa, Burfi, Kalakand, Rabri, Basundi and Halwa, etc. The MPV designed from AISI 316 stainless steel, was provided with all mountings like pressure gauge, thermodynamic type steam trap, steam safety valve, air release valve, steam regulating valve, worm and worm wheel mechanism for unloading of processed product and lid for covering the MPV.

The MPV is also consisting of specially designed scraper assembly provided with reduction gear for fixed rotation per minutes. The Variable Frequency Drive (VFD) is installed to have variations of speed in the range of 1 to 50 rpm to get desired attributes of the product. The steam pipe lines are connected with required accessories like strainer, steam control valve, expansion bend, safety valve, steam trap, etc. The design specifications of the MPV used for present investigation are given in Table 1.

Table 1. Specification of Multipurpose Processing Vat (MPV)

| Sr. No. | Particulars | Specifications |
| --- | --- | --- |
|  | Material of Construction | AISI SS 316 |
|  | Thermal Conductivity (K) | 16.2 W/m K |
|  | Diameter of Top (Inner Shell) | 1.080 m |
|  | Diameter of Bottom (Inner Shell) | 0.736 m |
|  | Thickness of Inner shell | 0.005 m |
|  | Height of Inner shell (Conical) | 0.380 m |
|  | Volume of kettle | 40 L |
|  | Frustum Slant Height | 0.420 m |
|  | Thickness of Jacket (Bottom) | 0.080 m |
|  | Thickness of Bottom Base | 0.010 m |
|  | Total Area of the kettle | 1.5952 m2 |
|  | Effective Heat Transfer Area of Kettle (Ae) | 0.4069 m2 |

The Mechanical vapour ejector system (MVES) was designed and installed to remove water vapours produced inside during milk concentration. The MVES consists of a radial type of blower for vapour removal with specially designed quadrilateral hood and a base for fixing the ejector to the MPV as per specification (Table 2). The rotation of radial blades causes suction of vapour from MPV and discharge at the outlet due to centrifugal force. The flow is tangential to the axis of shaft. Various components of MVES were casing, impeller, shaft, motor and suction/discharge ports. Motor was selected as per IP-55 specifications in which shaft has a liquid seal and the motor winding is completely enclosed to prevent moisture ingress and damage to motor winding. MVES was fitted on the top of the MPV using circular base. Suction created inside the MPV and removal of vapour depended on the MVES air flow rate. The air flow rate of MVES was controlled by potentiometric speed controller.

Table 2. Specification of Mechanical Vapour Ejector System (MVES)

| Sr. No. | Particulars | Details |
| --- | --- | --- |
|  | Motor Model | CB 100836 W |
|  | Rated power | 25 W |
|  | Voltage, Phase & Frequency | 230 V, 1 Phase, 50Hz |
|  | Suction port diameter (Ds) | 7.4 cm |
|  | Discharge port diameter (Dd) | 7.5 cm |
|  | Max. Static Pressure | 200 Pa |
|  | Rated Speed | 2800 rpm |
|  | Max. air flow | 100 CFM |

2.2 Experimental process variables

The effect of MVES on thermal performance of MPV during the milk concentration process was evaluated at various combinations of two different steam pressures (98.06 kPa and 196.12 kPa), three different (0.33 rps, 0.50 rps and 0.66 rps) scraper speed and four different levels of air flow rate of MVES (0.000 m3/s, 0.0094 m3/s, 0.0188 m3/s, 0.0282 m3/s) as the experimental process variables. Further, the data recorded during the experimental trials were used for the estimation of different thermal performance parameters.

2.3 Preparation of khoa

The khoa was manufactured using the combination of process variables in MPV employed with MVES (developed system) as well as MPV without MVES (control system). The process flow diagram for manufacturing khoa is illustrated in Figure 1.

| Standardized Pasteurized Milk Total Solids= 13.0 % (4.5 % Fat, 8.5 % SNF, Batch= 12 Kg) |  |  |
| --- | --- | --- |
| Filtration of milk |  |  |
| Milk poured in Multipurpose Processing Vat (MPV) |  |  |
| Steam Pressure 98.06 kPa |  | Scraper Speed 0.666 rps |
| First Boiling (Stage-I) |  |  |
|  | Air Flow Rate 0 m3/s & 0.0282 m3/s |  |
| Commencement of concentration |  |  |
| Concentration continued |  |  |
| Pre-pat formation stage (Stage-II) (Total Solids = 45.5%) |  |  |
| Pat formation (Stage-III) (60-65%TS) |  |  |
| Khoa (65-70% TS) |  |  |
| Collection of khoa in clean SS trays & Spreading into uniform thick layer |  |  |
| Cooling and storage at room temperature |  |  |

Figure 1. Process flow diagram of khoa preparation

2.4 Thermal performance evaluation during manufacture of khoa

The thermal performance evaluation was also carried out during manufacturing of khoa using the optimized process variables in a MVES employed MPV (Developed system) and MPV without MVES (Control system). The thermal performance evaluation involved the measurement and estimation of different parameters such as time for boiling, time for concentration, time for pat formation, evaporation rate, overall heat transfer coefficient (U-value for stage-I, stage-II and stage- III), steam consumption and electrical energy consumption.

2.4.1 Determination of evaporation rate

The evaporation rate was measured as kg water evaporated per hour. The rate of evaporation was calculated by Total Solids (TS) difference and mass balance. The quantity of water evaporated per hour was estimated by following equations:

Ratio of concentration (R) = CfC						(1)

Where,

Cf = final concentration of milk (% TS)

C = initial concentration of milk (% TS)

Quantity of concentrated milk (P)=  MR  (kg)					(2)

Where,

M =quantity of initial milk supplied for concentration (kg)

R= Ratio of concentration

Evaporation rate (E) = M-PT×60  (kg/h)							(3)

Where,

M = Initial quantity of milk taken for concentration (kg)

P= Quantity of concentrated milk obtained (kg)

T= Time required for concentration (min)

2.4.2 Time for boiling, concentration and pat formation

The time (in minutes) taken for boiling of milk from the initial temperature of 100C to reach a boiling point was measured as time for boiling. Further, time taken to achieve concentration of milk (~45% TS) from initial concentration (13.0 %TS) was measured as concentration time. While, the time taken after a milk concentration (65% TS) to reach a final stage called pat formation (~65% TS) was measured as time for pat formation. The total time for processing was measured by addition of time taken for boiling, concentration as well as pat formation.

2.4.3 Sensible heat transfer

QS= M x CpM x (Ti - Tp) = U Ae Δθm			(4)

Where,

QS = Sensible heat transfer, (kJ)

M = Mass of feed, (kg)

CpM = Specific heat of feed, (kJ/kg K)

Tp = Evaporation temperature, (K)

Ti = Temperature of feed, (K)

U = Overall heat transfer co-efficient, (W/m2 K)

Ae = Effective area of heat transfer, (m2)

Δɵm = Log mean temperature difference, (K)

2.2.4 Latent heat transfer

QL = E x L 						(5)				Where,

QL = Latent heat transfer, (kJ)

E = Rate of evaporation, (kg/h)

L = Latent heat of evaporation, (kJ/kg)

2.4.5 Determination of overall heat transfer co-efficient (U-value)

According to Fourier’s heat flow equation,

Qs + QL = U × A×(Ts−Tp)				(6)

Where,

U = Overall heat transfer co-efficient (U-value), W/m2K

A = Effective heat transfer area, m2

Ts = Temperature of steam corresponding to steam pressure, °C

Tp = Temperature of the product, °C

2.4.6 Steam consumption

The steam consumption was estimated by measuring the quantity of condensate collected at the steam trap. At the start of trial, 3 kg water was taken in a steel can and condensate pipe was dipped into water. Water is taken in can to avoid formation of flash vapour of condensate and it gives accurate quantity of condensate. Quantity of condensate was calculated as difference of initial and final weight of can. The steam consumption was measured in kilogram (kg). The dryness fraction of steam was also estimated (X= 0.86). The total steam consumption was measured by addition of steam utilized for boiling and concentration of milk.

2.4.7 Electrical energy consumption

A three-phase electric power supply is required to operate motor of MPV agitator and a single-phase electric power supply is required for MVES. During experimental trials, VFD that has an inbuilt energy meter measured electric power consumption of agitator motor. Hence, the total power consumption is calculated as:

Total power consumption = PMPV + PMVES			(7)

Where,

PMPV = Power consumed by MPV Agitator, (Wh)

PMVES= Power consumed by Motor of MVES, (Wh)

2.5 Physico-chemical and sensory analysis of khoa

The khoa prepared in developed and control system was evaluated for physico-chemical and sensory characteristics. Khoa’s desired colour, flavour, body and texture were evaluated by the BIS scorecard for sensory attributes. Results of the physico-chemical and sensory attributes of khoa prepared in developed and control system were analysed statistically.

3. results and discussion

3.1 Optimization of process variable for manufacturing of khoa

The data of thermal performance parameters was subjected for optimization of process variables for MVES. The Design Expert 13.0 statistical analysis software was used for the optimization of process parameters. As shown in Table 3, the optimized process variables were selected for the further preparation of traditional milk products viz Khoa.

Table 3. Optimization of process variables

| No. | Steam pressure (kPa) | Scraper speed (rps) | Air flow rate (m3/s) | Desirability |
| --- | --- | --- | --- | --- |
| 1 | 98.06 | 0.666 | 0.0282 | 0.832 |

3.2 Effect of MVES on thermal performance evaluation in manufacture of khoa

The thermal performance of developed system was evaluated by measurement and estimation of different parameters such as time for boiling, time for concentration, time for pat formation, total time for processing, evaporation rate, overall heat transfer coefficient (Stage-I= Boiling, Stage-II=Concentration, Stage-III= Pat formation), steam consumption and electrical energy consumption. Evaluation of the thermal performances of the developed system (MPV employed with MVES) and control system (MPV without MVES) were carried out using optimized levels of experimental variables and their effect was compared for the individual performance parameters as shown in the Table 4.

As shown in the Table 4, the comparative analysis of the thermal performance parameters for control and developed system in preparation of Khoa, time for boiling, concentration and pat formation was reduced 4.10%, 20.70% and 8.10%, respectively. Thus, total time for processing for developed system was reduced 15.80% in comparison to the control system. The statistical analysis shown that, the time for concentration was significantly (p<0.05) decreased, however, time for boiling and pat formation was comparable for both the systems. The evaporation rate was significantly (p<0.05) improved (17.50%) for the developed system as compared to control system. Further, the U value for stage-I, stage-II and stage-III were increased 0.20%, 17.80% and 3.50% for developed system in comparison to control system. The statistical analysis (Table 4) shown that, U value for stage-II was significantly (p<0.05) increased. Whereas, the U value for stage-I and stage-III was slightly higher for developed system but not significantly (p>0.05) different from control.

The systems were compared for power consumption, it was observed that the total power requirement was significantly (p<0.05) decreased (10.01%) for the developed system as compared to control system recompensing the additional power required for MVES. Similar effect was observed in case of the steam consumption, that was significantly (p<0.05) reduced (12.70%) for the developed system as compared to control system. The steam consumption was higher due to highly viscous semi-solid (about 60% TS) at pat formation stage which required great amount of energy due to the minimum heat penetration rate.

Table 4. Comparison of thermal performance parameters for control and developed system in manufacture of Khoa

| No | Parameter | Unit | Control System | Developed System | P value (p<0.05) | % Change |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Time for Boiling | min | 12.33±0.29 | 11.83±0.29 | NS | - 4.1% |
| 2 | Time for concentration | min | 55.67±0.76 | 44.17±0.58 | S | - 20.7% |
| 3 | Time for pat formation | min | 16.17±1.04 | 15.17±0.76 | NS | - 8.1% |
| 4 | Total time for processing | min | 84.17±0.58 | 71.17±1.26 | S | -15.8% |
| 5 | Evaporation rate | kg/h | 8.15±0.02 | 9.82±0.23 | S | +17.5% |
| 6 | U Value (Stage-I) | W/m2k | 261.92±0.71 | 262.4±1.09 | NS | +0.2% |
| 7 | U Value (Stage-II) | W/m2k | 526.47±7.19 | 640.67±3.02 | S | +17.8% |
| 8 | U Value (Stage-III) | W/m2k | 247.35±16.54 | 248.98±14.44 | NS | +3.5% |
| 11 | Power consumption | Wh | 514.82±3.53 | 464.96±8.22 | S | -10.01% |
| 12 | Steam consumption | kg | 55.7±1.71 | 48.6±1.08 | S | -12.7% |
| Control System= MPV without MVES, Developed System= MPV employed with MVES Each observation is an average of three replicate experiments (n=3), S = significant and NS= Non- significant |  |  |  |  |  |  |

3.3 Effect of MVES on physico-chemical characteristics of khoa

A khoa sample from the whole batch of khoa was taken as procedure described in Indian Standard: 2785-1964 [13]. The proximate analysis of chemical composition for khoa such as moisture content, protein, fat and ash were determined according to AOAC [14] standard method. While the total carbohydrate content was determined by difference. Further, the method prescribed by Deeth et al. [15] was used to estimate the Free Fatty Acids (FFA) content of khoa. While, the Titratable acidity of khoa was determined by the procedure as described in BIS: Part XI, 1981.

When the value for physico-chemical characteristics was compared, the moisture, protein, total carbohydrate, ash, FFA and acidity values were found non-significant (p>0.05) for control sample of khoa in comparison of experimental sample; while the fat values was found significantly (p<0.05) reduced due to loss of fat in sticking of khoa under the slight vacuum conditions (Table 5).

Table 5. Comparison of physico-chemical characteristics of khoa prepared by control and developed system

| No. | Parameters | Control Sample | Experimental Sample | P value (p<0.05) |
| --- | --- | --- | --- | --- |
| 1 | Moisture (%) | 30.50±0.93 | 31.81±0.81 | NS |
| 2 | Fat (%) | 23.76±1.06 | 22.23±0.35 | S |
| 3 | Protein (%) | 19.34±0.65 | 20.16±1.0 | NS |
| 4 | Total Carbohydrate (%) | 22.94±2.2 | 22.41±2.07 | NS |
| 5 | Ash (%) | 3.45±0.07 | 3.39±0.34 | NS |
| 6 | Free fatty acid (%oleic Acid) | 0.48±0.01 | 0.47±0.01 | NS |
| 7 | Acidity (%LA) | 0.46±0.02 | 0.5±0.04 | NS |
| Each observation is an average of three replicate experiments (n=3) S = significant and NS= Non- significant |  |  |  |  |

3.4 Effect of MVES on sensory attributes of khoa

The khoa was analyzed using 100-point sensory score card as per BIS (1980). The colour & appearance score of the khoa prepared using developed system was higher as compared to control system. The colour & appearance score was significantly (p<0.05) increased for the developed system as it observed the lower browning and therefore resulted in slightly more whitish in colour. However, the Khoa prepared in control system had a whitish yellow colour.

The body and texture score for khoa prepared in developed system has no significant difference (p>0.05) with the control system. It was observed that the body and texture of fresh khoa prepared in both the systems was slightly harder due to lower moisture content as shown in Table 6. Further, the flavour score was improved significantly (p<0.05) for experimental sample compared to control sample. It was perceived that the flavour of the khoa prepared in control system had a mildly cooked flavour, while the khoa prepared in the developed system had perceived with slightly nutty flavour. The total sensory score was significantly (p<0.05) higher for experimentally prepared khoa in the developed system in comparison to khoa prepared in the control system. The average sensory scores of khoa is presented in Table 6.

Table 6. Comparison of sensory scores of khoa prepared by control and developed system

| No | Khoa | Score | Control Sample | Experimental Sample | P value (p<0.05) |
| --- | --- | --- | --- | --- | --- |
| 1 | Colour & appearance | (15) | 12.45±1.12 | 13.6±0.53 | S |
| 2 | Body & texture | (30) | 24.13±2.07 | 23.96±1.38 | NS |
| 3 | Flavour | (50) | 40.54±2.72 | 43.97±1.99 | S |
| 4 | Total score | (100) | 82.13±5.05 | 86.53±2.93 | S |
| Total score includes score of 5.0 for package Each value is an average of three replicates (n=3), S = significant and NS= Non- significant |  |  |  |  |  |  |

4. Conclusion

In the present study, the effect of MVES on thermal performance of MPV was evaluated during manufacture of khoa. The combination of steam pressure (98.06kPa), scraper speed (0.666 rps) and air flow rate (0.0282 m3/s) were optimized statistically. Further, the khoa was manufactured in a control system (MPV without MVES) as well as developed system (MPV with MVES) and various thermal performance parameters were evaluated and compared. It was found that, the developed system reduced time for boiling, concentration, pat formation, and time for total processing by 4.1%, 20.7%, 8.1%, and 15.8%, respectively, compared to the control system during khoa preparation. While, evaporation rate and overall heat transfer coefficients for stages I, II, and III increased by 17.5%, 0.2%, 17.8%, and 3.5%, respectively. The power consumption and steam consumption were decreased by 10.1% and 12.7%, respectively, as compared to control system. Furthermore, the physico-chemical and sensory characteristics were compared for khoa manufactured in a control system as well as developed system and found that the khoa samples was comparable in terms of physico-chemical characteristics, while the experimental khoa sample found with improved sensory attributes. Thus, from the current study it can be concluded that the MPV employed with MVES could be effectively used for the manufacture of khoa with reduced energy requirements, lower processing times and improved sensory attributes.

References

- Baladhiya, C. S., Doshi, J. S., and Bhalodiya, V. B. (2020). Performance evaluation of mechanized manufacture of bottle gourd Halwa. International Journal of Agricultural Engineering, 2020, Vol. 13, No. 1, pp. 113-120.

- Basic Animal Husbandry Statistics-2023 (2024, May 22). Government of India: Ministry of Fisheries, Animal Husbandry & Dairying, Department of Animal Husbandry and Dairying, Krishi Bhawan, New Delhi.

- Patil, G. R. (2005) Innovative processes for indigenous dairy products. Indian Dairyman. 57(12): p. 82.

- Ghosh, D., Debnath, A., & Chetri, A. (2019). Effect of processing parameters on quality attributes of cow skim milk paneer. The Pharma Innovation Journal, 8(1), pp. 429–431.

- Pal, D., & Raju, P. N. (2010). Traditional Indian dairy products with functional attributes: status and scope. Indian Food Industry, 29(1), 13-21.

- Dodeja, A. K. (2014). Application of SSHE for mechanized production of Indian dairy products. Compendium of ICAR sponsored summer school on “Engineering interventions in processing and value addition of milk and milk products” organized by National Dairy Research Institutes, ICAR, Karnal, pp. 6-10.

- Patel, S. M. & Bhadania, A. G. (2015). Mechanized production of Traditional Indian Dairy Product: Present status, Opportunities and Challenges, Compendium of national seminar on “Indian Dairy Industry – Opportunities and Challenges” organized by SMC College of Dairy Science, Anand, pp. 214-222.

- Aneja, R. P., Mathur, B. N., Chandan, R. C., & Banerjee, A. K. (2002). Technology of Indian milk products: Handbook on process technology modernization for professionals, entrepreneurs and scientists. Dairy India Yearbook. Dairy India Publication, New Delhi. pp. 133-142.

- Rasane, P., Beenu, T., & Anirban, D. (2015). Khoa: a heat desiccated indigenous Indian dairy product. Research Journal of Pharmaceutical, Biological and Chemical Sciences, 6(5), pp. 39-48.

- Ganguli, N.C. (1981). Buffalo as a candidate for milk production. IDF Bulletin No. 137, FAO, Rome, Italy.

- Rajarajan, G., Kumar, C. N., & Elango, A. (2007). Distribution pattern of moulds in air and khoa samples collected from different sections of khoa plants. Indian Journal of Dairy Science, 60, 133–135.

- Indian Standard IS: 4883. Indian Standards specification for Khoa, Indian Standards Institution, New Delhi; 1980.

- Indian Standard. ISI Handbook of Food Analysis Part XI Dairy Products, Bureau of Indian Standards, New Delhi, India; (IS SP 18, 1981, Part XI).

- A. O. A. C. (2005). Official methods of analysis (16th ed.), Association of Official Analytical Chemists, Washington, DC.

- Deeth, H. C., Fitz-Gerald, C. H., & Wood, A. F. (1975). A convenient method for determining the extent of lipolysis in milk. Austrilian Journal of Dairy Technology, 30(3), pp. 109-111.