Assessment of Polycyclic Aromatic Hydrocarbon (PAH) and Polychlorinated Biphenyl (PCB) Congeners in Soil from Industrial Sites in Rivers State, Nigeria: Environmental and Health Implications

Abstract

Polycyclic aromatic hydrocarbons (PAHs) and polychlorinated biphenyls (PCBs) are two categories of persistent organic pollutants (POPs) often used in various industrial contexts.    Many individuals are concerned about them because of their frequency and potential detrimental effects on the environment and public health. This study investigated the levels of PAH and PCB congeners in soil samples collected from selected industrial sites in Rivers State, Nigeria. A total of 10 samples were collected from four different industrial sites and a non-industrial area. The samples were analyzed for PAH and PCB congeners using Gas Chromatography-Mass Spectrometry (GC-MS). The results showed that the levels of PAH and PCB congeners in the soil samples were significantly higher than the background levels. Mean PAH concentrations of the samples ranged from 0.24mg/kg to 37.08mg/kg, while mean PCBs concentrations ranged from 0.72mg/kg to 7.01mg/kg. The lowest concentration of PAH was recorded in the topsoil sample obtained from the Transformer oil storage depot at 0.24mg/kg. Additionally, the second-highest total PCB concentration was found in its surface soil (4.09mg/kg), suggesting significant contamination, particularly in deeper soil layers. The dominant PAHs congeners were benzo(a)pyrene, benzo(b)fluoranthene, and indeno(1,2,3-cd)pyrene, while the dominant PCBs congeners were 4,4'-Dichlorobiphenyl,  2,2',3,4,4',5',6-Heptachlorobiphenyl, 2,2',3,4,4',5',6-Heptachlorobiphenyl, and 2,2',3,3',4,5,6,6'-Octachlorobiphenyl. Results obtained from this study suggest that industrial activities in Rivers State contribute significantly to the contamination of the soil with PAH and PCB congeners, which can have adverse environmental and health impacts. The study recommended that regular soil testing should be conducted to monitor PAH and PCB levels in soil. Moreover, bioremediation and chemical treatment should be used to clean up PAH- and PCB-contaminated soil. Furthermore, proper waste management practices and collaboration between government agencies, industries, and communities are necessary for managing PAH- and PCB-contaminated soil.

Keywords: PAHs, PCBs, Industrial Sites, Soil Contamination, Environmental impacts, Health impacts

1.0 Introduction

Environmental pollution represents a threat to human health, quality of life and the natural function of ecosystems. The organic pollutants are qualified among the most harmful pollutants due to the eco-toxicological risks and their slow degradability, hence their persistence (Matei et al., 2023). Polycyclic aromatic hydrocarbons (PAHs) and polychlorinated biphenyls (PCBs) are two categories of persistent organic pollutants (POPs) often used in various industrial contexts (WHO, 2010; EPA, 2017).    Many individuals are concerned about them because of their frequency and potential detrimental effects on the environment and public health.    Concerns exist over the environmental dispersion of these poisons, their mobility, and their impact on populations, particularly in soil next to industrial zones.

1.1 Polycyclic Aromatic Hydrocarbons (PAHs)

Polycyclic aromatic hydrocarbons (PAHs) are produced during the incomplete combustion of fossil fuels. PAHs are aromatic hydrocarbons with two or more interconnected benzene rings that may be configured in various arrangements (Nguyen et al., 2014; Sharma, 2014).     They lack any heteroatoms or substituents.      Light PAHs possess up to four rings, while heavy PAHs have more than four rings.      Kuppusamy et al. (2016) discovered that heavy PAHs exhibit greater stability and pose a greater risk than light PAHs.      Moreover, PAHs consist solely of carbon and hydrogen atoms interconnected in various ring configurations.     The benzene rings exhibit distinct physical, chemical, and biological properties from one another.  PAHs existing in solid, liquid and gaseous phases could have an impact on human health through inhalation, dermal absorption and or ingestion (Akpan et al., 2022).

1.2 Polychlorinated Biphenyls (PCBs)

Polychlorinated biphenyls (PCBs) are a significant category of compounds that were produced extensively for various commercial applications from the 1930s to the 1970s.    PCBs enter the environment during their production due to accidental spills and leaks, and during transportation, use, disposal, etc. PCBs own “dioxin-like toxicity”, which makes it a probable carcinogen due to this reason (Sharma et al., 2018; Najam & Alam, 2023). The majority of their environmental dissemination originates from industrial applications (Jones and De Voogt, 1999).    PCBs, synthetic compounds, were extensively used as coolants and insulators in electrical apparatus (EPA, 2017).

In the 1970s, the manufacturing and use of PCBs were prohibited due to evidence of their detrimental effects on the environment and on human and animal health (Langer, 2005).    Due to their lipophilicity, they may accumulate in cells, ascend the food chain, and persist in soils, sediments, and water for extended periods (Mackay et al., 1997).    Indigenous marine and soil species may lack the capacity to rapidly degrade PCBs, which might be the primary cause for their persistence in the environment.

In the 1970s and 1980s, several nations implemented stringent regulations regarding the use and disposal of PCBs.    Consequently, their environmental impact has significantly diminished.  Nonetheless, PCBs continue to be discharged from contaminated aquatic environments such as lakes and marine sediments.    Recent studies indicate that contaminated regions may emit significant quantities of persistent organochlorines into the atmosphere, which might then disperse globally (Langer, 2005).    Many of them deteriorate with time, causing harm to animals and plants, which is detrimental to the ecosystem.    Research on PCB concentrations in many environmental sectors has been conducted for several years; nonetheless, there is an insufficiency of accurate data about PCB contamination, and the little information available is subject to significant scrutiny.

The majority of individual chlorobiphenyls exist as solids at ambient temperature.    They exhibit poor electrical conductivity, possess a high flash point (170–380°C), and have significant resistance to heat and chemicals.

PAHs and PCBs are recognised as hazardous substances associated with carcinogenicity and many health and environmental issues (IARC, 2010; ATSDR, 2015).    The extensive use of PAHs and PCBs in manufacturing has resulted in significant contamination of terrestrial, aquatic, and atmospheric environments (Kisku et al., 2017).    Nwachukwu et al. (2010) assert that the oil and gas sector in Nigeria, in conjunction with other sectors, is a significant producer of PAHs and PCBs.    Among the principal oil-producing states in Nigeria, Rivers State is more susceptible to environmental degradation caused by industrial facilities (Osuji et al., 2015).

Soil contamination with PAHs and PCBs is a significant issue due to its enduring impacts on human and environmental health (Baird et al., 2017).    PAHs and PCBs may persist in the environment for extended periods; nevertheless, flora and fauna can assimilate them, potentially introducing these compounds into the food chain (Wang et al., 2016).    Cancer, reproductive and developmental issues, and damage (Lauby-Secretan et al., 2013) are among the health concerns associated with PAHs and PCBs.

Exposure to PAHs and PCBs presents certain dangers; nevertheless, the extent and locations of these chemicals in the area around workplaces in Rivers State remain unknown (Nwachukwu et al., 2010).    This research aims to quantify the levels of PAH and PCB congeners in soil samples from several industrial locations in Rivers State and to investigate their potential environmental and health impacts.

PAHs and PCBs are two categories of POPs that have been extensively researched in environmental studies.

- Sources of PAHs and PCBs

PCBs are synthetic compounds extensively used as insulators and coolants in electrical apparatus.  PAHs are produced during the incomplete combustion of fossil fuels (WHO, 2010).    Industrial activities such as oil refining, petroleum extraction, lubricant manufacturing, and electronics manufacturing are significant contributors to the environmental presence of PAHs and PCBs (Kisku et al., 2017; Nwachukwu et al., 2010).

- Environmental Fate and Transport

PAHs and PCBs have prolonged environmental persistence and may disseminate via soil, water, air, and other mediums (Baird et al., 2017; Wang et al., 2016).    The contamination of the ground with PAHs and PCBs is alarming due to its enduring impact on human and animal health (Lauby-Secretan et al., 2013).

- Human Health Impacts

Cancer, reproductive and developmental issues, and neuronal impairment are among the health concerns associated with PAHs and PCBs (ATSDR, 2015; IARC, 2010).    The International Agency for Research on Cancer (IARC) has said that some PAHs and PCBs are carcinogenic to humans (IARC, 2010).

- Remediation and Management

Soil contaminated with PAHs and PCBs may be remediated using many methods, including excavation, bioremediation, and chemical treatment.    The efficacy of these technologies will be contingent upon the particular characteristics of the location and the contaminants present (Kang et al., 2018).

Researchers have investigated the PAH and PCB congeners present in water, soil, sediment, and components of ecosystems.    Van der Wat and Forbes (2015) used lichens as biomonitors for semi-volatile organic air pollutants, particularly PAHs, due to their ability to detect varying pollution levels, longevity, and capacity to indicate the presence and concentrations of PAHs.       Soxhlet and ultrasonic-assisted extractions are mostly used to get analytes from plants for environmental research.     Consequently, lichens have lately gained widespread recognition globally.       The majority of PAH profiles consist of phenanthrene, fluoranthene, naphthalene, and pyrene.  Nonetheless, lichens from various sites exhibit differing total PAH concentrations, notwithstanding the prevalence of 2- and 3-ring PAHs over 6-ring PAHs.       Accurate and reliable methods for detecting organic pollutants in lichens would facilitate the comparison of data across several investigations.

Rahmanpour et al. (2014) identified PAHs in four fish species from the Persian Gulf, representing various trophic levels.       Snyder and West.  Wei et al. (2015) investigated both solid and gaseous states, identifying 29 parent and alkyl polycyclic aromatic hydrocarbons (PAHs), 15 oxygenated PAHs (OPAHs), 11 nitrated PAHs (NPAHs), and 4 azaarenes (AZAs) in the atmosphere.  They also identified organic carbon, elemental carbon, char, and soot associated with the particles.

PAHs are present in home dust (Iwegbue 2014; Qi et al., 2014; Yang et al., 2015).       In several African nations, such as Nigeria, the extent of PAH contamination in the soil next to commercial establishments remains ambiguous.

3.0 Material and Methods

3.1 Study Area

Fig 1. Study area showing sampling points

The study area comprises 4 industrial and 1 non-industrial sites. The industrial sites were Iloabuchi Electronics Market (4°47'27"N 6°59’16.082"E), Indorama Petrochemical (4°48'28"N 7°06’6.121"E), Transformer Oil Storage Site (4°53'42"N 6°59’5.121"E) and Uniport Academy Colour factory (4°54'05"N 6°54' 6.150"E). The sample for the control was collected in the Elekahia residential area (4°49'20"N 7°01’22.132"E).

3.2 Industrial Activities in Study Sites

Iloabuchi electronics market is situated in Akokwa-Iloabuchi street, Mile 2 Diobu, Port Harcourt in Rivers State. The market specializes in the sales and repairs of electronic appliances, equipment, accessories, etc. Illegal electronic waste dumps exist within the market, which can be a source of contamination to the ecology and humans.

Indorama Petrochemicals Company is situated in the Eleme Local Government Area of Rivers State. It specializes in the production of polyolefins, majorly polyethylene and polypropylene resins. These are used in the manufacture of a wide range of polyethylene and polypropylene products including fertilizers, medical gloves, textiles, packaging materials etc.

The PHED Transformer Oil Storage site is located along Rukpoku in Obio Akpor LGA of Rivers State. It is a Depot that stores transformer oil used to achieve electrical stability and insulation in power transformers. It also acts as a cooling agent. Transformer oil is a blend of highly refined mineral oil with other additives such as antioxidants, etc. It could be of Napthanic, paraffinic or synthetic origin.

Uniport Academy Colour is a paint factory located in Abuja Park Road Choba, in Emuoha LGA of Rivers State. This factory/paint laboratory specializes in the manufacture, mixing and blending of paints and other paint products. Some of their products include emulsion paints (Total weather resistant emulsion paint and Academy weather resistant emulsion paint), gloss paint, texture paint, etc. Several chemical components are employed in the manufacture of the paint products including surfactants, co-solvents, defoamers, biocides etc.

## 3.2 Determination of Analytes

The determination of PAH and PCB congeners was carried out at Tidalflow Nigeria Limited Laboratory Department, Port Harcourt.

## 3.3	Sample Preparation

## The soil samples were dried at room temperature for 5-8 days. On confirmation that the samples are dried, they were ground using a ceramic mortar, sieved (212µm) and kept at 4oC prior to chemical analysis.

## 3.4	Sample Extraction

## The Extraction method used for PAH and PCB analysis in the soil samples was Soxhlet extraction. Two (2) grams of each sample was weighed into a glass beaker, and 20 ml of the organic solvent, Dichloromethane, was added to the weighed sample in the beaker and mixed properly with a stirring rod. Activated copper was added to remove inorganic sulphur in the extract.

## Samples were subsequently filtered with funnel, cottonwool, silica gel and sodium sulphate. The cotton wool is used to remove visible impurities, while silica gel is used to remove invisible impurities. Sodium sulphate removes water from the sample; thus, it is employed as a dehydrating agent. After the filtration process, the samples were allowed to concentrate for 1-2 hours.

## 3.5	Determination of PAH in Soil Samples.

A Gas Chromatograph-Mass Spectrometer (GC-MS) was used for the determination of PAH in the soil samples. One (1) ml of dichloromethane was added to the concentrated sample, and one (1) microliter of the sample was collected using an analytical syringe and injected into the injector compartment of the GC-MS. The sample was analyzed for 25 minutes. The GC uses gases such as air, Helium and Hydrogen. The function of air and hydrogen is to heat up the instrument while Helium is the carrier gas which carries the sample injected through the column to the detector in the GC.

3.5	Determination of PCB in Soil Sample

A Gas Chromatograph-Mass Spectrometer (GC-MS) was used to analyze the extract from the soil samples for PCB congeners. One (1) ml of dichloromethane was added to the concentrated sample, one (1) microliter of the sample was collected using an analytical syringe and injected into the injector compartment of the GC-MS.

4.0 Results and Discussions

Concentrations of Polycyclic Aromatic Hydrocarbon (PAH) Congeners in Samples obtained from Control Site

The concentrations of various PAHs detected in the control soil sample are shown in Table 1. In the topsoil sample, PAH concentrations were below the detection limit. Similarly, subsoil samples gave a concentration of PAH congeners that were below the detection limit.

Table 1:	Concentrations of Polycyclic Aromatic Hydrocarbon (PAH) Congeners in Control Samples

|  |  | Topsoil sample | Subsoil sample |
| --- | --- | --- | --- |
| S/N | Compound | Concentration (ppm) | Concentration (ppm) |
| 1 | Naphthalene | <0.01 | <0.01 |
| 2 | Acenaphthylene | <0.01 | <0.01 |
| 3 | Acenaphthene | <0.01 | <0.01 |
| 4 | Fluorene | <0.01 | <0.01 |
| 5 | Phenanthrene | <0.01 | <0.01 |
| 6 | Anthracene | <0.01 | <0.01 |
| 7 | Fluoranthene | <0.01 | <0.01 |
| 8 | Pyrene | <0.01 | <0.01 |
| 9 | Benz[a]anthracene | <0.01 | <0.01 |
| 10 | Chrysene | <0.01 | <0.01 |
| 11 | Benzo[b]fluoranthene | <0.01 | <0.01 |
| 12 | Benzo[k]fluoranthene | <0.01 | <0.01 |
| 13 | Benzo[a]pyrene | <0.01 | <0.01 |
| 14 | Dibenz(a,h)anthracene | <0.01 | <0.01 |
| 15 | Indeno(1,2,3-cd)pyrene | <0.01 | <0.01 |
| 16 | Benzo[g,h,i]perylene Total | <0.01 <0.01 | <0.01 <0.01 |

| Compound | Electronic Site (Topsoil) | Electronic Site (Subsoil) | Petrochemical Industrial site (Topsoil) | Petrochemical Industrial site (Subsoil) | Transformer Oil Storage Site (Topsoil) | Transformer Oil Storage Site (Subsoil) | Paint Company (Topsoil) | Paint Company (Subsoil) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Naphthalene | 0.13 | 0.14 | 1.43 | 1.76 | 0.02 | 0.02 | 0.03 | 0.03 |
| Acenaphthylene | 0.13 | 0.12 | 2.01 | 2.61 | 0.03 | 0.05 | 0.03 | 0.03 |
| Acenaphthene | 0.34 | 0.35 | 1.92 | 2.01 | 0.01 | 0.01 | 0.10 | 0.19 |
| Fluorene | 0.25 | 0.25 | 2.13 | 3.07 | 0.01 | 0.06 | 0.05 | 0.15 |
| Phenanthrene | 0.10 | 0.11 | 4.05 | 5.15 | 0.01 | 0.05 | 0.01 | 0.10 |
| Anthracene | 0.31 | 0.90 | 1.43 | 2.23 | 0.02 | 0.02 | 0.01 | 0.01 |
| Fluoranthene | 0.02 | 0.02 | 0.91 | 1.09 | 0.01 | 0.04 | 0.02 | 0.02 |
| Pyrene | 0.41 | 0.41 | 1.89 | 2.59 | 0.01 | 0.01 | 0.41 | 0.51 |
| Benz[a]anthracene | 0.05 | 0.09 | 1.93 | 2.64 | 0.02 | 0.09 | 0.03 | 0.05 |
| Chrysene | 0.03 | 0.23 | 3.43 | 4.03 | 0.01 | 0.06 | 0.03 | 0.05 |
| Benzo[b]fluoranthene | 0.03 | 0.18 | 2.71 | 3.01 | 0.03 | 0.03 | 0.01 | 0.03 |
| Benzo[k]fluoranthene | 0.12 | 0.12 | 2.02 | 2.12 | 0.01 | 0.04 | 0.02 | 0.03 |
| Benzo[a]pyrene | 0.06 | 0.06 | 0.50 | 1.31 | 0.01 | 0.03 | 0.02 | 0.05 |
| Dibenz(a,h)anthracene | 0.02 | 0.22 | 0.31 | 0.71 | 0.01 | 0.01 | 0.01 | 0.05 |
| Indeno(1,2,3-cd)pyrene | 0.02 | 0.02 | 0.27 | 1.77 | 0.01 | 0.01 | 0.01 | 0.04 |
| Benzo[g,h,i]perylene Total | 0.04 2.06 | 0.06 3.26 | 0.52 27.46 | 0.98 37.08 | 0.02 0.24 | 0.04 0.57 | 0.03 0.82 | 0.09 1.43 |

Table 2: Mean Concentrations of Polycyclic Aromatic Hydrocarbons (PAHs) in all

Polycyclic Aromatic Hydrocarbon (PAH) Congeners concentrations in all industrial sites sampled

The concentrations of various PAH congeners detected in the topsoil and subsoil samples from the different industrial sites are shown in Table 1. The PAH congener with the highest concentration in the electronic site was observed for Anthracene in the subsoil sample with a concentration of 0.90ppm. Samples from the Petrochemical site had the highest concentration for PAH congeners in Phenanthrene with a concentration value of 5.15ppm. The transformer oil storage site had the highest concentration of 0.09ppm in Benz(a)anthracene while the Paint industry recorded a concentration of 0.51 for the PAH congener Pyrene.

The mean concentrations of PAH congeners in the various industrial sites evaluated ranged from 0.24mg/kg to 37.08mg/kg. The lowest concentration of PAH was recorded in the topsoil sample obtained from the Transformer oil storage depot at 0.24mg/kg. The topsoil sample from the electronic site had a concentration of 2.06mg/kg, while the sub-surface soil sample had a concentration of 3.26mg/kg. Topsoil sample from the Petrochemical industrial site recorded a PAH concentration of 27.46mg/kg while the subsoil sample gave 37.08mg/kg. Similarly, topsoil sample from the Transformer oil storage site recorded a PAH concentration of 0.24mg/kg while that of the subsoil sample recorded a value of 0.57mg/kg. Additionally, the topsoil sample from the paint company had a PAH concentration of 0.82mg/kg and a value of 1.43mg/kg for the subsoil sample.

Result of Polychlorinated Biphenyl (PCB)

4.1.2	Concentration of Polychlorinated Biphenyl (PCB) Congeners in Control Sample

The result presents the concentrations of PCB congeners observed in topsoil and subsoil samples obtained from the Control site. All PCB congeners, ranging from 4,4’-Dichlorobiphenyl to Decachlorobiphenyl had concentration value below detection limit of >0.01mg/kg, thus were Not Detected (ND).

Table 3:	Concentrations of Polychlorinated Biphenyl (PCB) Congeners in Control Soil Sample

|  |  | Topsoil sample | Subsoil sample |
| --- | --- | --- | --- |
| S/N | Compound | Concentration (ppm) | Concentration (ppm) |
| 1 | 4,4'-Dichlorobiphenyl | ND | ND |
| 2 | 2,3,4,4',5-Pentachlorobiphenyl | ND | ND |
| 3 | 2,2',3,3',4,5-Hexachlorobiphenyl | ND | ND |
| 4 | 2,2',3,4,4',5-Hexachlorobiphenyl | ND | ND |
| 5 | 2,2',4,4',5,5'-Hexachlorobiphenyl | ND | ND |
| 6 | 2,2',3,3',4,4',6-Heptachlorobiphenyl | ND | ND |
| 7 | 2,2',3,4,4',5',6-Heptachlorobiphenyl | ND | ND |
| 8 | 2,2',3,4,5,5',6-Heptachlorobiphenyl | ND | ND |
| 9 | 2,3,3',4,4',5,5'-Heptachlorobiphenyl | ND | ND |
| 10 | 2,3,3',4,4',5,6-Heptachlorobiphenyl | ND | ND |
| 11 | 2,2',3,3',4,5,6,6'-Octachlorobiphenyl | ND | ND |
| 12 | 2,2',3,3',4,5,5',6-Octachlorobiphenyl | ND | ND |
| 13 | 2,2',3,3',4,4',5,5',6- Octachlorobiphenyl | ND | ND |
| 14 | Decachlorobiphenyl Total | ND ND | ND ND |

*ND= Not Detected

Table 4: Concentrations of Polychlorinated Biphenyl (PCB) in all Sample areas

| S/N | Compound | Electronics market Topsoil | Electronics market Subsoil | Petro-chemical industry Topsoil | Petro-chemical industry Subsoil | Transf-ormer oil depot Topsoil | Transf-ormer oil depot Subsoil | Paint Company Topsoil | Paint Company Subsoil |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4,4'-Dichlorobiphenyl | 0.09 | 0.11 | 0.53 | 0.72 | 0.04 | 0.09 | 0.22 | 0.52 |
| 2 | 2,3,4,4',5-Pentachlorobiphenyl | 0.22 | 0.29 | 0.29 | 0.49 | 0.05 | 0.07 | 0.09 | 0.39 |
| 3 | 2,2',3,3',4,5-Hexachlorobiphenyl | 0.09 | 0.21 | 0.16 | 0.26 | 0.11 | 0.15 | 0.21 | 0.51 |
| 4 | 2,2',3,4,4',5-Hexachlorobiphenyl | 0.25 | 0.34 | 0.40 | 0.40 | 0.04 | 0.18 | 0.31 | 0.36 |
| 5 | 2,2',4,4',5,5'-Hexachlorobiphenyl | 0.08 | 0.17 | 0.24 | 0.92 | 0.04 | 0.04 | 0.09 | 0.14 |
| 6 | 2,2',3,3',4,4',6-Heptachlorobiphenyl | 0.08 | 0.23 | 0.34 | 1.14 | 0.06 | 0.13 | 0.05 | 0.20 |
| 7 | 2,2',3,4,4',5',6-Heptachlorobiphenyl | 0.07 | 0.14 | 0.67 | 0.93 | 0.05 | 0.15 | 0.07 | 0.35 |
| 8 | 2,2',3,4,5,5',6-Heptachlorobiphenyl | 0.12 | 0.18 | 0.19 | 0.19 | 0.09 | 0.13 | 0.08 | 0.12 |
| 9 | 2,3,3',4,4',5,5'-Heptachlorobiphenyl | 0.08 | 0.10 | 0.43 | 0.43 | 0.03 | 0.10 | 0.14 | 0.22 |
| 10 | 2,3,3',4,4',5,6-Heptachlorobiphenyl | 0.15 | 0.17 | 0.21 | 0.31 | 0.07 | 0.11 | 0.19 | 0.32 |
| 11 | 2,2',3,3',4,5,6,6'-Octachlorobiphenyl | 0.14 | 0.18 | 0.12 | 0.42 | 0.04 | 0.09 | 0.09 | 0.10 |
| 12 | 2,2',3,3',4,5,5',6-Octachlorobiphenyl | 0.12 | 0.16 | 0.12 | 0.22 | 0.06 | 0.11 | 0.24 | 0.31 |
| 13 | 2,2',3,3',4,4',5,5',6-Octachlorobiphenyl | 0.03 | 0.13 | 0.19 | 0.29 | 0.02 | 0.06 | 0.08 | 0.10 |
| 14 | Decachlorobiphenyl Total | 0.06 1.58 | 0.18 2.59 | 0.20 4.09 | 0.29 7.01 | 0.02 0.72 | 0.05 1.46 | 0.11 1.97 | 0.21 3.85 |

Concentrations of Polychlorinated Biphenyl (PCB) Congeners in Soil Samples from All Industrial Sites

Results obtained for the concentration of PCBs across different sample areas, viz: the Electronics market, the Petrochemical Industry, Transformer Oil Storage site, and Paint Company are shown in Table 4. The results indicate that the highest total PCB concentration was found in the Petrochemical Industry sub-surface soil (7.01mg/kg), followed by its surface soil (4.09mg/kg), suggesting significant contamination, particularly in deeper soil layers. In contrast, the Transformer Oil Depot surface soil had the lowest total PCB concentration (0.72 mg/kg), indicating minimal contamination, though the sub-surface soil showed a slightly higher concentration (1.46 mg/kg).

5.0 Conclusion and Recommendations

5.1 Conclusion

PAHs and PCBs are persistent and toxic pollutants that can have significant environmental and health impacts. The literature review has shown that PAHs and PCBs can contaminate soil through various sources, including industrial activities, waste disposal, and atmospheric deposition. The review has also highlighted the importance of remediation and management strategies for PAH- and PCB-contaminated soil.

5.2 Recommendation

Based on the literature review, the following recommendations are made:

- Regular soil testing should be conducted to monitor PAH and PCB levels in soil, particularly in areas with high industrial activity.

- Remediation technologies, such as bioremediation and chemical treatment, should be used to clean up PAH- and PCB-contaminated soil.

- Proper waste management practices should be implemented to prevent PAH and PCB contamination of soil.

- Policies should be developed to regulate the use and disposal of PAHs and PCBs, and to provide guidelines for the remediation and management of contaminated soil.

- Public awareness campaigns should be conducted to educate people about the risks associated with PAH and PCB contamination of soil.

- Collaboration between government agencies, industries, and communities is necessary to develop effective strategies for managing PAH- and PCB-contaminated soil.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

References:

ATSDR (2015). Toxicological Profile for Polychlorinated Biphenyls (PCBs). Agency for Toxic Substances and Disease Registry.

Baird, C., Cann, M., & Jacob, R. (2017). Environmental chemistry. W.H. Freeman and Company.

EPA (2017). Polychlorinated Biphenyls (PCBs). United States Environmental Protection Agency.

IARC (2010). Some Non-Heterocyclic Polycyclic Aromatic Hydrocarbons. International Agency for Research on Cancer.

Iwegbue, C. M. A., Overah, L. C., Tesi, G. O., Bassey, F. I., & Martincigh, B. S. (2014). Polycyclic Aromatic Hydrocarbon Profiles of Some Brands of Canned Fish in the Nigerian Market. Human and Ecological Risk Assessment, 21, 157–168.

Jones, K. C., & De Voogt, P. (1999). Persistent organic pollutants (POPs): state of the science. Environmental pollution, 100(1-3), 209-221.

Kang, Y., Wang, W., & Huang, M. J. (2018). Remediation of polycyclic aromatic hydrocarbon-contaminated soil using bioremediation and chemical treatment. Environmental Science and Pollution Research, 25(11), 10341-10350.

Kisku, G. C., Pandey, P., & Kumar, R. (2017). Distribution and source apportionment of polycyclic aromatic hydrocarbons in soil of a coal mining area. Environmental Science and Pollution Research, 24(11), 10353-10364.

Kuppusamy, S., Thavamani, P., Megharaj, M., & Naidu, R. (2016). Biodegradation of polycyclic aromatic hydrocarbons (PAHs) by novel bacterial consortia tolerant to diverse physical settings - Assessments in liquid- and slurryphase systems. International Biodeterioration and Biodegradation, 108, 149–157.

Langer, P. (2005). persistent organochlorinated pollutants (POPs) and human thyroid--2005. Endocrine regulations, 39(2), 53-68.

Lauby-Secretan, B., Loomis, D., Grosse, Y., El Ghissassi, F., Bouvard, V., Benbrahim-Tallaa, L., ... & Straif, K. (2013). Carcinogenicity of polychlorinated biphenyls and polybrominated biphenyls. Lancet Oncology, 14(4), 287-288.

Mackay, D., Shiu, W. Y., & Ma, K. C. (1997). Illustrated handbook of physical-chemical properties of environmental fate for organic chemicals (Vol. 5). CRC press.

Nguyen, T. C., Loganathan, P., Nguyen, T. V., Vigneswaran, S., Kandasamy, J., Slee, D., Stevenson, G., & Naidu, R. (2014). Polycyclic aromatic hydrocarbons in road-deposited sediments, water sediments, and soils in Sydney, Australia: Comparisons of concentration distribution, sources and potential toxicity. Ecotoxicology and Environmental Safety, 104, 339–348.

Nwachukwu, M. A., Ezenwa, M. I., & Nwachukwu, E. O. (2010). Distribution of polycyclic aromatic hydrocarbons in soil and water around the Warri Refinery and Petrochemical Company, Nigeria. Journal of Environmental Science and Health, Part B, 45, 345-355.

Osuji, L. C., Ezenwa, M. I., & Okeke, P. O. (2015). Environmental impact of oil and gas exploration and production on the Niger Delta ecosystem. Journal of Environmental Science and Health, Part C, 33, 1-15.

Qi, H., Li, W. L., Zhu, N. Z., Ma, W. L., Liu, L. Y., Zhang, F., & Li, Y. F. (2014). Concentrations and sources of polycyclic aromatic hydrocarbons in indoor dust in China. Science of the Total Environment, 491–492, 100–107.

Rahmanpour, S., Farzaneh Ghorghani, N., & Lotfi Ashtiyani, S. M. (2014). Polycyclic aromatic hydrocarbon (PAH) in four fish species from different trophic levels in the Persian Gulf. Environmental Monitoring and Assessment, 186, 7047–7053.

Sharma, T. (2014). In silico investigation of polycyclic aromatic hydrocarbons against bacterial 1-2 dioxygenase. Journal of Chemical and Pharmaceutical Research, 6, 873–877.

Van der Wat, L., & Forbes, P. B. C. (2015). Lichens as biomonitors for organic air pollutants. TrAC Trends in Analytical Chemistry, 64, 165–172.

Wang, W., Huang, M. J., & Kang, Y. (2016). Distribution and risk assessment of polycyclic aromatic hydrocarbons in soil around a coal-fired power plant. Environmental Science and Pollution Research, 23(11), 10341-10350.

WHO (2010). Exposure to Polycyclic Aromatic Hydrocarbons (PAHs). World Health Organization.

Yang, Z. Z., Li, Y. F., & Fan, J. (2015). Polycyclic aromatic hydrocarbons in deposited bedroom dust collected from Xinxiang, a fast developing city in North China. Environmental Monitoring and Assessment, 187, 4150.

Sharma, J. K., Gautam, R. K., Nanekar, S. V., Weber, R., Singh, B. K., Singh, S. K., & Juwarkar, A. A. (2018). Advances and perspective in bioremediation of polychlorinated biphenyl-contaminated soils. Environmental Science and Pollution Research, 25, 16355-16375.

Matei, M., Zaharia, R., Petrescu, S. I., Radu-Rusu, C. G., Simeanu, D., Mierliță, D., & Pop, I. M. (2023). Persistent organic pollutants (POPs): A review focused on occurrence and incidence in animal feed and cow milk. Agriculture, 13(4), 873.

Akpan, A. D., Okori, B. S.-U., & Ekpechi, D. C. (2022). Human Health Risk Assessment of Polycyclic Aromatic Hydrocarbons in Water Samples around Eket Metropolis, Akwa Ibom State, Nigeria. Asian Journal of Environment & Ecology, 19(4), 58–71.

Najam, L., & Alam, T. (2023). Occurrence, distribution, and fate of emerging persistent organic pollutants (POPs) in the environment. In Emerging contaminants and plants: interactions, adaptations and remediation technologies (pp. 135-161). Cham: Springer International Publishing.