Water Quality Index of Selected Borehole Water along Pipeline Routes in Parts of Obio/Akpor Local Government Area, Rivers State Nigeria

ABSTRACT

Water is the principal component in the earth which supports the life of all living and groundwater is a very important source of water which must be managed in sufficient quantity and quality contributes to maintaining health. The study examined  water quality index of selected borehole water along pipeline routes in parts of obio/akpor local government area , Rivers State Nigeria. Water samples (15) were sourced from communities (Rumuoke, Rumukania and Egbelu) where decommissioned pipelined pass through and analysed for physicochemical and heavy concentration following the standard methods of the American Public Health Organization (APHA) and the American Society for Testing and Materials (ASTM).  The findings revealed that the pH mean concentration was 5.34 across the study area and all concentrations reported are within the WHO and NSDWQ allowable limits. The mean concentration of electrical conductivity (107.51 µS/cm), TDS (69.73 mg/l), NO3- (0.02 mg/l) and Cl- (4.52 mg/l) reported are within the WHO and NSDWQ allowable limits. The groundwater sample WQI was 21.61 and can be classified “Excellent Water”. It was concluded that the decommissioned pipelines had no direct impact on the quality of the groundwater.

Keywords: Water Quality Index (WQI), Groundwater, Drinking Water, Water Quality, Rivers State.

- INTRODUCTION

Water is essential for life and for all economic activities. It is used for domestic, industrial and agricultural purposes. Having sufficient water in sufficient quantity and quality contributes to maintaining health. The availability of water of good quality is essential to prevent diseases and to improve the quality of life. The use of water increased due to increasing in human population and activities (Al-sudani, 2018). Groundwater is one of the important components in development of any area. It is the major potable, agricultural and industrial source of water. In 2003, it was estimated that groundwater holds nearly 50% of the drinking water supply, 40% of the demand for industrial water, and 20% of the water used for irrigation (Al-sudani, 2019).

Assessment of groundwater quality is critical for resource planning and environmental management (Mohamed et al., 2019). Since water quality status can be described by a number of physical, chemical, and biological parameters, large amounts of data make assessment and comparison significantly difficult (Mester et al., 2020). The most important benefit of water quality indices (WQI) is to provide a simplified approach for the assessment of water quality by greatly minimizing the data volume and simplifying the expression of water quality status (Al-Omran et al., 2018; Mester et al., 2020). Thus, the use of these indices in ecosystem monitoring programs makes it possible to inform the general public and decision-makers about the state of the environment (Tiwari et al. 2016; Venkatramanan et al. 2017; Bilgin, 2018; Wu et al. 2020; Mester et al., 2020). This approach can also help to provide a benchmark for evaluating the successes and failures of management strategies in improving water quality, and it will also indicate what actions should be modified (Mester et al., 2020).

Water is the principal component in the earth which supports the life of all living. Groundwater is a very important source of water, specifically in the semi-arid and arid region. It supports all types of uses (drinking, irrigation, and industrial) (Hamzaoui-Azaza et al. 2020; Aouiti et al., 2021). However, groundwater is threatened by severe problems caused by natural/anthropogenic factors, such as the extensive agricultural activities, the marine intrusion, the population growth, and the industrial development (Zammouri et al. 2013; Aouiti et al., 2021). This factor engendered a degradation in the quality and the quantity and the quantity of groundwater in many countries: for example, Ameur et al. (2016) found that the water quality, in the northeast Tunisia, is at poor level due to the nitrate pollution that originate from the excessive use of nitrate-rich fertilizers. Adimalla (2019) conducted a study on the effect of the rapidly urban activities (South India) on water quality and the human health risk related to the nitrate and the fluoride pollution.

Several criterial are considered when carrying out groundwater quality assessment. Assessment can be carried using different methodologies (Satyajit, et al., 2020; Megahed, 2020; Knopek & Dabrowska, 2021; Afolabi et al., 2023) and based on specific parameter focus (Lofti et al., 2020, Ibe et al., 2020; Ogbaran & Uguru, 2021; Saleh et al., 2022). However, the study focused on assessment of groundwater quality along decommissioned pipeline routes using various water pollution indices specifically water quality index (WQI). The outcome of the study can provide future reference for groundwater quality studies in the study area and beyond, and create the needed awareness about environmental implication of human activities influence on groundwater sustainability.

- MATERIAL AND METHODS

Study Area

The study was carried out in Rumuoke, Rumukania and Egbelu communities in Obio/Akpor LGA. Obio/Akpor LGA is one of the 23 local governments of Rivers state, found in the south southern part of Nigeria, otherwise called the Niger Delta Region of Nigeria, located approximately between latitude 4º 45” N through 4º 56” N and longitude 6º 52” E through 7º 6” E (Figure 1). Consequent on rapid urbanization and the rising industrial and commercial growth of the city of Port Harcourt, more goods and services are being made available, thus the springing up of fuelling stations, to meet up with the demand of the growing population arises.  Reconnaissance survey was carried out on the 30th November, 2024 to help familiarize with the study area. GARMIN Extech 76 held Global Positioning System (GPS) was  deployed during the reconnaissance survey to obtain the coordinates of the sampling points in the study. The study area was selected due to the presence of decommissioned pipelines within the  residential area. The focus on the decommissioned pipeline was to determine the potential influence on the nearby water quality.

2.1 Data (Sampling) Collection and Procedure

Sampling Points/Sites

The water samples for the study was collected from boreholes;  groundwater sources around the study area [Rumuoke, Rumukania and Egbelu communities all within Obio/Akpo local government area of Rivers State.], Specifically, water samples were collected from sixteen [16] boreholes during the in December 21st 2024) and was  tested for various physicochemical, and heavy metals parameters. To prevent confusion and mix up of the water sample, each sample  was tagged accordingly with the generic name and symbols.

Figure 1: Overview of the Study Area and Sampling Points

Sample Collection Procedure

With the aid of labeled lucid bottle, water samples were collected  from various designated water source. Prior to the water collection, the lucid bottles were cleaned with 70% sterilizer in order prevent impurities and other form of contamination. Afterwards, the water samples were collected from each designated point and the bottles were be fill to the prim. Thereafter, the filled bottles were be immediately placed in the ice-parked cooling medium to arrest continuous microbial activities and preserve the water before been taken to the laboratory for analysis.

Data Analysis

The concentration of groundwater quality parameters (pH, Temperature, Turbidity, Total Dissolved Solid (TDS), Electricity Conductivity (EC),  Chloride, Salinity, Dissolved Oxygen, Sodium, Calcium, Magnesium, Sulphate, Nitrate and Copper (Cu) were analysed. All physicochemical parameters were analyzed by following the standard methods of the American Public Health Organization (APHA) and the American Society for Testing and Materials (ASTM), similar to those described by Khalid et al. (2018) and Latif et al. (2024). The acidified water samples were filtered using Whatman ashless filter paper and thereafter analysed with Atomic Absorption Spectrophotometer (Shimadzu AA-6650) using standard method (ASTM D1971/4691) to determine the level of heavy metals Copper in the sample (Afolabi et al., 2022). All findings were presented using tables and histogram while the mean value of sample points provided.

Water Quality Index (WQI)

In this study, weights (wi) were assigned to water quality indicators based on the health consequences of those parameters for drinkable water (Chegbeleh et al., 2020). WQI is taken into consideration for human consumption. Weight values (wi = 1–5) for the parameters were adopted based on similar studies (Ibe et al., 2020; Peterside et al., 2022), and Cu was assigned weight values of 5 because it is an essential factor that affect groundwater quality and thought to be health-associated. In addition, WQI entails calculating the subindex (SI) (equation (3)), water quality index (WQI) (equation (4)), and relative weights (Wi) (equation (1)) and the quality rating scale (qi) (equations (2).

Wi=wiwi                                      (Eq.  1)

qi=CiSi x 100                                     (Eq. 2)

SI=Wi x qi                                        (Eq. 3)

WQI= n=1nSI                                  (Eq. 4)

Where wi= weight values for parameters, Ci = Concentration of parameters and Si = Standard value of Parameters, qi = Quality rating scale. The standard value of WHO for parameters was adopted for the study (Afolabi et al., 2022). The estimated WQIs were categorized based on Sahu and Sikdar (2008) categorization where <50: Excellent water, 50-100: Good water, 100-200: Poor water, 200-300: Very poor water, and >300: Water unsuitable for drinking.

- RESULTS AND DISCUSSION

The concentration of groundwater quality parameters used for analysing WQI was collected from boreholes (BH 1-15)  and the outcome was presented in Table 1. The pH mean concentration across the sampled points was 5.34 across the study area and the reported concentrations were within those reported by Addisie (2022) at 6.5 to 7.45. The temperature mean concentration across the sampled points was 28.4 °C. Temperature in this study was found within permissible limit of WHO (30 °C). The salinity mean concentration across the sampled points was 0.01 was similar to those reported by Emmanuel et al. (2024). The EC mean concentration across the sampled points was 107.51 µS/cm. All concentrations reported are within the WHO (2018) and NSDWQ

Table 1: Physiochemical Concentration of Groundwater Quality

|  | BH1 | BH2 | BH3 | BH4 | BH5 | BH6 | BH7 | BH8 | BH9 | BH10 | BH11 | B12 | BH13 | BH14 | BH15 | CON | Mean | WHO | NSDWQ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pH | 6.57 | 5.49 | 5.34 | 5.71 | 4.95 | 5.29 | 6.41 | 5.09 | 5.00 | 5.69 | 4.92 | 4.23 | 6.62 | 4.53 | 4.31 | 7.88 | 5.34 | 6.5 - 8.5 | 6.5 - 8.5 |
| Temp ºC | 27.8 | 27.9 | 27.0 | 27.2 | 27.7 | 24.4 | 28.4 | 33.8 | 30.0 | 30.1 | 30.3 | 29.9 | 30.2 | 30.0 | 21.2 | 29.9 | 28.39 | Amb. | Amb. |
| Salinity | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.02 | 0.01 | <0.01 | 0.01 | 0.01 | 0.01 | 0.0 | 0.01 | 0.01 | 0.02 | 0.0 | 0.01 | NS | NS |
| EC (µS/cm) | 166.2 | 156.9 | 123.1 | 126.2 | 101.9 | 132.7 | 77.71 | 99.20 | 15.67 | 16.73 | 33.89 | 428.5 | 37 | 57.47 | 39.53 | 82.95 | 107.51 | 1000 | 1000 |
| TDS | 105.0 | 98.74 | 79.25 | 79.47 | 64.06 | 83.12 | 49.9 | 57.99 | 8.71 | 10.33 | 21.18 | 281.0 | 45.29 | 36.5 | 25.38 | 82.92 | 69.73 | 1500 | 500 |
| DO | 5.61 | 6.12 | 5.1 | 4.72 | 5.4 | 5.2 | 5.10 | 4.7 | 6.62 | 4.99 | 4.78 | 5.10 | 4.71 | 5.0 | 6.7 | 5.46 | 5.32 | 5.0 | > 4 |
| Nitrate | 0.037 | 0.041 | 0.031 | 0.023 | 0.02 | 0.029 | 0.017 | 0.018 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | 0.012 | <0.001 | 0.02 | 50 | 50 |
| Chloride | 7.99 | 7.99 | 5.99 | 6.99 | 4.99 | 7.99 | 3.99 | 3.99 | 0.99 | 1.99 | 1.99 | 2.99 | 2.99 | 2.99 | 3.99 | 2.99 | 4.52 | 600 | 100 |
| Sulphate | 0.049 | 0.053 | 0.047 | 0.042 | 0.037 | 0.042 | 0.033 | 0.036 | 0.002 | 0.013 | 0.016 | 0.018 | 0.026 | 0.022 | 0.035 | 0.014 | 0.03 | 400 | 100 |
| Phosphate | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | - | 5 | 5 |
| Sodium | 0.088 | 0.121 | 0.062 | 0.079 | 0.063 | 0.104 | 0.069 | 0.084 | 0.009 | 0.073 | 0.052 | 0.072 | 0.044 | 0.061 | 0.044 | 0.052 | 0.07 | - | - |
| Calcium | 0.221 | 0.285 | 0.211 | 0.279 | 0.187 | 0.192 | 0.203 | 0.211 | 0.048 | 0.076 | 0.061 | 0.052 | 0.061 | 0.082 | 0.077 | 0.081 | 0.15 | 100 | - |
| Magnesium | 0.013 | 0.027 | 0.031 | 0.012 | 0.034 | 0.041 | 0.038 | 0.026 | 0.012 | 0.027 | 0.033 | 0.012 | 0.018 | 0.032 | 0.039 | 0.027 | 0.02 | 50 | - |
| Copper | 0.022 | 0.016 | <0.001 | 0.009 | 0.016 | 0.027 | <0.001 | 0.011 | 0.009 | 0.011 | <0.001 | 0.007 | 0.013 | 0.015 | <0.001 | <0.001 | 0.014 | 2 | 0.3 |

BH: Borehole, CON: Control, WHO: World Health Organisation, NSDWQ: Nigeria Standard for Drinking Water Quality, TDS: Total Dissolve Solid, DO: Dissolved Oxygen

(2007) allowable limits. The concentration reported for the study lower than those reported by Meride and Ayenew (2016). The TDS mean concentration across the sampled points was 69.73 mg/l. All concentrations reported are within the WHO (2018) and NSDWQ (2007) allowable limits but lower in concentration to those reported Afolabi et al. (2022) for groundwater around dumpsites. The DO mean concentration across the sampled points are 5.01 mg/l across the study area. The higher the concentration of DO the better the water quality (Ojekunle & Lateef, 2017).

The NO3- mean concentration across the sampled points are 0.01 mg/l across the study area. All concentrations reported are within the WHO (2018) and NSDWQ (2007) allowable limits and within the range reported by Meride and Ayenew (2016). The Cl- mean concentration across the sampled points are 4.52 mg/l across the study area. The Cl reported for this study was similar to those reported by Asaomaku (2022) while the higher than those reported by Khalid et al. (2018). The SO42- mean concentration across the sampled point was 0.03 mg/l across the study area.  All the reported concentrations are similar to those reported by Afolabi et al. (2022) for groundwater and Magaji (2020) for Sachet water produced. All the inorganic elements (Na, Ca and Mg) reported are similar in concentration across the study area while the reported concentrations are below the WHO allowable limit. The concentration reported for inorganic elements are lower to those reported by Afolabi et al. (2022). According to Adimalla and Wu (2019), inorganic elements are from natural sources, although anthropogenic activities can also increase their concentration in groundwater. The Cu concentration ranged from 0.007 mg/l at BH9 to 0.027 mg/l at BH6 with mean concentration of 0.014 and control sample concentration of <0.001 mg/l. All the reported concentrations were below the WHO (2018) and NSDWQ (2007) allowable limit. The outcome is similar to the concentration reported by Laniyan and Adewumi (2019), Naminata et al. (2018) and Afolabi et al. (2022) for similar Cu concentration for groundwater.

The WQI of groundwater samples (based on mean concentration) across the water samples were evaluated for the quality for consumption and other purposes and the outcome was presented in Table 2. The groundwater sample had WQI of 21.61. From the WQI analysis, the groundwater sample can be classified “Excellent Water”. The values obtained in this current study is lower than those reported by Asomaku (2022) with WQI values (35.12 – 62.97) for selected groundwater. Similar outcome was reported by the study conducted by Akter et al. (2016). The procedure adopted for the study was similar to those reported by Manna and Biswas (2023) for drinking water quality index.

### Table 2: WQI of groundwater of the Study Area

| Parameters |  | wi | Si | Wi | Groundwater |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Ci | qi | SI |
| PH |  | 4 | 8.5 | 0.06 | 5.34 | 62.82 | 3.77 |
| Temp |  | 4 | 40 | 0.06 | 28.4 | 71.0 | 4.26 |
| Salinity |  | 4 | 1000 | 0.06 | 0.011 | 0.0011 | 0.00006 |
| TDS |  | 4 | 50 | 0.06 | 69.73 | 139.46 | 8.37 |
| Conductivity |  | 4 | 1000 | 0.06 | 107.51 | 10.75 | 0.65 |
| DO |  | 4 | 7.5 | 0.06 | 5.32 | 70.93 | 4.26 |
| Nitrate |  | 5 | 50 | 0.08 | 0.03 | 0.06 | 0.0048 |
| Chloride |  | 3 | 250 | 0.04 | 4.52 | 1.808 | 0.072 |
| Sulphate |  | 4 | 250 | 0.06 | 0.03 | 0.012 | 0.0007 |
| Sodium |  | 2 | 200 | 0.025 | 0.07 | 0.035 | 0.0009 |
| Calcium |  | 3 | 200 | 0.04 | 0.15 | 0.075 | 0.003 |
| Magnesium |  | 2 | 150 | 0.025 | 0.03 | 0.02 | 0.0005 |
| Copper |  | 4 | 2 | 0.06 | 0.014 | 0.7 | 0.042 |
| Iron |  | 3 | 0.3 | 0.05 | 0.011 | 3.67 | 0.183 |
|  | ΣWi=50 |  |  |  | WQI=21.61 |  |  |

- CONCLUSION

The groundwater sourced through boreholes along a decommissioned oil and gas pipeline routes was assessed for its quality status based on water quality index based on the concentration of various groundwater parameters. The water quality index of the groundwater was “Excellent water”. Based on the outcome of the study, it was concluded that the decommissioned pipelines had no direct impact on the quality of the groundwater. The study suggested the need for further assessment using other water quality assessment metric for more comprehensive outcome.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

REFERENCES

Addisie, M. B. (2022). Evaluating Drinking Water Quality Using Water Quality Parameters and Esthetic Attributes. Air, Soil and Water Research, 15(15), 117862212210750. https://doi.org/10.1177/11786221221075005

Adimalla N & Wu, F. (2019) Groundwater quality for drinking and irrigation purposes and potential health risks assessment: a case study from semiarid region of south india. Expo Health 11:109–123. https://doi.org/10.1007/s12403-018-0288-8

Afolabi, O. O.,  Wali, E. & Nwankwoala, H. O. (2023). Hydrogeochemical process of ions in groundwater of communities along Southern-axis Atlantic Ocean in Bayelsa, Nigeria, Water Science, 37:1, 399-408, DOI: 10.1080/23570008.2023.2285124

Afolabi, O.O., Wali, E. Ihunda, E.C., Orji, M.C., Emelu, V.O., Bosco-Abiahu, L.C., Ogbuehi, N.C., Asomaku, S.O. Wali, O.A. (2022). Potential environmental pollution and human health risk assessment due to leachate contamination of groundwater from anthropogenic impacted site, Environmental Challenge. 9:100627. https://doi.org/10.1016/j.envc.2022.100627

Al-Sudani, H. I. Z. (2019). A Review on Groundwater Pollution. International Journal of Recent Engineering Science, 6(5)

Ameur M, Hamzaoui-Azaza F, GueddariM(2016) Nitrate contamination of Sminja aquifer groundwater in Zaghouan, northeast Tunisia: WQI and GIS assessments. Desalin Water Treat 57:1–11. https://doi.org/10.1080/19443994.2015.1137495

Aouiti, M., Azaza, F. H., El-Melki, F., Hamdi, M., Celico, F. and Zammouri, M. (2021). Groundwater quality assessment for different uses using various water quality indices in semi-arid region of central Tunisia. Environmental Science and Pollution Research, 28:46669–46691, https://doi.org/10.1007/s11356-020-11149-5

Asomaku, S. O. (2023). Quality assessment of groundwater sourced from nearby abandoned landfills from Industrial City in Nigeria: Water pollution indices approach. HydroResearch, 6, 130-137

Bilgin, A. (2018). Evaluation of surface water quality by using Canadian Council of Ministers of the Environment Water Quality Index (CCME WQI) method and discriminant analysis method: a case study Coruh River Basin. Environmental Monitoring and Assessment, 190(9), 554.

Chegbeleh, L., Akurugu, B. A. and Yidana, M. S. (2020). Assessment of Groundwater Quality in the Talensi District, Northern Ghana. The Scientific World Journal, https://doi.org/10.1155/2020/8450860

Hamzaoui-Azaza F,AmeurM, Chaouch R, Cheikha L, Gueddari M, JoséJoel R (2020) Assessment of groundwater quality based on GIS and geochemical methods: coastal aquifer of Bouficha (north-eastern Tunisia). Journal of Coast Conservation, 24:45. https://doi.org/10.1007/s11852-020-00762-8

Ibe, F. C., Enyoh, E. C., Opara, A. I & Ibe, B. O. (2020). Evaluation of pollution status of groundwater resources of parts of Owerri metropolis and environs, Southeastern Nigeria, using health risk and contamination models. International Journal of Energy and Water Resources, https://doi.org/10.1007/s42108-020-00071-8

Meride, Y., & Ayenew, B. (2016). Drinking water quality assessment and its effects on residents health in Wondo genet campus, Ethiopia. Environmental Systems Research, 5(1). https://doi.org/10.1186/s40068-016-0053-6

Mester, T., Balla, D and Szabó, G. (2020). Assessment of groundwater quality changes in the rural environment of the hungarian great plain based on selected water quality indicators. Water Air Soil Pollution, 231: 536

Mohamed, A. K., Dan, L., Kai, S.,Mohamed,M. A., Aldaw, E.,& Elubid, B. A. (2019). Hydrochemical analysis and fuzzy logic method for evaluation of groundwater quality in the North Chengdu Plain, China. International Journal of Environmental Research and Public Health, 16(3); 302-314

Peterside, A. N., Hart, A. I. & Nwankwoala, H. O. (2022). Assessment of Groundwater Quality Using Water Quality Index (WQI) Method in Southern Ijaw Local Government Area of Bayelsa State, Nigeria. World Scientific News, 171 (2022) 35-48

Sahu, P. & Sikdar, P. K. (2008). Hydrochemical framework of the aquifer in and around East Kolkata Wetlands, West Bengal, India. Environmental Geology, 55(4), 823–835.

Tiwari, A. K., Singh, P. K., Singh, A. K., & De Maio, M. (2016). Estimation of heavymetal contamination in groundwater and development of a heavy metal pollution index by using GIS technique. Bulletin of Environmental Contamination and Toxicology, 96(4), 508–515.

Venkatramanan, S., Chung, S. Y., Selvam, S., Lee, S. Y., and Elzain, H. E. (2017). Factors controlling groundwater quality in the Yeonjegu District of Busan City, Korea, using the hydrogeochemical processes and fuzzy GIS. Environmental Science and Pollution Research, 24(30), 23679–23693.

Wu, J., Zhang, Y.,&Zhou,H. (2020). Groundwater chemistry and groundwater quality index incorporating health risk weighting in Dingbian County, Ordos basin of Northwest China. Geochemistry, 12 (5); 60-67.