Dual-Fungal Bioconversion Model for Ethanol Production from Agro-Waste Using Trichoderma viride and Aspergillus niger

ABSTRACT: The exponential generation of agro-industrial waste in India estimated at over 350 million metric tons annually poses a dual threat of environmental degradation and missed valorization opportunities. While rich in cellulose and hemicellulose, the complex lignocellulosic matrix of such biomass limits its direct fermentability. In this study, a decentralized, fungal-only bioconversion model is proposed for sustainable ethanol production using Trichoderma viride and Aspergillus niger, both isolated from pond and compost niches due to their adaptation to high organic stress environments. The biomass matrix comprising fruit pulp, sugarcane

bagasse, and vegetable residues was pretreated via overnight soaking in 10% H₂SO₄ followed by a 5-minute steam explosion at 180 °C and 1.2 MPa, leading to effective delignification and the release of 42.6 g/L of fermentable sugars. Fungal enzymatic hydrolysis was enhanced by the addition of 0.5% carboxymethyl cellulose (CMC), which improved matrix loosening and enzyme substrate interaction. Fermentation was carried out by an ethanol-tolerant Saccharomyces cerevisiae strain, isolated from dried fig, under semi-anaerobic conditions at 32 °C. The process achieved a final ethanol yield of 10.6% (v/v), with a pH drop from 6.2 to 4.8 and refractive index stabilization at 1.341, confirming active fermentation. Viability tests revealed sustained fungal growth (2-1 × 10⁷ CFU/mL) after 30 days, confirming that the process is biologically stable and potentially reusable.

Comparative analysis against previous bacterial fungal systems showed that this dual-fungal consortium offers superior substrate adaptability, reduced contamination risk, and higher ethanol recovery from heterogeneous feedstock. The approach avoids commercial enzyme dependency and high-energy demands, offering a low-cost, modular strategy aligned with India’s bioenergy and circular economy goals. Although this study focused on ethanol generation, the fungal-rich residue presents additional potential for biofertilizer applications reserved for future work

KEYWORDS: Fermentation; Agro-waste valorization; Lignocellulosic Hydrolysis; Trichoderma viride; Aspergillus niger; Saccharomyces cerevisiae; Bioethanol; Biorefinery; CMC-enhanced saccharification; Ethanol Distillation

ABBREVIATIONS: CMC – Carboxymethyl Cellulose, DNS – Dinitrosalicylic Acid, RI – Refractive Index, CFU – Colony Forming Units, MPa – Megapascal, SDG – Sustainable Development Goals, SI – Système International (International System of Units), H₂SO₄ – Sulfuric acid

- INTRODUCTION

The exponential rise in agro-industrial biomass generation presents a significant environmental and bioeconomic challenge. India alone produces over 350 million metric tons of crop residues and food processing waste annually, much of which is discarded or incinerated due to lack of decentralized processing technologies (Magalhães et al., 2019, Kumar et al., 2023). These waste streams, rich in lignocellulosic content such as cellulose, hemicellulose, and lignin, hold immense potential as renewable feedstocks for bioethanol generation a clean liquid fuel with established blending infrastructure and policy support under India’s Ethanol Blending Programme (EBP). (Valera, H., Agarwal, A.K. 2025).

However, the recalcitrant architecture of lignocellulose limits its digestibility by standard fermentation organisms. Pretreatment becomes essential for disrupting the crystalline structure of hemicellulose  and releasing fermentable sugars. Methods such as dilute acid soaking followed by steam explosion have shown promising results in breaking down hemicellulose and partially solubilizing lignin (Alawad, I., & Ibrahim, H. 2024). Steam-assisted pressure disruption at moderate temperature-pressure combinations (180 °C, 1.2 MPa) offers a balance between sugar yield and energy input, especially for small-scale rural applications (Dębowski et al., 2021)

In recent years, fungal systems have gained attention for their enzymatic versatility in biomass degradation. Fungi such as Trichoderma spp. and Aspergillus spp. naturally secrete a suite of hydrolytic enzymes including cellulases, hemicellulases, and pectinases (Nevalainen et al., 2024). Unlike mono-culture yeast systems that rely on pre-released glucose, fungal consortia can actively digest and liquefy complex substrates prior to fermentation (Alawad, I., & Ibrahim, H. 2024). Trichoderma viride is widely known for its role in cellulose breakdown and composting, while Aspergillus niger contributes pectinase and acid-tolerant enzymatic activity, often complementing other microbes in fermentation ecosystems (Maji et al., 2015, Sindhu et al., 2016)

The synergistic potential of fungal-fungal consortia has been supported by various co-cultivation models that report enhanced saccharification and better stress tolerance under semi-aerobic or ambient conditions Furthermore, these fungi thrive in nutrient-limited and high organic matter environments  as seen in pond water and compost-rich niches  making them naturally adapted to agro-waste systems. Their ability to operate without industrial-level sterile conditions adds to their feasibility in field-scale or village-scale ethanol production settings. (Sharma, A., Aggarwal, N. K. 2020)

Enzymatic hydrolysis often benefits from the inclusion of facilitators such as carboxymethyl cellulose (CMC), which modifies substrate accessibility and promotes enzyme-substrate interaction by increasing water retention and matrix looseness. CMC-assisted fungal saccharification has been shown to increase reducing sugar yield and shorten fermentation time, especially in heterogeneous waste mixtures. (Scheffer et al., 2022, Hafid et al.,2017)

The fermentative stage typically relies on Saccharomyces cerevisiae, which is industrially preferred for its ethanol tolerance, rapid glucose uptake, and regulatory GRAS status. However, its performance is tightly linked to upstream saccharification efficiency and inhibitor levels post-pretreatment (Moya et al., 2024). Stress-adapted strains isolated from dried fruits or natural ferments often show improved tolerance to acidic pH and osmotic stress, making them suitable partners in fungal-integrated systems (Bustos et al., 2025).

Despite multiple advances, few studies have integrated non-bacterial, dual-fungal enzymatic systems with scalable ethanol fermentation models under realistic field conditions. Most reports rely on purified enzymes or monocultures, limiting their rural adaptability. This study introduces a two-stage fungal fermentation platform using pond-isolated Trichoderma viride and Aspergillus niger, supported by CMC-enhanced hydrolysis and semi-anaerobic fermentation using adapted S. cerevisiae. The system was applied to mixed agro-waste under low-energy pretreatment and evaluated for ethanol yield, pH drift, refractive index change, and fermentation robustness.

This approach aligns with India’s climate-smart agriculture initiatives, the National Bio-Energy Mission, and broader SDG targets for sustainable fuel generation from waste.

- LITRATURE REVIEW

The conversion of lignocellulosic biomass into fermentable sugars for ethanol production has been a focus of bioenergy research for decades. Traditionally, chemical and thermochemical pretreatment methods like dilute acid hydrolysis and steam explosion have been employed to disrupt the complex structure of plant biomass (Sun, L, et al., 2020, Alvira et al., 2010). Steam explosion at moderate pressures (1.0–1.5 MPa) and temperatures (170–200 °C) has been especially effective in solubilizing hemicellulose and increasing cellulose accessibility, particularly when combined with prior acid soaking (Shukla et al.,2023)

Fungal bioconversion, using organisms such as Trichoderma spp. and Aspergillus spp., offers a more sustainable approach due to their ability to secrete multiple lignocellulolytic enzymes, including endoglucanases, β-glucosidases, and pectinases. Trichoderma viride has been widely used in composting, bio-enzyme production, and cellulose degradation, while Aspergillus niger is known for its acidic tolerance and its role in the production of industrial enzymes such as glucoamylase and pectinase (Benatti et al., 2023)

Several studies have examined the enzymatic synergy between fungal strains for biomass liquefaction. For instance, co-cultivation of T. reesei with A. niger was reported to enhance xylanase and pectinase activity, improving saccharification of mixed agro-residues (Bhardwaj et al., 2021) Similarly, T. viride paired with Penicillium chrysogenum increased the release of reducing sugars from sugarcane bagasse, due to complementary action on cellulose and hemicellulose fractions (Thamy et al., 2022). However, many of these studies were limited to laboratory-scale enzyme assays or used pure cellulose rather than heterogeneous field-collected biomass.

The inclusion of carboxymethyl cellulose (CMC) as a saccharification aid has also been shown to increase reducing sugar yield by loosening the substrate matrix and increasing water retention (Scheffer et al., 2022, Hafid et al.,2017). When used alongside fungal enzymes, CMC improves contact between enzyme and substrate, particularly in complex substrates like fruit-vegetable pulp mixtures.

Fermentation following enzymatic hydrolysis typically relies on Saccharomyces cerevisiae, which is capable of tolerating ethanol concentrations up to 14% (v/v) under optimized conditions (Tareen et al., 2021).

Some studies have reported the use of wild-type yeast isolates adapted to higher sugar and ethanol concentrations from natural ferments, such as dried figs and palm sap. These strains exhibit higher stress tolerance, especially under pH fluctuations and in the presence of residual inhibitors from pretreatment steps (Kooprasertying et al., 2022)

Despite advances in fungal fermentation, most ethanol studies still focus on mono-culture models or require expensive enzyme purification. Few have attempted to develop integrated, fungal-only systems that are capable of both hydrolysis and fermentation support using real, mixed agro-waste under low-energy pretreatment. There is also a lack of scalable models employing field-adapted fungal isolates, particularly those sourced from nutrient-rich environments like pond sediments or composting zones.

Therefore, this study aims to bridge these gaps by using a compatible, pond-isolated fungal consortium (Trichoderma viride and Aspergillus niger) for enzymatic hydrolysis, enhanced by CMC, followed by fermentation using stress-adapted S. cerevisiae. The approach is designed for small-scale, decentralized ethanol production using mixed agro-residues under moderate processing conditions, aligning with sustainable biofuel strategies.

- MATERIALS AND METHODS

- Collection and preparation of substrates

Agro-waste samples were collected from roadside vegetable vendors, juice stalls, and agro-industrial waste points near the MIET campus, Meerut (Uttar Pradesh, India). The biomass included sugarcane bagasse 20%, mixed fruit pulp and peel 40% (mainly orange, pomegranate, papaya), and decaying vegetable residues 20% (cabbage,  potato and tomato), kitchen waste 20% (spoiled food items). All samples were collected, peels are washed with sterile water, chopped into 2-4 cm fragments, and stored at 4 °C until further use (Magalhães et al. 2019).The mixture was homogenized manually and weighed to maintain a dry matter content of 30-40%. Equal quantities were pooled into sterile flasks for pretreatment.

- Fig 1(Vegetable waste collected from kitchen)

- Fig 2 (wasted agro-waste near meerut,INDIA

- Pre-Treatment of hemicellulosic agro-waste

Pretreatment was performed in two sequential stages:

- Acid Soaking: Hemicellulose biomass (sugarcane bagasse, fruit and vegetable peels) was soaked in 10% (v/v) H₂SO₄ overnight (16 hours) at room temperature (~28 °C) for hemicellulose disruption and microbial deactivation (Zhang et al., 2024).The slurry was filtered and washed with distilled water until near-neutral pH, then pH was manually adjusted to 6.0 using 1N NaOH.

- Steam Explosion: The acid-treated biomass was transferred to a stainless-steel pressure vessel and subjected to 180 °C at 1.2 MPa for 10 minutes, followed by sudden decompression. This facilitated further breakdown of lignocellulosic structure and enhanced enzyme accessibility. Post-explosion slurry was cooled, and the final pH was rechecked and adjusted to 6.0 prior to enzymatic hydrolysis. (Wang, L., Liu, Y., & Chen, H. 2019).

- Isolation and cultivation of fungal consortium

Pond water and surrounding compost-soil samples were screened for cellulolytic fungi by serial dilution and plating on Potato Dextrose Agar (PDA) amended with 2% cellulose.

Two dominant morphotypes were isolated and identified through microscopic and phenotypic analysis as: Trichoderma viride, Showing green conidial mass, fast sporulation, and clear zone on CMC agar, Aspergillus niger: Characterized by black, conidial heads with radial growth and pectinolytic activity on citrus peel agar. Fungal inocula were maintained on PDA slants at 4 °C and subcultured regularly for enzyme production (Asis et al., 2021, Akpomie et al., 2021)

- Fig.3 ( Aspegillus niger microscopy 10x)

- Fig 4 ( Trichoderma viride microscopy, 40x)

- Enzymatic saccharification

Submerged fermentation was carried out in 1000 mL Erlenmeyer flasks containing 500 mL of sterilized pretreated slurry. Each flask was inoculated with 10% v/v spore suspensions of both T. viride and A. niger in equal ratio (1:1), and incubated at 30 °C for 96 hours under static conditions (Woo et al., 2024) fermentation for centrifugation 0.5% Carboxymethyl Cellulose (CMC) was added to each flask as an inducer and structural enhancer for enzymatic action (Li et al., 2022), Sampling was done every 24 hours, and reducing sugar concentration was measured using the DNS method (McCleary et al., 2015), Final sugar yield (g/L) was recorded at 96 h for further fermentation.

- Fig. 5 Agro-waste slurry after pH set

- Fig. 6 Filtration of biomass after primary fungal

- Fermentation using adapted Saccharomyces cerevisiae

The hydrolysate was centrifuged to remove fungal biomass and inoculated with an adapted strain of Saccharomyces cerevisiae, isolated from dried fig ferment and selected for ethanol tolerance up to 14% v/v (Narayanan et al., 2016).

Fermentation was carried out in 1 L flasks with 500 mL working volume under semi-anaerobic conditions at 32 °C for 5–7 days, Occasional shaking was applied for uniform distribution, Refractive index (RI), ethanol concentration (% v/v), and pH were monitored at 48 h intervals, Final ethanol was recovered via fractional distillation, and % alcohol was confirmed using alcoholmeter (Jitrwung et al.,2015)

- Microbial viability and CFU determination

After fermentation, residual fungal viability was assessed by plating on PDA and serial dilution technique. Viable cell count (CFU/mL) for both fungi was measured after 30 days of liquid culture storage to check formulation compatibility (Rimkus et.al., 2023)

- Analytical methods and controls

Reducing sugars: Measured by DNS assay at 540 nm (McCleary et al., 2015), Ethanol content: Alcoholmeter and refractometer reading (RI ~1.341) cross-verified., pH: Digital pH meter, calibrated before use, CFU: Standard pour plate technique (10⁻⁶ dilution, triplicates) (Plugatar et al., 2023).Control sets (uninoculated, acid-treated but non-fermented) were maintained to validate natural degradation vs enzymatic conversion.

- RESULTS AND DISCUSSION

- Ethanol yield and fermentation profile

The pretreated agro-waste slurry yielded 42.6 g/L of reducing sugars after enzymatic hydrolysis by the fungal consortium. Following fermentation with stress-adapted S. cerevisiae, a final ethanol yield of 10.6% v/v was recorded by day 6. The pH dropped from 6.2 to 4.8 during fermentation, indicating active metabolic conversion, while the refractive index stabilized at 1.341, aligning with values reported in ethanol-dominant broths (Manochio et al., 2017)

- Fig. 7 Enzyme hydrolysed hemicellulose ethanol level almost (0) before Saccharomyces cerevisiae fermentation.

- Fig 8. Spectrophotometer reading of enzyme hydrolysed hemicellulose mass (529 nm ABS) at 1100 nm beam.

TABLE 1. Fermentation Indicators for Ethanol Production

| Parameter | Value |
| --- | --- |
| Reducing sugars (g/L) | 42.6 |
| Ethanol yield (% v/v) | 10.6 |
| Final pH | 4.8 |
| Refractive index (RI) | 1.341 |
| Distillation temp (°C) | 78.5 |

These values are consistent with prior work using dual fungal hydrolysis and wild yeast strains, though slightly higher ethanol conversion was observed, likely due to CMC inclusion and substrate diversity.

- Fungal viability post-fermentation

Table 2.  shelf life of fungus after 30 days for checking its compatibility.

| Parameter | T. viride | A. niger |
| --- | --- | --- |
| Initial CFU (day 0) | 2 × 10⁷ | 1 × 10⁷ |
| Final CFU (day 30) | 1 × 10⁷ | 1 × 10⁶ |
| Spore viability (%) | 85% | 80% |
| Shelf-stability (30 days) | Maintained | Maintained |

This confirms that the residual biomass post-distillation has potential for further compost-based valorization  to be explored in a separate study. Both T. viride and A. niger remained viable post-saccharification, with final CFU counts of 2 × 10⁷ CFU/mL and 1 × 10⁶ CFU/mL, respectively, after 30 days in stored broth. Their co-existence without zone inhibition confirms compatibility, supported by earlier findings on fungal consortiums in compost ecosystems (Plugatar et al., 2023, Rimkus et.al., 2023, Woo et al., 2024 ).

- Fig. 9 (Fermentation in 5L pot ,2nd trail)

- Fig. 10 (Final ethanol reading 10.5 v/v)

- Fig. 11 (Distillation assembly for pure ethanol recovery.

- Fig 12 (Extracted Ethanol after primary distillation)

- Summary of Results

- High ethanol conversion from mixed substrates with only fungal enzymes

- Dual fungal culture showed strong compatibility and shelf stability

- CMC inclusion boosted hydrolysis efficiency

- Low-energy pretreatment (overnight acid + 10-min steam) was effective

- Residual broth retained fungal load indicates future reuse possibilities

- CONCLUSION

This study successfully demonstrates a field-relevant bioconversion model for ethanol production using a dual-fungal consortium of Trichoderma viride and Aspergillus niger on heterogeneous agro-industrial waste. The combination of overnight 10% H₂SO₄ soaking and steam explosion proved effective in releasing fermentable sugars. Enzymatic hydrolysis supported by CMC yielded 42.6 g/L of reducing sugars, and subsequent fermentation using stress-tolerant Saccharomyces cerevisiae resulted in an ethanol yield of 10.6% (v/v). The fungal consortium remained viable post-fermentation, indicating both enzymatic efficiency and strain stability. The scalable and decentralized nature of this model aligns with rural biofuel solutions and national bioenergy missions. While this study focused on ethanol generation, the fungal-rich residue also shows potential for future compost or bioinput applications, which will be explored in a follow-up study.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

Author(s) hereby declare that generative AI technologies such as Google NotebookLM, Chat GPT, have been used for the rephrasing, spelling check, and language optimization  during the writing or editing of manuscripts.

- REFERENCES

- Magalhães Jr, A. I., de Carvalho, J. C., de Melo Pereira, G. V., Karp, S. G., Câmara, M. C., Medina, J. D. C., & Soccol, C. R. (2019). Lignocellulosic biomass from agro-industrial residues in South America: Current developments and perspectives. Biofuels, Bioproducts and Biorefining, 13(6), 1505–1519. https://doi.org/10.1002/bbb.2048

- Kumar, R., Kumar, V., & Nagpure, A. S. (2023). Bio-energy potential of available livestock waste and surplus agriculture crop residue: An analysis of 602 rural districts of India. Science of the Total Environment, 889, 163974. https://doi.org/10.1016/j.scitotenv.2023.163974

- Valera, H., & Agarwal, A. K. (2025). India’s growing Ethanol Blending Program and implications of scalable and sustainable Methanol Blending Program for transport sector. Transport Policy, 165, 179–193. https://doi.org/10.1016/j.tranpol.2025.01.007

- Alawad, I., & Ibrahim, H. (2024). Pretreatment of agricultural lignocellulosic biomass for fermentable sugar: Opportunities, challenges, and future trends. Biomass Conversion and Biorefinery, 14(5), 6155–6183. https://doi.org/10.1007/s13399-022-02981-5

- Dębowski, M., Dudek, M., Zieliński, M., Nowicka, A., & Kazimierowicz, J. (2021). Microalgal hydrogen production in relation to other biomass-based technologies—a review. Energies, 14(19), 6025. https://doi.org/10.3390/en14196025

- Nevalainen, H., Aggarwal, S., & Adlakha, N. (2024). Sources, properties, and modification of lignocellulolytic enzymes for biomass degradation. In Handbook of Biorefinery Research and Technology: Biomass Logistics to Saccharification (pp. 567–605). Dordrecht, Netherlands: Springer.

- Maji, D., Singh, M., Wasnik, K., Chanotiya, C. S., & Kalra, A. (2015). The role of a novel fungal strain Trichoderma atroviride RVF3 in improving humic acid content in mature compost and vermicompost via ligninolytic and celluloxylanolytic activities. Journal of Applied Microbiology, 119(6), 1584–1596. https://doi.org/10.1111/jam.12959 Oxford Academic+1ScienceDirect+1

- Sindhu, R., Gnansounou, E., Binod, P., & Pandey, A. (2016). Bioconversion of sugarcane crop residue for value-added products—An overview. Renewable Energy, 98, 203–215. https://doi.org/10.1016/j.renene.2016.02.057

- Sharma, A., & Aggarwal, N. K. (2020). Water hyacinth: A potential lignocellulosic biomass for bioethanol. Cham, Switzerland: Springer. https://doi.org/10.1007/978-3-030-35632-3

- Scheffer, G., Rachel, N. M., Ng, K. K., Sen, A., & Gieg, L. M. (2022). Preparation and identification of carboxymethyl cellulose-degrading enzyme candidates for oilfield applications. Journal of Biotechnology, 347, 18–25. https://doi.org/10.1016/j.jbiotec.2022.02.001

- Hafid, H. S., Nor’Aini, A. R., Mokhtar, M. N., Talib, A. T., Baharuddin, A. S., & Kalsom, M. S. U. (2017). Overproduction of fermentable sugar for bioethanol production from carbohydrate-rich Malaysian food waste via sequential acid–enzymatic hydrolysis pretreatment. Waste Management, 67, 95–105. https://doi.org/10.1016/j.wasman.2017.05.005

- Moya, E. B., Syhler, B., Dragone, G., & Mussatto, S. I. (2024). Tailoring a cellulolytic enzyme cocktail for efficient hydrolysis of mildly pretreated lignocellulosic biomass. Enzyme and Microbial Technology, 175, Article 110403. https://doi.org/10.1016/j.enzmictec.2024.110403

- Bustos, A. Y., Taranto, M. P., Gerez, C. L., Agriopoulou, S., Smaoui, S., Varzakas, T., & Enshasy, H. A. E. (2025). Recent advances in the understanding of stress resistance mechanisms in probiotics: Relevance for the design of functional food systems. Probiotics and Antimicrobial Proteins, 17(1), 138–158. https://doi.org/10.1007/s12602-024-10273-9 hittinger.genetics.wisc.edu

- Alvira, P., Tomás-Pejó, E., Ballesteros, M., & Negro, M. J. (2010). Pretreatment technologies for an efficient bioethanol production process based on enzymatic hydrolysis: A review. Bioresource Technology, 101(13), 4851–4861. https://doi.org/10.1016/j.biortech.2009.11.093

- Sun, L., & Jin, Y. S. (2020). Xylose assimilation for the efficient production of biofuels and chemicals by engineered Saccharomyces cerevisiae. Biotechnology Journal, 16(4), Article 2000142. https://doi.org/10.1002/biot.202000142

- Shukla, A., Kumar, D., Girdhar, M., et al. (2023). Strategies of pretreatment of feedstocks for optimized bioethanol production: Distinct and integrated approaches. Biotechnology for Biofuels, 16, Article 44. https://doi.org/10.1186/s13068-023-02295-2

- Benatti, A. L. T., & Polizeli, M. L. T. M. (2023). Lignocellulolytic biocatalysts: The main players involved in multiple biotechnological processes for biomass valorization. Microorganisms, 11(1), 162. https://doi.org/10.3390/microorganisms11010162

- Bhardwaj, N., Kumar, B., Agrawal, K., & Verma, P. (2021). Current perspective on production and applications of microbial cellulases: A review. Bioresources and Bioprocessing, 8(1), 95. https://doi.org/10.1186/s40643-021-00447-6

- Corrêa, T. L. R., Román, E. K. B., Cassoli, J. S., Santos, L. V. D., & Pereira, G. A. G. (2022). Secretome analysis of Trichoderma reesei RUT-C30 and Penicillium oxalicum reveals their synergic potential to deconstruct sugarcane and energy cane biomasses. Microbiological Research, 260, 127017. https://doi.org/10.1016/j.micres.2022.127017

- Tareen, A. K., Sultan, I. N., Songprom, K., Laemsak, N., Sirisansaneeyakul, S., Vanichsriratana, W., & Parakulsuksatid, P. (2021). Two-step pretreatment of oil palm trunk for ethanol production by thermotolerant Saccharomyces cerevisiae SC90. Bioresource Technology, 320, 124298.

https://doi.org/10.1016/j.biortech.2020.124298

- Kooprasertying, P., Vanichsriratana, W., Sirisansaneeyakul, S., Laemsak, N., Tareen, A. K., Ullah, Z., Parakulsuksatid, P., & Sultan, I. N. (2022). Ethanol production through optimized alkaline pretreated Elaeis guineensis frond waste from Krabi Province, Thailand. Fermentation, 8(11), 648. https://doi.org/10.3390/fermentation8110648

- Zhang, L., Huang, H., He, T., Sun, J., Chen, K., & Yue, F. (2024). Highly selective extraction of lignin and hemicellulose with retained original structure from sugarcane bagasse via low-temperature soaking and acidic precipitation. Industrial Crops and Products, 209, 118015. https://doi.org/10.1016/j.indcrop.2024.118015

- Wang, L., Liu, Y., & Chen, H. (2019). A steam-explosion-based hydrolysis and acidification technology for cornstalk bioconversion. BioEnergy Research, 12(1), 103–111. https://doi.org/10.1007/s12155-018-9961-z

- Asis, A., Shahriar, S. A., Naher, L., Saallah, S., Fatihah, H. N. N., Kumar, V., & Siddiquee, S. (2021). Identification patterns of Trichoderma strains using morphological characteristics, phylogenetic analyses and lignocellulolytic activities. Molecular Biology Reports, 48(4), 3285–3301. https://doi.org/10.1007/s11033-021-06318-w

- Akpomie, O. O., Okonkwo, K. E., Gbemre, A. C., Akpomie, K. G., Ghosh, S., Ahmadi, S., & Banach, A. M. (2021). Thermotolerance and cellulolytic activity of fungi isolated from soils/waste materials in the industrial region of Nigeria. Current Microbiology, 78(7), 2660–2671. https://doi.org/10.1007/s00284-021-02528-3

- Li, X., Tang, Z., Sun, Z., Simonsen, J., Luo, Z., Li, X., & Morrell, J. J. (2022). Chemical and enzymatic fiber modification to enhance the mechanical properties of CMC composite films. Polymers, 14(19), 4127. https://doi.org/10.3390/polymers14194127

- Woo, W. X., Tan, J. P., Wu, T. Y., Yeap, S. K., Luthfi, A. A. I., Manaf, S. F. A., … Hui, Y. W. (2024). An overview on the factors affecting enzymatic saccharification of lignocellulosic biomass into fermentable sugars. Reviews in Chemical Engineering, 40(2), 279–303. https://doi.org/10.1515/revce-2022-0019

- McCleary, B. V., & McGeough, P. (2015). A comparison of polysaccharide substrates and reducing sugar methods for the measurement of endo-1,4-β-xylanase. Applied Biochemistry and Biotechnology, 177(5), 1152–1163. https://doi.org/10.1007/s12010-015-1803-z

- Narayanan, V., Sánchez i Nogué, V., van Niel, E. W., & Gorwa-Grauslund, M. F. (2016). Adaptation to low pH and lignocellulosic inhibitors resulting in ethanolic fermentation and growth of Saccharomyces cerevisiae. AMB Express, 6(1), 59. https://doi.org/10.1186/s13568-016-0228-5

- Jitrwung, R., & Yargeau, V. (2015). Biohydrogen and bioethanol production from biodiesel-based glycerol by Enterobacter aerogenes in a continuous stir tank reactor. International Journal of Molecular Sciences, 16(5), 10650–10664. https://doi.org/10.3390/ijms160510650

- Rimkus, A., Namina, A., Dzierkale, M. T., Grigs, O., Senkovs, M., & Larsson, S. (2023). Impact of growth conditions on the viability of Trichoderma asperellum during storage. Microorganisms, 11(4), 1084. https://doi.org/10.3390/microorganisms11041084

- Plugatar, Y., Johnson, J. B., Timofeev, R., Korzin, V., Kazak, A., Nekhaychuk, D., … Rotanov, G. (2023). Prediction of ethanol content and total extract using densimetry and refractometry. Beverages, 9(2), 31. https://doi.org/10.3390/beverages9020031

- Manochio, C., Andrade, B. R., Rodriguez, R. P., & Moraes, B. S. (2017). Ethanol from biomass: A comparative overview. Renewable and Sustainable Energy Reviews, 80, 743–755. https://doi.org/10.1016/j.rser.2017.05.063