Physicochemical and Bacteriological Quality Assessment of Ogbunike Abattoir Wastewater Anambra State, Nigeria for Irrigation

ABSTRACT

The physicochemical and bacteriological properties of abattoir wastewater from Ogbunike abattoir in Anambra State, Nigeria was evaluated to ascertain its fitness for irrigation purposes. Physicochemical parameters which include the heavy metal concentrations, temperature, pH, electrical conductivity, colour, total dissolved solid (TDS), total suspended solid (TSS), turbidity, nitrate, total amount of sulphate, total amount of phosphate, total amount of chloride, biochemical oxygen demand (BOD) and chemical oxygen demand (COD) were evaluated according to standard methods for the examination of water and wastewater. Bacteriological quality of abattoir wastewater was also conducted using standard microbiological methods. Results from the physicochemical analyses show mean values of 1,431.81 pcu for colour, temperature (28.9 oC),  pH (6.30), conductivity (4,560 µS/cm), TDS (3,420 mg/L), TSS (367 mg/L),  turbidity (13564 NTU),  BOD (895 mg/L), COD (4,280 mg/L), phosphate (10.19 mg/L), sulphate (180.85 mg/L), nitrate (77.78 mg/L) and chloride (1,086.75 mg/L). These values exceeded the WHO/NESREA limits for water quality, except pH and temperature with values within the permissible limits. Data from the heavy metal analysis presented mean concentrations of Cadmium (0.111 mg/L), Chromium (0.065 mg/L), Lead (0.738 mg/L), Copper (1.467 mg/L) and Nickel (0.075 mg/L). All the heavy metals analyzed exceeded the WHO/NESREA permissible limits. The abattoir wastewater presented total coliform counts of 8.2 x 105 CFU/100mL. The study revealed that the discharge of effluents from Ogbunike abattoir into the environment poses a great threat to the environment and pretreatment should be carried out before discharge.

Keywords: Abattoir, Wastewater, Heavy metal, Coliform, Ogbunike.

1.0 Introduction

Slaughterhouses, otherwise known as abattoirs are facilities set aside for the slaughtering of animals for meat consumption (Nwanta et al., 2008). In Nigeria, abattoirs are a crucial component of the domestic meat supply chain resulting in the meat industry being regarded as the major contributor to liquid waste [1]. Environmental pollutants released through wastewater by abattoirs may be complicated due to changes brought about by additional substance used during animal processing. Wastewater is usually released from abattoirs directly into the ecosystem without treatment, thereby posing serious threats to surface water quality and greatly impact environmental health. The abattoir industry is an important component of the livestock industry in Nigeria, providing domestic meat supply to over 200 million people and also employment opportunities and source of livelihood (Okoye et al., 2023). However, majority of the activities going on in most abattoir sites in Nigeria are not monitored and facilities for the treatment of abattoir effluents before discharge into nearby water bodies are not lacking. Abattoir activities are generally known to pollute the receiving environments such as water and land from their various processes, either directly or indirectly (Bala et al., 2016). Abattoir wastewater contain high amounts of biodegradable organic matter, suspended matter and colloidal matter such as fats, proteins, and cellulose and these directly or indirectly impact the water quality parameters such as pH, temperature, electrical conductivity, salinity, turbidity, total dissolved solids, total suspended solids, dissolved oxygen (DO) and coliforms (Mujere, 2015).

In the natural environment, pollutants including heavy metals are present in various chemical forms and display varied effects in terms of chemical interactions, mobility, biological abundance, and consequently, they have different potential toxicity (Nazir et al., 2015). Some of these heavy metals can be persistent environmental contaminants since they cannot be degraded easily (Osu et al., 2014; Okoye et al., 2023). One of the ways to determine threats to surface water quality is to assess the biochemical oxygen demand (BOD), chemical oxygen demand (COD), total solids (TS), pH, temperature, turbidity, and a lot of heavy metals, etc. (Ogbonna and Ideriah, 2014; Ogunlade et al., 2021).

Polluted water is home to millions of pathogenic microorganisms, which are responsible for various water-related diseases such as typhoid fever, cholera, diarrhoea, dysentery, giardiasis (WHO, 2011). Although chemical composition may affect the safety, taste and appearance, bacterial contamination cannot be detected by appearance, taste or smell. This can only be detected by testing the water sample for the presence of indicator organisms such as Escherichia coli and other coliform organisms present in the water samples (Ogunlade et al., 2021; Okoye et al., 2023). Ideally, drinking water should not contain any pathogenic organism, and should be free from bacteria indicative of faecal pollution (WHO, 2011). With the level of contamination around the slaughterhouses, there is need to evaluate the physical, chemical and bacteriological qualities of wastewater generated in abattoir sites before discharging into the environment. Hence, this study is aimed at examining the physicochemical and bacteriological quality of Ogbunike abattoir wastewater, in Anambra State, Nigeria for Irrigation.

2.0 Materials and methods

2.1 Study Area:

The study was carried out in Ogbunike town in Oyi Local Government Area of Anambra State Nigeria. Ogbunike Town lies at Longitude 6.40oE and Latitude 6.20oN. A major abattoir is situated in Ogbunike town along Onitsha –Enugu express way, Anambra State, Nigeria.

Figure 1: Map showing the location of Ogbunike Abattoir

2.2 Sample collection

The wastewater samples were collected from two points; the wastewater drainage and the wash drums, with the aid of a plastic scoop into a sterilized 25-L container. The wastewater was homogenized, labelled and transported to the laboratory for analysis.

2.3 Physicochemical Evaluation of the wastewater samples

The physicochemical properties of the abattoir wastewater samples were evaluated immediately after collection. Heavy metal concentration, temperature, pH, conductivity, colour, total dissolved solid (TDS), total suspended solid (TSS), turbidity, nitrate, total amount of sulphate, total amount of phosphate, total amount of chloride, biochemical oxygen demand (BOD) and chemical oxygen demand (COD) were evaluated according to standard methods for the examination of water and wastewater (APHA, 2012).

Heavy metal analysis was conducted using Agilent FS240AA Atomic Absorption Spectrophotometer according to the method of APHA 2012 (American Public Health Association).

Working principle: The atomic absorption spectrometer's working principle is based on the sample being aspirated into the flame and atomized when the AAS's light beam is directed through the flame into the monochromator, and onto the detector that measures the amount of light absorbed by the atomized element in the flame. Since metals have their characteristic absorption wavelength, a source lamp composed of that element is used, making the method relatively free from spectral or radiational interferences. The amount of energy of the characteristic wavelength absorbed in the flame is proportional to the concentration of the element in the sample.

Procedure: The wastewater sample was thoroughly mixed by shaking and 100ml of the water sample was transferred into a glass beaker of 250ml volume, to which 5ml of conc. nitric acid was added and heated to boil till the volume was reduced to about 15-20ml, by adding conc. nitric acid in increments of 5ml till all the residue was completely dissolved. The mixture was cooled, transferred and made up to 100ml using metal-free distilled water. The sample was aspirated into the oxidizing air-acetylene flame. When the aqueous sample was aspirated, the sensitivity for 1% absorption was observed.

Preparation of reference solutions

A series of standard metal solutions in the optimum concentration range were prepared, and the reference solutions were prepared daily by diluting the single stock element solutions with water containing 1.5 mL concentrated nitric acid/litre.  A calibration blank was prepared using all the reagents except the metal stock solutions. A calibration curve for each metal was prepared by plotting the absorbance of standards versus their concentrations.

2.3.1 Colour determination

According to the method of Singh et al. (2017), the sample was centrifuged at 1,000 rpm for
30 min to remove all the suspended matter. The pH was adjusted to 7.6 with 2 M NaOH and then used for the measurement of absorbance at 465 mm. The absorbance values were transformed into colour unit (CU) using the following relationship:

CU =   500 x A1

A2

---

Where A1 = Absorbance of 500 cu platinum cobalt standard solution and A2 = Absorbance of the effluent sample.

2.3.2 Determination of Temperature, pH, Conductivity and Total dissolved solid

The temperature, pH, conductivity and total dissolved solids of the prepared samples were determined using multi meter analytical instrument (Model Ph-2603, China). The samples were dispensed in beakers and triplicate readings were taken after calibrations of the instrument as instructed by the manufacturer (APHA, 2012).

2.3.3 Determination of total suspended solid (TSS)

According to the method of Singh et al. (2017), 100 mL of the sample was centrifuged at 2000 rpm for 10 minute. The supernatant was removed and the residue was washed three times by suspending it in distilled water and recollecting by centrifugation. The residue was finally transferred quantitatively to pre - weighted dish (X1g). The dish was weighed again after drying (X2g) to a constant weight at 105 0C. TSS was calculated by using the following formula:

TSS (mg/L) = X2 – X1 X 1,000

Volume of sample

2.3.4 Turbidity

The turbidity of the samples was determined after calibrations of the turbidometer instrument with blank and standard solutions and triplicate measurements were taken (APHA, 2012).

2.3.5 Determination of Dissolved Oxygen and Biochemical Oxygen Demand (BOD5)

The amount of biological oxygen demand was determined using Winkler’s method according to the description of APHA (2012). The samples were collected in a BOD bottle using D.O sampler. Then, 1 mL of MnSO4 followed by 1 mL of alkali-iodide-azide reagent was added to a sample collected in 250 to 300 mL bottle up to the brim and mixed well by inverting the bottle 2 - 3 times and allowing the precipitate to settle down and leaving 150 mL clear supernatant. The precipitate is white if the sample is devoid of oxygen, and becomes increasingly brown with rising oxygen content. At this stage, 1 mL of concentrated H2SO4 was added and the stopper replaced and mixed well till the precipitate goes into solution. Thereafter, 201 mL of this solution was taken in a conical flask and titrated against standard Na2S2O3 solution. Then, 2 drops of starch indicator were added and continued to titrate till the colour of the solution became either colourless or changes to its original sample color and the volume of 0.025N sodium thiosulfate consumed was noted down. As 1 mL of sodium thiosulfate of 0.025N equals to 1 mg/L dissolved oxygen. Therefore, dissolved oxygen (D.O.) (in mg/L) = mL of sodium thiosulfate (0.025N) consumed. Finally, the BOD5 calculation is given by the formula below:

Where: D1 = DO of diluted sample immediately after preparation (mg/L); D2 = DO of diluted sample after a 5day incubation at 20 °C (mg/L); P = Decimal volumetric fraction of sample used (mL of the sample taken (201 mL) divided by total volume of the BOD bottle (400 mL).

2.3.5 Determination of Chemical Oxygen Demand (COD)

The amount of chemical oxygen demand was determined according to APHA (2012). The culture tubes and caps were washed with 20 % H2SO4 before using to prevent contamination. Then, 2.5 mL of the industrial effluent sample and 1.5 mL of K2Cr2O7 digestion solution were placed in culture tube. Three point five (3.5 mL) of sulphuric acid reagent was carefully ran down inside of vessel so an acid layer is formed under the sample - digestion solution layer and tightly cap tubes or seal ampules and invert each several times to mix completely. The tubes were placed in water bath preheated to 100 °C and refluxed for 3 hours. The culture vessels were cooled to room temperature and vessels placed in test tube rack. Two drops of Ferron indicator were added and stir rapidly with magnetic stirrer while titrating with standardized 0.10 M ferrous ammonium sulphate (FAS). A sharp colour change from blue - green to reddish brown appeared as end - point, although the blue green reappeared within minutes. In the same manner, a blank containing the reagents and a volume of distilled water equal to that of the sample was refluxed and titrated. The COD is given by:

COD (mg O2 /L) = [(A - B) × M × 8,000) / (V sample)

Where: A = volume of FAS used for blank (mL); B = volume of FAS used for sample (mL); M = molarity of FAS; 8,000 = milli equivalent weight of oxygen (8) ×1,000 mL/L.

2.3.6 Determination of Phosphate

The amount of phosphate was determined using molybdenum blue phosphorous method in conjunction with UV - Visible spectrophotometer according to APHA (2012). In order to prepare a calibration curve for the standard, 2 mL of the standard solution concentrations of 0.5, 1.0, 1.5, 2.0, 2.5, 3.0 and 3.5 ppm, respectively, 1 mL of ammonium molybdate and 0.4 mL of hydrazine sulphate were added and was made up to the mark with double distilled water in a 10 mL standard volumetric flask. The same procedure above was repeated but with 4 mL of the industrial effluent samples. These were kept for 30 min in a water bath for heating at 60 °C. On heating, a blue colour was observed due to the formation of phosphomolybdate complex and was cooled. The absorbance was measured using UV- Visible Spectrophotometer at 860 nm. sterilized water was used as experimental blank solution. The analysis was carried out in triplicate.

2.3.7 Determination of Sulphate

The sulphate concentration was determined according to the standard method of APHA (2012) and as described by Sharma and Kaur (2016). The calibration curve was prepared by preparing series of standard solution of sulphate (anhydrous sodium sulphate Na₂SO4) concentrations (5, 10, 20, 30 and 40 mg/L) by diluting stock solution of sulphate and the blank with distilled water. Reading of each standard was taken on turbidity meter/spectrophotometer and calibration curve was prepared. The industrial effluent sample was filtered through 0.45 μm if there is any turbidity and 20 mL of the water sample was taken in 100 mL conical flask. Then, 1 mL HCl and 1 mL conditioning reagent were added and mixed well for 30 seconds. After 10 minutes, the turbidity was read on turbidity meter and concentration of sulphate in sample was directly found from the curve.

2.3.8 Determination of Chloride

Chloride was obtained using Argentometric titration method and as described by Adelowo and Agele (2016). Potassium chromate indicator solution was prepared by dissolving 50 g K2CrO4 in a distilled water and AgNO3 solution was added until a definite red precipitate was formed. This solution was allowed to stand for 12 hours, filtered and diluted to 1 L with distilled water. Then, 2.395 g AgNO3 was dissolved in distilled water and diluted to 1000 mL and stored in a brown bottle. This is the standard silver nitrate solution. Thereafter, 50 mL of the effluent sample was measured into 250 mL conical flask followed by addition of 1 mL K2CrO4 solution (indicator) and was titrated with AgNO3 (titrant) to a pinkish yellow end point. The process was repeated for blank using 50 mL of distilled water. The chloride content was determined as follow:

Mg Cl-/L = (A - B) × N × 35 × 450

Volume of sample

Where A = mL titration for sample, B = mL titration for blank, N = Normality of AgNO3

2.4 Coliform evaluation of abattoir wastewater samples

To evaluate the total coliform in the wastewater samples, 1 ml each of the wastewater samples was serially diluted using the ten-fold serial dilution. After the serial dilution, Chromocult Coliform Agar was prepared following the manufacturer’s instructions. Then 1ml each of the 3rd diluents were plated out in duplicates using the pour plating technique with the already prepared medium. The plates were incubated aerobically at 37 0C for 24 hours. The Chromocult Coliform Agar is a selective and differential chromogenic medium for the microbiological analysis of water samples. Within 24 hours, this medium enables the simultaneous detection, differentiation and enumeration of E. coli and other coliform bacteria in water. The presence of coliform bacteria results in salmon red-coloured colonies while E. coli takes a dark blue-coloured colony.

3.0 Results and Discussion

The result of the physicochemical evaluation of the abattoir wastewater is presented in Table 1. The colour of the abattoir waster was dark brown with a mean value of 1,431.81. Colour, as a physicochemical parameter measures the aesthetic value of water (Mulu and Ayenew, 2015). Colour appearance in water indicates the presence of dissolved and suspended particles. The presence of a pronounced colour in water is a clear indicator of poor water quality. Coloured water often suggests contamination and the presence of substances that could be harmful to both human health and aquatic ecosystems.

Table 1: Physicochemical profile of the sewage and abattoir wastewater samples

| Parameter | Abattoir wastewater | WHO (2011) Standard | NESREA (2007) standard |
| --- | --- | --- | --- |
| Colour (Unit) | 1,431.81±2.00 | 5.13 | 7.00 |
| Temperature (°C) | 28.90±0.10 | Ambient | 30.00 |
| pH | 6.30±0.01 | 6.00 – 9.00 | 6.00 – 9.00 |
| Conductivity (µS/cm) | 4,560.00±0.10 | 1,250.00 | 1,000.00 |
| TDS (mg/L) | 3,420.00±1.00 | 2,000.00 | 2,000.00 |
| TSS (mg/L) | 367.00±1.00 | 30 - 200 | 30.00 |
| Turbidity (NTU) | 135.64±1.00 | 5.00 – 10.00 | 10.00 |
| BOD5 (mg/L) | 895.52±1.00 | 30.00 – 100.00 | 20.00 |
| COD (mg/L) | 4,280.00±3.00 | 250.00 | 80.00 – 100.00 |
| Phosphate (mg/L) | 10.19±0.10 | 5.00 | 5.00 |
| Sulphate (mg/L) | 180.85±0.10 | 500.00 | 500.00 |
| Nitrate (mg/mL) | 77.78±0.20 | 50.00 | 40.00 |
| Chloride (mg/L) | 1,086.75±0.20 | 350.00 | 600.00 |

The  mean pH and temperature of the abattoir wastewater were 6.30 and 28.90 oC respectively, which are within the WHO and NESREA permissible limits and corroborate with the pH and temperature values recorded in the work of Fadeyibi et al. (2011) and Okoye et al. (2023). Other parameters analyzed were observed to be above the WHO and NESREA permissible limits. The abattoir wastewater recorded a mean electrical conductivity value of 4,560 µS/cm. The electrical conductivity measures dissolved ions and the ability of an environmental sample to conduct electricity (Isoken and Ita, 2018). Thus, electrical conductivity and total dissolved solids are strongly positively correlated; hence, an increase in dissolved solids increases the electrical conductivity (Bakume et al., 2019). The high electrical conductivity value recorded in the abattoir wastewater could be attributed to the use of confined containers for washing and processing of the cow hides. This may give room for accumulation of organic and inorganic compounds and build-up of suspended solids, as the abattoir workers do not discard the wash water immediately after use not until they are done for the day. The World Health Organization recommends that the electrical conductivity of water should be no higher than 1000 µS/cm (WHO, 2011).

The Chemical Oxygen Demand (COD) of the abattoir wastewater showed a mean value of 4,280, which is higher than the WHO and NESREA permissible limits (50-100 mg/L). COD measures the amount of oxygen required to degrade the organic pollutant in water. High COD level shows the presence of significant amount of chemical oxidants in the abattoir effluent while low COD levels show otherwise (Ogbuene et al., 2020). In the work by Adeyemi et al. (2019) high COD levels of 338.42 mg/L upstream and 382.09 mg/L downstream, were recorded both of which exceeded the WHO permissible limit. Similarly, Okoye et al. (2023) recorded COD values of 48.00 mg/L upstream and 234.70 downstream. Result recorded mean BOD value of 895, which is higher than WHO/NESREA permissible limit. High BOD level indicates the presence of large amounts of organic matter, which, when decomposed by microorganisms, leads to a depletion of dissolved oxygen. The implication of high BOD values is that abattoir-affected water is largely unsafe and unsuitable for any beneficial purposes. BOD values recorded in this study (895.52±1.00 mg/L) far exceeds the standard permissible limits as recommended by the WHO and NESREA (30–100 mg/L and 20.0 mg/L), respectively.

The abattoir wastewater showed high TDS (3,420 mg/L), TSS (367 mg/L) and turbidity (13564 NTU) values. The high TSS and TDS values in the abattoir wastewater could be as a result of contents from blood, gut and organic matters from scaling of the burnt cow hides. The values from the result revealed the high level of pollution in Ogbunike abattoir. Turbidity is a measure of water clarity. A major source of increased turbidity in abattoir wastewater is the presence of suspended particles such as blood and animal urine. Wizor and Nwankwoala (2019) recorded high turbidity values. Several studies have reported elevated turbidity levels in areas affected by abattoir wastes, often surpassing the permissible limits set by the World Health Organization (WHO). Okoye et al. (2023) reported that turbidity levels of abattoir wastewater in Ezu River, Uguwoba, Enugu State, Nigeria ranged from 20.53 to 118.5 NTU. Similarly, Alabi et al. (2020) reported, turbidity measurements of 227 to 647 NTU, significantly higher than the WHO’s maximum acceptable limit of 25 NTU.

The level of phosphate, sulphate and nitrate detected were 10.19 mg/L, 180.85 mg/L and 77.78 mg/L respectively. These values were clearly above the WHO/NESREA limit. Similarly, chloride concentration (1,086.75 mg/L) was above the permissible limit. The presence of chloride may be as a result of the cleaning agents such as detergents and soaps used in the cleaning of animal hides after roasting and removal of blood.

Result of the evaluation of heavy metal concentrations of the abattoir wastewater is presented in Table 2.

Table 2: Heavy metal profile of abattoir wastewater

| Metal (mg/L) | Abattoir wastewater | WHO Limit | NESREA (2007) Limit |
| --- | --- | --- | --- |
| Cadmium | 0.111±0.02 | 0.05 | 0.05 |
| Chromiun | 0.065±0.56 | 0.05 | 0.01 |
| Lead | 0.738±0.05 | 0.4 | 0.02 |
| Copper | 1.467±0.94 | 0.05 | 0.01 |
| Nickel | 0.075±0.02 | 0.1- 0.02 | 0.05 |

Key: NESREA = National Environmental Standards and Regulations Enforcement Agency; mg/L = Milligram per litre

The heavy metals analyzed in this work were cadmiun, chromiun, lead, copper and nickel, and all  presented values higher than the WHO (2011) and NESREA (2007) permisible limits in water. The mean concentrations of the heavy metals are  0.111±0.02 mg/L, 0.065±0.56 mg/L, 0.738±0.05 mg/L, 1.467±0.94 mg/L and 0.075±0.02 mg/L for cadmiun, chromiun, lead and copper and nickel, respectively. Heavy metal accumulation in the wastewater could be as a result of the presence of various chemicals, organic and inorganic compound resulting as wastes from cleaning agents and cumbustion of tyres in the slaughter house processing of the cowhides. Okoye et al. (2023) also reported elevated levels of heavy metals (Pb, Cu, Cd and Cr) in Ugwuoba abattoir effluent from Ezu River.

The presence of these heavy metals in high concentration in the abattoir wastewater reveals the high level of pollution and health risk to humans and the environment.  According to the World Health Organization (WHO, 2011) report, the heavy metals of most immediate concern in water pollution monitoring are Al, Cr, Mn, Fe, Co, Ni, Cu, Zn, Cd, and Pb. The presence of any of these metals in water may pose an environmental and health risk at concentrations higher than the set limit (Nazir et al., 2015). However, some heavy metals, including Al, Cu, Cd, Fe, Pb, Mn and Ni are not hazardous when present in minute concentrations. Nevertheless, even at trace concentrations, some heavy metals have also been found to be toxic (Onianwa, 2001).

Table 3 shows the result of  the coliform evaluation of the abattoir wastewater and presented  total coliform counts of 8.2 x 105.

Table 3: Coliform count for the untreated sewage and abattoir wastewater

| Wastewater sample | Total coliform (CFU/mL) | Log CFU/mL |
| --- | --- | --- |
| Abattoir wastewater | 8.2 × 105 | 5.9031 |

Similar total coliform range was reported in abattoir wastewater by Bosede and Omokaro (2021). The presence of bacteria and coliforms in abattoir wastewater is an indication of high organic content.

Conclusion

The study has revealed the level of contamination of the effluent from Ogbunike abattoir and the environmental health implications of discharging and irrigation use of the wastewater in nearby farmlands. Therefore, regulation should be enacted to ensure pre- treatment of the effluents before discharge and improved sanitation of the abattoir environment and workers.

References

- Adelowo, F. and Agele, S. (2016). Spectrophotometric analysis of phosphate concentration in agricultural soil samples and water samples using molybdenum blue method. Brazilian Journal of Biological Sciences, 3:407–412.

- Adeyemi-Ale, O.A. (2014). Impact of abattoir effluent on the physico-chemical parameters of Gbagi stream (Odo-eran), Ibadan, Nigeria. Ilorin Journal of Science, 1(1):100–109

- Alabi, J.O., Sanni, J.E., Ashiru, M.A., Yusuf, M.J. and Adeoye, A.S. (2020). The Effect of Nasarawa Central Abattoir Effluent on the Receiving River, Nasarawa, Nasarawa State, Nigeria. World Journal of Innovative Research, 9(1):31–35

- Anunobi, I.A., Adedayo, F.O., Oyetola, A.S., Umar, D.S., and Ayuba, P. (2015). An evaluation of the effects of abattoir wastes/effluents on its residential environs in Minna, Niger state, Nigeria. Herald Journal of Geography and Regional Planning, 4(1):8–15

- American Public Health Association (APHA) (2012). Standard Methods for Examination of Water and Wastewater. 22nd edition. American Public Health Association, Washington, DC.  pp. 1360.

- Bakume, B.Q., Asong, T.V. and Essiais, U. (2019). Abattoir waste management and its potential effects on humans and surface water quality: South west region, Cameroon.  International Journal of Science and Research, 8:2218–2223.

- Bala, J., Moumouni, A. and Musa, A.A. (2016). Influence of abattoir wastes on groundwater quality in Keffi, North Central, Nigeria. Journal of Environmental Science, Toxicology and Food Technology, 13(3):71–80.

- Benka-Coker, M. and Ojioro, O. (1995). Effect of Slaughterhouse Wastes on the Water Quality of Ikpoba River, Nigeria. Bioresource Technology, 52:5–12.

- Fadeyibi, A., Halilu, A. and Osunde, Z. D. (2011). Analysis of Physicochemical Properties of Abattoir Wastewater for Irrigation in Minna, Niger State. Continental Journal of Water, Air and Soil Pollution, 2(2):12–15.

- Isoken, H.I. and Ita, U.T (2018). Effect of abattoir effluents on the physicochemical properties of a receiving watershed in an urban community. Ife Journal of Science, 20(2):219–228.

- Mujere, N. (2020). Water Quality Impacts of Abattoir Activities in Southern Africa. In: Waste Management. pp.405–415

- Mulu, A. and Ayenew, T. (2015). Characterization of Abattoir Wastewater and Evaluation of the Effectiveness of the Wastewater Treatment Systems in Luna and Kera Abattoirs in Central Ethiopia. International Journal of Scientific and Engineering Research, 6(4):1026–1041.

- Nazir, R., Khan, M., Masab, M., Ur Rehman, H., Ur Rauf, N. and Shahab, S. (2015). Accumulation of heavy metals (Ni, Cu, Cd, Cr, Pb, Zn, Fe) in soil, water and plants and analysis of physicochemical parameters of soil and water collected from Tanda Dam, Kohat. Journal of Pharmaceutical Science Research, 7(3):89–97

- Ogunlade, T.M., Babaniyi, B.R., Afolabi, F.J. and Babaniyi, G.G. (2021). Physicochemical, heavy metal and microbiological assessment of wastewater in selected abattoirs in Ekiti State, Nigeria. Journal of Environmental Treatment Techniques, 9(4):788–795.

- Okoye, C.K., Ogunjiofor, E.I., Okoye, E.C.S. and Okoye, L.C. (2023). Impact of abattoir waste effluent on water bodies: A case study of Ugwuoba abattoir activities on Ezu River, Enugu State, Nigeria. International Journal of Ecology and Environmental Sciences, 5(2):19–25.

- Osu, C.I., Ogoko, E.C. and Emeziem, D. (2014). Metal Species, Mobility and Bioavailability In: An Oil Impacted Soil From Ikoku And Alaoji Automobile Spare Parts Markets, Niger- Delta Nigeria. American Academic and  Scholarly Research Journal, 6(3):1–10

- Ogbonna, D.N. and Ideriah, T.J.K. (2014). Effect of Abattoir Waste Water on Physico-chemical Characteristics of Soil and Sediment in Southern. Nigeria. Journal of Science Research Reports, 3(12):1612–1632.

- Ogbuene, E.B., Eseile, E.A., Oroke, A.M. and Eze, C.A (2020). Assessment of the Impact of Abattoir Waste on Ikpoba River Water Quality, Benin City Nigeria. IAMURE Multidisciplinary Research, 31(4):231–239.

- Sharma, Y. and Kaur, K. (2016). Determination of nitrates and sulphates in water of Barnala (Punjab, India) region and their harmful effects on human lives. International Journal of Advanced Research in Education and Technology, 3(3):79–82.

- Singh, P., Srivastava, N. and Singh, P. (2017). Analysis of various iron nanoparticles and compounds in pulp and paper mill waste water treatment. Integrated Research Advances, 4(2):24–28.

- Wizor, C.H. and Nwankwoala, H.O. (2019). Effects of Municipal Abattoir Waste on Water Quality of Woji River in Trans-Amadi Industrial Area of Port Harcourt, Nigeria: Implication for Sustainable Urban Environmental Management. International Journal of Geography and Geology, 8(2):44–57.

- World Health Organization (2011). Guidelines for drinking water quality. WHO Geneva, 2nd Edition, Vol 1.