# Short communication

Latest Applications of Ferroelectric Materials

.

ABSTRACT

Ferroelectric materials although have their origin many decades ago but since than these are expanding and developing. These materials find their application in every branch of Science. Researchers are working hard on their newer applications. Some latest applications include Energy storage devices, wearable electronics, Field effect transistors, Infrared detectors, Memristors, Photodetection, Neuromodulation and Electro-optic applications.

Keywords: Ferroelectric Materials, Oscillator, Memristor, Actuators, Transistor

1. INTRODUCTION

Ferroelectric materials possess spontaneous polarization that can be switched by applied electric field. Conventionally, ferroelectric materials are used in Capacitors, Actuators, Sensors, Memory Devices, Thermistors, Switches, Oscillators, Filters [1-4] etc. Although Ferroelectric Materials have been used from several decades but Scientists are still exploring its applications. This manuscript summarizes some of the latest applications of ferroelectric materials.

2. results and discussion

2.1 Ferroelectric materialS in Energy storage application

Liu et al. [5] have used electro-thermally induced ferroelectric phase transition in percolative ferroelectric polymer nanocomposites to achieve high strain performance in electric-field-driven actuation materials. They demonstrate a strain of over 8% and an output mechanical energy density of 11.3 J cm−3 at an electric field of 40 MV m−1 in the composite.

Yang et al. [6] have explored PbZr0.52Ti0.48O3 (PZT)/PbZrO3 (PZ) composite films, prepared by sol-gel method for energy storage applications. These films possess the high energy storage density of 10.0 J cm−3 and the energy storage efficiency of 84.8%.

Pappachan et al. [7] have examined Lead-free ternary solid solutions of (1−2x) K0.5Bi0.5TiO3-xBaTiO3-xLiNbO3 (KBLN100x) with x = 0.03, 0.05, 0.07, and 0.09. A recoverable energy density of 1.3 J/cm3 was achieved for the KBLN7 sample, making it potential candidate for energy storage application.

2.2 FERROELECTRIC MATERIALS IN FIELD EFFECT TRANSISTOR (FET) APPLICATION

Zhu et al. [8] have used a single vertical nanowire ferroelectric tunnel field-effect transistor (ferro-TFET) that can modulate an input signal with diverse modes including signal transmission, phase shift, frequency doubling, and mixing with significant suppression of undesired harmonics for reconfigurable analogue applications. Conventional Ferroelectric Materials cannot be used directly in making CMOS devices due to fatigue nature, however Chen et al. [9] demonstrated the use of hafnium based ferroelectric material in making high density and high performance devices.

Liu et al. [10] have used the ferroelectric material in insulator gate in the TFT Structure to form ferroelectric field effect transistors. Li et al. [11] have demonstrated the high performance wearable ferroelectric field effect transistors. In this, Zr doped HfO2 was linked with ultra-thin layer of Indium Tin Oxide (ITO). The device can be used as high flexible wearable memory. The device exhibit thermal budget of 400OC, memory window of 2.78 V, current on/off ratio of 108, bending reliability of 5×105 pulse cycles with bending radius of 5 mm and high endurance of 2×107 cycles. This resulted in highly bendable and transparent memory device.

Razumnaya et al. [12] have used ferroelectric logic unit comprising of multiple nanodots stacked between two electrodes and coated by dielectric material in novel topologically-controlled implementation of discrete synaptic states in neuromorphic computing. They have devised the integration of the ferroelectric logic unit, providing topologically configurable non-binary logic into a gate stack of the field-effect transistor as an analog-like device with resistive states. By controlling the charge of the gate, they demonstrated the various routes of the topological switchings between different polarization configurations in ferroelectric nano-dots. Hysteresis loops were considered for finding the switching routes between different logic levels. Different logic levels were achieved by varying external fields and temperature used for measuring hysteresis loops.

2.3 FERROELECTRIC MATERIALS IN MEMORY APPLICATION

Xue et al. [13] have demonstrated the origin of ferroelectric resistance switching using planar vander Walls ferroelectric memristors and volatile and nonvolatile ferroelectric memristor, with large on/off ratios above 104.

Wen et al. [14] developed electro-optic memristor based on ferroelectric PMN-PT system. These can be used in programmable photonic integrated circuits and is based on electro-optic effect. Phase of light is modified without affecting its transparency. Non-volatile wave-plate shifts the phase of the light by 90°. The resulted memristor possess energy consumption of switching 234 nJ and acquired states are retained up to 12 h without needing any static power consumption.

2.4 FERROELECTRIC MATERIALS IN OPTICAL APPLICATION

Whatmore et al. [15] have tested various ferroelectric materials such as Pb[(Mg⅓Nb⅔)xTi1−x]O3  and Na½Bi½TiO3–K½Bi½TiO3 based lead-free crystals and ceramics system for Pyroelectric Infrared Detectors in the wide frequency range of 0.1 to100 Hz.

He et al. [16] have studied TiO2/BFO/WO3 device in the form of Core-Shell Ferroelectric Nanowire for Photovoltaic applications. These films exhibit an open-circuit voltage (Voc) and short-circuit current density (Jsc) of 1.32 V and 1.14 mA/cm2, respectively, when the scanning bias is 1.75 V with photoelectric conversion efficiency of 0.93%. Wu et al. [17] have explored ferroelectric crystals of KTa1−xNbxO3 (KTN) for ultraviolet photodetection. Head to Head domain structure was achieved by tuning the Curie temperature (Tc) producing charged domains hence conducting channel for electrons and holes. This resulted into giant photoresponsivity and high electro-optic coefficient. This material find potential application in all optical devices and photonic chips.

Jadhav et al. [18] studied the role of Hf doped BaTiO3 nanoparticles on electro-optic and dielectric properties of ferroelectric liquid crystal. It was observed that these nanoparticles significantly modifies various properties of KCFLC 7S liquid crystal such as photoluminescence and spontaneous polarization resulting in application of these crystals in liquid crystal displays.

Pal et al. [19] investigated light controlled modification in polarization in free-standing BaTiO3 membranes. These membranes exhibit optical controlled polarization switching which is 1200 times faster than their clamped counterparts. Information and Communication Technology are largely benefitted by these materials resulting in wireless sensing. Resistance of these membranes changes as a result of polarization reversal caused by illumination.

Liu et al. [20] have studied the application of ferroelectric material Relaxor-lead titanate (PbTiO3) crystals for electro-optic applications. Generally optical transparency of ferroelectric materials is very low due to scattering and reflection of light by domain walls, limiting its electro-optic applications. Through synergistic design of a ferroelectric phase, crystal orientation, and poling technique, light scattering by domains was significantly reduced. High transmittance of 99.6% in antireflection film-coated crystals with an ultrahigh electro-optic coefficient of 900 pm/V was obtained, which is almost 30% of conventionally used crystals.

Han et al. [21] utilized property of ferroelectric polarization in photocatalytic degradation of nitrogen containing heterocyclic compound quinoline which act as pollutant in cooking waste water. BaTiO3/CuFe2O4 is used in Photocatalytic system. Polarized BaTiO3 resulted in directional flow of photogenerated electrons and enhanced adsorption and activation due to polarized surface sites. This resulted in 99.65% removal of quinoline from waste water.

2.5 FERROELECTRIC MATERIALS IN WEARABLE ELECTRONICS AND BIOMEDICAL APPLICATION

Ito et al. [22] have tested the use of soft functional polymers in stretchable devices used in wearable tactile electronics. Blend ratio of copolymer PMMA-PBA has been considered with a ferroelectric polymer PVDF-TrFE). By making a capacitor like device, these films can stretch upto 400%. Ren et al. [23] have developed flexible piezoelectric nano-generator (PENG) devices based on PZT resulted in wearable electronic devices which can convert biomechanical energy to useful electrical energy. PENG device was prepared from free standing orientation controlled, single-crystalline Lead Zirconate Titanate membranes supported by water soluble polydimethylsiloxane (PDMS) polymer layer. PZT membranes designed this way exhibit better properties than there clamped version.  Fig. 1 illustrates the use of this PENG device in wearable electronics application.  These nano-generators exhibit very high output density, strain tolerance factor of more than 3.4 % and superior mechanical stability in cycling tests more than 60000.

Fig. 1(a) Measured output voltage of the PENG device in forward connection (top) and reverse connection (bottom). (b) Variation of output voltage with strain ratio. (c) Output electrical signal test by attaching the PENG to different body parts (from top to bottom: finger, wrist, elbow), demonstrating its versatility and performance. (d) The durability of the flexible PZT energy harvester is shown over 60,000 bending cycles, highlighting its mechanical stability. (e) A photograph demonstrating the ability of the device to power commercial LEDs to display the letters “HIT” using the electrical energy generated from bending the human hand. (Reprinted from [23] with the permission from Nature Communications

Das et al. [24] have developed Piezoelectric Nano Generator (PENG) devices based on MWCNTs/BSTO/PVDF system. These nano-composites can generate energy from waste heat and human moments. These resulted in sustainable, renewable and ecofriendly sources of Energy. It was observed that these devices give power output of 31.5μW and output current of 9μA, which is enough to power a small electronic device.

Zhang et al. [25] have designed Potassium Sodium Niobate based miniaturized ultrasonic devices having variable frequency for neuromodulation of brain i.e. for biomedical applications due to its high piezoelectric coefficient (~680 pC/N). These hetrojunction resulted in adjustable focus based ultrasonic device. Even these perform better than traditionally available PZT based hetrojunctions.

Becker et al. [26] demonstrated ferroelectric material for microelectrode in bioelectronics and bipolar impedance switching. This behavior was studied in 18 nm thick BaTiO3 film in an electrolyte–ferroelectric–semiconductor (EFS) configuration. Cyclic voltammetry measurements in EFS configuration, with a phosphate-buffered saline solution acting as the liquid electrolyte top contact, indicate characteristic ferroelectric switching peaks in the bipolar current–voltage loop. Further, maximum ratio of two different impedance magnitudes of ~1.5 was observed at frequency of 100 Hz.

Shi et al. [27] have given revolutionized idea of fabricating dense ferroelectric metamaterials using latest technology just as 3D printing resulting into ceramics with superior properties then their bulk counterparts and naturally occurring or chemically obtained materials. With the latest technology it is possible to engineer grain size, grain growth and domains. Truss based metamaterials can be ultralight and ultrastiff. These modified metamaterials can be used in non-contact sensors, wearable electronics etc.

2.6 FERROELECTRIC MATERIALS IN SOME OTHER IMPORTANT APPLICATIONS

Spandana et al. [28] have examined PLZT ceramics for ferroelectric cathode applications. PLZT was prepared by a unique method of uniaxial pressing followed by high energy, mechanical activation method. This result in high current density, high dielectric constant (~5195) and low piezoelectric coefficient so it can be a potential candidate for cathode applications.

Ma et al. [29] have studied room temperature ferroelectric modulated tunneling and negative differential resistance behavior in BaTiO3/SrRuO3/BaTiO3 Quantum-Well Structures. The resonant current amplitude and voltage are tunable by the switchable polarization of the BaTiO3 ferroelectric with the NDR ratio modulated by ≈3 orders of magnitude and an OFF/ON resistance ratio exceeding a factor of 2 × 104.

Zakeri et al. [30] have tested Al-Ce co-doped BaTiO3 for application as electrode in electrochemical energy conversion and storage devices. This material possess excellent electrocatalytic property due to its high conductivity and inherent oxygen vacancies. Since, fossil fuels cause excessive environmental issues, so there is an urgent need to shift to the supercapacitors and water splitting cells. These can convert renewable sources into electricity. Al-Ce co-doped BaTiO3 nanofiber can act like effective electrode for abundantly present water via oxygen evolution reaction (OER) and hydrogen evolution reaction (HER) indicated by their low onset potential of 11 mV and -174 mV vs. RHE respectively and over potentials of -894  mV and -570 mV respectively. This electrode exhibit capacitance of 224.18F/g at a scan rate of 10mV/s.

Park et al. [31] explored ferroelectric material PMN-PT for thermal switch application. Their behavior is tested at high temperature and low frequency poling resulting in reduced EC. These materials shows 25 % modulation at low electric field.

3. Conclusion

Though Scientists are working hard to discover new ferroelectric materials, on various method of preparation of ferroelectric materials from several decades. But the factor which is more prominent is application of these materials in daily life. In addition to traditional applications, Scientist are continuously working on latest applications of these materials. Some of the latest applications are in energy storage, tunnel field effect transistor (TFET), memristor, infrared detector, photodetector, neuromorphic computing, stretchable devices, microelectrode etc.

Reference

# [1] Yuan Zi-lin, Sun Yu, Wang Dan, Chen Ke-Qiu and Tang Li-Ming (2021). A review of ultra-thin ferroelectric films. Journal of Physics: Condensed Matter, 33 (40), 403003.

[2] Rabe Karin M., Dawber Matthew, Lichtensteiger Céline,  Ahn Charles H.,  Triscone Jean-Marc. Modern Physics of Ferroelectrics: Essential Background, 105, 1-30.

[3] Y. Xu, Ferroelectric Materials and Their Applications, 1st Edition - November 15, 1991.

[4] Zhang Lei, Pu Yongping, Peng Xin, Wang Bo, Shang Jing (2023). Design strategies of perovskite energy-storage dielectrics for next-generation capacitors. Journal of European Ceramic Society, 43 (14), 5713-5747.

[5] Liu Yang, Zhou Yao, Qin Hancheng, Yang Tiannan, Chen Xin, Li Li, Han Zhubing, Wang Ke, Zhang Bing, Lu Wenchang, Chen Long-Qing, Bernholc J., Wang Qing (2023). Electro-thermal actuation in percolative ferroelectric polymer nanocomposites. Nature Materials, 22, 873–879.

[6] Yang F., Yuan Z., Wu S. J., Chen J. Y.,  Hou M. Z., Liu A. D., Yu K. X., Zhang Y. H.,  Li X. F., Hu Y. C., Shang J., Yin S. Q., Wang X. W. (2023). Energy storage performance of PZT/PZ composite films obtained by sol-gel method. Physica Status Solidi A, 17, 220.

[7] Pappachan Seethal,  Kuruvila Krupa Maria, Duraisamy Dhayanithi, Rashid A., Lu Teng,  Liu Yun,  Venkatesan Giridharan Nambi (2022). Enhancing the relaxor character in K0.5Bi0.5TiO3 based ternary system for energy storage applications. Journal of Applied Physics, 132, 214105.

[8] Zhu Zhongyunshen, Persson Anton E. O., Wernersson Lars-Erik. (2023). Reconfigurable signal modulation in a ferroelectric tunnel field-effect transistor. Nature Communications, 14, 2530.

[9] Chen Pei-Yao, He Zheng-Yu, Ming-Yang.Cha, Liu Hao, Zhu Hao, Chen Lin,  Sun Qing-Qing,  Ding Shi-Jin,  Zhang David Wei (2021). Hafnium Oxide-Based Ferroelectric Devices for Computing-in-Memory Applications. Physica Status Solidi, 218 (9), 2000635.

[10] Liu X., Wang D., Kim K.H., Katti K., Zheng J., Musavigharavi P., Miao J., Stach E. A., Olsson R. H., Jariwala D. (2021) Post-CMOS Compatible Aluminum Scandium Nitride/2D Channel Ferroelectric Field-Effect-Transistor Memory. Nano Lett., 21, 3753–3761.

[11] Li Qingxuan, Wang Siwei, Li Zhenhai, Hu Xuemeng, Liu Yongkai, Yu Jiajie, Yang Yafen, Wang Tianyu, Meng Jialin, Sun Qingqing, Zhang David Wei, Chen Lin (2024). High-performance ferroelectric field-effect transistors with ultra-thin indium tin oxidechannels for flexible and transparent electronics. Nature Communications, 15, 2686. https://doi.org/10.1038/s41467-024-46878-5.

[12] Razumnaya Anna G, Tikhonov Yuri A,  Vinokur Valerii M, Lukyanchuk Igor A (2023). Ferroelectric topologically configurable multilevel logic unit. Neuromorph. Comput. Eng., 3 (2), 024003.

[13] Xue Fei, He Xin, Ma Yinchang, Zheng Dongxing, Zhang Chenhui,  Li Lain-Jong, He Jr-Hau, Yu Bin, Zhang Xixiang (2021). Unraveling the origin of ferroelectric resistance switching through the interfacial engineering of layered ferroelectric-metal junctions. Nature Communications, 7291.

[14] Wen Yiyang, Cao Yilin, Ren Hongda, Du Xiaona, Guo Jiaxing, Wu Zhenping, Liu Weiwei, Du Jiangbing, Zhang Yang (2025). Ferroelectric Optical Memristors Enabled by Non-Volatile Electro-Optic Effect. Advanced Materials. 37 (8), 2417658.

[15] Whatmore Roger W., Ward Samuel J. (2023). Pyroelectric infrared detectors and materials—A critical perspective. Journal of Applied Physics, 133, 080902.

[16] He Jizhuang, Li Jiahua, He Yunfei, Ren Yin, Li Sisi, Xing Shulin, Gao Rongli, Cai Wei, Fu Chunli. (2023). Core-Shell ferroelectric nanowire arrays for photovoltaic applications. Journal of Alloys and Compounds, 960, 170692.

[17] Wu Yabo, Wang Xuping, Tian Gang, Zheng Limei, Liang Fei, Zhang Shujun, Yu Haohai, Zhang Huaijin (2022). Inverse Design of Ferroelectric-Order in Perovskite Crystal for Self-Powered Ultraviole6t Photodetection. Advanced Materials, 34 (9), 2105108.

[18] Reshma Jadhav, Swapnil Doke, Onkar A. Ramdasi, Rahul C. Kambale, Santosh Mani, Raghavendra V. Reddy, R.R. Deshmukh, Prasun Ganguly, Shrikrishna D. Sartale (2025). Enhanced electro-optical properties offerroelectric liquid crystal incorporated with Hf-BaTiO3 nanoparticles. Journal of Molecular Liquids, 437 B, 128480.

[19] Pal Subhajit, Hsu Lan-Tien, Sun Haoying, Teng Sheng-Han, Dwij Vivek, Emanuele, Palladino, Nie Yuefeng, John Samuel, Prabhu S. S., Grünebohm Anna & Briscoe Joe (2025). Subsecond optically controlled domain switching in freestanding ferroelectric BaTiO3 membrane. Nature Communications, 16, 7940.

[20] Liu Xin, Tan Peng, Xue M, Wang Danyang, Jin Xinyu, Liu Yao, Xu Bin, Qiao Liao, Wang Chaorui Qiu Bo, Zhao Weigang, Wei Chaojie, Song Kexin, Chen Hai Qing, Xu Zhuo, Li Fei, Tian Hao, Zhang Shujun (2022). Ferroelectric crystals with giant electro-optic property enabling ultracompact Q-switches. Science, 376 (6591), 371-377.

[21] Han Rong, Huang Wenbao, Sun Bihan, Li Houfen, Li, Wang Sufang, Zhou Aijuan, Yue Xiuping, Gao Cong, Yu Hongtao, Zhao Xu (2025). Persulfate-assisted photocatalytic pollutantdegradation on ferroelectric BaTiO/CuFe2O4 material: Ferroelectric polarization enhancedelectron transfer and persulfate activation. Applied Catalysis B: Environment and Energy, 376 (5), 125452.

[22] Ito Keita, Sekine Tomohito, Shouji Yoshinori, Wang Yi-Fei, Takeda Yasunori, Kumaki Daisuke,  Santos Fabrice Domingues Dos, Tong Huang,  Miyabo Atsushi, Tokito Shizuo (2023). Optimization of the blend ratio of soft functional polymers for high piezoelectric performance stretchable device. Appl. Phys. Express, 16, 011004.

[23] Ren Zhongqi, Deng Shiqing, Shao Junda, Si Yangyang, Zhou Chao, Luo Jingjing, Wang Tao, Li Jinyang, Li Jingxuan, Liu Haipeng, Qi Xue, Wang Peike, Yin Ao, Wu Lijun, Yu Suzhu, Zhu Yimei, Chen Jun, Das Sujit, Wei Jun & Chen Zuhuang (2025). Ultrahigh-power-density flexible piezoelectric energy harvester based on freestanding ferroelectric oxide thin films. Nature Communications, 16, 3192.

[24] Das Tupan, Biswas Piyali, Dev Amar, Mallick Jyotirekha, Kar Manoranjan (2024). Multi-functional piezoelectric nanogenerator based on relaxor ferroelectric materials (BSTO) and conductive fillers (MWCNTs) for self-powered memristor and optoelectronic devices. Chemical Engineering Journal, 479 (1), 147900.

[25] Tao Zhang, Haoyuan Hu, Hong Jiang, Wang Zhen, Lin Jinfeng, Cheng Ye, Guo Wei, Hang Di Ke, Hai, Ta Mengshu, JunOu-Yang, Zhai Jiwei, Yang Xiaofei, Wang Songyun & Zhu Benpeng (2025). KNN-based frequency-adjustable ferroelectric heterojunction and biomedical applications. Nature Communications, 16, 7120.

[26] Becker Maximilian T., Oldroyd Poppy, Strkalj Nives, Müller Moritz L., Malliaras George G., MacManus-Driscoll Judith L. (2023). Impedance switching for neural recording with ferroelectric microelectrode. Appl. Phys. Lett. 122, 173701.

[27] Shi Jiahao, Ju Kang, Chen Haoyu, Orsat Valérie, Sasmito Agus P., Ahmadi Ali, Akbarzadeh Abdolhamid (2025). Ultrahigh Piezoelectricity in Truss-Based FerroelectricCeramics Metamaterials. Advanced Functional Materials, 35, 2417618.

[28] Spandana K., Reddy Ch. Gopal, Ravi Meduri, James A.R. (2023). Structural, electrical and electron emission property studies of high dielectric constant (Pb0.88La0.12)(Zr0.65Ti0.35)O3 ceramics for ferroelectric cathode applications. Ceramics International, 49 (17), 27939-27948.

[29] Ma Zhijun, Zhang Qi, Tao Lingling, Wang Yihao, Sando Daniel, Zhou Jinling, Guo Yizhong, Lord Michael, Zhou Peng, Ruan Yongqi, Wang Zhiwei, Hamilton Alex, Gruverman Alexei,  Tsymbal Evgeny Y., Zhang Tianjin, Valanoor Nagarajan (2022). A Room-Temperature Ferroelectric Resonant Tunneling Diode. Advanced Materials, 34 (35), 2205359.

[30] Zakeri Fatemeh, Javid Abbas, Orooji Yasin, Fazli Arezou, Khataee Amirreza & Alireza Khataee (2024). Al‑Ce co‑doped BaTiO3 nanofibers as a high‑performance bifunctional electrochemical supercapacitor and water‑splitting electrocatalyst. Nature, 14, 9833, https://doi.org/10.1038/s41598-024-54561-4.

[31] Park Gimin, Yoon Byoungjin, Lee Jeyeon , Kim Kichan, Song Hyun Seok, Kim Rokhyeon, Kim Hyun Soo, Lee Dong-Gyu, Hwang Junphil, Kim Jungwon , Kang Heemin, Lee Sang-Goo ,  Ryu Jungho ,  Yan Yongke , Baik Jeong Min,  Kim Tae Heon, Song  Hyun-Cheol ,  Hur Sunghoon (2025). Low-voltage driven ferroelectric thermal switch. Nano Energy, 145, 111410.