Advancements in Thermoelectric Technology: Development of a Portable Cooler and Heater System

ABSTRACT:

Emissions of greenhouse gases are increasing globally because of continuous increase in demand of electricity, use of vehicles, desertification and improper waste management, etc. The developed portable heater and cooler by thermoelectric effect consist of 4 thermoelectric peltier modules, heat sink with 4 fans, 2 batteries, charge controller and solar panel. The 4 peltier modules each of them having capacity of 50 watt are connected in series and parallel connection on the cover of thermocol box. For better effective heating and cooling in entire work area all the modules are attached with heat sink and fan having 12 V and 0.28 A . For effective cooling and heating we have made a sandwich of peltier module with heat sink with the help of thermal paste so when the power is supplied to the peltier modules the one side peltier module become hot and another side become cold. These sandwich connections are placed on the top of the thermocol box. The developed portable cooler and heater was tested with different load condition includes zero loads, 1 liter of water and 2 liter of water with battery in cooling as well as heating. The fully charged batteries can run the whole experimental setup for 21 hours in each of case i.e., cooling and heating. The developed portable cooler and heater by thermoelectric effect has successfully achieved the lowest temperature of working area (thermocol box) is 18.8 °C during cooling for no load condition and lowest temperature of water is 18.6 °C during cooling for 1 liter of water. The experimental setup achieved the highest temperature of working area (thermocol box) is 42.4°C and highest temperature of water is 33.3 °C during heating for 1 liter of water.

Keywords: Thermoelectric effect, Peltier effect, Peltier module, Heat sink, solar photovoltaic system, battery, Voltage stabilizer.

INTRODUCTION:

The term of refrigeration may be defined as the process of removing heat from substance under controlled condition. Refrigeration is also described as the scientific field concerned with lowering and sustaining the temperature of an area or substance below that of its surroundings. 
Recently, there has been a resurgence of interest in thermometric heating or cooling technology because of its unique characteristics when compared to traditional methods like vapor-compression and electric heating or cooling systems. Thermoelectric (TE) Modules are solid-state devices that act as heat pumps (used for both cooling and heating purposes) and operate based on the Peltier Effect occurring between the junctions of two semiconductors. 
In 1821, a significant breakthrough in thermoelectricity was made by German scientist Thomas Seebeck. He observed that when two dissimilar metals were joined to form a closed circuit and the junctions of the metals were kept at different temperatures, an electric current would flow continuously in the circuit. In 1821, a significant breakthrough in thermoelectricity was made by German scientist Thomas Seebeck. He observed that when two dissimilar metals were joined to form a closed circuit and the junctions of the metals were kept at different temperatures, an electric current would flow continuously in the circuit.

Principles of Thermoelectric Effect:

The thermoelectric effect can be described as the conversion of energy between electricity and heat. These effects are found to be thermodynamically reversible and act in conjunction with irreversible effects such as Joule heating and thermal conduction. 
The thermoelectric effect refers to the direct transformation of temperature differentials into electric voltage and conversely through a thermocouple. Thermoelectric gadgets generate voltage when there exists a temperature contrast between their sides. On the flip side, applying a voltage to it results in the transfer of heat from one side to another, leading to the formation of a temperature disparity. On an atomic scale, the application of a temperature gradient causes charge carriers within the material to move from the warmer side to the cooler side through diffusion.

There are three types of thermoelectric effects that occur:

1. The See beck Effect

2. The Peltier Effect

3. The Thomson Effect.

Peltier Effect

Conversely, the Peltier Effect is mainly associated with thermoelectric systems used for cooling or heating purposes.  It functions by transferring heat between two unlike conductors or semiconductors as an electric current flow through a closed circuit loop containing these materials. In this phenomenon, heat is taken in at one junction and conveyed to the opposite junction, leading to either heating or cooling depending on the direction of the electric current flow.

When comparing the two effects:

The Thomson Effect involves the generation of a voltage difference due to a temperature gradient, while the Peltier Effect involves the transfer of heat due to an electric current passing through a junction.

- Joule heating, which is quadratic with respect to current, is associated with irreversible heat generation in a conductor, unlike the Peltier heat, which is linearly related to current.

- The Seebeck Effect, which converts heat flow into an electric current, contrasts with the Peltier Effect, where an electric current induces a heating or cooling effect depending on the direction of current flow.

One significant application of the Peltier Effect is in the creation of compact refrigeration systems that do not require circulating fluids or moving parts. These thermoelectric refrigerators leverage the Peltier Effect to achieve cooling effects by passing currents through suitable thermoelectric materials in a closed-loop configuration.

Figure 1 Peltier Effect

MATERIAL AND METHODS

In this project the thermoelectric refrigeration concept using solar panel. Solar panel system is based on the principle of solar cell. Solar cell principle is that in which solar cell consist of the utilization of the photovoltaic (PV) cells. When light is incident on the solar panel, then electron-hole pair is generated. The quantity of electron-hole pairs formed is directly related to the brightness or intensity of light. Subsequently, electrons migrate towards the N-type semiconductor while holes move towards the P-type semiconductor. Both the electrons and holes are gathered at the electrodes, leading to the development of a potential difference between them. When a wire is connected to the electrode, current flows between them, and this current is gathered by the battery as electric charge. This charge is provided to the thermoelectric refrigerator continuously. 
In experimental settings, the efficiency of solar cells used in thermoelectric refrigeration is greatly influenced by factors such as the intensity of solar exposure and the temperature differential between the hot and cold sides of the thermoelectric module. The examined refrigerator is capable of sustaining temperatures within the refrigerated space ranging from 14 to 39 degrees Celsius, with a coefficient of performance (COP) of approximately 0.3 under specific operating conditions.

Cooling load calculations

Cooling load = Refrigeration Effect = m × Cp × ΔTt

Where, m= mass, Kg

Cp= specific heat of fluid, kJ/kg-K

ΔT= temperature difference, K

t = time required in sec

This formula gives cooling load and then it is possible to select number of Peltier Modules.

Selection of Number of Peltier Modules:-

- Mass of water in kg,

- Cp for Water kJ/kg-K,

- Temperature Difference in K,

- Time in second

Number of Modules = Refrigeration Effect (W)Capacity of one Peltier Module (W)

For better effect of heating and cooling in this system select 4 Peltier Modules for design of portable cooler and heater by thermoelectric effect for safer side.

Components and Description:-

- Peltier Module

- Thermocol Box

- Cooling Fan

- Heat Sink

- Solar Panel

- Charge Controller

- Battery

- Thermal Paste

RESULTS AND DISCUSSION:

The chapter discusses the outcomes of evaluating the performance of a portable cooler and heater using the thermoelectric effect. We conducted a total of 11 experiments from 04/01/2020 to 17/01/2020. Cooling and heating experiments were performed by supplying the necessary electricity/power using a stabilizer (from the grid) and a battery (from solar power without the grid)

Table 1: Experimental results with Stabilizer (On grid supply)

|  | Cooling |  |  |  |  |  |  |  | Heating |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Zero load |  | 1 ltr of water |  |  | 2 ltr of water |  |  | 1 ltr of water |  |  | 2 ltr of water |  |  |
|  | T2 (ºC) | T3 (ºC) | T1 (ºC) | T2 (ºC) | T3 (ºC) | T1 (ºC) | T2 (ºC) | T3 (ºC) | T1 (ºC) | T2 (ºC) | T3 (ºC) | T1 (ºC) | T2 (ºC) | T3 (ºC) |
| Initial Temperature | 27.9 | 25.8 | 34.5 | 25.4 | 22 | 30.2 | 27.5 | 28.2 | 14.9 | 16.6 | 16.2 | 14.5 | 16.5 | 17.8 |
| Final Temperature | 19.6 | 23.5 | 21.4 | 23.9 | 22.8 | 22.3 | 24.5 | 24.7 | 32.5 | 40.3 | 18.7 | 31.2 | 39 | 19.2 |

Table 2: Experimental results with Battery (Solar power)

|  | Cooling |  |  |  |  |  |  |  | Heating |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Zero load |  | 1 ltr of water |  |  | 2 ltr of water |  |  | Zero load |  | 1 ltr of water |  |  | 2 ltr of water |  |  |
|  | T2 (ºC) | T3 (ºC) | T1 (ºC) | T2 (ºC) | T3 (ºC) | T1 (ºC) | T2 (ºC) | T3 (ºC) | T2 (ºC) | T3 (ºC) | T1 (ºC) | T2 (ºC) | T3 (ºC) | T1 (ºC) | T2 (ºC) | T3 (ºC) |
| Initial Temperature | 14.3 | 16.2 | 24.8 | 23.1 | 21.3 | 28 | 21.7 | 19.3 | 17 | 20.1 | 15.4 | 15.9 | 14.9 | 16.2 | 17.7 | 18.1 |
| Final Temperature | 18.8 | 20.5 | 18.6 | 19.9 | 21.4 | 17.7 | 18.9 | 20.7 | 39.1 | 28.6 | 25.1 | 29.3 | 20.9 | 19.3 | 22 | 20.6 |

Here,   T1 = Temperature of water (ºC)

T2 = Temperature inside thermocol box (ºC)

T3 = Temperature of atmosphere (ºC)

As depicted in both tables above, a total of 11 experiments were carried out involving heating and cooling operations using a stabilizer (grid supply) and a battery (solar power).

During Stabilizer operation for cooling experiment for zero load, 1 liter of water and 2 liter of water minimum temperature with stabilizer operation was 19.6 ºC, 23.5 ºC and 21.4 ºC respectively against the atmospheric temperature of 23.5 ºC, 22.8 ºC, 24.7 ºC respectively and for heating experiment for 1 liter of water and 2 liter of water maximum temperature was 32.5 ºC and 31.2 ºC respectively against the atmospheric temperature of 18.7 ºC and 19.2 ºC respectively.

During the cooling experiment with the battery and zero load, the minimum temperatures reached for 1 liter of water and 2 liters of water were 18.8 ºC and 18.6 ºC, respectively, compared to the atmospheric temperatures of 20.5 ºC and 21.4 ºC. For the heating experiment with the battery and zero load, the maximum temperatures recorded for 1 liter of water and 2 liters of water were 39.1 ºC and 25.1 ºC, respectively, against the atmospheric temperatures of 28.6 ºC and 20.9 ºC.

CONCLUSIONS:

- If the project receives power from the grid via a voltage stabilizer, the average cooling and heating values are 9.76 ºC and 17.15 ºC, respectively.

- If the project utilizes power from solar PV technology, the average cooling and heating values are 4.06 ºC and 11.53 ºC, respectively.

- The battery has the capacity to operate the entire setup for around 21 hours, contingent upon atmospheric conditions.

REFERENCES:

- Mayank Awasthi, K V Mali, “Design and development of thermoelectric refrigerator”, International journal of mechanical engineering and robotics research, Int. J. Mech. Eng. & Rob.Res. 2012.

- Manoj Kumar Rawat, Lal Gopal Das, Himadri Chattopadhyay, SubhasisNeogi, “An experimental investigation on thermoelectric refrigeration system: A potential green refrigeration technology”, journal of environment research and development”, vol.6 NO.4, april-june 2012.

- Umesh V. Sangale, Prof. Priyanka Jhavar, Dr. G.R Soleskar S, “Thermoelectric refrigeration using solar energy for domestic appliance”, International journal of research in Advent technology,Vol.3, NO.1,January 2015,E-ISSN:2321-9637.

- Sagar D. Patil, Kiran D. Devade, “Review on thermoelectric refrigeration: Application and technology”, Scientific journal impact factor (SJIF):1.711.

- Jaspal Sinh. B.Dabhi, Nimesh.B. Parmar, Dr, Nirvesh S. Mehta, “Consideration for design of thermoelectric refrigeration system”, International journal on advanced engineering research and studies”, E-ISSN: 2249-8974.

- Robert A. Taylor, Dr. Gary Solbrekken, Thesis Supervisor, “Comprehensive Optimization for Thermoelectric Refrigeration Devices”.

- Sagar D. Patil, Kiran D. Devade, “Review on thermoelectric refrigeration: Application and technology”, Scientific journal inpact factor (SJIF):1.711.

- Umesh V. Sangale, Prof. Priyanka Jhavar, Dr. G.R Soleskar S, “Thermoelectric refrigeration using solar energy for domestic appliance”, International journal of research in Advent technology,Vol.3, NO.1,January 2015,E-ISSN:2321-9637.

- Y. Dai. R. Wang and L. Ni, "Experimental investigation on a thermoelectric refrigerator driven by solar cells”, Renewable Energy, vol. 28, no. 6, pp. 949-959, May 2003.