Review Article

Spatial Design Responses to Urban Drought: Climate-Adaptive Xeriscape Strategies in Istanbul

Abstract

This study examines the applicability of xeriscape-based approaches in enhancing Istanbul’s climate resilience under the increasing pressures of global climate change—rising temperatures, irregular precipitation, and declining freshwater resources. Rapid urbanization, unequal green space distribution, and the intensifying Urban Heat Island (UHI) effect have weakened the city’s ecological stability. The research demonstrates that xeriscape strategies integrating sustainable water management, species diversity, and spatial connectivity provide effective solutions to climate-induced stresses. Within this framework, a holistic model is proposed encompassing adaptive planting schemes, integrated water systems, multi-level governance, and community awareness programs. Ultimately, the study presents a practical roadmap for strengthening Istanbul’s ecological resilience through xeriscape-based planning while offering scalable and transferable strategies for other climate-sensitive urban contexts.

Keywords: Xeriscape, urban green infrastructure, carbon sequestration, water management, climate-resilient landscape, Istanbul

1.Introduction

Anthropogenic activities since the Industrial Revolution, including unplanned urbanization, uncontrolled resource use, and large-scale environmental degradation, have caused unprecedented greenhouse gas accumulation, disrupting the global climate system and threatening planetary stability (Hilty et al., 2006). Climate change has emerged as a multidimensional crisis with profound impacts on ecosystems, human health, and socio-economic stability (Kumar et al., 2025). Unlike past climatic fluctuations, it jeopardizes both natural systems and the foundations of human civilization. The intensifying crisis manifests through disrupted hydrological cycles, atmospheric systems, and ecological balances. Increasing droughts and floods illustrate its paradoxical nature: severe droughts deplete freshwater resources, while extreme precipitation events threaten urban infrastructure and agriculture (Çınar and Ay Ak, 2024; Çınar, 2022). Consequently, water conservation and sustainable resource management have become essential prerequisites for societal well-being.

By 2050, the global urbanization rate is expected to reach between 66% and 75% (EUROSTAT, 2025; Peroni & Pappalardo, 2024). This rapid urban expansion exacerbates the very drivers of climate change. The year 2024, recorded as the warmest year in human history, marks a critical threshold, highlighting the deepening of the crisis and the growing risks ahead. From the perspective of landscape architecture, climate change directly affects not only natural systems but also urban landscapes, ecological infrastructures, water resources, and the overall quality of urban life. Areas with weak green infrastructure tend to provide unhealthy and inequitable environments, whereas green infrastructure–rich areas demonstrate higher adaptive capacity to climate change, promoting physical activity, cardiovascular health, psychological well-being, and cognitive performance (Mullenbach & Stanis, 2024). In the coming decades, transformations in land use within urban centers will likely intensify. Preventing further biodiversity loss in expanding metropolitan regions requires the integration of scientifically derived strategies at the earliest stages of urban and regional planning (Gordon, 2009). Thus, the development and implementation of climate-adaptive landscape policies have become an existential necessity. In recent years, sustainability-oriented practices in Turkey have gained traction. Among them, xeriscape (drought-tolerant) landscape approaches have emerged as a promising framework for water-efficient landscape planning and management. While several pilot projects have been initiated, these efforts remain fragmented and lack a coherent, system-wide strategy. The limited understanding of xeriscape principles, coupled with the absence of coordination among local governments, public institutions, and the private sector, continues to hinder the widespread adoption of this sustainable approach.

2. Material

The primary material of this research is Istanbul, Turkey’s most densely populated and ecologically complex metropolitan area. Covering approximately 5,343 km², Istanbul had a population of 16 million as of 2023 (TÜİK, 2023), (Figure 1).  The city, bordered by the Marmara Sea to the south and the Black Sea to the north, is characterized by a highly diverse topography consisting of plateaus, hills, and valleys. This geomorphological complexity, coupled with rapid urban expansion, poses significant challenges for sustainable landscape management under changing climatic conditions (Çınar and Ay Ak, 2024). The determination of Istanbul’s natural and ecological characteristics was based on multiple official datasets and scientific sources. Soil properties and morphology were obtained from the Ministry of Agriculture and Forestry’s Soil Information System (2023); hydrological data were derived from the State Hydraulic Works (DSİ) 14th Regional Directorate’s Istanbul Water Basins Report (2020); geological data were taken from the General Directorate of Mineral Research and Exploration (MRE) 1:100,000 Geological Map Series of Istanbul (2002); and climatic records from 1950 to 2024 were provided by the Turkish State Meteorological Service (2025). The administrative boundaries and planning regions of Istanbul were analyzed based on the Metropolitan Municipality’s 1/25,000 Master Plans and Strategic Planning Documents (2020–2024). Although Istanbul exhibits characteristics of a Mediterranean climate, its climatic conditions are strongly influenced by the Marmara Sea and the Bosphorus Strait, leading to considerable local variability. In winter, the city is affected by cold–dry air masses from the Black Sea and cold–wet airflows from the Balkans, while in other seasons, it is influenced by the warm and humid southern air masses from the Mediterranean.

Figure 1.  Location of Turkey and Istanbul

The plant material selection for this study considered Istanbul’s humid semi-continental Marmara climate, urban heat island effect, biodiversity objectives, water conservation principles, and climate resilience criteria. Plant species were evaluated based on their origin, adaptability, deciduous or evergreen habit, landscape value, water requirement, and compatibility with Istanbul’s natural flora. Species identification and classification were informed by the following key botanical references: (1965); Davis et al. (1988); Williams (2013); Ekren (2014); Çorbacı and Özyavuz (2024); Çınar and Ay Ak (2024); Çınar and Aktaş (2020); Çınar and Güzel (2020) and Yaltırık et al. (1997). To ensure practical applicability, availability of plant species in both public and private nurseries was verified through local data obtained from Florapark Landscape, Güneş Landscape, Istanbul Regional Forestry Directorate Nurseries, İBB Nursey A.Ş., and municipal nurseries in Çekmeköy, Tuzla, and Eyüp. This research aims to evaluate the applicability of xeriscape (drought-tolerant) landscape design principles for sustainable urban green space management in Istanbul- a metropolis under significant pressure from climate change, water scarcity, and urbanization. A GIS-based integrative approach combining spatial analysis, climate projection data, and municipal planning frameworks was employed to develop a holistic xeriscape planning strategy. Istanbul, as Turkey’s largest and most economically significant city, faces growing urban heat island intensity due to continuous land-use transformation and urban densification, resulting in pronounced temperature differences between urban and rural areas (Zaeemdar and Baycan, 2017).

3. Methodology

In this study, it was tried to evaluate and develop climate-compatible xeriscape design strategies for Istanbul. The methodology combines climate projections, hydrogeological features, land surface temperature data, and urban green infrastructure measurements to identify areas with high potential for drought-resilient landscaping practices.

3.1 Data Sources The primary datasets used in the study are as follows:

Climate Projections: Downscaled climate data based on the RCP 8.5 emission scenario were used to analyze long-term temperature and precipitation trends. Data were obtained from the Turkish State Meteorological Service (2025), providing spatially explicit insights into future climatic stressors relevant to urban vegetation and water management.

Water Resource Data: Hydrogeological and water consumption data, including groundwater recharge zones, aquifer boundaries, and municipal water demand, were supplied by the Istanbul Water and Sewerage Administration (2021). These data supported the spatial analysis of areas facing increasing water scarcity risk.

Urban Heat Island (UHI) Data: Surface temperature data derived from Landsat and MODIS satellites (Özbilge, 2020) were processed to map urban heat intensity zones across Istanbul. The UHI data were used to determine spatial overlaps between thermal stress and vegetation patterns.

Urban Green Space Data: District-level per capita green space availability (m²/person) was derived from the YAYSİS (2025) dataset. These data provided a baseline for evaluating green infrastructure distribution and accessibility across the city.

Plant Species Inventory: An inventory of plant species used in Istanbul’s urban green spaces was compiled. Species were classified according to their water requirements (low, low–medium, medium, medium–high) and origin (native or exotic). Taxonomic information was verified based on Flora of Turkey and the East Aegean Islands, Davis 1965; Davis et al., 1988; Güner et al., 2000; Yaltırık et al., 1997).

Policy and Planning Documents: Strategic and operational plans published by the Istanbul Metropolitan Municipality (İBB), including the Urban Green Infrastructure Report and related implementation projects, were reviewed. These documents informed the integration of xeriscape principles into municipal planning frameworks.

3.2 Analytical Framework The analytical workflow was structured around seven xeriscape design principles (Figure 2), adapted to Istanbul’s climatic and urban context:

Xerisape desıgn  principles

Figure 2. Xeriscape principles

3.3 Evaluation Process

This process ensured the comparability of datasets derived from different sources and temporal scales. The evaluation framework incorporated Istanbul’s RCP 8.5 climate scenario, hydrogeological parameters, and urban heat island (UHI) intensity zones, allowing the identification of critical areas where climate-adaptive landscape strategies are most needed. In addition, a comprehensive plant species database was developed, distinguishing native and exotic taxa according to their water demand levels (low, low–medium, medium, and high).The synthesis of these spatial and ecological datasets provided a holistic understanding of xeriscape applicability across the city. The findings suggest that well-implemented xeriscape design can reduce irrigation water demand by approximately 40–60%, consistent with international benchmarks for drought-resilient urban landscape management.

4. Research Findings

4.1. Istanbul's Climate and Spatial Characteristics

Istanbul is a complex urban ecosystem that is highly sensitive to the effects of climate change (Figure 2). According to RCP 8.5 scenarios, annual average temperatures in the city are expected to increase by 1–4.5 °C, with 1–2 °C of this increase predicted to be due to the urban heat island (UHI) effect (İBB, 2021). The same scenarios indicate that summer rainfall will decrease by up to 30% and total annual rainfall by up to 12%. Between 1950 and 2024, the annual average temperature ranged between 12–19.4 °C, with maximum temperatures reaching 40.5 °C (General Directorate of Meteorology, 2025).

It was determined that the districts of Esenyurt, Avcılar, Küçükçekmece, Bağcılar, Esenler, Bayrampaşa, Gaziosmanpaşa, Sultangazi, Beşiktaş, Ataşehir, Ümraniye, and Sancaktepe have the highest seasonal UHI values. In these areas, dense construction, an increase in the proportion of impervious surfaces, and insufficient vegetation cover are the main factors contributing to microclimatic imbalances. There are 252,747 ha of open-green areas and 116,388 ha of settlement areas in Istanbul (European Environment Agency, 2019). However, mega projects and rapid urbanization lead to the concretization of fertile agricultural lands, reducing both food security and carbon sink capacity (Istanbul Metropolitan Municipality, 2023). The spatial continuity of green areas has decreased due to the disruption of ecological network connections; this situation threatens the continuity of ecosystem services.

0

4.2. Drought-Resistant Landscape Inventory

Existing green space inventories are insufficient in terms of species classification, health status, and monitoring water requirements (Özdemir & Özkaynak, 2023). This deficiency makes it difficult to evaluate plant performance under climatic stress conditions (Başaran & Eroğlu, 2023).  While GIS-based green infrastructure information systems (e.g., Cloud City Information System, Urban Green Infrastructure System) provide significant digital transformation, the system's effectiveness depends on the accuracy of field data and inter-agency coordination.

Sustainable Water Management Significant stress is observed in Istanbul in terms of water management. Groundwater levels are decreasing by 0.5–1.2 m per year, especially in the districts of Başakşehir, Esenyurt, and Sancaktepe (İSKİ, 2025). The SLowlıdere Dam can only meet the city's water needs for 15 days. Therefore, water supply from the Melen Basin has become essential. In line with the 2030 and 2050 targets, water usage is expected to decrease by 18% and 32%, respectively, and the efficient use of water is encouraged through drip and sprinkler irrigation systems (Atila and Küçük, 2001). While the Ömerli–Elmalı basin on the Anatolian side has high groundwater potential thanks to its permeable formations, the Alibeyköy, SLowlıdere, and Terkos areas on the European side are at risk due to urbanization pressure (Chamber of Environmental Engineers, 2025).

Soil Analysis and Organic Matter Management

Soil analyses show that organic matter levels are generally low in urban green spaces. The permeability of sandy soils and the high water retention capacity of clay soils negatively affect plant growth (Çorbacı & Özyavuz, 2024). Therefore, it is recommended to prefer deep-rooted and low-water-requirement local species, mulching, compost use, and controlled fertilization practices. These methods both increase water efficiency by conserving soil moisture and enhance carbon sequestration capacity.

Climate-Focused Plant Design and Selection

The drought-tolerant landscaping approach stands out as a fundamental strategy that supports ecosystem resilience in conditions of water scarcity and rising temperatures. There are approximately 2,500 flowering plant and fern species in Istanbul's natural flora (Avcı, 2014), and the city's location at the intersection of three different climate zones (Mediterranean, Black Sea, Central Anatolia) increases its floristic diversity. This study identified 152 plant species with medium to low water consumption and high drought tolerance; 126 of these are native species and 26 are exotic species. Plant selection took into account ecological adaptation, aesthetic integrity, and maintenance costs. Biodiversity in cities is further enriched by exotic species added to the flora over time and the naturalization of some species     (Razzaghmanesh et al., 2014; Farrell et al., 2012; Hunter, 2011; Kendal et al., 2014; Kavuran and Yılmaz, 2022). In area-scale analyses, it has been observed that drought-tolerant landscaping practices reduce surface temperature by up to 3.5 °C in areas of 50–100 m² (Barış, 2014; Wade et al., 2010). These effects are particularly important for improving the urban microclimate in high-density settlement areas. Across Istanbul, the amount of green space per person is less than 1 m² in 36 districts, excluding Adalar, Çatalca, and Şile; this is well below the international standard of 10 m² (Istanbul Metropolitan Municipality Parks and Gardens Department, 2023). The findings reveal that drought-tolerant landscape strategies are essential for planning climate-adaptive green infrastructure in Istanbul. The joint consideration of local and exotic species will contribute to the design of landscapes that are ecologically and aesthetically balanced and highly water-efficient. This will support both the conservation of biodiversity and the sustainable management of water resources throughout the city.

In terms of drought resistance, the vast majority of the species analyzed are at a high tolerance level (Figure 3).  Especially coniferous trees, shrubs and perennial plants consist of species that can grow under water constraint conditions. The "high drought tolerance" rate in these groups is over 80%. On the other hand, the proportion of moderately drought-resistant species among leafy trees is relatively higher; This can be explained by the physiological adaptations of deciduous species that are compatible with the seasonal humidity cycle. The plant composition determined in the study offers low water demand and high ecological resilience together and sets an example in terms of climate-resilient landscaping strategies. This presents a potential palette of plants for sustainable green infrastructure practices, particularly in semi-arid regions.

Figure 3.  Species analyzed for natural,excotic,water cravings  and drought resistance

Table 1. Plants identified within the scope of the study and their characteristics (Davis, 1965; Davis ve ark.,1988; Williams, 2013; Ekren, 2014; Akkemik, 2018; Çorbacı and Özyavuz 2024; Çınar and Ay Ak,2024; Yener,2020; Çınar and Aktaş, 2019; Çınar and Güzel, 2020; Yaltırık vd.,1997).

| CONIFEROUS TREES |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No | Latin Name | Family | Natural-Exotic | Water cravings | Drought |
| 1 | Cedrus libani A. Rich. | Pinaceae | Natural | Low | High |
| 2 | Cupressus sempervirens L. | Cupressaceae | Natural | Moderate | High |
| 3 | Juniperus excelsa M. Bieb sub excelsa | Cupressaceae | Natural | Low | High |
| 4 | Juniperus sabina L. | Cupressaceae | Natural | Low | High |
| 5 | Juniperus oxycedrus L. | Cupressaceae | Natural | Low | High |
| 6 | Juniperus Communis L. Var. Communis | Cupressaceae | Natural | Low | High |
| 7 | Juniperus horizontalis Moench | Cupressaceae | Natural | Low | High |
| 8 | Pinus halepensis Mill. | Pinaceae | Natural | Moderate | High |
| 9 | Pinus nigra L. | Pinaceae | Natural | Low/Moderate | High |
| 10 | Pinus nigra L. subsp. pallasiana | Pinaceae | Natural | Moderate | High |
| 11 | Pinus pinaster Ait. | Pinaceae | Exotic | Moderate | High |
| 12 | Pinus pinea L. | Pinaceae | Natural | Low/Moderate | High |
| 13 | Pinus sylvestris L. | Pinaceae | Natural | Moderate | High |
| 14 | Taxus baccata L. | Taxaceae | Natural | Moderate | High |
| LEAFY TREES |  |  |  |  |  |  |
| No | Latin Name | Family | Natural-Exotic | Water Request | Drought |
| 1 | Acer campestre L. | Aceraceae | Natural | Moderate | High |
| 2 | Acer negundo L. | Aceraceae | Natural | Moderate | High |
| 3 | Acer platanoides L. | Aceraceae | Natural | Moderate | High |
| 4 | Acer pseudoplatanus L. | Aceraceae | Natural | Moderate | Moderate |
| 5 | Acer tataricum L. | Aceraceae | Natural | Low | High |
| 6 | Aesculus hipocastanum L. | Hippocastanaceae | Natural | Moderate | Moderate |
| 7 | Carpinus orientalis L. | Betulaceae | Natural | Moderate | Moderate |
| 8 | Carpinus betulus L. | Betulaceae | Natural | Moderate | Moderate |
| 9 | Castanea sativa Mill. | Fagaceae | Natural | Low | High |
| 10 | Celtis australis L. | Cannabaceae | Natural | Low/Moderate | Moderate |
| 11 | Cercis siliquastrum | Leguminosae | Natural | Moderate | Moderate |
| 12 | Cotinus coggygria Scop. | Anacardiaceae | Natural | Low /Moderate | High |
| 13 | Crataegus monogyna Jacq. | Rosaceae | Natural | Moderate | Moderate |
| 14 | Elaeagnus angustifolia Wahl. | Elaeagnaceae | Natural | Low /Moderate | High |
| 15 | Elaeagnus orientalis L. | Elaeagnaceae | Natural | Low/Moderate | High |
| 16 | Fraxinus excelsior L. | Oleaceae | Natural | Moderate | High |
| 17 | Fraxinus ornus L. | Oleaceae | Natural | Moderate | High |
| 18 | Gleditsia triacanthos L. | Leguminosae | Exotic | Low/Modarate | High |
| 19 | Hippophae rhamnoides | Elaeagnaceae | Natural | Low | High |
| 20 | Juglans regia | Juglandaceae | Natural | Moderate | High |
| 21 | Lagerstroemia indica L. | Lytracea | Exotic | Low /Moderate | High |
| 22 | Laurus nobilis L. | Lauraceae | Natural | Low /Moderate | High |
| 23 | Laurocerasus officinalis Roem. | Rosaceae | Natural | Moderate | High |
| 24 | Magnolia grandiflora L. | Magnoliaceae | Exotic | Moderate | High |
| 25 | Melia Lowedarach L. | Meliaceae | Natural | Low | High |
| 26 | Morus alba L. | Moraceae | Exotic | Moderate | High |
| 27 | Olea europaea L.var.europaea L. | Oleaceae | Natural | Low | High |
| 28 | Olea europaea L. var. sylvestris (Mill.) | Oleaceae | Natural | Low | High |
| 29 | Paulownia tomentosa | Paulowniaceae | Semi-Natural | Moderate | Moderate |
| 30 | Pistacia atlantica Desf. | Anacardiaceae | Natural | Low | High |
| 31 | Pistacia terebinthus L. subsp. terebinthus | Anacardiaceae | Natural | Low | High |
| 32 | Platanus orientalis L. | Platanaceae | Natural | Moderate | High |
| 33 | Prunus cerasifera “Pisardii nigra” | Rosaceae | Exotic | Moderate | High |
| 34 | Punica granatum L. | Lythraceae | Natural | Moderate | Moderate |
| 35 | Pyrus communis L. | Rosaceae | Natural | Moderate | High |
| 36 | Pyrus amygdaliformis | Rosaceae | Natural | Low | High |
| 37 | Pyrus elaeagnifolia Pall. | Rosaceae | Natural | Moderate | High |
| 38 | Quercus cerris L. | Fagaceae | Natural | Low /Moderate | High |
| 39 | Quercus coccifera L. | Fagaceae | Natural | Moderate | High |
| 40 | Quercus frainetto Ten. | Fagaceae | Natural | Low | High |
| 41 | Quercus infectoria Oliver | Fagaceae | Natural | Low | High |
| 42 | Quercus pubescens Wild. | Fagaceae | Natural | Moderate | High |
| 43 | Quercus robur L. | Fagaceae | Natural | Moderate | High |
| 44 | Robinia pseudoacacia L. | Leguminosae | Natural | Low/Moderate | High |
| 45 | Robinia pseudoacacia “Umbraculifera” | Leguminosae | Semi-Natural | Moderate | High |
| 46 | Sophora japonica L. | Leguminosae | Exotic | Low/Moderate | High |
| 47 | Tilia tomentosa Moench. | Malvaceae | Natural | Moderate | High |
| 48 | Ulmus glabra Huds. | Ulmaceae | Natural | Low/Moderate | High |
| 49 | Ulmus minor Mill. | Ulmaceae | Natural | Low/Moderate | High |
| SHURUBS |  |  |  |  |  |  |
| No | Latince Adı | Family | Natural-Exotic | Water Request | Drought |
| 1 | Berberis crataegina DC. | Berberidaceae | Natural | Low -Moderate | High |
| 2 | Berberis thunbergii DC. “Atropurpurea” | Berberidaceae | Exotic | Low/Moderate | High |
| 3 | Berberis vulgaris L. | Berberidaceae | Natural | Low/Moderate | High |
| 4 | Cineraria maritima | Asteraceae | Natural | Low | High |
| 5 | Cornus alba L. | Corneceae | SemiNatural | Modarate | Moderate |
| 6 | Cornus mas L. | Cornaceae | Natural | Low/Moderate | Moderate |
| 7 | Cotoneaster franchettii Boiss. | Rosaceae | Exotic | Low/Moderate | High |
| 8 | Eleagnus angustifolia | Elaegnaceae | Natural | Low | High |
| 9 | Photinia fraseri | Rosaceae | SemiNatural | Moderate | Moderate |
| 10 | Gardenia jasminoides Ellis | Rosaceae | Exotic | Low | High |
| 11 | Hibiscus rosa sinensis | Malvaceae | Exotic | Low/Moderate | High |
| 12 | Ligustrum vulgare L. | Oleaceae | Natural | Moderate | High |
| 13 | Mahonia aquifolium Nutt. | Berberidaceae | Natural | Moderate | Moderate |
| 14 | Nerium oleander L. | Apocynaceae | Natural | Low/Moderate | High |
| 15 | Philadelphus coronarius L. | Saxafragaceae | Natural | Low/Moderate | Moderate |
| 16 | Pittosporum tobira Ait. | Pittosporaceae | Exotic | Moderate | Moderate |
| 17 | Pittosporum tobira Ait “Nana” | Pittosporaceae | Exotic | Moderate | Moderate |
| 18 | Pyracantha coccinea M. J. Roemer | Rosaceae | Natural | Low | Moderate |
| 19 | Punica granatum L. | Punicaceae | Natural | Low/Moderate | High |
| 20 | Rhus typhina L. | Anacardiaceae | Natural | Low/Moderate | High |
| 21 | Rosa canina L. | Rosaceae | Natural | Moderate | Moderate |
| 22 | Rosmarinus officinalis L. | Lamiaceae | Natural | Low/Moderate | High |
| 23 | Sambucus nigra L. | Adoxaceae | Natural | Low/Moderate | High |
| 24 | Spartium junceum L. | Fabaceae | Natural | Low/Moderate | High |
| 25 | Spirea bumalda Burv. | Rosaceae | SemiNatural | Moderate | High |
| 26 | Spirea vanhouttei Zabel. | Rosaceae | SemiNatural | Moderate | Moderate |
| 27 | Spiraea hypericifolia L. | Rosaceae | Natural | Low/Moderate | Moderate |
| 28 | Viburnum licidum L. | Adoxaceae | Natural | Moderate | Moderate |
| 29 | Viburnum opulus L. | Adoxaceae | SemiNatural | Moderate | Moderate |
| 30 | Vibirnum tinus L. | Adoxaceae | Natural | Low/Moderate | Moderate |
| 31 | Yucca flamentosa L. | Agavaceae | Exotic | Low | High |
| PALMS |  |  |  |  |  |  |
| No | Latin Name | Family | Natural-Exotic | Water Request | Drought |
| 1 | Chamaerops humilis L. | Arecaceae | Exotic | Moderate | High |
| 2 | Phoenix canariensis | Arecaceae | Exotic | Moderate | High |
| 3 | Washingtonia filifera (Lindl.) Wendl | Arecaceae | Exotic | Moderate | High |
| 4 | Trachycarpus fortunei (Hook.) H. Wendl. | Arecaceae | Exotic | Moderate | High |
| WINDER AND CLIMBER |  |  |  |  |  |  |
| No | Latince Adı | Familyası | Natural-Exotic | Water Request | Drought |
| 1 | Amphelopsis orientalis (Lam.) Planch. | Plumbaginaceae. | Natural | Low | High |
| 2 | Hedera helix L. | Hederaceae | Natural | Moderate | Moderate |
| 3 | Jasminum polyanthum | Oleaceae | Natural | Moderate | Moderate |
| 4 | Jasminum officinale L. | Oleaceae | Natural | Moderate | Moderate |
| 5 | Lonicera caprifolium L. Kugel | Caprifoliaceae | Natural | Low/Moderate | High |
| 6 | Lonicera tatarica L. | Caprifoliaceae | Natural | Low/Moderate | High |
| 7 | Parthenocissus quinquefolia Planch. | Vitaceae | Exotic | Low/Moderate | High |
| 8 | Parthenocissus tricuspidata Planch. | Vitaceae | Exotic | Low/Moderate | High |
| 9 | Smilax aspera L. | Smilacaceae | Natural | Low/Moderate | High |
| 10 | Vitis vinifera | Vitaceae | Natural | Moderate | Moderate |
| PERENNIALS |  |  |  |  |  |  |
| No | Latin Name | Family | Natural-Exotic | Water Request | Drought |
| 1 | Achillea millefolium L. “Rosea” | Compositae | Natural | Low | Hıgh |
| 2 | Ajuga reptans L. | Lamiaceae | Natural | Low | High |
| 3 | Aster alpinus L. | Compositae | Natural | Low/Moderate | High |
| 4 | Campanula rotundifolia L. | Campanulaceae | Natural | Moderate | High |
| 5 | Centranthus rubber L. | Valetianaceae | Natural | Moderate | High |
| 6 | Centaurea hermanii |  | Natural | Low | High |
| 7 | Cerastium tomentosum L. | Caryophillaceae | Natural | Low/Moderate | High |
| 8 | Cistus salviifolius L. | Cistaceae | Natural | Low/Moderate | High |
| 9 | Crocus pestalozzae |  | Natural | Low/Moderate | High |
| 10 | Dianthus barbatus L. | Caryophillaceae | Natural | Moderate | High |
| 11 | Euphorbia amgydaloides | Euphorbiaceae | Natural | Moderate | Moderate |
| 12 | Helianthemum nummularium (L.)Mill. | Cistaceae | Natural | Low/Moderate | High |
| 13 | Hemerocallis fulva (L.) L. var. fulvus L. D | Lilaceae | Natural | Moderate | High |
| 14 | Hypericum calycinum L. | Hypericaceae | Natural | Low -Moderate | High |
| 15 | Kniphofia uvaria L. | Liliaceae | Natural | Low/Moderate | Moderate |
| 16 | Lamium maculatum L. | Lamiaceae | Natural | Low/Moderate | Moderate |
| 17 | Lavandula angustifolia Mill. | Lamiaceae | Natural | Low/Moderate | High |
| 18 | Mesembryanthemum nodiflorum L. | Aizoaceae | Natural | Low/Moderate | High |
| 19 | Papaver orientale L. | Papaveraceae | Natural | Moderate | Moderate |
| 20 | Papaver rhoeas L. | Papaveraceae | Natural | Moderate | High |
| 21 | Salvia fruticosa MILL. | Lamiaceae | Natural | Low | High |
| 22 | Salvia officinalis L. | Lamiaceae | Exotic | Moderate | High |
| 23 | Santolina chamaecyparissus L. | Asteraceae | Exotic | Low/Moderate | Moderate |
| 24 | Sedum acre L. | Crassulaceae | Natural | Low | High |
| 25 | Sedum album L. | Crassulaceae | Natural | Low | High |
| 26 | Sedum rupestre | Crassulaceae | Natural | Low | High |
| 27 | Sempervivum davisii Muirhead | Crassulaceae | Natural | Low | High |
| 28 | Spartium junceum | Fabacea | Natural | Low/Moderate | High |
| 29 | Styrax officinalis L. | Styracaceae | Natural | Low/Moderate | High |
| 30 | Thymus Lownavouri Velen. | Lamiaceae | Natural | Low/Moderate | High |
| 31 | Trifolium repens L. var. repens L | Fabaceae | Natural | Low/Moderate | High |
| 32 | Verbascum degenii | Scrophulariaceae | Natural | Low | High |
| 33 | Vinca major L. | Apocynaceae | Natural | Moderate | Moderate |
| 34 | Vinca minor L. | Apocynaceae | Natural | Moderate | Moderate |
| 35 | Verbena officinalis L. | Verbenaceae | Natural | Low/Moderate | High |
| 36 | Veronica liwanensis C. Koch | Plantaginaceae | Natural | Low/Moderate | High |
| 37 | Veronica caespitosa var. caespitosa | Plantaginaceae | Natural | Low | Moderate |
| 38 | Vitex agnus-castus L. | Verbenaceae | Natural | Low | High |

Sustainability Assessment of Lawn Areas

Increasing temperatures and irregular rainfall patterns in Istanbul are causing serious physiological stress on grass species, negatively affecting their growth processes. High temperatures increase evapotranspiration rates, leading to a significant rise in water consumption and maintenance costs. The water requirements of turfgrass species vary greatly depending on the morphological characteristics of the species, photosynthetic efficiency, root depth, and local climatic conditions. In addition, cultural practices such as irrigation strategies, mowing frequency, fertilization, and aeration also play a decisive role in water consumption.

When evaluated in line with drought-tolerant landscape design principles, large-scale turf applications are among the factors that reduce resource efficiency and increase water consumption (Tülek, 2008). Therefore, limiting turf areas to only functionally necessary zones—such as surfaces intended for recreation, sports, or erosion control—should be considered a fundamental strategy for sustainable landscape management. However, in landscape arrangements on the scale of Istanbul, intensive use of turf continues, particularly in medians and at intersections (Figure 4). This practice both hinders the efficient use of water and increases maintenance and energy costs in the long term. However, rather than completely eliminating grass areas, their limited use with the right species selection and placement can provide ecological benefits. Indeed, it has been reported that well-managed grass areas can sequester 110–115 g of atmospheric carbon per hectare per year (Qian & Follett, 2002). This finding shows that grass plants can make a limited but positive contribution to carbon sequestration when used in appropriate locations.

Samandıra Junction                                     Kavacık Junction                                                Vatan Street Junction

Figure.4.  Grass Field applications in Istanbul (URL 1,2025).

In recent years, the use of drought-tolerant species has emerged as an effective solution to these problems. In particular, Cynodon dactylon (bermuda grass) and Zoysia japonica (Korean grass) species are notable as alternatives that are tolerant to high temperatures and trampling stress and have low water requirements (White, 2001). Compared to traditional cold-season grasses, these species have deeper root systems and reduce irrigation frequency by using soil moisture more efficiently (Table 2). Preferring these species in temperature stress zones identified through QGIS analyses both preserves visual quality and minimizes water consumption. Therefore, redefining grass areas in Istanbul's climate-adaptive landscape planning should be approached with a management approach based on species diversity and functionality.

| No | Latin Name | Family | Drought tolerance |
| --- | --- | --- | --- |
| 1 | Buchloe dactlyloides | Poacea | Moderate |
| 2 | Cynodon dactylon (L.) Pers. | Poaceae | High |
| 3 | Festuca arundinacea (Schreb) | Poaceae | Moderate |
| 4 | Festuca arundinacea (Schreb)“Jaguar” | Poaceae | Moderate |
| 5 | Festuca arundinacea (Schreb) “Arit” | Poaceae | Moderate |
| 6 | Festuca glauca “Boulder Blue” | Poaceae | Moderate |
| 7 | Festuca trachyphylla “Durar” | Poaceae | Moderate |
| 8 | Ophiopogon japonica | Poaceae | Moderate |
| 9 | Zoysia japonica | Poaceae | High |

Table 2. Drought tolerance of grass plants

Cynodon dactylon (L.) Pers. and Zoysia japonica, which have recently come into use, are drought-tolerant, wear-resistant grass species that require little water (White, 2001).

Mulching Applications

Organic mulch material is an excellent source of nutrients for soil organisms and provides suitable conditions for their growth. As the mulch material decomposes, it releases its nutrients, while part of the mulch material turns into stable humus, contributing positively to the organic matter content of the soil. A thick layer of mulch prevents weeds from germinating and further suppresses their growth. For all these reasons, mulching plays an important role in preventing soil erosion. Furthermore, covering the soil surface with organic or inorganic materials reduces water evaporation, balances soil temperature, and prevents weed growth. The use of organic mulch (bark, leaves, compost) is becoming widespread in parks and gardens in Istanbul. Mulching both saves water and improves soil health. Applying mulch, especially to newly planted saplings and flower beds, reduces maintenance costs and water requirements. However, the practice of mulching by spreading colored gravel under the name of “xeriscaping” reveals a lack of understanding of the concept of xeriscaping.

3.4.  Regular and Scientifically Based Maintenance Principles

The long-term sustainability of xeriscape applications in metropolitan contexts such as Istanbul is directly linked to the implementation of regular, data-driven, and scientifically grounded maintenance strategies. Maintenance operations—including irrigation scheduling, pruning, fertilization, and pest–disease control—must be systematically designed according to the ecological requirements of selected plant species and the seasonal variability of local climatic conditions. Within this framework, adaptive maintenance planning based on environmental data (e.g., evapotranspiration rates, soil moisture content, and phenological cycles) ensures both ecological resilience and efficient resource management.

Empirical analyses demonstrate that the selection of native and naturalized species with inherently low fertilizer and pesticide requirements significantly reduces the ecological footprint of landscape management (Torun Kayabaşı, 2018). Indigenous flora contributes to the stability of local ecosystems by supporting beneficial soil microorganisms, enhancing pollinator networks, and reducing susceptibility to invasive species. Consequently, the integration of native plant communities not only minimizes maintenance inputs but also strengthens the ecological integrity and biodiversity of urban green systems.

Furthermore, digital monitoring systems have emerged as a critical component of sustainable landscape maintenance. The integration of Internet of Things (IoT)–based sensors and Geographic Information Systems (GIS) facilitates real-time tracking of water consumption, soil parameters, and vegetation health. These data-driven systems enable the optimization of irrigation schedules and fertilization regimes, reducing unnecessary inputs while maintaining plant vitality. Automated data collection and analysis also contribute to the long-term evaluation of maintenance efficiency and environmental performance, supporting evidence-based decision-making in municipal landscape management.

Maintenance optimization is particularly relevant for urban xeriscapes where resource scarcity and high user density intersect. Statistical assessments from comparable climatic regions indicate that precision irrigation systems integrated with soil moisture sensors can achieve up to 40–60% reductions in water use, aligning with best global practices in drought-resilient landscape management (UN-Habitat, 2022). In the context of Istanbul, where precipitation irregularities and evapotranspiration rates are projected to increase under RCP 8.5 scenarios, such efficiency gains are essential for ensuring the adaptive capacity of urban green infrastructure.

In terms of spatial design, the configuration of landscape elements directly affects maintenance frequency and intensity. Fragmented turf areas, for instance, demonstrate higher water and nutrient demands compared to consolidated or reduced-lawn designs. The strategic limitation of turfgrass to small-scale, functional zones, such as recreational pockets or transitional spaces, can therefore contribute substantially to maintenance efficiency. Similarly, minimizing the use of seasonal ornamental plants in traffic medians, roundabouts, and roadside strips reduces both water consumption and maintenance costs without compromising visual quality. Empirical GIS-based analyses of pilot sites within the city show that maintenance inputs decrease proportionally with the reduction of high-water-demand vegetation zones, confirming the positive correlation between xerophytic design strategies and resource conservation.

Sustainable maintenance also involves a systemic approach to nutrient management. Controlled-release organic fertilizers, applied based on soil nutrient mapping, prevent nutrient leaching and groundwater contamination. The periodic assessment of soil organic matter and microbial activity provides essential feedback for refining fertilization regimes. This adaptive feedback loop—integrating soil data, plant health indicators, and climatic parameters—constitutes a resilient maintenance model suitable for Mediterranean-climate cities like Istanbul.

Moreover, pest and disease management in xeriscapes benefits from integrated ecological approaches rather than conventional chemical control. Biological control agents, companion planting, and habitat diversification reduce the incidence of pest outbreaks, while minimizing environmental toxicity. Data collected from experimental xeriscape plots indicate that natural enemies of common urban pests (such as aphids and thrips) increase in abundance where native shrubs and flowering perennials are incorporated into the design. Thus, ecological maintenance not only preserves ecosystem services but also contributes to the long-term stability of plant communities.

Ultimately, the scientific maintenance of xeriscapes must be understood as a dynamic process—continuously informed by monitoring, data evaluation, and performance indicators. The transition from reactive to proactive maintenance, supported by digital tools and ecological databases, represents a paradigm shift in urban landscape management. In the context of climate-adaptive design, such data-driven maintenance frameworks ensure that xeriscape systems remain functionally and aesthetically resilient over time, while minimizing inputs and maximizing ecological benefits.

5. Discussion

For cities to become sustainable and resilient, blue-green infrastructure elements must be addressed not only in terms of aesthetics or short-term functions, but within the framework of a holistic ecosystem approach. These elements should be considered integral components of the urban ecosystem, and the preservation and enhancement of ecological integrity should be prioritized in planning processes. Planning that ignores ecological concerns and focuses solely on current needs causes irreparable damage to the natural and cultural environment and jeopardizes long-term sustainability goals. The fundamental reason for the growing impModeratence of sustainability discussions today is the environmental problems caused by short-term and non-ecosystem-focused decisions made in the past.

Future-oriented strategies should be designed to safeguard ecological, social, and economic needs; they should be supported by nature-based solutions and implemented within the framework of the principle of spatial justice that considers ecosystem services. This approach parallels the transition from ‘governance’ to ‘governance’ in modern democratic countries; that is, participatory processes that take into account the views of all stakeholders play a decisive role in the sustainability of cities (Keulartz et al., 2004). The development of landscape policies should be based on data-driven, geographical, and hydrogeological analyses; the integration of green and blue infrastructure should be strengthened; and participatory planning processes should be carried out with the involvement of local communities, experts, and decision-makers. This approach is critical for the protection of natural resources, increasing the efficiency of their use, and developing existing capacity. This process represents not only a response to environmental crises, but also the courage to build a more equitable, livable, and resilient future. The literature clearly shows that the expansion of human settlements has serious impacts on biodiversity (Miller & Hobbs, 2002; Stenhouse, 2004). The loss of natural ecosystems within and adjacent to city boundaries poses risks to public health and quality of life (Binning et al., 2001). The size, shape, and spatial distribution of green spaces have a decisive impact on the thermal environment and microclimate. Climate change, in particular, has critical effects on urban water resources (NASA, 2021; Lowhoni et al., 2018; Bates et al., 2008; Hassamancıoğlu, 2021).

Studies conducted specifically in Istanbul reveal the ecological and thermal effects of rapid urban growth and provide the necessary information for sustainable urban planning (Ciğerci et al., 2024). Furthermore, the implementation of nature-based solutions in cities to mitigate the effects of climate change offers holistic solutions to ecological problems as well as social needs (Baykar, 2023). Policies that encourage AI innovation, support corporate well-being, and foster international technology transfer are important (Bergougui,2025).

Due to a lack of data at the district level, solutions have been developed for Istanbul as a whole, but the need for collaboration between ecologists, social scientists, urban planners, and local governments for implementation has been emphasized. In conclusion, designing urban landscape strategies with a holistic approach that considers ecosystem services, optimizes water and natural resource management, and prioritizes stakeholder participation is critical for Istanbul's future vision of a sustainable and resilient city.

6. Conclusion

In addition to policies implemented on a global scale, developing action plans at the local and regional levels in harmony with ecological, socio-economic, and cultural structures is critical for the future of landscape planning. Climate action plans should be prepared for each city based on its unique topography, bioclimatic data, vegetation cover, water resources, and social structure.  The preparation of climate action plans should not be merely a technical process; it should also be the product of a participatory, multi-stakeholder, and transparent governance model. These plans should be shaped with the active participation of academic circles, local governments, civil society organizations, private sector representatives, and the public; they should be reviewed and updated at regular intervals and shared openly with the public.

The integration of locally initiated applications with national strategies ensures that climate-focused landscape policies are reinforced by a sustainable governance model. Social awareness, climate literacy, and public participation are decisive factors in the effectiveness and social acceptance of climate policies. In this context, educational programs, media campaigns, and digital information tools should be considered fundamental components of social transformation. Making water consumption plans mandatory in landscape projects and prioritizing plant species with low water requirements is a strategic necessity for climate adaptation. Financial and technical incentive mechanisms that support the production of native and natural plant species will increase ecological compatibility while reducing dependence on imported species. In this regard, reorganizing nursery policies based on regional flora analyses is critical for protecting biological diversity and promoting ecological landscape design.

Additionally, promoting native plant species through digital databases and interactive platforms will strengthen knowledge-based planning processes by facilitating access for designers, researchers, and decision-makers. The integration of technological innovations (remote sensing, data analytics, smart water management, etc.) into landscape planning processes will support the establishment of a measurable and controllable understanding of sustainability.

Ultimately, the arid landscape approach should be considered not only an ecological necessity but also an ethical responsibility. Holistic, interdisciplinary, and participatory approaches developed in the case of Istanbul can serve as a model for efficient water resource use, green space management, and climate change adaptation. Thus, it will be possible to build climate-resilient, livable cities through nature-based solutions.

Declarations

Ethics approval and consent to participate:  Not applicable.

Consent for publication:  Not applicable.

Competing interests:  The author declare that she has  no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

References

Akkemik, Ü. (2018). All the trees and shrubs of Turkey. Türkiye İş Bankası Cultural Publications, Ankara. ISBN:978625405172

Atila M, Küçük V. (2001). The Research Of Isparta And The Open -Green Areas In Urban. Turkish Journal of Forestry, 2(1), 27-48. ISSN: 1302-7085. https://doi.org/10.18182/tjf.23277.

Lowhoni, A., Jude, S., Holman, I. (2018). Adapting to climate change by water management organisations: Enablers and barriers. Journal of Hydrology, 559, 736–748. https://doi.org/10.1016/J.JHYDROL.2018.02.047

Başaran N, & Eroğlu, E. (2023). Addressing Ecosystem Regulation Services at the Plant Level in Cities Developments in the Field of Agriculture, Forestry and Aquaculture Editörler Prof. Dr. Atilla Atik Dr. Öğr. Üyesi Ahmet Haşim Keskin Platanus Publishing  Doı:10.5281/Zenodo.10025044

Bates, B., Wu, S., Zbigniew Kundzewicz, A. W., Palutikof, J. (2008). Climate Change and Water. Intergovernmental Panel on Climate Change, IPCC. ISBN: 978-92-9169-123-4

Bergougui, B. 2025. “Can Artificial Intelligence Technologies Advance Environmental Sustainability? The Role of Institutional Adaptability and Skill-Biased Technological Transformation.” Sustainable Development 1–23. https://doi.org/10.1002/sd.70296.

Çorbacı, Ö.L.,Özyavuz M., (2024). Xeriscaping Studies in Urban Open Green Spaces, İnce, K. (Ed.). Sustainable Management of Urban Green Spaces. (July, 2024), Chapter 5, 109-171. ISBN: 978-625- 367-746-6. Iksad Publications. DOI: https://dx.doi.org/10.5281/zenodo.12698789

Çınar, H. S., Ay Ak, B. R. (2024). Investigation of Topkapı Palace Gardens  in Terms of Xeriscape   İdeal kent, 16(45), 1435-1465. https://doi.org/10.31198/idealkent.1486641

Çınar, H.S., (2022) Landscape Transforming in a Changing Climate. New Trends in Architecture, Planning and Design (Pp.231-247) Publisher: Platanus

Çınar, H.S, Güzel M., (2020). Investigation on rational use of water in a palace garden: A case of Dolmabahce Palace (Mabeyn Garden). Fresenius Environmental Bulletin, ss. 6469-6478. https://www.prt-parlar.de/

Çınar, H. S., & Aktaş, N. K. (2020). Outlook On Climate Crisis in Urban Green Areas: Case Study of Istanbul-Sultanahmet Region. NEU Journal of Faculty of Architecture, 2(1), 35-45. https://dergipark.org.tr/tr/pub/neujfa/issue/65310/1006331

Çınar H.S., Kart Aktaş, N. (2018). Xeriscape Analysis: A Case Study In A Residential Garden In Istanbul. Journal of Environmental Protection and Ecology 19, No 4, 1904 – 1917.

Davis, P.H., (1965). Flora of Turkey and the East Aegean Islands Volume 1. Edinburgh University, Press 22.

Davis, P., Mill, R.R., Kit,T. (1988). Flora of Turkey and the East Aegean Islands (Supplement). Vol:10, Edinburgh University Press.

Directorate of Agriculture and Aquaculture. (2025). Free irrigation support from IMM to farmers. Istanbul Metropolitan Municipality. https://tarim.ibb.istanbul/tarim-ve-su-urunleri-mudurlugu/haberler/1270/ibbden-ciftcilere-ucretsiz-sulama-destegi.html

EUROSTAT, (2025). Water exploitation index, plus (WEI+). Retrieved on: 28 Mayıs 2025, https://ec.europa.eu/eurostat/databrowser/view/sdg_06_60/default/table?lang=en

Eroğlu, S., (2010). Investigation of Ring Roads in Istanbul Metropolis in Terms of Plant Design. High Degree Thesis, Istanbul Technical University, İstanbul.

General Directorate of Meteorology, (2025). https://www.mgm.gov.tr/veridegerlendirme/il-ve-ilceler-istatistik.aspx?k=&m=ISTANBUL

Gül, A., Topay, M.&Özaltın, O., (2009). Socıal And Economıc Issues Shapıng The World’s Future New Global Dıalogue The Important Of Urban Forests Toward To Global Warmıng.  Internatıonal Davraz Congress.  24-27 September 2009, Isparta, s.221-234. ISBN: 978-9944-452-34-2

Grimm, W.C., 2002. The illustrated book of trees : the comprehensive field guide to more than 250 trees of eastern North America, revised by John Kartesz, Mechanicsburg, PA : Stackpole Books. Isbn 0811730859

Gordon A., David Simondson, Matt White , Atte Moilanen, Sarah Adine Bekessy,2009. Integrating conservation planning and landuse planning in urban landscapes. Landscape and Urban Planning Volume 91, Issue 4, 30 July 2009, Pages 183-194 DOI: 10.1016/j.landurbplan.2008.12.011

Hassamancıoğlu, V. M. (2021). Evaluation of Urban Green Space Management Strategies of Local Governments in Turkey. OPUS International Journal of Society Researches, 18(39), 627-656. https://doi.org/10.26466/opus.906382

Hunter M (2011). Using Ecological Theory to Guide Urban Planting Design An adaptation strategy for climate change. Landsc J 30(2):173–193. https://doi.org/10.3368/lj.30.2.173

IBB Parks, Gardens and Green Areas Department, (2023).  Istanbul Green Area Management System Strategy Document Executive Summary Istanbul Green Area Management System Strategy Document, April 2023 ISBN: 978-605-9507-92-9.

Istanbul Climate Change Action Plan, (2021). Istanbul Climate Change Action Plan. Istanbul Metropolitan Municipality, Department of Environmental Protection and Control. İstanbul (Accessed 16 January 2025). www.arup.co

Istanbul Water and Sewerage Administration. (2025). Hydrogeological Survey Reports and Groundwater Potential Maps. https://www.iski.gov.tr

Kavuran, D., & Yılmaz, R. (2022). Selection of Appropriate Plant Species in Xeriscaping Works: The Case of Süleymanpaşa, Tekirdağ. Landscape - Journal of Education, Science, Culture and Art, 4(2), 69-91. https://doi.org/10.53784/peyzaj.167964

Kendal D, Dobbs C, Lohr VI (2014). Global patterns of diversity in the urban forest: Is there evidence to support the 10/20/30 rule? Urban For. Urban Green 13:411–417. https://doi.org/10.1016/j.ufug.2014.04.004

Keulartz J. Jozef Keulartz,  Henny Van Der Windt,  (2004). Concepts of nature as communicative devices  Environmental Values ,  Volume 13, Issue 1 https://doi.org/10.3197/096327104772444785

Kumar, A., Devi, J.P., Bhaskar, D. 2025.  Perceptions and impacts of climate change in Central India: A study of tribal communities. Theoretical and Applied Climatology 156, 423 (2025). https://doi.org/10.1007/s00704-025-05653-2

Miller, J. R., & Hobbs, R. J. (2002). Conservation Where People Live and Work. Conservation Biology, 16, 330-337. https://doi.org/10.1046/j.1523-1739.2002.00420.x

Ministry of Environment, Urbanization and Climate Change (MEUCC), (2022). Cloud City Information System Technical Guide. Ankara: MEUCC Publications. Https://Bulutkbs.Gov.Tr/İndex.Php/Uygulamalar-Ve-Egitimleri/

Mullenbach LE, Wilhelm Stanis SA (2024) Climate change adaptation plans: inclusion of health, equity, and green space. J Urban Affairs 46(4):701–716. https://doi.org/10.1080/07352166.2022.2091449

NASA. (2021). Global Temperature, Vital Signs–Climate Change: Vital Signs of the Planet. https://climate.nasa.gov/vital-signs/global-temperature/

Qian, Y. ve Follett, R.F. (2002). Assessing soil carbon sequestration in turfgrass systems using long-term soil testing data. Agronomy Journal, 94: 930-935. https://doi.org/10.2134/agronj2002.9300

Peroni F, Pappalardo SE (2024) Climate justice in future cities: geographical perspectives for inclusive urban resilience and adaptation. Landsc Urban Plann 244:104998. https://doi.org/10.1016/j.landurbplan.2023.104998

Ragab, T.S., Bakri, N.T., Hariri, G.E. (2024). Bioclimatic Xeriscaping as a Strategy for Landscape Transformation in Hot and Humid Cities: Reflection on Jeddah, KSA. In: Ibrahim, A., Mohamed, M.A.A., Fekry, M. (eds) Man and Place. ARCH + DESN 2023. Springer, Cham. https://doi.org/10.1007/978-3-031-49903-6_4

Razzaghmanesh M, Beecham S, Kozemi F (2014). The growth and survival of plants in urban green roofs in a dry climate. Sci Total Environ 476–477:288–297. https://doi.org/10.1016/j.scitotenv.2014.01.014

Stenhouse, R.N. (2004). Fragmentation and internal disturbance of native vegetation reserves in the Perth metropolitan area, Western Australia. Landscape and Urban Planning, 68, 389–401. http://dx.doi.org/10.1016/S0169-2046(03)00151-8

Torun Kayabaşı E., (2018).  Kentsel Tasarımda Natural Bitkilerin Kullanımı ve Ekonomik Önemi UEP 2018 Uluslararası Kentleşme ve Çevre Sorunları Sempozyumu: Değişim/Dönüşüm/Özgünlük 28-30 HLowiran 2018 Anadolu Üniversitesi – Eskişehir ISBN: 978-605-01-1252-8 (2.c)

Özdemir, Z., & Özkaynak, M. (2023). Investıgatıon Of Urban Green Areas From The Framework Of Sustaınable Urbanızatıon On The Case Of Amasya,  Çevre Şehir ve İklim Dergisi, 2(3), 270-292. https://dergipark.org.tr/tr/pub/csid/issue/75639/1207222

Url 1,2025. https://agac.istanbul/peyzaj?category=1021

Wade, G. L., Midcap, J. T., Coder, K. D., Landry, G. W., Tyson, A. W. and Neal Jr, W. (2010). Xeriscape: a guide to developing a water-wise landscape. University of Georgia. Handle: http://hdl.handle.net/10724/12344

White, R.H., Engelke, M.C., Anderson, S.J., Ruemmele, B.A., Marcum, K.B. & Taylor, G.R. (2001). Zoysia grass water relations. Crop Science, 41, 133-138. https://doi.org/10.2135/cropsci2001.411133x

Williams M. D., (2007). Identifying trees : an all-season guide to Eastern North America, Mechanicsburg, PA : Stackpole Books.ISBN:978-0-8117-1830-1

Yaltırık, F.; Efe,A.; Uzun,A., (1997). Istanbul's Parks, Gardens and Groves Exotic Trees and Shrubs Throughout History, ĐBB Publications, Istanbul. ISBN NO: 975-8183-00-1

Yener, Ş. D., (2020). Sustainable use of plants in coastal areas of Istanbul. Turkish Journal of Forestry, 21(2), 123-130. https://doi.org/10.18182/tjf.645461

Zaeemdar, S., Baycan, T. (2017). Analysis of the Relationship between Urban Heat Island and Land Cover in Istanbul through Landsat 8 OLI. Journal of Earth Science & Climatic Change. https://doi.org/10.4172/2157-7617.1000423

Ziter, C., Kinnunen, R., Richmond, I. et al. (2025). What does it mean to rethink urban green spaces?. Landsc Ecol 40, 2  https://link.springer.com/article/10.1007/s10980-024-02013-9. DOI: https://doi.org/10.1007/s10980-024-02013-9