Original Research Article

Hydrogeological Parameters and Climate Variability Impacts on Groundwater Availability in M’Bahiakro, Central-East Côte d’Ivoire

.

ABSTRACT

In the context of increasing climate variability in West Africa, this study analyzes groundwater dynamics in M’Bahiakro in relation to climatic conditions and aquifer characteristics. The data used include daily rainfall (1944–2016), monthly temperatures (1964–2015), and potential evapotranspiration (1971–1998) from the Bouaké station. The methodology combines the analysis of Nicholson indices after seasonal filtering using a Hanning low-pass filter. It also relies on hydrodynamic measurements of hydraulic conductivity, effective porosity, and groundwater flow velocities in four representative districts. The results identify three climatic phases: wet (1944–1972), normal (1973–1996), and dry (1997–2016), which directly influence aquifer recharge. Hydraulic conductivity ranges from 7.1 × 10⁻⁵ to 1.5 × 10⁻⁴ m/s, with permeable zones in N’guattakro and Koko and less permeable zones in Dougouba and Baoulekro. Effective porosity (20–40%) enhances water storage and infiltration in sandy areas but limits flow in clay-rich sectors. Groundwater recharge represents 34.85% of rainfall, while evapotranspiration accounts for 62.07%. Groundwater flow velocities, on the order of 10⁻⁵ m/s, indicate slow circulation and tend to decrease with declining rainfall and rising temperatures. These results highlight that groundwater availability depends on both lithology and climate, emphasizing the need for integrated and sustainable water resource management. They also provide critical information for local planning and climate adaptation, contributing to improved water-resource management in tropical environments.

Keyword: Climate variability, Groundwater availability, groundwater table, Water resources management, Hydrogeology, M’Bahiakro, Côte d’Ivoire

1. INTRODUCTION

Groundwater resources play a fundamental role in the socio-economic development of tropical regions, particularly in West Africa, where they often constitute the primary source of drinking water and irrigation (Tirogo et al., 2023; Magaji et al., 2024). In Côte d’Ivoire, phreatic aquifers represent a crucial source of water for domestic, agricultural, and industrial uses. However, these resources are increasingly under pressure due to climate variability, population growth, and intensified agricultural activities (Soro et al., 2013).

Climate variability, characterized by interannual and seasonal fluctuations in rainfall and temperature, directly affects groundwater recharge and, consequently, the availability of groundwater resources (Adhikari et al., 2022). In West Africa, several studies have highlighted a declining trend in precipitation since the 1970s, leading to reduced water table levels and alterations in aquifer hydrodynamic parameters (Kouakou et al., 2014; Tefera et al., 2025). The city of M’Bahiakro, located in the central-eastern part of Côte d’Ivoire, falls within the Sudanian-Guinean climatic zone, characterized by alternating dry and wet seasons. The region hosts the country’s first hydro-agricultural dam, covering approximately 450 hectares dedicated to irrigated rice cultivation, aimed at reducing food insecurity by increasing rice production (Kouassi et al., 2013; Bai et al., 2019). The implementation of this project has fostered rapid population growth, with a predominantly low-income population relying almost entirely on groundwater for drinking water, thereby increasing pressure on the phreatic aquifer and raising the risk of overexploitation. In M’Bahiakro, previous studies (N’cho et al., 2024) have mainly focused on the quantitative and qualitative evolution of the phreatic aquifer, without integrating an assessment of hydrodynamic parameters or the aquifer’s water availability. Hydrodynamic parameters, such as transmissivity, hydraulic conductivity, porosity, and storage coefficient, govern the behavior of aquifers and their capacity to sustainably supply water (Bear, 1979; Soro et al., 2010). Furthermore, water balance analysis allows for the assessment of effective groundwater availability and the aquifer’s response to climatic variations and anthropogenic withdrawals. Therefore, this study aims to support sustainable water resource management by improving the understanding of interactions between climate, human activities, hydrodynamic parameters, and groundwater availability in M’Bahiakro.

2. material and methods

2.1 Material

2.1.1 Study Area

The M’Bahiakro Department is located in the central-eastern part of Côte d’Ivoire, approximately 400 km from Abidjan. It belongs to the Iffou region within the Lacs District and covers an area of 5,538 km² It is bordered to the north by Dabakala, to the south by Prikro and Daoukro, to the east by Bocanda, and to the west by Bouaké. The town of M’Bahiakro, the departmental capital, is situated between longitudes 4°19’35’’ and 4°20’53’’ West and latitudes 7°26’30’’ and 7°28’12’’ North. It comprises four neighborhoods: Baoulekro, N’guattakro, Koko, and Dougouba (Figure 1). The relief consists of gently undulating plateaus with an average elevation of 300 meters, interspersed with shallow valleys that facilitate water flow. The hydrographic network is dominated by the N’Zi River, a tributary of the Bandama, which crosses the region and contributes to the recharge of phreatic aquifers. The climate is of the sub-equatorial type, with two rainy seasons and two dry seasons. Annual rainfall ranges between 1,100 and 1,500 mm, and the mean temperature is 27°C. The soils, primarily ferrallitic, are suitable for agriculture but prone to erosion. Vegetation consists of wooded savannas and open forests, which are declining due to human activities.

Figure 1: Presentation of the Study Area

2.1.2 Data

The data collected for this study concern daily rainfall from 1944 to 2016 at the M’Bahiakro station, as well as monthly temperature (1964–2015) and evapotranspiration (1971–1998) data from the Bouaké station. The climatological data were provided by the Directorate of the Société de Développement et d’Exploitation Aéroportuaire, Aéronautique et Météorologique (SODEXAM). The limitation of potential evapotranspiration (PET) data up to 1998 is due to the absence of measurements from 1999 onward, associated with the period of socio-political unrest in Côte d’Ivoire.

2.2 Methods

2.2.1 Determination of Rainfall Trends

The analysis of the evolution of climatic variables was carried out using various methods, notably the Nicholson index and the second-order Hanning low-pass filter method. These approaches make it possible to highlight, at the spatio-temporal scale, periods of rainfall deficit and surplus.

2.2.1.1 Nicholson Index Method

The Nicholson Index method (Nicholson, 1998) is used to highlight rainfall, temperature, and streamflow trends (excess and deficit periods) within a time series. A positive value indicates a wet year, while a negative value indicates a dry year. The Nicholson Index is expressed by the following equation:

Ii=Xi-Xδ                                                                        (Éq.1)

where:

- Ii= Nicholson index for year i,

- Xi= observed value of the variable (rainfall, temperature, or discharge) for year i,

- X= mean of the series,

- δ= standard deviation of the series.

2.2.1.2 Second-Order Hanning Low-Pass Filter Method

The second-order Hanning low-pass filter method helps to better distinguish deficit and excess periods in rainfall or streamflow series. Its principle is to smooth out small fluctuations in a time series to highlight long-term variations. In this case, the totals of annual rainfall or streamflow series are weighted using the following equations (Bodian, 2014; Nicholson, 2000):

X(t)=0,06X(t-2) +0,25X(t-1) +0,38Xt+0,25X(t-1) +0,06X(t-2)                     (Éq.2)

Pour 3 ≤ t ≤ (n-2)

where:

X(t): weighted total of rainfall or streamflow series at term t;

X(t-2) and X(t-1): observed totals of rainfall or streamflow series for the two terms immediately preceding term t;

X(t+1) and X(t+2): observed totals of rainfall or streamflow series for the two terms immediately following term t.

The weighted totals of the first two ([X (1), X (2)]) and the last two ([X(n-1), X(n)]) terms of the series are calculated using the following expressions (where n is the length of the series):

X1 = 0, 54X1 + 0, 46X2                                                                   (Éq.3)

X2 = 0, 25X1 + 0, 50X2 + 0, 25X3                                                  (Éq.4)

X (n-1) = 0, 25X (n-2) + 0, 50 X (n-1) + 0,25X (n)                           (Éq. 5)

X (n) = 0,54X (n) + 0, 46 X (n-1)                                                      (Éq.6)

To better visualize the deficit and excess periods in rainfall or streamflow series, the moving averages were centered and standardized using the following formula (Bodian, 2014; Nicholson, 2000)):

Y't=Xt-mδ                           (Éq.7)

Where:

- (m): mean of the weighted moving average series;

- δ: standard deviation of the weighted moving average series.

Wet period: when the annual mean of rainfall or streamflow exceeds the mean observed over the reference period.

Normal period: when the annual mean is close to the mean of the observation period.

Dry period: when the annual mean of rainfall or streamflow is lower than that of the reference period.

2.2.2 Measurement of Hydrodynamic Parameters

The hydrodynamic parameters measured were hydraulic conductivity and effective porosity. Measurements were carried out at seven sites in the town of M’Bahiakro.

2.2.2.1 Hydraulic Conductivity

Hydraulic conductivity (K) represents the ability of a soil or rock to allow water to flow through it under the influence of a hydraulic gradient. It has the dimension of velocity and is expressed in m/s. It was determined in situ using a double-ring infiltrometer. The setup involves inserting the central metal cylinder, on which a Mariotte bottle is mounted, approximately 10 cm into the soil. To prevent lateral divergence of the water under the central cylinder, a second ring of larger diameter is inserted 2 to 3 cm around the first. A constant water level is maintained in the space between the two rings. After the infiltration rate stabilizes, the volume of water infiltrated (V) over a time period T is measured. The infiltration rate is then given by:

q=VT                (Éq.8)

By applying Darcy’s law, with infiltration occurring through a surface 𝑆 corresponding to the cylinder’s cross-section and for a hydraulic gradient equal to 1, the hydraulic conductivity is given by:

K=VS×T  (Éq.9)

Where:

- K: hydraulic conductivity (m/s),

- V: volume of water infiltrated (m³),

- S: cross-sectional area of the cylinder (m²),

- T : infiltration time (s).

The hydraulic conductivity was estimated using a double-ring infiltrometer due to its ability to provide reliable in situ measurements while preserving the natural soil structure. The outer ring limits lateral flow, concentrating vertical infiltration within the inner ring, making the measurement representative of actual infiltration. This method, suitable for different soil types, allows multiple tests to obtain an average value. Its limitations mainly concern sensitivity to soil heterogeneity, compaction, and saturation conditions. The method has been successfully applied by Kafando (2021) and Keita (2023) for measuring hydraulic conductivity in unsaturated soils.

It should be noted that field measurements of hydraulic conductivity (K) are subject to uncertainties related to soil heterogeneity, operational conditions, measurement duration, and the effectiveness of the rings in limiting lateral flow. In general, these uncertainties range from 10% to 20%, in accordance with the observations of Bagarello et al. (2004), Lai and Ren (2007), and Alaoui (2023). In this study, an uncertainty of 10% was adopted as it represents a conservative and realistic value for field measurements of hydraulic conductivity and for porosity measurements from disturbed soil samples. This estimate accounts for minor disturbances to soil structure, experimental variations, and instrument limitations while remaining scientifically credible. Hydraulic conductivity measurements were carried out at seven sites in the town of M’Bahiakro (Figure 2).

Figure 2: Measurement points of hydraulic conductivity and effective porosity

2.2.2.2 Effective Porosity

Soil samples were collected in situ using a manual auger. The sampling process involves applying vertical pressure on the auger while rotating it clockwise. This movement allows the tool to gradually penetrate the soil and retain a representative amount of material. The extracted sample is carefully collected and stored in a labeled bag to ensure traceability.

In the laboratory, a representative soil mass of approximately 450 g was weighed using a precision balance. The samples were then moistened by adding known quantities of water until full saturation was reached. After a drainage period of 48 hours, the volume of gravitationally drained water was collected and measured using a graduated cylinder. These measurements allowed the estimation of the soil’s effective porosity. As it was calculated from disturbed soil samples, the effective porosity carries an uncertainty due to the disturbance of the natural soil structure, which may influence the actual value. In the literature, this uncertainty is typically estimated to be between ±10% and ±20% of the measured value (Zou et al., 2021). In this study, an uncertainty of ±10% was adopted, as it represents a conservative and realistic value for porosity measurements Effective porosity is determined using the following relationship:

ne%=VdVt×100  (Éq.10)

Where :

- nₑ: effective porosity (%),

- Vd: volume of gravitationally drained water (mL or cm³),

- Vt: total volume of the sample (mL or cm³).

2.2.3 Flow Velocity

Flow velocities (Darcy velocities) were determined from seven flow lines, drawn in areas corresponding to the sites where hydraulic conductivity values were measured. For each flow line, two piezometric points are considered:

NPA: minimum piezometric level,

NPB: maximum piezometric level, separated by a distance (d) measured on the map.

The actual distance AB is calculated from the map scale using the following relationship:

DAB=dEou encored=DAB×E (Éq.11)

Where :

- Dₐᵦ: actual distance (m),

- d: distance on the map (cm),

- E: map scale (1/50,000).

The flow velocity is then determined using the following equation:

V=K×ΔHD×n    (Éq.12)

Where:

- V: flow velocity (m/s),

- K: hydraulic conductivity (m/s),

- n: effective porosity,

- D: actual distance between two piezometric points (m),

- ΔH: difference in piezometric head between NPB and NPA (m).

2.2.4 Estimation of the Hydrological Balance Using the Thornthwaite Method

The hydrological balance of a basin or an aquifer allows for the assessment of water availability by integrating inputs (precipitation) and losses (evapotranspiration and runoff). In this study, the balance was estimated using the Thornthwaite method (1948), which is widely used for tropical and subtropical regions (Sharma, 1988; Omar et al., 2021; Tagne et al., 2023). The monthly results were then aggregated into annual data for the analysis of the hydrological balance. The choice of the Thornthwaite method is justified by its simplicity, low data requirements, and proven relevance in tropical environments, where long-term continuous climate records are often limited. Numerous studies (Dienf et al., 2020; Todote et al., 2021) have successfully applied it in West Africa for hydrological balance estimation, making it a robust and suitable approach for the climatic context of the study area.

2.2.4.1 Principle of the Method

The Thornthwaite method is based on the calculation of potential evapotranspiration (PET) from monthly mean temperatures and a local climate coefficient. The PET is then compared to precipitation to determine:

- Water surplus (precipitation > PET), available for infiltration and storage in the aquifer,

- Water deficit (precipitation < PET), corresponding to the water lacking to meet evapotranspiration needs,

- Net balance, which integrates aquifer recharge and surface runoff.

The calculation of potential evapotranspiration (PET) was performed using the following equation:

ETP=16(10⋅TmI)a          (Éq.13)

Where:

- Tm: monthly mean temperature (°C)

- I: annual heat index

- a: empirical coefficient depending on I

The water deficit and surplus were calculated using the following equations:

D=PET-P if PET > P               (Éq.14)

E=P-PET if P > PET             (Éq.15)

where P represents monthly precipitation.

The reconstruction of the annual water balance was carried out as follows:

- Summation of monthly surpluses to estimate the effective recharge of the aquifer;

- Summation of monthly deficits to quantify periods of water stress.

3. RESULTS AND DISCUSSION
3.1 Results
3.1.1 Evolution of Rainfall
3.1.1.1 Annual Rainfall Variability

The analysis of Nicholson indices, after removal of the seasonal component using the Hanning filter, reveals three major climatic phases in M’Bahiakro between 1944 and 2016: a humid period (1944- 1972) with above-average rainfall, a normal phase (1973–1996) with values close to the mean, and a dry period (1997–2016) characterized by a significant rainfall deficit (Figure 3).

Figure 3: Interannual variation of rainfall in M’Bahiakro (1944–2016).

3.1.1.2 Interannual Temperature Variation

Figure 5 illustrates the annual evolution of temperature at the Bouaké station over the period 1964–2015. A general upward trend is observed, with an increase of about 0.5 °C per year. Between 1964 and 1972, temperatures remained generally below 26 °C, while from 1973 onward they exceeded this threshold. This warming trend intensified from the 2000s, reaching values close to 26.9 °C (Figure 4).

Figure 4: Interannual variation of temperature in Bouaké (1964–2015)

3.1.2 Hydrodynamic Characteristics

3.1.2.1 Hydraulic Conductivity

The hydraulic conductivity values of the surface layer generally range between 7.1±0.71×10⁻⁵ m/s and 1.50±0.15×10⁻⁴ m/s. In the neighborhoods of Baoulekro and Dougouba, the measured conductivity is 7.1±0.71 × 10⁻⁵ m/s, while it reaches (1.50±0.15) ×10⁻⁴ m/s in N’Guattakro and 8.4±0.84× 10⁻⁴ m/s in Koko (Figure 5).

Figure 5: Hydraulic Conductivity Map of M’Bahiakro

3.1.2.2 Effective Porosity

The effective porosity measurements reveal notable spatial variability among the different neighborhoods of M’Bahiakro (Figure 6). The observed values generally range between 20%±10% and 31.9%± 10%. The highest porosity is recorded in N’Guattakro (31.9%±10%), followed by Koko (29.5%±10%). The neighborhoods of Dougouba and Baoulekro show lower values, with 24.5% ±10% and 24.1%± 10% respectively.

Figure 6: Effective Porosity Map of M’Bahiakro

3.1.3 Flow Velocity

The estimated flow velocities for the years 2012, 2013, and 2015 remain relatively low, on the order of 10⁻⁵ m/s. A slight decrease in these values is also observed over the years, indicating a gradual slowdown in groundwater flow dynamics (Table 1).

| Année | 2012 | 2013 | 2015 |
| --- | --- | --- | --- |
| Vitesse de Darcy (m/s) | 1,6 × 10⁻⁴ | 1,8 × 10⁻⁴ | 1,9 × 10⁻⁴ |

Table 1: Groundwater Flow Velocity Values

3.1.4 Hydrological balance

The annual hydrological balance for the period 1971–1998 (table 2) shows an average rainfall of 1680.09 mm. Actual evapotranspiration (aet) is estimated at 1042.81 mm/year, representing 62.07% of total precipitation, while 37.93% of rainfall corresponds to mobilizable water. Surface runoff (q) is low, around 51.81 mm/year (3.08% of precipitation). In contrast, infiltration reaches 585.47 mm/year, or 34.85% of rainfall, providing a significant contribution to groundwater recharge.

Table 2: Summary of the hydrological balance (1971–1998) in m’Bahiakro

| Parameters | M’Bahiakro |
| --- | --- |
| Rainfall (mm·yr⁻¹) | 1680.09 |
| Actual evapotranspiration, AET (mm·yr⁻¹) | 1042.81 |
| Runoff (mm·yr⁻¹) | 51.81 |
| Infiltration (mm·yr⁻¹) | 585.47 |

3.2 Discussion

3.2.1 Evolution of Rainfall and Temperature

The analysis of Nicholson’s rainfall indices, after removing the seasonal component using the Hanning filter, reveals three major climatic phases in M’Bahiakro between 1944 and 2016, illustrating the progressive evolution of the rainfall regime and its connection with regional climate dynamics in West Africa. The first phase (1944–1972), a humid period, is characterized by above-average rainfall, which enhanced groundwater recharge and improved water availability.

The second phase (1973–1996), considered a normal period, shows rainfall values close to the long-term mean but with increased variability, alternating between wet and dry years and indicating a gradual shift from the previous regime. The last phase (1997–2016) is a dry period marked by a significant rainfall deficit and predominantly negative Nicholson indices, leading to reduced aquifer recharge and increased water vulnerability. These findings are consistent with the observations of Soro et al. (2013). In parallel, data from the Bouaké meteorological station show a clear warming trend between 1964 and 2015 (Nicholson, 2005). The 1964–1972 period, relatively cool and stable, was favorable to infiltration, whereas from 1973 onward, temperatures frequently exceeded 26 °C (New et al., 2006), reaching nearly 26.9 °C in the 2000s (Sylla et al., 2016; Dosio et al., 2022). This warming increases evapotranspiration, limits groundwater recharge, and aligns with the “regional amplification” described by the IPCC (IPCC, 2021), directly affecting water availability and population vulnerability. Overall, these combined results highlight the dual impact of decreasing rainfall and rising temperatures on groundwater resources in M’Bahiakro, emphasizing the need for adaptive water-resource management strategies in the context of climate change.

3.2.2 Hydrodynamic Characteristics

The hydraulic conductivity values measured in the superficial layer of M’Bahiakro show significant spatial variability, ranging from 7.1 × 10⁻⁵ to 8.4 × 10⁻⁴ m/s across different neighborhoods. Low hydraulic conductivity values, around 7.1 × 10⁻⁵ m/s, were recorded in Dougouba and Baoulekro, while higher values were observed in N’guattakro (1.5 × 10⁻⁴ m/s) and Koko (8.4 × 10⁻⁴ m/s). According to Cui et al. (2008), hydraulic conductivity tends to be low in clay-rich formations and higher in sandy formations. These findings suggest that the aquifers in N’guattakro and Koko are predominantly sandy, favoring faster flow and improved groundwater recharge, whereas those in Baoulekro and Dougouba consist mainly of clays, resulting in limited permeability and restricted infiltration. García‑Gutiérrez et al. (2018) highlight that hydraulic conductivity directly affects the time required for water to infiltrate and reach the groundwater table, underscoring the importance of lithology and porosity in aquifer recharge dynamics. These patterns are consistent with the work of Bear (1979), who note that hydraulic conductivity varies according to grain size distribution and the heterogeneity of surface formations, significantly influencing aquifer recharge and groundwater flow.

Effective porosity, representing the fraction of pore space that actively contributes to flow, also shows marked spatial contrasts in M’Bahiakro, ranging from 20% to 31.9%. N’guattakro (31.9%) and Koko (29.5%) exhibit the highest values, indicating a greater capacity for water storage and groundwater recharge. Conversely, Dougouba (24.5%) and Baoulekro (24.1%) display lower values, and the clays present-although porous-tend to limit water circulation, thereby slowing aquifer recharge (Meyer et al., 2018; Uhlemann et al., 2022).

According to Pakparvar et al. (2018), the combined consideration of porosity (or effective porosity) and permeability (here via soil hydraulic conductivity) is fundamental to assess the suitability of soils for infiltration and groundwater recharge.

Sandy areas such as N’guattakro and Koko are highly favorable for groundwater recharge, whereas clay-rich sectors like Dougouba and Baoulekro exhibit reduced potential. These observations are also consistent with the findings of Vogt et al. (2024), which show that infiltration and aquifer recharge are strongly influenced by soil texture and clay content, with sandy areas promoting water penetration while clay-rich sectors restrict flow.

Furthermore, Cui et al. (2008) emphasize that hydraulic conductivity directly influences the time required for water to infiltrate down to the aquifer. Overall, the spatial variability of hydraulic conductivity and effective porosity in M’Bahiakro reflects the lithological heterogeneity of the basin and governs the dynamics of the groundwater table. This heterogeneity must be considered in water resource management, as it determines zones that are favorable or unfavorable for recharge and influences water availability for domestic and agricultural uses.

3.2.3 Flow Velocity

The groundwater flow velocities estimated for the years 2012, 2013, and 2015 on the order of 10⁻⁵ m/s indicate a slow movement of water through the aquifers of M’Bahiakro. The slight decrease observed over time reflects a gradual slowdown of subsurface circulation, possibly linked to reduced recharge induced by climate variability, changes in the hydrodynamic properties of the medium, or increasing pressure on aquifers due to water extraction for domestic and agricultural purposes (Fan, 2015).

This low-flow dynamic implies that aquifers require more time to recharge after rainfall events, making them particularly vulnerable during prolonged periods of rainfall deficit. As a result, the water security of local populations is further weakened, as groundwater availability may significantly decrease during dry periods (Delle Rose, 2022). These observations highlight the need for integrated and cautious water-resource management, taking into account the slow recharge rates and the spatial and temporal variability of aquifer properties to prevent overexploitation and ensure sustainable groundwater supply for domestic and agricultural needs.

3.2.4 Hydrological Balance

The annual hydrological balance for the period 1971–1998 indicates that M’Bahiakro receives an average rainfall of 1680.09 mm, of which 62.07% is lost through actual evapotranspiration (1042.81 mm/year). This substantially limits the volume of water available for groundwater recharge and surface runoff (Penman, 1948; Thornthwaite, 1948; Oki & Kanae, 2006). Surface runoff remains low (51.81 mm/year, or 3.08% of rainfall), reflecting the basin’s moderate capacity to generate runoff, while infiltration (585.47 mm/year, 34.85% of rainfall) provides a significant contribution to aquifer recharge (Meyer et al., 2018; Uhlemann et al., 2022). These results indicate a relatively stable climatic context during this period, allowing aquifers to be regularly replenished and supporting water availability for domestic and agricultural uses (Delle Rose, 2022).

However, from 2012 onwards, analysis of rainfall, temperature, and groundwater flow velocity data reveals a progressive decline in aquifer recharge in M’Bahiakro. This decline results in reduced groundwater availability, underscoring the sensitivity of aquifers to climate variability. Rising temperatures increase evapotranspiration, while decreasing rainfall and low flow velocities slow down groundwater circulation (Fan, 2015; Meyer et al., 2018; Uhlemann et al., 2022); Delle Rose, 2022). These observations confirm that climate variability is a key factor influencing groundwater dynamics and vulnerability in M’Bahiakro.

These findings have important implications for water resource management in M’Bahiakro. The observed reduction in recharge suggests that reliance on groundwater without adaptive strategies could lead to water scarcity, particularly for domestic and agricultural uses. Sustainable management measures such as the promotion of rainwater harvesting, controlled groundwater abstraction, and the protection of recharge zones are therefore essential to maintain aquifer levels and ensure long-term water security in the face of climate variability.

CONCLUSION

The hydrological and hydrodynamic assessment in M’Bahiakro reveals that groundwater availability is closely linked to rainfall patterns, temperature variations, and aquifer properties. The aquifers of M’Bahiakro exhibit notable hydrodynamic heterogeneity. Hydraulic conductivity ranges from 7.1 × 10⁻⁵ to 1.5 × 10⁻⁴ m/s, with permeable zones in N’guattakro and Koko, and less permeable zones in Dougouba and Baoulekro. Effective porosity ranging between 20% and 40% is highest in the sandy areas of N’guattakro and Koko, promoting storage and infiltration, while the clay-dominated sectors of Baoulekro and Dougouba restrict water circulation. The 1971-1998 hydrological balance shows that 34.85% of rainfall contributes to groundwater recharge, while 62.07% is lost through evapotranspiration and surface runoff remains low. Groundwater flow velocities, on the order of 10⁻⁵ m/s, indicate a slow circulation dynamic, which has slowed even further since 2012 due to declining rainfall and rising temperatures-thereby reducing recharge and overall groundwater availability. Overall, these findings highlight the increasing vulnerability of groundwater resources in M’Bahiakro under the influence of climate variability, emphasizing the need for sustainable and adaptive water management strategies to ensure long-term water security for local populations.

These results highlight that groundwater availability depends both on the lithological characteristics of the aquifers and on climate variability, underscoring the need for integrated water-resources management in M’Bahiakro.

Limitations and Perspectives of the Study

Although this study provides essential insights into the dynamics of groundwater in M’Bahiakro, it presents several limitations:

- Data availability and continuity: The hydrological and climatic time series cover only specific periods (1971–1998 and 2012–2015), which restricts the analysis of long-term trends and the detection of major climatic shifts.

- Spatial representativeness of measurements: The hydraulic conductivity and effective porosity data come from only a few sites, which does not fully reflect the heterogeneity of aquifers at the regional scale.

- Empirical methods and unaccounted impacts: The methods used for the hydrological balance include approximations that may affect the accuracy of the estimates, and the study does not incorporate anthropogenic impacts such as groundwater abstraction, urbanization, or land-use changes.

- Perspectives for future studies: Integrating field data with climatic models (GCM, RCM) and hydrological models (SWAT, MODFLOW) would allow simulation of water fluxes, assessment of future groundwater recharge, and testing of sustainable management scenarios.

Thus, an integrated approach combining field observations, climate modelling, and hydrological simulation would provide a more comprehensive and forward-looking understanding of the vulnerability of groundwater resources in M’Bahiakro.

Competing interests

The authors declare that they have no competing interests.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

- Adhikari, R. K., Yilmaz, A. G., Mainali, B., Dyson, P., & Imteaz, M. A. (2022). Methods of Groundwater Recharge Estimation under Climate Change: A Review. Sustainability, 14(23), 15619. DOI: doi.org/10.3390/su142315619

- Alaoui, A. (2023). A Simple Method to Assess Key Soil Hydraulic Properties. Water, 15(3), 467. https://doi.org/10.3390/w15030467

- Baï, R., Kouame, I.K., Kouassı, L.K., Konan, S.K. and N’Cho, H.A. (2019) Assessment of the Physicochemical Quality of Irrigation Water and Soil for Sustainable Irrigated Rice Cultivation: Case of Irrigated Perimeter of M’Bahiakro (Central-East of Côte d’Ivoire). Journal of Environmental Protection, 10, 1536-1552. https://doi.org/10.4236/jep.2019.1011091

- Bagarello, V., Iovino, M., & Elrick, D. E. (2004). A simplified falling‑head technique for rapid determination of field‑saturated hydraulic conductivity. Soil Science Society of America Journal, 68(1), 66–73. https://doi.org/10.2136/sssaj2004.6600

- Bear, J. (1979). Hydraulics of groundwater (567 pp.). McGraw-Hill.

- Bodian, A. (2014). Caractérisation de la variabilité temporelle récente des précipitations annuelles au Sénégal (Afrique de l’Ouest). Physio‑Géo. Géographie physique et environnement, 8, 297‑312. https://doi.org/10.4000/physio-geo.4243

- Cui, Y.-J., Tang, A.-M., Loiseau, C., & Delage, P. (2008). Determining the unsaturated hydraulic conductivity of a compacted sand–bentonite mixture under constant-volume and free-swell conditions. Physics and Chemistry of the Earth, Parts A/B/C, 33(Supplement 1), S462–S471. https://doi.org/10.1016/j.pce.2008.10.017

- Delle Rose, M. (2022). Sinkhole flooding and aquifer recharge in arid to dry sub‑humid regions: Sensitivity to climate change. Hydrology, 9(2), 25. https://doi.org/10.3390/hydrology9020025

- Dosio, A., Lennard, C., & Spinoni, J. (2022). Projections of indices of daily temperature and precipitation based on bias adjusted CORDEX Africa regional climate model simulations. Climatic Change, 170, article 13. DOI: doi.org/10.1007/s10584-022-03307-0

- Fan, Y. (2015). Groundwater in the Earth's critical zone: Relevance to large scale patterns and processes. Water Resources Research, 51(5), 3052–3069. DOI: doi.org/10.1002/2015WR017037

- García Gutiérrez, C., Pachepsky, Y., & Martín, M. A. (2018). Technical note: Saturated hydraulic conductivity and textural heterogeneity of soils. Hydrology and Earth System Sciences, 22(7), 3923–3931. DOI: doi.org/10.5194/hess-22-3923-2018

- IPCC (2021). Climate Change 2021: The Physical Science Basis. Contribution of Working Group I to the Sixth Assessment Report of the Intergovernmental Panel on Climate Change [Masson-Delmotte, V., P. Zhai, A. Pirani, S.L. Connors, C. Péan, S. Berger, N. Caud, Y. Chen, L. Goldfarb, M.I. Gomis, M. Huang, K. Leitzell, E. Lonnoy, J.B.R. Matthews, T.K. Maycock, T. Waterfield, O. Yelekçi, R. Yu, and B. Zhou (eds.)]. Cambridge University Press, Cambridge, United Kingdom and New York, NY, USA, 2391 pp. DOI: doi:10.1017/9781009157896.

- Kafando, M. B., Koïta, M., Le Coz, M., Yonaba, O. R., Fowe, T., Zouré, C. O., Faye, M. D., & Leye, B. (2021). Use of Multidisciplinary Approaches for Groundwater Recharge Mechanism Characterization in Basement Aquifers: Case of Sanon Experimental Catchment in Burkina Faso. Water, 13(22), 3216. DOI: doi.org/10.3390/w13223216

- Keïta, A., Zorom, M., Faye, M. D., Damba, D. D., Konaté, Y., Hayde, L. G., & Lidon, B. (2023). Achieving Real-World Saturated Hydraulic Conductivity: Practical and Theoretical Findings from Using an Exponential One-Phase Decay Model. Hydrology, 10(12), 235. DOI: doi.org/10.3390/hydrology10120235

- Kouakou, E., Koné, B., N’Go, A., Cissé, G., Ifejika Speranza, C., & Savané, I. (2014). Ground water sensitivity to climate variability in the White Bandama basin, Ivory Coast. SpringerPlus, 3, 226. doi: 10.1186/2193-1801-3-226. eCollection 2014

- Kouassi, K.L., Kouame, K.I., Konan, K.S., Dibi, B., Soumahoro, M., Kone, D. and Dago, G. (2013). Prospects for Minimizing the Potential Environmental Impacts of the Hydro-Agricultural Dam of M’Bahiakro (Côte d’Ivoire). Journal of Water Re source and Protection, 5, 847-853. DOI: doi.org/10.4236/jwarp.2013.59085

- Lai, J., & Ren, L. (2007). Assessing the size dependency of measured hydraulic conductivity using double ring infiltrometers and numerical simulation. Soil Science Society of America Journal, 71(6), 1667–1675. DOI: doi.org/10.2136/sssaj2006.0227

- Magaji Ibrahim, M., Ibrahim, M., & Mubi Aisha, M. (2024). Groundwater storage variability in West Africa using gravity recovery and climate experiment (GRACE) and global land data assimilation system (GLDAS) data. EQA - International Journal of Environmental Quality, 64, 87–102. DOI: doi.org/10.6092/issn.2281-4485/20167

- Meyer, R., Engesgaard, P., Hinsby, K., Piotrowski, J. A., and Sonnenborg, T. O. (2018). Estimation of effective porosity in large-scale groundwater models by combining particle tracking, auto-calibration and 14C dating, Hydrol. Earth Syst. Sci., 22, 4843–4865. DOI: doi.org/10.5194/hess-22-4843-2018, 2018.

- N’cho, H.A., Koffi, K., Konan, S.K., Ruth Baï., Innocent Kouassi Kouame., Lazare Kouakou Kouassi (2024). Simulation of the Nitrate Concentrations in Consumed Well Water Using the Error Gradient Backpropagation Neural Network: A Case Study: M’bahiakro (Central-Eastern Ivory Coast). Environ Model Assess 29, 855–869 (2024). DOI: doi.org/10.1007/s10666-024-09970-0

- New, M., Hewitson, B., Stephenson, D. B., Tsiga, A., Kruger, A., Manhique, A., Gomez, B., Coelho, C. A. S., Masisi, D. N., Kululanga, E., Mbambalala, E., Adesina, F., Saleh, H., Kanyanga, J., Adosi, J., Bulane, L., Fortunata, L., Mdoka, M. L., & Lajoie, R. (2006). Evidence of trends in daily climate extremes over southern and west Africa. Journal of Geophysical Research: Atmospheres, 111(D14), D14102. DOI: doi.org/10.1029/2005JD006289

- Nicholson, S.E. (1998). Historical Fluctuations of Lake Victoria and Other Lakes in the Northern Rift Valley of East Africa. In: Lehman, J.T. (eds) Environmental Change and Response in East African Lakes. Monographiae Biologicae, vol 79. Springer, Dordrecht. DOI: doi.org/10.1007/978-94-017-1437-2_2

- Nicholson, S. E. (2000). The nature of rainfall variability over Africa on time scales of decades to millennia. Global and Planetary Change, 26(1–3), 137–158. DOI: doi.org/10.1016/S0921-8181(00)00040-0

- Nicholson, S. E. (2005). On the question of the “recovery” of the rains in the West African Sahel. Journal of Arid Environments, 63(3), 615–641. DOI: doi.org/10.1016/j.jaridenv.2005.03.004

- Omar, G. M., Paturel, J.-E., Salles, C., Mahé, G., & Jalludin, M. (2021). Caractérisation hydro climatique, analyse comparative des termes du bilan hydrologique du bassin versant d'Ambouli (République de Djibouti). Proceedings of the International Association of Hydrological Sciences, 384, 225–231. DOI: doi.org/10.5194/piahs-384-225-2021

- Oki, T., & Kanae, S. (2006). Global hydrological cycles and world water resources. Science, 313(5790), 1068–1072. DOI : doi.org/10.1126/science.1128845

- Pakparvar, M., Hashemi, H., Rezaei, M., Cornelis, W. M., Nekooeian, G., & Kowsar, S. A. (2018). Artificial recharge efficiency assessment by soil water balance and modelling approaches in a multi-layered vadose zone in a dry region. Hydrological Sciences Journal, 63(8), 1183–1202. DOI: doi.org/10.1080/02626667.2018.1481962

- Penman, H. L. (1948). Natural evaporation from open water, bare soil and grass. Proceedings of the Royal Society of London. Series A, Mathematical and Physical Sciences, 193(1032), 120–145. DOI: doi.org/10.1098/rspa.1948.0037

- Sharma, T. C. (1988). An evaluation of evapotranspiration in tropical central Africa. Hydrological Sciences Journal, 33(1), 31–40. DOI: doi.org/10.1080/02626668809491220

- Soro, G., Soro, N., Ahoussi, K. E., Lasm, T., Kouamé, F. K., Soro, T. D., & Biémi, J. (2010). Assessment of the hydraulic properties of fractured aquifers in crystalline and metamorphic formations in the region of Lacs (Central Cote d'Ivoire) ; Evaluation des proprietes hydrauliques des aquiferes fractures des formations cristalline et metamorphique dans la region des Lacs (centre de la Cote d'Ivoire). Estudios Geológicos, 66(2), 227–242. DOI : doi.org/10.3989/egeol.40123.093

- Soro, T. D., Djè, K. B., Soro, N., Oga, M. S., & Soro, G. (2013). Climate variability impact on groundwater resources in the Highest Bandama watershed at Tortiya (Northern Côte d’Ivoire). American Journal of Environmental Protection, 2(4), 103–112. DOI : doi.org/10.11648/j.ajep.20130204.11

- Sylla, M.B., Nikiema, P.M., Gibba, P., Kebe, I., Klutse, N.A.B. (2016). Climate Change over West Africa: Recent Trends and Future Projections. In: Yaro, J., Hesselberg, J. (eds) Adaptation to Climate Change and Variability in Rural West Africa. Springer, Cham. DOI : doi.org/10.1007/978-3-319-31499-0_3

- Tagne, M. M., Simo-Tagne, M., Nwakuba, N. R., Effoudou-Priso, E. N., Ndam Ngoupayou, J. R., Mbessa, M., Tagne Tagne, A., & Kharchi, R. (2023). Influence of infiltrations on the recharge of the Nkoabang aquifer located in the Center Region, Cameroon: a Thornthwaite water-balance analysis. Earth, 4(1), 23–39. DOI : doi.org/10.3390/earth4010002

- Tefera, M. L., Seddaiu, G., Carletti, A., & Awada, H (2025). Rainfall variability and drought in West Africa: challenges and implications for rainfed agriculture. Theor Appl Climatol 156, 41. DOI: doi.org/10.1007/s00704-024-05251-8

- Thornthwaite, C. W. (1948). An Approach toward a Rational Classification of Climate. Geographical Review, 38(1), 55–94. https://doi.org/10.2307/210739/ https://www.jstor.org/stable/210739

- Tirogo, J., Jost, A., Biaou, A., Koussoubé, Y., Ribstein, P., & Dakouré, D. (2023). Impacts of climate change and pumping on groundwater resources in the Kou River basin, Burkina Faso. Comptes Rendus Géoscience, 355(S1), 439–464. DOI: 10.5802/crgeos.207

- Todote, V., Lyra, G., & Gabreu, M. (2021). Climatological water balance and Thornthwaite–Mather climate classification for Benin, West Africa, during the period 1970–2015. Revista Engenharia na Agricultura – Reveng, 29, 291–302. DOI : doi.org/10.13083/reveng. v29i1.12387

- Uhlemann, S., Ulrich, C., Newcomer, M., Fiske, P., Kim, J., & Pope, J. (2022). 3D hydrogeophysical characterization of managed aquifer recharge basins. Frontiers in Earth Science, 10, 942737. DOI : doi.org/10.3389/feart.2022.942737

- Vogt, M.-L. A., Zwahlen, F., Pera, S., Mahamat, H. B., Hunkeler, D., & Brunner, P. (2024). Infiltration and recharge dynamics in the Nubian Sandstone Aquifer System of northern Chad. Hydrogeology Journal, 32, 417-431. DOI: doi.org/10.1007/s10040-024-02765-3

- Zou, C., Zhao, L., Xu, M., Chen, Y., & Geng, J. (2021). Porosity prediction with uncertainty quantification from multiple seismic attributes using Random Forest. Journal of Geophysical Research: Solid Earth, 126(7), e2021JB021826. DOI: doi.org/10.1029/2021JB021826