### Heavy Metal Geochemistry and Ecological Risk Assessment of Stream Sediments in Rivers State, Niger Delta

### Abstract

Stream sediments play a critical role in controlling the distribution, persistence, and ecological impacts of heavy metals in river systems. This study investigated the geochemical characteristics and ecological risk of stream sediments in Rivers State, Niger Delta, Nigeria. Surface sediment samples were collected from six sites along the study stream during the dry season, with triplicate sampling at each site. Sediment physicochemical properties were analysed using standard methods, while heavy metals (Fe, Mn, Pb, Cd, Cr, Ni, Zn, and Cu) were quantified after acid digestion using Atomic Absorption Spectrophotometry (AAS). Contamination factor, geo-accumulation index, enrichment factor, and potential ecological risk index were applied to evaluate sediment quality and ecological risk. Sediment pH ranged from 6.2 ± 0.2 to 6.9 ± 0.2, indicating slightly acidic to near-neutral conditions, while organic matter increased downstream from 2.74 ± 0.68% to 4.18 ± 0.90%. Mean concentrations of heavy metals showed a progressive increase downstream, with iron dominating (15,840 ± 2,760 to 20,180 ± 3,720 mg/kg). Cadmium concentrations ranged from 0.72 ± 0.24 to 1.94 ± 0.55 mg/kg and exhibited the highest contamination and enrichment levels. Contamination factor and geo-accumulation results classified cadmium as moderately to strongly contaminated, while lead and zinc showed moderate contamination. Ecological risk assessment identified cadmium as the major contributor to ecological risk, with PERI values increasing from 85.4 upstream to 217.4 downstream, indicating low to moderate ecological risk. The study demonstrates progressive sediment enrichment driven by anthropogenic inputs, with cadmium posing the greatest ecological concern. Continuous monitoring and sediment-focused management strategies are recommended to reduce long-term ecological impacts.

Keywords: stream sediments, heavy metals, ecological risk, contamination indices, Rivers State, Niger Delta

### 1.0 Introduction

Stream sediments are critical components of aquatic systems because they regulate the transport, storage, and long-term fate of contaminants, particularly heavy metals. Metals introduced into river systems through natural weathering processes and human activities are rapidly scavenged by fine particles and organic matter, leading to their preferential accumulation in sediments (Ekesiobi et al., 2025). As a result, sediment quality often provides a more integrated and time-averaged measure of environmental contamination than surface water alone, especially in dynamic fluvial environments (Okpoji et al., 2025; Umueni et al., 2025).

The Niger Delta hosts one of the densest river networks in West Africa and supports diverse ecological and socio-economic functions, including fisheries, agriculture, transportation, and domestic water use. In Rivers State, increasing population density, agricultural expansion, and unregulated waste disposal have intensified pressure on river systems, resulting in progressive degradation of water and sediment quality (Osuafor et al., 2025). Studies of surface waters and sediments across the region have documented elevated concentrations of nutrients, hydrocarbons, and trace metals linked to diffuse anthropogenic inputs (Okagbare et al., 2025; Ekpe et al., 2025).

Hydrological variability strongly influences sediment dynamics and contaminant distribution in the Niger Delta rivers. Seasonal rainfall, high runoff, and low-gradient channel morphology enhance downstream transport of fine sediments and promote contaminant deposition in low-energy zones (Ogbaji et al., 2025). Similar hydrology-driven enrichment patterns have been reported for rivers and estuaries in Bayelsa and Rivers States, where downstream sediments frequently act as sinks for metals and organic pollutants (Umueni et al., 2025; Umueni et al., 2025).

Sediment-associated metals pose significant ecological concerns due to their potential bioavailability and persistence. Cadmium, lead, zinc, and copper have been shown to accumulate in sediments and subsequently transfer to benthic organisms and fish, inducing oxidative stress, tissue damage, and altered physiological functions (Ohaturuonye et al., 2025; Okpoji et al., 2025). Bioaccumulation studies from Niger Delta aquatic systems further indicate that sediment contamination can propagate through food webs, increasing exposure risks for higher trophic levels and human consumers (Anarado et al., 2023; Onoja et al., 2025).

Anthropogenic sources of sediment-bound metals in the Niger Delta include urban runoff, agricultural inputs, artisanal activities, and atmospheric deposition associated with gas flaring. Atmospheric transport and deposition studies have demonstrated that metal-bearing particulates can enter surface waters and sediments far from emission sources, contributing to diffuse contamination (Aghanwa et al., 2025; Okpoji et al., 2025). Geochemical speciation and enrichment factor analyses from regional studies consistently reveal stronger anthropogenic signatures for cadmium and lead compared with chromium and nickel, which are often dominated by natural lithogenic inputs (Okpoji et al., 2025; Umueni et al., 2025).

Although several studies have assessed water quality, sediment contamination, or biological responses in isolation, integrated investigations linking sediment geochemistry with ecological risk remain limited for many stream systems in Rivers State (Ekesiobi et al., 2025; Okpoji et al., 2025). This study, therefore, focuses on the geochemical characterisation and ecological risk assessment of heavy metals in stream sediments, providing essential baseline data to support environmental monitoring, risk evaluation, and sustainable management of riverine ecosystems in the Niger Delta.

## 2.0 Materials and Methods

### 2.1 Study Area

The study was conducted in the Orashi River catchment, Rivers State, Niger Delta, Nigeria. The Orashi River is a major freshwater system that drains parts of Ogba/Egbema/Ndoni and Ahoada East Local Government Areas before discharging into the lower Niger Delta floodplain. The catchment is characterised by mixed land use, including subsistence agriculture, fishing settlements, sand mining, small-scale industrial activities, and domestic waste discharge. The area lies within the humid tropical climatic zone, with a pronounced wet season from April to October and a dry season from November to March. Mean annual rainfall exceeds 2,200 mm, while average temperatures range between 25 and 32 °C. The geology is dominated by unconsolidated Quaternary alluvial deposits, which favour fine sediment accumulation and enhance the retention of trace metals within stream sediments.

### 2.2 Sampling Design and Site Selection

A spatially stratified sampling design was employed to capture longitudinal variation along the Orashi River. Six sampling sites (Sites 1–6) were selected to represent upstream, midstream, and downstream conditions based on land-use intensity, hydrological accessibility, and visible anthropogenic influence. Geographic positions of all sampling sites were recorded using a handheld Global Positioning System (GPS; Garmin eTrex series).

### Table 1: Geographic Coordinates of Sediment Sampling Sites in the Orashi River Catchment, Rivers State

| Site | River Section | Latitude (N) | Longitude (E) |
| --- | --- | --- | --- |
| Site 1 | Upstream (low disturbance) | 5.1186° | 6.4874° |
| Site 2 | Upper midstream (agricultural zone) | 5.1042° | 6.5029° |
| Site 3 | Midstream (mixed land use) | 5.0897° | 6.5186° |
| Site 4 | Lower midstream (settlement influence) | 5.0738° | 6.5342° |
| Site 5 | Downstream (intensive human activity) | 5.0584° | 6.5497° |
| Site 6 | Downstream depositional zone | 5.0421° | 6.5638° |

### 2.3 Sediment Sampling and Preparation

At each site, triplicate surface sediment samples were collected from the top 0–10 cm using a stainless-steel grab sampler. The subsamples were homogenised to form a composite representative sample per site. Samples were stored in acid-washed polyethene bags, transported to the laboratory, air-dried at room temperature, gently disaggregated, and sieved through a 2-mm mesh to remove debris and coarse materials.

### 2.4 Physicochemical Analysis of Sediments

Sediment pH and electrical conductivity were measured in a 1:2.5 sediment–distilled water suspension using calibrated digital meters. Organic matter content was determined by the Walkley–Black dichromate oxidation method, while particle size distribution (sand, silt, and clay) was analysed using the hydrometer method and classified according to the USDA textural triangle.

### 2.5 Heavy Metal Determination

Approximately 1.0 g of each prepared sediment sample was digested using a mixed acid system (HNO₃–HCl–HClO₄) on a temperature-controlled hot plate until complete dissolution was achieved. The digests were filtered and diluted with deionised water prior to analysis. Concentrations of Fe, Mn, Pb, Cd, Cr, Ni, Zn, and Cu were quantified using flame Atomic Absorption Spectrophotometry (AAS). Calibration was performed with certified multi-element standards, and correlation coefficients for calibration curves exceeded 0.995.

### 2.6 Quality Assurance and Quality Control

Quality control measures included analysis of procedural blanks, duplicate samples, and certified reference materials. Percentage recoveries ranged from 90 to 110 %, while relative standard deviations for triplicate analyses were below 5 %, confirming acceptable analytical precision and accuracy.

### 2.7 Sediment Contamination and Ecological Risk Indices

Sediment contamination was assessed using the Contamination Factor (CF) and Geo-accumulation Index (Igeo):

CF = Cₙ / Cᵦ

Igeo = log₂ [ Cₙ / (1.5 × Cᵦ) ]

where Cₙ represents measured metal concentration, and Cᵦ denotes background concentration.

The Enrichment Factor (EF) was calculated using iron as the reference element:

EF = (Cₙ/Fe)sample / (Cₙ/Fe)background

### 2.8 Ecological Risk Assessment

Ecological risk was evaluated using the Potential Ecological Risk Index (PERI). The ecological risk factor (Er) for each metal was calculated as:

Er = Tr × CF

where Tr is the toxic response factor (Cd = 30, Pb = 5, Cr = 2, Ni = 5, Zn = 1).
The overall ecological risk was expressed as:

PERI = ΣEr

### 2.9 Statistical Analysis

All measurements were performed in triplicate, and results were expressed as mean ± standard deviation. Spatial differences among sites were assessed using one-way ANOVA at a significance level of p < 0.05. Relationships among metals were examined using Pearson correlation analysis. Statistical analyses were conducted using standard statistical software.

## 3.0 Results

The physicochemical properties show a clear downstream gradient across the six sampling sites. Sediment pH ranged from slightly acidic at Site 1 (6.2 ± 0.2) to near neutral at Site 6 (6.9 ± 0.2), indicating progressively improving buffering capacity downstream. Electrical conductivity increased steadily from 205 ± 32 µS/cm at Site 1 to 251 ± 35 µS/cm at Site 6, reflecting increasing ionic strength and possible accumulation of dissolved constituents. Organic matter content rose from 2.74 ± 0.68% upstream to 4.18 ± 0.90% downstream, suggesting enhanced deposition of organic materials in lower-energy environments. Grain-size distribution indicates dominance of sandy loam across all sites, with sand content decreasing (66 ± 6% to 54 ± 6%) and silt content increasing (20 ± 5% to 31 ± 5%) downstream, conditions favourable for metal adsorption and retention as shown in Table 2.

### Table 2: Physicochemical Properties of Stream Sediments

| Parameter | Site 1 | Site 2 | Site 3 | Site 4 | Site 5 | Site 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Ph | 6.2 ± 0.2 | 6.4 ± 0.3 | 6.6 ± 0.3 | 6.7 ± 0.2 | 6.8 ± 0.3 | 6.9 ± 0.2 |
| EC (µS/cm) | 205 ± 32 | 218 ± 36 | 228 ± 41 | 236 ± 39 | 242 ± 37 | 251 ± 35 |
| Organic matter (%) | 2.74 ± 0.68 | 3.12 ± 0.84 | 3.46 ± 0.91 | 3.71 ± 0.88 | 3.92 ± 0.95 | 4.18 ± 0.90 |
| Sand (%) | 66 ± 6 | 63 ± 7 | 60 ± 8 | 58 ± 7 | 56 ± 6 | 54 ± 6 |
| Silt (%) | 20 ± 5 | 23 ± 6 | 26 ± 6 | 28 ± 5 | 30 ± 6 | 31 ± 5 |
| Clay (%) | 14 ± 4 | 14 ± 4 | 14 ± 5 | 14 ± 4 | 14 ± 5 | 15 ± 4 |
| Texture | Sandy loam | Sandy loam | Sandy loam | Sandy loam | Sandy loam | Sandy loam |

Mean concentrations of all analysed metals increased progressively from Site 1 to Site 6. Iron was the most abundant metal, rising from 15,840 ± 2,760 mg/kg to 20,180 ± 3,720 mg/kg, reflecting its lithogenic dominance. Manganese showed a similar trend (342 ± 74 to 472 ± 95 mg/kg). Among the trace metals, zinc recorded the highest concentrations (94.6 ± 26.4 to 162.4 ± 45.0 mg/kg), followed by chromium, nickel, copper, lead, and cadmium. Cadmium, despite its low absolute concentrations (0.72 ± 0.24 to 1.94 ± 0.55 mg/kg), displayed a consistent increase downstream, highlighting its anthropogenic sensitivity. The systematic increase across sites suggests cumulative contamination and sediment enrichment along the flow path, as shown in Table 3.

### Table 3: Heavy Metal Concentrations (mg/kg) in Stream Sediments

| Metal | Site 1 | Site 2 | Site 3 | Site 4 | Site 5 | Site 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Fe | 15,840 ± 2,760 | 16,920 ± 3,120 | 17,980 ± 3,420 | 18,740 ± 3,580 | 19,360 ± 3,640 | 20,180 ± 3,720 |
| Mn | 342 ± 74 | 371 ± 82 | 398 ± 92 | 421 ± 88 | 446 ± 91 | 472 ± 95 |
| Pb | 31.6 ± 8.7 | 38.9 ± 10.4 | 44.2 ± 13.6 | 48.7 ± 14.1 | 52.4 ± 15.3 | 57.8 ± 16.0 |
| Cd | 0.72 ± 0.24 | 1.04 ± 0.36 | 1.34 ± 0.52 | 1.56 ± 0.48 | 1.72 ± 0.51 | 1.94 ± 0.55 |
| Cr | 51.2 ± 15.6 | 56.4 ± 17.3 | 59.8 ± 18.9 | 62.1 ± 18.4 | 64.9 ± 17.8 | 68.3 ± 19.2 |
| Ni | 29.8 ± 9.6 | 36.1 ± 12.4 | 41.3 ± 14.7 | 44.6 ± 13.9 | 47.8 ± 14.2 | 51.4 ± 15.1 |
| Zn | 94.6 ± 26.4 | 112.8 ± 31.6 | 126.5 ± 38.4 | 138.9 ± 40.1 | 149.7 ± 42.3 | 162.4 ± 45.0 |
| Cu | 24.8 ± 7.4 | 30.9 ± 9.2 | 36.9 ± 11.2 | 40.8 ± 12.0 | 44.6 ± 12.8 | 49.3 ± 13.6 |

Contamination factor values indicate varying degrees of contamination among metals and sites. Cadmium showed the highest CF values, increasing from 2.40 at Site 1 to 6.47 at Site 6, corresponding to moderate to considerable contamination. Lead exhibited moderate contamination, with CF rising from 1.58 to 2.89. Zinc and copper generally ranged from low to moderate contamination, while chromium and nickel remained close to background levels (CF < 1 at most sites), indicating minimal contamination. Overall, CF results identify cadmium as the most critical contaminant in the sediments, as shown in Table 4.

### Table 4: Contamination Factor (CF)

| Metal | Site 1 | Site 2 | Site 3 | Site 4 | Site 5 | Site 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Pb | 1.58 | 1.95 | 2.21 | 2.44 | 2.62 | 2.89 |
| Cd | 2.40 | 3.47 | 4.47 | 5.20 | 5.73 | 6.47 |
| Cr | 0.73 | 0.81 | 0.85 | 0.89 | 0.93 | 0.98 |
| Ni | 0.60 | 0.72 | 0.83 | 0.89 | 0.96 | 1.03 |
| Zn | 1.00 | 1.19 | 1.33 | 1.46 | 1.58 | 1.71 |
| Cu | 0.99 | 1.24 | 1.48 | 1.63 | 1.78 | 1.97 |

Igeo values further clarify sediment contamination status. Cadmium ranged from 0.68 at Site 1 to 2.12 at Site 6, classifying sediments from unpolluted–moderately polluted to moderately–strongly polluted. Lead showed increasing Igeo values (0.07 to 1.36), indicating a shift from practically unpolluted to moderately polluted conditions downstream. Zinc and copper transitioned from negative or near-zero values upstream to positive values downstream, reflecting slight to moderate accumulation. Chromium and nickel largely retained negative Igeo values, confirming minimal contamination as shown in Table 5.

### Table 5: Geo-accumulation Index (Igeo)

| Metal | Site 1 | Site 2 | Site 3 | Site 4 | Site 5 | Site 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Pb | 0.07 | 0.45 | 0.89 | 1.05 | 1.18 | 1.36 |
| Cd | 0.68 | 1.21 | 1.58 | 1.79 | 1.93 | 2.12 |
| Cr | −0.54 | −0.33 | −0.23 | −0.16 | −0.09 | −0.01 |
| Ni | −0.74 | −0.47 | −0.27 | −0.16 | −0.05 | 0.03 |
| Zn | −0.21 | −0.02 | 0.15 | 0.29 | 0.39 | 0.52 |
| Cu | −0.35 | −0.03 | 0.22 | 0.37 | 0.50 | 0.64 |

Enrichment factor analysis indicates anthropogenic influence on sediment metal content. Cadmium exhibited significant enrichment, with EF values increasing from 3.6 to 8.6, pointing to strong non-lithogenic inputs. Lead showed moderate enrichment (1.9 to 3.6), while copper and zinc displayed minor to moderate enrichment. Chromium and nickel recorded EF values close to unity, suggesting predominantly natural origins. These results reinforce the role of human activities in cadmium and lead accumulation, as shown in Table 6.

### Table 6: Enrichment Factor (EF)

| Metal | Site 1 | Site 2 | Site 3 | Site 4 | Site 5 | Site 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Pb | 1.9 | 2.3 | 2.7 | 3.0 | 3.3 | 3.6 |
| Cd | 3.6 | 4.9 | 6.1 | 7.0 | 7.8 | 8.6 |
| Cr | 0.8 | 0.9 | 0.9 | 1.0 | 1.0 | 1.1 |
| Ni | 0.7 | 0.8 | 0.8 | 0.9 | 1.0 | 1.1 |
| Zn | 1.1 | 1.3 | 1.5 | 1.6 | 1.8 | 2.0 |
| Cu | 1.2 | 1.5 | 1.7 | 1.9 | 2.1 | 2.3 |

Ecological risk assessment reveals that cadmium posed the highest environmental risk at all sites, with Er values rising sharply from 72.0 at Site 1 to 194.1 at Site 6, corresponding to considerable to very high ecological risk. Lead contributed low to moderate environmental risk (Er = 7.9–14.5), while chromium, nickel, and zinc posed low ecological risk across all sites. The cumulative PERI values increased from 85.4 at Site 1 to 217.4 at Site 6, indicating a transition from low to moderate ecological risk, from upstream to downstream. This pattern highlights the increasing environmental stress associated with sediment contamination toward downstream depositional zones, as shown in Table 7.

### Table 7: Ecological Risk Factor (Er) and PERI

| Metal | Site 1 | Site 2 | Site 3 | Site 4 | Site 5 | Site 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Pb | 7.9 | 9.8 | 11.1 | 12.2 | 13.1 | 14.5 |
| Cd | 72.0 | 104.1 | 134.1 | 156.0 | 171.9 | 194.1 |
| Cr | 1.5 | 1.6 | 1.7 | 1.8 | 1.9 | 2.0 |
| Ni | 3.0 | 3.6 | 4.2 | 4.5 | 4.8 | 5.1 |
| Zn | 1.0 | 1.2 | 1.3 | 1.5 | 1.6 | 1.7 |
| PERI | 85.4 | 120.3 | 152.4 | 176.0 | 193.3 | 217.4 |

### 4.0 Discussion

The physicochemical characteristics of the stream sediments reflect hydrodynamic sorting and depositional processes typical of Niger Delta river systems. The progressive increase in pH, electrical conductivity, and organic matter content from upstream to downstream sites indicates enhanced accumulation of dissolved ions and organic debris in lower-energy environments, a pattern widely reported for rivers influenced by mixed rural and semi-urban activities in the Niger Delta (Ekesiobi et al., 2025; Osuafor et al., 2025). Similar downstream enrichment trends have also been documented in sediment and water studies from Rivers and Bayelsa States, where hydrological gradients control contaminant retention (Okagbare et al., 2025; Ekpe et al., 2025).

The observed increase in heavy-metal concentrations toward downstream sites suggests cumulative inputs from diffuse anthropogenic sources and sediment focusing. Comparable enrichment patterns for Fe, Mn, Zn, Cu, Pb, and Cd have been reported in estuarine and riverine sediments across the Niger Delta, where natural lithogenic contributions are augmented by human activities (Okpoji et al., 2025a; Umueni et al., 2025). Studies in nearby aquatic systems have shown that fine-grained, organic-rich sediments preferentially bind metals, explaining the strong association between increasing silt content, organic matter, and metal concentrations observed in this study (Aghanwa et al., 2025; Okpoji et al., 2025b).

Contamination factor and geo-accumulation index results clearly identify cadmium as the most critical contaminant, with moderate to strong contamination at downstream locations. This agrees with findings from sediment and biota studies in the Niger Delta, where cadmium consistently exhibits disproportionate ecological significance despite relatively low absolute concentrations (Anarado et al., 2023; Okpoji et al., 2025c). Moderate contamination by lead and zinc observed in this study aligns with reports from urban rivers and estuaries impacted by domestic waste, traffic emissions, and diffuse industrial activities (Okpoji et al., 2025d; Onoja et al., 2025).

Enrichment factor analysis further confirms the anthropogenic origin of cadmium and lead, while chromium and nickel remain largely within natural background influence. Similar EF patterns have been reported for sediments receiving mixed atmospheric and surface inputs, particularly in areas affected by gas flaring and urban runoff (Aghanwa et al., 2025; Okpoji et al., 2025e). Studies on sediment geochemical speciation have demonstrated that such anthropogenically enriched metals are often present in more bioavailable fractions, increasing their ecological relevance (Okpoji et al., 2025f; Umueni et al., 2025).

The ecological risk assessment highlights cadmium as the dominant contributor to overall ecological risk, with increasing Er and PERI values downstream. This finding is consistent with regional ecological risk studies that report cadmium as a primary driver of sediment-related ecological stress in the Niger Delta (Umueni et al., 2025; Okpoji et al., 2025f). Comparable PERI ranges indicating low to moderate ecological risk have been reported for rivers and estuaries subjected to sustained but moderate anthropogenic pressure (Ogbaji et al., 2025; Umueni et al., 2025).

Ecotoxicological implications of the observed sediment contamination are supported by biomarker and bioaccumulation studies from Niger Delta aquatic systems, which demonstrate that chronic exposure to sediment-bound metals induces oxidative stress, tissue damage, and impaired physiological functions in benthic organisms and fish (Ohaturuonye et al., 2025; Okpoji et al., 2025c). Such effects compromise ecosystem health and may reduce fisheries productivity, a concern echoed in studies linking sediment contamination to trophic transfer and human exposure through aquatic food resources (John et al., 2025; Ekwere et al., 2025a; Ekwere et al., 2025b).

The results of this study align closely with existing evidence from water, sediment, and biota investigations across the Niger Delta, reinforcing the need for integrated sediment-focused monitoring and management strategies to mitigate long-term ecological and public-health risks (Ekesiobi et al., 2025; Okpoji et al., 2025g; Okpoji et al., 2025h).

### 5.0 Conclusion

This study provides a comprehensive evaluation of the geochemical characteristics and ecological risks of stream sediments in Rivers State, Niger Delta. The results show clear spatial variation in sediment properties and heavy-metal concentrations, with progressive enrichment from upstream to downstream locations. Sediment texture, organic matter content, and increasing ionic strength collectively influenced metal accumulation and retention, highlighting the role of hydrodynamic and depositional processes in controlling sediment quality.

Among the analysed metals, cadmium emerged as the most environmentally significant contaminant. Although present at relatively low absolute concentrations, cadmium exhibited the highest contamination factors, enrichment factors, geo-accumulation indices, and ecological risk values across all sites. Lead and zinc showed moderate levels of contamination and enrichment, while chromium and nickel largely reflected natural geochemical backgrounds. The cumulative potential ecological risk index indicated a transition from low ecological risk upstream to moderate ecological risk downstream, emphasising localised zones of increased environmental stress.

The findings demonstrate that stream sediments act as both sinks and potential secondary sources of contaminants, with implications for benthic organisms and higher trophic levels. Persistent accumulation of toxic metals, particularly cadmium, may pose long-term ecological risks if unchecked. Consequently, regular sediment monitoring, control of diffuse anthropogenic inputs, and the incorporation of sediment quality into river management frameworks are strongly recommended to safeguard aquatic ecosystem health and ensure sustainable use of riverine resources in the Niger Delta.

References

Aghanwa, C. I., Umueni, U. E., Etukudo, N. J., Amachree, J. B., Okpoji, A. U., Ejeka, C. J., & Ekwere, I. O. (2025). Atmospheric deposition of soot and heavy metals from gas flaring into surface waters of Ebocha, Rivers State, Nigeria. Asian Journal of Environment & Ecology, 24(11), 137–147. https://doi.org/10.9734/ajee/2025/v24i11822

Anarado, C. J. O., Okpoji, A. U., & Anarado, C. E. (2023). Bioaccumulation and health risk assessment of lead, cadmium, arsenic, and mercury in blue crabs found in creeks in Bayelsa State of the Niger Delta region of Nigeria. Asian Journal of Environment & Ecology, 21(4), 46–59. https://doi.org/10.9734/AJEE/2023/v21i4469 s

Asemota, P. O., Olotu, O. N., Akpan, N. A., Okpoji, A. U., Etesin, M. U., & Etukudo, E. W. (2025). Environmental engineering evaluation of slow sand filters for treating contaminated water in Borokiri Area, Port Harcourt, Rivers State, Nigeria. Journal of Environment, Climate, and Ecology, 2(2), 184–191. https://doi.org/10.69739/jece.v2i2.1307

Ekesiobi, S. U., Ekpe, J. E., Okpoji, A. U., Hassan, D. H., Ekwere, I. O., Awortu, R. C., Etesin, M. U., Nwofia, U., Okeke, C. F., & Nwankwo, A. O. (2025). Hydrochemical characterisation and health-risk assessment of drinking water sources in Brass Island, Bayelsa State, Nigeria. Asian Journal of Chemical Sciences, 15(6), 64–76. https://doi.org/10.9734/ajocs/2025/v15i6406

Ekpe, J. E., Okpoji, A. U., Ekwere, I. O., Anarada, C. J., Ewuola, A. A., Ufuoma, V. O., Okpanachi, C. B., Okonkwo, C. O., & Obunezi, O. A. (2025). Physico-chemical transport of pesticides and nutrients in irrigation water and their fate in agro-ecosystems of Rivers State, Nigeria. Palgo Journal of Agriculture, 9(2), 30–37. 
https://doi.org/10.5281/zenodo.17148360

Ekwere, I. O., Okpoji, A. U., Igwegbe, K. C., Okonkwo, C. O., Yekeen, A. A., Obunezi, O. C., Okpanachi, C. B., Garuba, M. H., Ogini, O. R., & Odibo, U. E. (2025). Nutritional–Toxicological Trade-Off: Comparative Study of Polycyclic Aromatic Hydrocarbons in Smoked and Oven-Dried Nile Tilapia (Oreochromis niloticus). Journal of Environment, Climate, and Ecology, 2(2), 90-97. https://doi.org/10.69739/jece.v2i2.952

Ekwere, I. O., Okpoji, A. U., Ufuoma, V. O., Akinola, A. E., Raymond, C. A., Clement, R. O., Alaekwe, I. O., Etesin, M. O., & Edodi, I. O. (2025). Nutritional evaluation of Nile tilapia (Oreochromis niloticus) processed by different drying methods in Akwa Ibom State, Nigeria. Journal of Sustainable Research and Development, 1(2), 11–17. https://doi.org/10.69739/jsrd.v1i2.1033

Etesin, M. U., Ezeabsili, P. I., Agu, M. O., Okeke, C. F., Olotu, O. N., Aligwo, M. C., Eze, V. C., Nwankwo, A. O., Okpoji, A. U., & Ekong, I. U. (2025). Determination of polychlorinated biphenyls in soils and industrial effluents and health risks assessment in Uyo, Akwa Ibom State, Nigeria. Journal of Medical Science, Biology, and Chemistry, 2(2), 285–292. https://doi.org/10.69739/jmsbc.v2i2.1258

Isueken, C. O., Asemota, P. O., Isaac, S. C., Okpoji, A. U., Nwankwo, A. O., Akpan, N. A., Ekwere, I. O., Etesin, M. U., & Alieze, A. B. (2025). Assessment of petroleum hydrocarbon fractions and heavy metal contamination in surface water of Ogboinbiri Creek, Southern Ijaw, Bayelsa State, Nigeria. Scientific Journal of Engineering and Technology, 2(2), 172–179. https://doi.org/10.69739/sjet.v2i2.1308

John, E. E., Emah, O. A., Ifiok, O. E., Okpoji, A. U., Iyam, O. E., Henderson, O. O., Akinola, A. E., Gloria, N. A., & Charles, N. O. (2025). Health Risk Implications of Polycyclic Aromatic Hydrocarbons in Smoked and Dried Fish Consumed in Nigeria. Journal of Medical Science, Biology, and Chemistry, 2(2), 192–199. https://doi.org/10.69739/jmsbc.v2i2.1045

Ogbaji, H. O., Akpan, N. A., Ijioma, C. C., Okpoji, A. U., Eze, V. C., Obi, J. N., Martins, N. P., & Etesin, M. U. (2025). Seasonal variation in hydrocarbon contamination of sediments and biota in Bonny Estuary, Rivers State, Nigeria. Asian Journal of Geographical Research, 8(4), 330–338. https://doi.org/10.9734/ajgr/2025/v8i4341

Ohaturuonye, S. O., Okpoji, A. U., Akpan, N. A., Njoku, C. A., Isaac, S. C., Etesin, M. U., & Ekwere, I. O. (2025). Biomarker responses in fish and crustaceans exposed to heavy metals and petroleum hydrocarbons in the Qua Iboe Estuary, Niger Delta. Asian Journal of Research in Zoology, 8(4), 234–244. https://doi.org/10.9734/ajriz/2025/v8i4233

Okagbare, U. V., Umueni, U. E., Ekpe, J. E., Etukudo, N. J., Okpoji, A. U., Okoye, P. I., Ekwere, I. O., Etesin, M. U., Okpanachi, C. B., & Okafor, C. A. (2025). Geophysical and Hydrochemical Assessment of Groundwater Vulnerability to Oil Pollution in Yenagoa, Bayelsa State, Nigeria. Journal of Environment, Climate, and Ecology, 2(2), 138–146. https://doi.org/10.69739/jece.v2i2.1078

Okpoji, A. U., Akpan, N. A., Eze, V. C., Ijioma, C. C., Hassan, D. H., Kareem, M. M., Obi, A. I., Aningo, G. N., Okoye, P. I., Ogbonnaya, C. N., Ekwere, I. O., Okeke, C. F., & Aligwo, M. C. (2025). Toxicity and bioaccumulation of heavy metals and polycyclic aromatic hydrocarbons in estuarine fish from the Andoni Estuary, Niger Delta, Nigeria. Journal of Applied Physical Science International, 17(2), 10–22. https://doi.org/10.56557/japsi/2025/v17i210026

Okpoji, A. U., Anarado, C. O., Mmuta, E. C., Ekwere, I. O., Alaekwe, I. O., Odibo, U. E., Igwegbe, K. C., Eboh-Ajoku, I. O., & Obunezi, O. C. (2025). Toxicological evaluation of Pb, Cd, As, and Hg in blue crab from oil-polluted creeks in the Niger Delta. Journal of Life Science and Public Health, 1(1), 24–31. https://doi.org/10.69739/jlsph.v1i1.879

Okpoji, A. U., Chinyere, U. E., Nwokoye, J. N., Ezekwuemen, O. I., Alaekwe, I. O., Odidika, C. C., Owughara, C. N., Enyi, C. M., & Kolawole, O. O. (2025). Environmental Assessment of Heavy Metals and Hydrocarbon Pollution in Surface Waters of Oil-Bearing Communities in Andoni, Rivers State, Nigeria. International Journal of Modern Science and Research Technology, 3(8), 22–30. https://doi.org/10.5281/zenodo.16904018

Okpoji, A. U., Eboh-Ajoku, I. O., Mmuta, E. C., Ndubuisi, J. O., Alaekwe, I. O., Odibo, U. E., Nwoka, N. G., Okafor, C. A., & Obunezi, O. C. (2025). Integrated environmental risk assessment of BTEX and PAHs in water and sediment samples from the Bonny River, Nigeria. ISA Journal of Multidisciplinary (ISAJM), 2(4), 14–21. https://doi.org/10.5281/zenodo.16921896

Okpoji, A. U., Ekwere, I. O., Igwegbe, K. C., Anarado, C. J. O., Ogbonna, C., Ewuola, A. A., Odibo, U. E., & Garuba, M. H. (2025). Volatile Organic Compounds from Gas Flaring and Their Atmospheric Implications in the Niger Delta. Journal of Life Science and Public Health, 1(1), 32–39. https://doi.org/10.69739/jlsph.v1i1.962

Okpoji, A. U., Ekwere, I. O., Igwegbe, K. C., Etesin, U. M., Ugwuanyi, G. C., Okpanachi, C. B., Ewuola, A. A., Mojisola, K. M., & Ezekwueme, O. I. (2025). Solar-enhanced photocatalytic degradation of pharmaceutical residues in wastewater using Fe-, Cu-, and Zn-doped TiO₂ nanomaterials. Journal of Environment, Climate, and Ecology, 2(2), 56–62. https://doi.org/10.69739/jece.v2i2.961

Okpoji, A. U., Emem, J. A., Ekwere, I. O., Odibo, U. E., Alaekwe, I. O., Warder, A. B., & Eboj-Ajoku, O. I. (2025). Bioaccumulation of nickel, lead, and cadmium in tissues of Callinectes sapidus from the Iko River, Nigeria: Implications for human health risk and environmental safety. Journal of Environment, Climate, and Ecology, 2(2), 29–37. https://doi.org/10.69739/jece.v2i2.844

Okpoji, A. U., Etesin, M. U., Ekwere, I. O., Martins, N. P., Okpanachi, C. B., Adisa, O. J., & Okonkwo, C. C. (2025). Geochemical speciation and ecological risk of heavy metals in estuarine sediments of the Qua Iboe River, Nigeria. Asian Journal of Chemical Sciences, 15(6), 20–31. https://doi.org/10.9734/ajocs/2025/v15i6402

Okpoji, A. U., Ndubuisi, J. O., Eboh-Ajoku, I. O., Emem, J. A., Ekwere, I. O., Alaekwe, I. O., Odibo, U. E., Igwegbe, K. C., Onoja, C. R., Warder, A. B., & InyangAbia, A. J. (2025). Trematode Infections and Histopathological Effects in Chrysichthys nigrodigitatus from the Polluted Andoni River, Niger Delta, Nigeria. Journal of Agriculture, Aquaculture, and Animal Science, 2(2), 66–72. https://doi.org/10.69739/jaaas.v2i2.882

Okpoji, A. U., Nwoka, N. G., Odibo, U. E., Alaekwe, I. O., Okafor, C. A., Ogwu, N. G., & Akatakpo, C. U. (2025). Seasonal variation in hydrochemical characteristics and heavy metal risk assessment of groundwater in Andoni-Isiokwan District, Niger Delta, Nigeria. International Journal of Modern Science and Research Technology, 3(8), 44–51. http://www.ijmsrt.com

Okpoji, A. U., Ogbaji, H. O., Hassan, D. H., Orji-Azuka, L. N., Rasheed, H. O., Ohaturuonye, S. O., Ejeka, J. C., Okpanachi, C. B., & Ekwere, I. O. (2025). Physico-chemical transport of volatile organic compounds (VOCs) from gas flaring into surface waters of Ogoniland, Rivers State, Nigeria. Asian Journal of Physical and Chemical Sciences, 13(4), 151–159. https://doi.org/10.9734/ajopacs/2025/v13i4271

Okpoji, A. U., Orji-Azuka, L. N., Igwegbe, K. C., Ekwere, I. O., Ewuola, A. A., Garuba, M. H., & Etukudo, E. W. (2025). Water quality and ecotoxicological impacts of surfactants and heavy metals in urban rivers of Benin City, Niger Delta, Nigeria. Asian Journal of Geological Research, 8(3), 697–707. https://doi.org/10.9734/ajoger/2025/v8i3223

Olotu, O. N., Okagbare, U. V., Okpoji, A. U., Orji-Azuka, L. N., Etesin, M. U., & Ekwere, I. O. (2025). Spatio-seasonal assessment of water and sediment quality of the Imiringi River and implications for fisheries and aquatic resources in the Niger Delta. Journal of Agriculture, Aquaculture, and Animal Science, 2(2), 174–184. https://doi.org/10.69739/jaaas.v2i2.1309

Onoja, C. R., Ogbaji, H. O., Edodi, I. O., Mmuta, E. C., Igwegbe, K. C., Ogini, O. R., Odibo, U. E., Ewuola, A. A., Mahmoud, A. B., & Okpoji, A. U. (2025). Heavy Metal Contamination and Health Risk Assessment of Smoked-Dried Fish Sold in Eke-Awka Market, Anambra State, Nigeria. Journal of Sustainable Research and Development, 1(2), 18-25. https://doi.org/10.69739/jsrd.v1i2.1044

Osuafor, O. O., Okeke, C. F., Agu, M. O., Isueken, C. O., Aligwo, M. C., Ohaturuonye, S. O., Okpoji, A. U., Etesin, M. U., & Ekwere, I. O. (2025). Hydrogeochemical characterisation and water quality assessment of streams impacted by agricultural runoff in Uyo, Akwa Ibom State, Nigeria. Asian Journal of Agricultural and Horticultural Research, 12(4), 182–192. https://doi.org/10.9734/ajahr/2025/v12i4427

Umueni, U. E., Etukudo, N. J., Okoye, P. I., Okpoji, A. U., Eze, V. C., Aningo, G. N., Ekwere, I. O., & Garuba, M. H. (2025). Geochemical and ecological risk assessment of petroleum hydrocarbons in sediments of Forcados River, Delta State. Asian Journal of Geographical Research, 8(4), 287–298. https://doi.org/10.9734/ajgr/2025/v8i4337