Geochemical signatures and waste-soil interactions of heavy metals in open dumpsite soils:

Implications for contaminant dynamics and soil health

ABSTRACT

This work investigated the physicochemical characteristics, heavy metal concentrations, and depth-release properties of soils impacted by the deposition of automechanic (A), paint-processing (P), and abattoir (AB) waste in a coastal milieu in Southern Nigeria. Composite soil samples were collected directly beneath the waste piles at various subsurface depths to assess the vertical distribution and mobility of heavy metals within the waste-impacted soils. The samples were analyzed for their pH, electrical conductivity, organic carbon, organic matter, total nitrogen, available phosphorus, exchangeable calcium, exchangeable magnesium, exchangeable acidity, effective cation exchange capacity, and base saturation. The samples were also analyzed for their aqua regia-extractable concentrations of Lead (Pb), Cadmium (Cd), Chromium (Cr), Nickel (Ni), and Vanadium (V). The results indicated that waste inputs significantly altered the chemical characteristics of the soil, producing significant shifts in soil physicochemical properties. Maximum concentrations of Pb (3918 mg/kg), Cd (116.5 mg/kg), and Ni (226.1mg/kg) were recorded in soils impacted by automechanic wastes, while maximum concentrations of Cr (511.0 mg/kg) and V (179.3mg/kg) were recorded in soils impacted by paint wastes. These concentrations exceeded international guideline limits for residential soils and indicated severe anthropogenic contamination. Measures of contamination factor (CF) indicated severe and widespread heavy-metal contamination across all sites. Heavy Metal Release Percentage (HMR) indicated a vertical gradient in metal release, with the highest percentages of metals released within the topsoil (0-20cm), with attenuation occurring with depth. Overall, these results indicate that indiscriminate disposal of activity-specific wastes has degraded soil quality at the study sites and poses a threat to underlying groundwater resources. Enforcement of stringent waste management regulations, recycling at source, and phytoremediation of contaminated soils are recommended to mitigate further contamination.

Keywords: Heavy metals, soil contamination, physicochemical properties, waste-soil interaction, environmental geochemistry, remediation

INTRODUCTION

Traditional waste disposal methods such as unregulated landfilling, open dumping, and incineration have persisted despite their well-documented adverse environmental impacts (Ali et al., 2014; Oketola et al., 2015; Kebede et al., 2016; Nwaogu et al., 2017; Sharma et al., 2018; Sharma and Jain, 2020; Agbeshie et al., 2020; Nta et al., 2020; Mekonnen et al., 2020; Alao, 2023;  Sanga et al., 2023; Azuka and Ezeme, 2023; Alao 2024; Omada et al., 2024). These waste disposal systems lack engineered liners, leachate containment mechanisms, and gas recovery infrastructure, allowing for unregulated interactions between waste and the surrounding environment (Ogundiran et al., 2008; Obianefo et al., 2017; Olagunju et al.,2018; Mouhoun-Chouaki et al., 2019; Kulkarni et al., 2024; Udosen et al., 2024a; Drall et al., 2025).

The decomposition of waste produces large volumes of leachate, a chemically complex solution enriched with organic acids, dissolved solids, heavy metals, and nutrients (Ebong et al., 2020; Udosen, 2022). Upon infiltration, leachate alters the natural physicochemical balance of soils, modifying pH, cation exchange capacity, etc., and promotes the downward migration of pollutants into the unsaturated and saturated zones (Kanmani et al., 2013; Zhao et al., 2016; Jilani and Rashid, 2020; de Jesus et al., 2022; Sarma Bora et al., 2023; Bisht et al., 2024; Udo et al., 2025a; Varier et al, 2025). Simultaneously, the waste decomposition process emits greenhouse gases such as methane (CH4) and carbon dioxide (CO2), aggravating atmospheric warming. Volatile sulfur and nitrogen compounds that are also emitted cause offensive odors and photochemical smog (FAO, 2018). These interlinked soil-air-water interactions have adverse effects that can persist for decades (Knox et al., 1999; Prechthai et al., 2008; He et al. 2015; Chen et al., 2016; Breza-Boruta et al., 2016; Osim et al., 2020; Soubra et al., 2021; Alao, 2024; Ale et al., 2024; Dixit et al., 2024; Das et al., 2025).

Among the types of contaminants released from dumpsites, heavy metals pose the most critical environmental and public health threats (Ahmed et al. 2014; Ihedioha et al. 2017; Kaparwan et al., 2020; Oluyinka et al., 2024). Unlike organic pollutants, heavy metals are non-biodegradable and chemically stable; hence, they accumulate in soils and sediments over time (Wuana and Okieimen, 2011). Once mobilized, they enter terrestrial food webs, causing chronic toxicity in plants, animals, and humans (Gushit et al. 2019; Regmi et al., 2022; Marinho et al., 2022; Junior et al., 2023; Abd-Elhalim et al., 2025). Prolonged exposure to these elements has been associated with kidney and liver dysfunction, neurodegenerative disorders, reproductive impairment, and cancer (WHO, 2011; Maas et al., 2010; Sanga and Pius, 2024). Their persistence and bioaccumulation potential therefore makes the remediation of heavy metal pollution a priority concern for soil scientists, toxicologists, and environmental policymakers (Ghafourian et al., 2016; Ojegu 2016; Orodu and Leizou 2017; Ferronato et al., 2018; Omeiza et al., 2022; Omeiza et. al., 2023; Benhamdoun et al., 2023; Udo et al., 2025b).

This study therefore aimed to evaluate the extent and spatial pattern of heavy metal contamination in soils impacted by three distinct waste streams: automechanic, paint-processing, and abattoir dumpsites in Akwa Ibom, southern Nigeria. It was hypothesized that the waste type would significantly influence the magnitude and pattern of heavy metal accumulation and soil physicochemical alteration. To test this hypothesis, this study integrated physicochemical analysis (pH, electrical conductivity, cation exchange capacity, organic carbon, etc.) and heavy metal concentration analysis to assess contamination levels, controlling factors of metal distribution, and contamination signatures. In addition, Contamination Factor (CF) and Heavy Metal Release Percentage (HMR) analysis were undertaken to assess the severity of heavy-metal pollution and evaluate the metal mobility depth-dependence, respectively. This work’s significance lies in its assessment of waste-derived soil contamination via the integration of physicochemical characterization, total heavy metal concentrations, pollution indices, depth-wise distribution, and release behavior. Some past studies examined dumpsites as uniform entities, overlooking how waste type uniquely modulates soil chemistry and metal mobility. This approach, however, limited the ability to establish causal interrelations between waste composition and soil geochemistry. This work, on the other hand, examined waste profiles from three distinct waste streams: automechanic, paint-processing, and abattoir, the goal being to evaluate their geochemical signatures. Furthermore, unlike previous studies that relied on surface or peripheral soil sampling, soils in this investigation were collected directly beneath active waste piles to capture peak contamination conditions and provide a more realistic assessment of environmental risk. This study therefore contributes globally relevant insights for evaluating and mitigating the long-term environmental implications of open dumpsites in tropical environments.

LOCATION AND GEOLOGY OF THE STUDY SITES

The investigation areas are located in Akwa Ibom State, southern Nigeria, within the humid tropical rainforest belt. Geographically, the State lies between latitudes 4°32′ and 5°33′ N and longitudes 7°25′ and 8°25′ E. The State is bordered by Cross River State to the east, Abia State to the north, Rivers State to the west, and the Atlantic Ocean to the south.

The investigated sites were situated in Etinan, Uyo, and Ikot Ekpene (Figure 1), all in Akwa Ibom. Etinan hosts the defunct Peacock Paint Limited, previously one of the region’s largest paint manufacturing facilities, which contributed to heavy metal contamination through pigment-rich waste discharges. Uyo, the state capital, is undergoing rapid urbanization, industrial expansion, and increased waste production (Opara et al., 2005; Bello et al., 2023). Ikot Ekpene is characterized by extensive automobile repair workshops and food processing industries. Three dumpsite types, abattoir, paint-processing, and auto-mechanic waste sites, were selected within Etinan, Uyo, and Ikot Ekpene to represent diverse contamination sources. Corresponding control sites with no known history of industrial or waste-disposal activities were established in each region to ensure comparable soil type and land use history.

The soils of Akwa Ibom are from the Benin Formation (coastal plain sands), which account for approximately 70% of the area (Ekanem and Udosen 2023a, b). Other parent materials are beach ridge sands, sandstones, shales, and alluvial deposits (Udoh and Ibia 2022; Udosen et al. 2024b; Udosen et al. 2025). The dominance of coarse-textured coastal plain sands has contributed to the generally well-drained nature of the soils (Udo et al. 2009a, b).

Figure 1: Geologic map of study area

MATERIAL AND METHODS

Soil/waste sampling design and preparation

Soil sampling was undertaken at three major waste dumpsites representing automechanic, paint-processing, and abattoir activities.  Composite samples were collected directly beneath waste piles at three depth intervals (0–20 cm, 20–40 cm, and 40–60 cm). At automobile and paint-processing sites, solid residues and sludge-like materials were collected, while semi-solid materials were collected at abattoir sites. The waste samples (500g) were collected with sterilized plastic scoops, sealed in polyethylene containers, and transported to the laboratory for analysis. To remove large debris, the samples were air-dried at ambient temperature, disaggregated, and sieved through a 2 mm mesh. Subsamples were analysed for physicochemical parameters and heavy metal concentrations (APHA, 2017)

The total concentrations of Pb, Cd, Cr, Ni, and V were evaluated using the aqua regia digestion method (Abegunde et al. 2018). One gram of soil was placed in a digestion flask to which 20 ml of aqua regia (HCl: HNO3 = 3:1) was added. This mixture was heated gradually on a mantle under a fume hood until the volume reduced to about 5 ml. After cooling, the digest was filtered and transferred into a 50 ml volumetric flask, which was then diluted with deionized water. The metal concentrations were then quantified using an Atomic Absorption Spectrophotometer (AAS, Model AA500) operated under standard conditions.

For quality assurance, all glassware and plastic containers were pre-cleaned by soaking in 10% HNO3 for 24 hours and rinsed with double-distilled water. Analytical blanks were processed with every batch of samples. The detection limits for analyzed metals were <0.01 mg/l for Cd and Pb, and <0.05 mg/l for Cr, Ni, and V.  All analyses were performed in triplicate, and results were expressed as mean values. Precision was maintained below 5% for all measurements, ensuring the reliability of the analytical data.  Statistical analyses were performed using two-way analysis of variance (ANOVA) to evaluate differences in physicochemical properties and metal concentrations among sites and soil depths. Where significant differences were observed (p < 0.05), the means were separated using Least Significant Difference (LSD).

Contamination Factor (CF)

Contamination factor (CF) was computed to assess anthropogenic metal enrichment within the soils. This pollution index evaluated contamination intensity by comparing metal concentrations at impacted sites with their corresponding background levels, the aim being to differentiate anthropogenic inputs from natural geochemical inputs.  The contamination factor is given as:

CF=CiCb							(1)
where Ci = concentration of a given metal in soil collected from waste-impacted locations, and Cb = concentration of the same metal in uncontaminated control soils. According to established classification criteria, CF < 1 denotes low contamination; 1 ≤ CF < 3 denotes moderate contamination; 3 ≤ CF < 6 denotes considerable contamination; and CF ≥ 6 denotes very high contamination.

Heavy Metal Release Percentage

The percentage of heavy metals released from waste materials into the surrounding soils (de Vries and Bakker 1996) was expressed by:

HMR (%)=Ci-CbW×100					(2)		
where HMR (%) = heavy metal release percentage, Ci = concentration of a given metal in soil collected from waste-impacted locations, Cb = concentration of the same metal in uncontaminated control soils, and W = concentration of the metal in the corresponding waste (mg/kg). This computation provided a relative index of the transfer potential of each heavy metal from the waste materials to the surrounding soil.

RESULTS AND DISCUSSION

Physicochemical properties of the dumpsite soils

The physicochemical attributes of the soil samples are summarized in Table 1. The results show that all the soils had a predominantly sandy texture with negligible variations (p > 0.05) in sand, silt, and clay proportions. This uniformity corresponds to the coastal plain sand formation characteristic of Akwa Ibom (Akpan-Idiok, 2012; Obi et al., 2020; Udoh and Ibia, 2022).

Soil pH values varied significantly, ranging from strongly acidic (4.63) in control soils to slightly alkaline (7.99) in paint waste-impacted soils. The automechanic soils exhibited a near-neutral pH (7.37), while abattoir soils remained mildly acidic (6.49). These variations highlighted the strong influence of waste type on the soil’s acid-base balance.  For example, paint residues often contain alkaline additives and pigments which elevate pH, while organic matter in abattoir-waste soils release organic acid, causing acidity and decreased pH values. Slightly alkaline conditions enhance the precipitation of metal hydroxides, whereas acidic conditions promote solubilization and bioavailability (Kabata-Pendias and Pendias, 2001).

| Soil property | Range |  | Dumpsite |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Control | Automechanic | Paints | Abattoir |
| Sand (k/kg) | Min Max | 719.00 779.00 | 645.00 899.00 | 603.00 875.00 | 693.00 883.00 |
|  | Mean | 739.00a | 795.66a | 795.88a | 826.33a |
| Silt (k/kg) | Min Max | 30.00 90.00 | 30.00 98.00 | 28.00 118.00 | 34.00 114.00 |
|  | Mean | 63.33a | 65.56a | 66.44a | 73.00a |
| Clay (k/kg) | Min Max | 131.00 251.00 | 71.00 287.00 | 67.00 323.00 | 73.00 267.00 |
|  | Mean | 187.67a | 138.78ab | 139.67ab | 107.89b |
| pH (H2O) | Min Max | 4.50 4.80 | 6.70 8.20 | 7.30 8.40 | 5.70 7.30 |
|  | Mean | 4.63d | 7.36b | 7.99a | 6.49c |
| EC (µS/cm) | Min Max | 27.00 99.10 | 40.30 337.00 | 82.02 352.32 | 167.80 1259.00 |
|  | Mean | 56.73b | 146.01b | 166.40b | 536.42a |
| OC (g/kg) | Min Max | 10.73 18.13 | 10.15 33.95 | 2.43 7.35 | 26.25 46.25 |
|  | Mean | 13.57c | 21.31b | 4.98d | 37.10a |
| OM (g/kg) | Min Max | 18.46 31.26 | 17.49 58.23 | 4.22 12.67 | 45.26 79.74 |
|  | Mean | 23.37c | 36.74b | 8.58d | 63.96a |
| TN (k/kg) | Min Max | 0.98 3.22 | 0.56 3.64 | 0.56 0.84 | 0.98 21.98 |
|  | Mean | 1.77b | 1.31b | 0.67b | 13.22a |
| Available P. (mg/kg) | Min Max | 16.37 22.28 | 16.34 42.77 | 24.92 58.39 | 33.62 63.15 |
|  | Mean | 20.53b | 24.58b | 39.79b | 48.30a |
|  | Min | 1.60 | 1.60 | 3.60 | 25.20 |
| Ca (Cmol/kg) | Max | 3.20 | 27.00 | 11.60 | 40.00 |
|  | Mean | 2.49d | 20.47b | 7.20c | 32.53a |
| Mg (Cmol/kg) | Min Max | 0.80 1.60 | 5.20 12.20 | 2.00 7.60 | 6.40 18.40 |
|  | Mean | 1.20c | 8.47b | 4.00c | 12.71a |
| K (Cmol/kg) | Min Max | 0.06 0.15 | 0.95 4.69 | 0.33 0.90 | 3.16 6.58 |
|  | Mean | 0.10c | 2.77b | 0.58c | 4.80a |
| Na (Cmol/kg) | Min Max | 0.04 0.10 | 0.31 2.35 | 0.13 0.64 | 0.04 4.61 |
|  | Mean | 0.07c | 2.35b | 0.48c | 3.06a |
| EA (Cmol/kg) | Min Max | 0.24 2.00 | 0.24 0.56 | 0.32 0.48 | 0.56 0.80 |
|  | Mean | 1.68a | 0.41c | 0.42c | 0.68b |
| ECEC (Cmol/kg) | Min Max | 4.51 70.23 | 19.82 47.04 | 6.53 21.13 | 36.53 70.23 |
|  | Mean | 5.43c | 33.30b | 12.54c | 53.78a |
|  | Min | 55.60 | 97.17 | 93.69 | 98.02 |
| BS (%) | Max | 99.40 | 99.40 | 98.11 | 99.09 |
|  | Mean | 67.91c | 98.63a | 96.21b | 98.69a |

Table 1: Physicochemical properties of the dumpsite soils (Udo et al. 2025a). Symbols a, b, c and d indicate means which are statistically different and are compared horizontally across the table.

Organic carbon (OC) content ranged between 4.98 g/kg (paint soils) and 37.10 g/kg (abattoir soils), following the order: abattoir > automechanic > control > paint. The elevated OC in abattoir soils reflected the presence of degradable organic matter, while elevated OC in automechanic soils reflected the presence of hydrocarbons within the wastes (Johnbosco et al., 2020). Conversely, the low OC in paint-contaminated soil indicated the minimal organic input of paints within such soils and the possible oxidative degradation of carbon compounds by paint solvents.

Electrical conductivity (EC) varied between 56.73 µS/cm in the control and 536.42 µS/cm in abattoir soils, indicating substantial ionic enrichment. The cation exchange capacity (ECEC) and base saturation (BS) were consistently higher in contaminated soils, particularly at the abattoir sites (ECEC = 53.78 cmol/kg; BS = 98.69%). These suggested enhanced nutrient retention and exchangeable cation accumulation within the sites. Collectively, the physicochemical results confirm that waste inputs substantially altered soil electrochemical equilibrium, influencing nutrient cycling, buffering potential, and contaminant mobility.

Concentrations of heavy metals

An analysis of the trend of heavy metal concentrations (mg/kg) (Table 2) indicated that mean Pb values were 7.0, 3918.0, 1548.0, and 54.0 mg/kg for control (C), automechanic (A), paints (P), and abattoir (AB) soils, respectively, with LSD (0.05) value of 400.0.

| Dumpsite | Pb | Cd | Cr | Ni | V |
| --- | --- | --- | --- | --- | --- |
|  | mg/kg |  |  |  |  |
| Control | 7.0 | 0.6 | 1.0 | 4.0 | 1.4 |
| Auto mechanic | 3918.0 | 116.5 | 402.8 | 226.1 | 152.7 |
| Paints | 1548.0 | 90.1 | 511.0 | 137.6 | 179.3 |
| Abattoir | 54.0 | 20.7 | 81.5 | 99.2 | 81.0 |
| LSD (0.05) | 400.0 | 12.3 | 33.6 | 38.9 | 31.9 |

Table 2: Mean concentrations of heavy metals extracted with aqua regia. Key: LSD = Least significance difference

Values for C and AB were not significantly different from each other and were the lowest, followed by P, with A showing the highest Pb concentration. The sequence for Pb levels was C < AB < P < A (Figure 2a). The 7.0 mg/kg recorded for C was likely a natural background level. The Pb level in A soil (3918.0 mg/kg) was extremely high compared to the maximum limit of 300 mg/kg set by FAO (2004) for soils in residential zones. The results here are higher than those from previous studies like Okoro et al. (2013); Osu and Okereke (2015); Johnbosco et al. (2020), as those researchers collected soil samples around the dumpsites, and not directly underneath the waste piles. Collecting soil samples away from the actual dumping sites may yield results that do not accurately reflect the true impact of the wastes.

Mean values of Cd (Table 2) were 0.6, 116.5, 90.1, and 20.7 mg/kg for C, A, P, and AB soils, respectively, with an LSD (0.05) value of 21.3. Concentrations of Cd in all soils were significantly different from each other and followed the sequence C < AB < P < A (Figure 2b). The lowest Cd value, 0.64 mg/kg, was found within the control soil, indicating a natural background level from natural sources. High Cd levels within automechanic waste sites were likely due to battery wastes (nickel-cadmium batteries), brake pads, and used oils/fluids. In the paint processing dumpsites, Cd probably originated from paint pigments and coatings such as cadmium sulfide and cadmium selenide.

Cr values were 1.0, 402.8, 511.0, and 81.5 mg/kg for C, A, P, and AB soils, respectively, with an LSD (0.05) value of 33.6. Concentrations of Cr followed the sequence C < AB < A < P (Figure 2c). The 1.0 mg/kg in C soil was assumed to be from natural sources. The 402.8 mg/kg in A soil likely originated from chrome-plated vehicle parts and waste oils/fluids. The 511.0 mg/kg in P soil resulted from chrome-based paint wastes such as pigments. Only soils from A and P were contaminated beyond the permissible limit of 300 mg/kg for residential zones set by FAO (2004).

Mean Ni levels were 4.0, 226.1, 137.6, and 99.2 mg/kg for C, A, P, and AB soils, respectively, with an LSD (0.05) value of 38.9, following the sequence C < AB < P < A (Figure 2d). The 4.0 mg/kg Ni found in C soils represented background levels from natural sources. In A soils, Ni (probably from stainless steel, waste oils, and alloyed vehicle parts) was higher than 150 mg/kg, the limit set by FAO (2004) for residential zones. The Ni levels of 137.6 and 99.2 mg/kg in P and AB soils, respectively, were below this limit.

Vanadium (V) concentrations were 1.4, 152.7, 179.3, and 81.0 mg/kg for C, A, P, and AB soils, respectively, with an LSD value of 31.0 (Table 2). The V concentration in C soil was significantly different from the others; however, levels in A and P soils did not differ significantly. The sequence followed was C < AB < A < P (Figure 2e). All V values were below the critical threshold of 300 mg/kg for residential zones set by FAO (2004).

(a)

(d)

(c)

(e)

(b)

Figure 2: (a) Concentration of lead in dumpsite soils (b) concentration of cadmium in dumpsite soils (c) concentration of chromium in dumpsite soils (d) concentration of nickel in dumpsite soils (e) concentration of vanadium in dumpsite soils. Key: C = control site, AB = abattoir wastes dumpsite, P = paint processing wastes dumpsite, A =automechanic wastes dumpsite.

Comparison between heavy metal concentrations in soils and waste

Figure 3 illustrates the comparative concentrations of Pb, Cd, Cr, Ni, and V in waste materials and corresponding dumpsite soils. The results demonstrated that metal concentrations were consistently higher in waste materials than in the underlying soils, indicating that the wastes represented the primary contamination sources, while the soils only acted as secondary sinks for leaching, adsorption, and downward migration processes.

Lead (Pb) concentrations (Figure 3a) showed. elevated Pb levels in the associated soils, particularly at the automechanic (A) and abattoir (AB) sites, suggesting substantial transfer from wastes into the soil matrix. Cadmium (Cd) concentrations (Figure 3b) exhibited marked variability among dumpsite types, with high levels in paint processing (P) and automechanic (A) wastes. Although Cd concentrations decreased in soils relative to wastes, detectable levels across all dumpsite soils indicated their high mobility. Chromium (Cr) concentrations (Figure 3c) peaked in paint processing (P) wastes, a likely consequence of metal alloys, corrosion inhibitors, pigments, and industrial residues (Wuana and Okieimen, 2011; Igwe and Nwachukwu, 2016; Kaparwan et al., 2020). The observed attenuation from waste to soil indicated some degree of geochemical stabilization.

Nickel (Ni) concentrations (Figure 3d) displayed relatively high values across all dumpsite wastes, particularly at automechanic (A) sites, consistent with sources such as engine oils, fuel residues, metal scraps, and mechanical wear products. Elevated Ni levels in soils beneath these dumpsites suggest enhanced vertical transfer. Vanadium (V) concentrations (Figure 3e) were prominently associated with automechanic (A) and paint (P) processing wastes, attributable to petroleum products, combustion residues, alloys, and pigments. Although soil concentrations were reduced relative to wastes, persistence of Vanadium contamination indicated gradual incorporation into the soil system (Eissa et al., 2017).

(a)

(d)

(c)

(e)

(b)

Figure 3: (a) Concentration of lead in dumpsite soils vs wastes (b) concentration of cadmium in dumpsite soils vs wastes (c) concentration of chromium in dumpsite soils vs wastes (d) concentration of nickel in dumpsite soils vs wastes (e) concentration of vanadium in dumpsite soils vs wastes.  Key: AB = abattoir wastes dumpsite, P =paint processing wastes dumpsite, A =automechanic wastes dumpsite

Contamination Factor (CF)

Contamination Factor (CF) values indicated severe heavy-metal contamination across all sites, with CF for all metals exceeding the very high contamination threshold (CF > 6). Lead showed the most extreme enrichment, particularly in automechanic soils.  CF values ranged from 6.54 at an abattoir site (20–40 cm) to 4715.3 at 40–60 cm at an automechanic site. Paint-processing soils also exhibited high Pb contamination (CF ≈ 110–1907), while abattoir soils showed lower but still elevated values (6.54–41.52). Cadmium contamination was also pronounced, with CF values exceeding 100 at most sites and peaking at 658.7 and 648.3 at 40–60 cm depths in automechanic and paint-processing soils, respectively. Chromium recorded exceptionally high CF values, reaching values as high as 3621.6.  Nickel and vanadium also showed high levels of enrichment, with maximum CF values of 569.7 and 963.4, respectively. Overall, CF values indicated severe contamination, highlighting intense anthropogenic loading and potential groundwater risk.

Heavy Metal Release Percentage (HMR)

The HMR results indicated a clear vertical gradient in metal release, with the highest percentages consistently recorded in the topsoil (0–20 cm), followed by a progressive decline at 20–40 cm and 40–60 cm depths across most sites and metals (Table 3). This pattern reflected the dominant influence of surface waste deposition, leachate generation, and recent anthropogenic inputs, which preferentially enrich the upper soil horizons (Winegardner, 2019).

Across all dumpsites, Pb and Ni had higher mean release percentages compared to Cd, Cr, and V, suggesting greater mobility or weaker binding of these metals in the dumpsite soils. Among the dumpsite groups, the abattoir (AB) sites displayed the highest Pb release. This suggested intense Pb loading and possible downward migration due to high leachate fluxes, soil permeability, or complexation with organic matter. The automechanic sites showed comparatively moderate but consistent release patterns, with Ni and Cr displaying relatively elevated mean values, indicating mixed sources and sustained mobility. The paint sites were characterized by notably high Ni release in surface soils and variable Cd behavior.

Depth-wise, the sharp reduction in release percentages at 40–60 cm for most metals, particularly Cr and V, suggested attenuation mechanisms such as adsorption onto clay minerals and reduced organic matter availability at depth. However, the persistence of relatively high Pb and Ni values in some subsurface layers indicated a potential risk of vertical transport, which may pose long-term threats to underlying groundwater systems.

| Dumpsite | Soil depth | Metal release percentages |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Pb | Cd | Cr | Ni | V |
| A 1 | 0 -20 | 48.61 | 46.46 | 47.05 | 41.97 | 51.92 |
|  | 20 - 40 | 49.16 | 28.00 | 39.22 | 29.02 | 33.10 |
|  | 40 - 60 | 37.82 | 15.67 | 25.31 | 20.05 | 15.33 |
|  | Mean | 45.20 | 30.04 | 37.19 | 36.35 | 33.45 |
| A 2 | 0 - 20 | 52.79 | 46.50 | 40.41 | 53.37 | 41.01 |
|  | 20 - 40 | 45.99 | 30.29 | 36.10 | 41.97 | 28.49 |
|  | 40 - 60 | 40.91 | 16.86 | 31.44 | 24.71 | 12.42 |
|  | Mean | 46.56 | 31.22 | 35.98 | 40.02 | 27.31 |
| A 3 | 0 - 20 | 48.02 | 43.14 | 47.04 | 60.10 | 43.00 |
|  | 20 - 40 | 44.48 | 25.52 | 42.66 | 45.54 | 30.26 |
|  | 40 -60 | 40.35 | 12.87 | 37.88 | 32.19 | 15.97 |
|  | Mean | 44.28 | 27.18 | 42.53 | 49.94 | 29.74 |
| P 1 | 0 - 20 | 31.21 | 43.08 | 42.36 | 58.79 | 49.50 |
|  | 20 - 40 | 27.45 | 18.19 | 38.19 | 39.44 | 25.64 |
|  | 40 - 60 | 25.11 | 9.94 | 27.31 | 24.99 | 14.00 |
|  | Mean | 27.92 | 23.74 | 35.95 | 41.07 | 29.71 |
| P 2 | 0 - 20 | 35.12 | 73.06 | 39.51 | 47.31 | 45.17 |
|  | 20 - 40 | 32.15 | 36.29 | 35.50 | 36.10 | 39.24 |
|  | 40 - 60 | 27.74 | 22.12 | 33.66 | 19.73 | 22.42 |
|  | Mean | 31.67 | 43.82 | 36.22 | 34.38 | 35.61 |
| P 3 | 0 - 20 | 49.12 | 44.40 | 46.33 | 61.89 | 51.14 |
|  | 20 - 40 | 46.03 | 36.74 | 40.99 | 42.39 | 41.88 |
|  | 40 - 60 | 34.30 | 17.02 | 32.90 | 26.16 | 31.92 |
|  | Mean | 43.15 | 32.72 | 40.07 | 43.48 | 41.65 |
| AB1 | 0 - 20 | 36.82 | 45.10 | 43.40 | 48.40 | 54.32 |
|  | 20 - 40 | 21.42 | 33.94 | 28.73 | 39.18 | 36.56 |
|  | 40 - 60 | 15.37 | 25.99 | 24.98 | 26.99 | 28.34 |
|  | Mean | 24.54 | 35.01 | 32.37 | 38.19 | 39.71 |
| AB 2 | 0 - 20 | 74.37 | 42.63 | 44.95 | 40.66 | 44.66 |
|  | 20 - 40 | 89.60 | 31.32 | 33.44 | 31.10 | 39.92 |
|  | 40 - 60 | 50.91 | 24.35 | 22.13 | 19.36 | 28.15 |
|  | Mean | 71.63 | 32.77 | 33.51 | 30.37 | 35.58 |
| AB 3 | 0 - 20 | 52.01 | 41.59 | 44.40 | 49.58 | 40.56 |
|  | 20 - 40 | 88.24 | 33.62 | 32.91 | 57.76 | 32.51 |
|  | 40 - 60 | 50.94 | 23.96 | 22.24 | 30.36 | 24.45 |
|  | Mean | 63.72 | 33.06 | 33.18 | 45.90 | 32.51 |

Table 3: Percentages of release of heavy metals into dumpsites soils. Key: AI, A2, A3= automechanic sites at Etinan, Uyo and Ikot Ekpene, respectively.  PI, P2, P3 = paint processing sites at Etinan, Uyo and Ikot Ekpene, respectively.  AB1, AB2, AB3 = abattoir sites at Etinan, Uyo and Ikot Ekpene, respectively.

CONCLUSION

This study integrated physicochemical characterization, total metal concentrations, depth-wise distribution, Contamination Factor (CF), and Heavy Metal Release Percentage (HMR) analysis to differentiate geochemical signatures associated with distinct anthropogenic activities (automechanic, paint-processing, and abattoir wastes).The strong positive correlations between metal concentrations in waste materials and the corresponding soils indicate that the wastes are the primary sources of contamination. The soils’ sandy texture and moderate organic matter content have facilitated downward metal mobility through leaching and infiltration. The persistence and mobility of these metals pose serious risk to soil health, groundwater quality, food safety, and the overall integrity of the local terrestrial ecosystem.

The following recommendations should be considered for implementation to achieve sustainable environmental development. National and local environmental agencies should enforce stringent waste management regulations and conduct periodic environmental audits in contamination-prone regions. Recyclable materials should be recovered and processed through licensed recycling facilities to reduce environmental loading. Contaminated soils should be rehabilitated using phytoremediation, biochar application, or microbial-assisted metal immobilisation, which are sustainable and cost-effective for tropical sandy soils.

In summary, the study has demonstrated that anthropogenic waste inputs have significantly deteriorated soil quality in the study area, necessitating immediate remediation and policy-driven waste management reforms to mitigate further damage to groundwater systems and the ecosystem. The study has contributed to the global database on soil contamination in tropical regions, and results obtained are expected to aid policy-makers in ongoing waste management reforms. The integrated framework and methodologies employed in this study are generalizable to other regions worldwide.

STATEMENTS AND DECLARATIONS

COMPETING INTERESTS STATEMENTS

The authors have no competing interests to declare that are relevant to the content of this article.

DATA AVAILABILITY

All relevant data are included in the paper or its Supplementary Information

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCES

Abd-Elhalim, B. T., Gideon, M., Anton, K., & Boyi, M. O. (2025). Impact of dumpsite compost on heavy metal accumulation in some cultivated plants. BMC Research Notes, 18(1), 20. https://doi.org/10.1186/s13104-025-07083-9

Abegunde, S. M., Oyebanji, A. O. & Osibanjo, O. (2018) Evaluation of Digestion Procedures on Heavy Metals in Soil of a Dumpsite in Ibadan, South-western Nigeria. Suan Sunandha Science and Technology Journal, 5(2): 1-5

Agbeshie, A. A., Adjei, R., Anokye, J. & Banunle, A., (2020). Municipal waste dumpsite: Impact on soil properties and heavy metal concentrations, Sunyani, Ghana. Scientific African, 8, p.e00390. https://doi.org/10.1016/j.sciaf.2020.e00390

Ahmed, I., Prasad Rao, T. V. D. & Hussain, M. (2014). Quality Assessment of Soil at municipal solid waste dumpsite and possibilities of reclamation of land. Int. J. Innovative Sci. Engin. & Tech, 1(7), pp.274-294.

Akpan-Idiok, A. U. (2012). Physicochemical properties, degradation rate and rate of vulnerability potential of soil formed in coastal plain sands in Southeast Nigeria. International Journal of Agricultural Research, 7(7), 358–366. https://doi.org/10.3923/ijar.2012.358.366

Alao, J. O. (2023). Impacts of open dumpsite leachates on soil and groundwater quality. Groundwater for sustainable development, 20, 100877. https://doi.org/10.1016/j.gsd.2022.100877

Alao, J. O. (2024). The factors influencing the landfill leachate plume contaminants in soils,

surface and groundwater and associated health risks: A geophysical and geochemical view. Public Health and Environment, 1(1), 20–43. https://doi.org/10.70737/7ejde223

Ale, T. O., Ogunribido, T. H., Ademila, O., & Akingboye, A. S. (2024). Soil–water contamination assessment due to dumpsite-impacted leachates in a metamorphic environment. Journal of African Earth Sciences, 216, 105312. https://doi.org/10.1016/j.jafrearsci.2024.105312

Ali, S. M., Pervaiz, A., Afzal, B., Hamid, N. & Yasmin, A. (2014). Open dumping of municipal solid waste and its hazardous impacts on soil and vegetation diversity at waste dumping sites of Islamabad city. Journal of King Saud University-Science, 26(1), pp.59-65. https://doi.org/10.1016/j.jksus.2013.08.003

APHA (2017). Standard Methods for the Examination of Water &Waste Water. American People Health Association, 23rd ed., Washington DC

Azuka, C.V. & Ezeme, C. (2023). Influence of municipal solid waste dumpsite on soil physicochemical properties and vertical distribution of heavy Metals. Journal of Bioresource Management, 10(3), p.8.

Bello, A. L., Adekanye, U. O., Orakpoghenor, O., & Markus, T. P. (2023). Knowledge, attitude and practices of abattoir workers and veterinarians toward meat safety in abattoir or slaughter slabs within Uyo Metropolis, Akwa Ibom State, Nigeria. Journal of Health Science Research, 8, 30–37. https://doi.org/10.25259/jhsr_4_2023

Benhamdoun, A., Achtak, H., Vinti, G., & Dahbi, A. (2023). Soil contamination by trace metals and assessment of the risks associated: the dumping site of Safi city (Northwest Morocco). Environmental Monitoring and Assessment, 195(8), 941. https://doi.org/10.1007/s10661-023-11467-4

Bisht, A., Kamboj, V., Kamboj, N., Bharti, M., Bahukahndi, K.D. & Saini, H. (2024). Impact of solid waste dumping on soil quality and its potential risk on human health and environment. Environmental Monitoring and Assessment, 196(8), p.763. https://doi.org/10.1007/s10661-024-12914-6

Breza-Boruta, B., Lemanowicz, J. & Bartkowiak, A. (2016). Variation in biological and physicochemical parameters of the soil affected by uncontrolled landfill sites. Environmental earth sciences, 75(3), p.201. https://doi.org/10.1007/s12665-015-4955-9

Chen, P., Xie, Q., Addy, M., Zhou, W., Liu, Y., Wang, Y., Cheng, Y., Li, K., & Ruan, R. (2016). Utilization of municipal solid and liquid wastes for bioenergy and bioproducts production. Bioresource Technology, 215, 163–172. https://doi.org/10.1016/j.biortech.2016.02.094

Das, S., Chakraborty, S., & Sengupta, S. (2025). Soil Quality Assessment and Future Agricultural Land Use Potential of a Municipal Dumpsite (Jharkhand, India). Soil and Sediment Contamination: An International Journal, 1-27. https://doi.org/10.1080/15320383.2025.2509738

de Vries, W., & Bakker, D. J. (1996). Manual for calculating critical loads of heavy metals for soils and surface waters. Preliminary guidelines for environmental quality criteria, calculation methods and input data. Wageningen (The Netherlands): DLO Winand Staring Centre. Report 114.

de Jesus, A. O. C., de Farias Araujo, G., Saggioro, E. M., Mannarino, C. F., & Ritter, E. (2022). Contamination assessment of soil and groundwater of a deactivated dumpsite in Brazil. Environmental Monitoring and Assessment, 194(3), 216. https://doi.org/10.1007/s10661-022-09790-3

Drall, J. K., Rautela, R., Jambhulkar, R., Kataria, A. K. & Kumar, S. (2025). Effect of heavy metals contamination due to leachate migration from uncontrolled dumpsites: A comprehensive analysis on soil and groundwater. Journal of Environmental Management, 373, p.123473. https://doi.org/10.1016/j.jenvman.2024.123473

Dixit, A., Singh, D., & Shukla, S. K. (2024). Assessment of human health risk due to leachate contaminated soil at solid waste dumpsite, Kanpur (India). International Journal of Environmental Science and Technology, 21(1), 909-924. https://doi.org/10.1007/s13762-023-04868-y

Ebong, G. A., Ettesam, E. S. & Dan, E. U. (2020) Impact of abattoir wastes on trace metal accumulation, speciation, and human health–related problems in soils within Southern Nigeria. Air, Soil and Water Research, 13: 1–14. https://doi.org/10.1177/1178622119898430

Ekanem, A. M. & Udosen, N. I. (2023). Hydrogeochemical–geophysical investigations of

groundwater quality and susceptibility potential in Ikot Ekpene–Obot Akara Local Government Areas, Southern Nigeria. Water Practice & Technology, 18(11), pp.2675-2704. https://doi.org/10.2166/wpt.2023.187

Ekanem, A. M. & Udosen, N. I. (2023). Evaluation of groundwater potentiality and quality in Ikot Ekpene-Obot Akara Local Government Areas, Southern Nigeria. Environmental Contaminants Reviews, 6(1): 46-57. https://doi.org/10.26480/ecr.01.2023.46.57

Eissa, D., Abou-Shady, A., Ismaeil, S. & Nabil Bahnasawy, N. (2017). Distribution and Mobility of Vanadium in Cultivated Calcareous Soils and Some Food Chain Crops. Egyptian Journal of Soil Science, 57 (4): 385 – 392

FAO. (2004). Maximum limits for dangerous substances in soil and groundwater. Regulation of the Minister of the Environment No. 12 of 2 April 2004 (RTL[1] 2004, 40, 662). https://faolex.fao.org/docs/pdf/est97999E.pdf  (Retrieved on 08/01/2026)

FAO. (2018). Food loss and waste and the right to adequate food: Making the connection right to food discussion paper. Food and Agriculture Organization of the United Nations, Rome. Available at: www.fao.org/3/ca1397en/CA1397EN.pdf  (Retrieved on 08/01/2026)

Ferronato, N., Gorritty Portillo, M. A., Guisbert Lizarazu, E. G., Torretta, V., Bezzi, M., & Ragazzi, M. (2018). The municipal solid waste management of La Paz (Bolivia): Challenges and opportunities for a sustainable development. Waste Management and Research, 36(3), 288–299. https://doi.org/10.1177/0734242x18755893

Ghafourian, K., Mohamed, Z., Ismail, S., Malakute, R., & Abolghasemi, M. (2016). Construction and demolition waste management. Research Journal of Applied Sciences, Engineering and Technology, 13(9), 741–749. https://doi.org/10.19026/rjaset.13.3348

Gushit J. S., Omadefu K. O. & Egila J. N. (2019) Mobility and bioavailability of heavy metals in soils obtained from open-air automobile repair shop in Jos North LGA, Plateau State, Nigeria. Ewemen Journal of Analytical and Environmental Chemistry. 5(1):198 – 203. https://doi.org/10.15406/japlr.2018.07.00281

He, Z., Shentu, J., Yang, X., Baligar, V. C., Zhang, T., & Stofella, P. J. (2015). Heavy metal contamination of soils: Sources, indicators, and assessment. Journal of Environmental Indicators, 9, 17–18.

Igwe, A. C. & Nwachukwu, O. I. (2016). Soil Health, Our Health: Effect of Paint Effluent Contaminated Soil on the Heavy Metal Content of Okra (Abelmoschus esculentus Moench). Nigerian Journal of Soil Science, 26: 68 -77.

Ihedioha, J. N., Ukoha, P. O. & Ekere, N. R. (2017). Ecological and human health risk assessment of heavy metal contamination in soil of a municipal solid waste dump in Uyo, Nigeria. Environmental geochemistry and health, 39(3), pp.497-515. https://doi.org/10.1007/s10653-016-9830-4

Jilani, S. & Rashid, R. (2020). Municipal solid waste dumping and its impact on soil quality in Karachi. EQA-International journal of environmental quality, 36, pp.9-14.

Johnbosco, U., Chimezie, A. B., & Njoku, R. E. (2020). Impact of used motor oil on soil qualities of Orji Mechanic Village Owerri, Nigeria. International Journal of Engineering Technology and Management Research, 7(2), 1–12.  https://doi.org/10.29121/ijetmr.v7.i2.2020.524

Junior, S. F. S., Mannarino, C. F., de Farias Araújo, G., Bila, D .M., Hauser-Davis, R .A., Saint’Pierre, T., da Costa, G. L., Oliveira, M .M. E., Parente, C. E. T., Correia, F. V. & Saggioro, E. M., (2023). A comprehensive assessment of leachate contamination at a non-operational open dumpsite: mycoflora screening, metal soil pollution indices, and ecotoxicological risks. Environmental Monitoring and Assessment, 195(1), p.243. https://doi.org/10.1007/s10661-022-10885-0

Kabata-Pendias, A., & Pendias, H. (2001). Trace elements in soils and plants (3rd ed.). CRC Press LLC. https://doi.org/10.1201/b10158

Kanmani, S. and Gandhimathi, R. (2013). Investigation of physicochemical characteristics and heavy metal distribution profile in groundwater system around the open dump site. Applied Water Science, 3(2), pp.387-399. https://doi.org/10.1007/s13201-013-0089-y

Kaparwan, D., Rana, N.  S., & Dhyani, B. P. (2020). Heavy metals toxicity in agricultural soils – Critical review of possible sources, influence on soil health and remedial measures to remove, reduce and stabilize contaminants in soil. International Journal of Current Microbiology and Applied Sciences, 9(6), 1467–1482. https://doi.org/10.20546/ijcmas.2020.906.182

Kebede, A. A., Olani, D. D., Edesa, T. G. & Damtew, Y. T. (2016). Heavy metal content and physico-chemical properties of soil around solid waste disposal sites. American Journal of scientific and Industrial research, 7(5), pp.129-139.

Knox, A. S., Gamerdinger, A. P., Adriano, D. C., Kolka, R. K., & Kaplan, D. I. (1999). Sources and Practices Contributing to Soil Contamination In: D. C. Adriano, J. M. Bollag, W. T. Frankenberger (Jr) and R. C. Sims. Remediation of Contaminated Soil. No 37,Agronomy. American Society of Agronomy. https://doi.org/10.2134/agronmonogr37.c4

Kulkarni, B.N. & Anantharama, V. (2024). Comparative analysis of soil contamination caused by existing municipal solid waste management facilities. Environmental Nanotechnology, Monitoring & Management, 22, p.100979. https://doi.org/10.1016/j.enmm.2024.100979

Maas, S., Scheifler, R., Benslama, M., Crini, N., Lucot, E., Brahmia, Z., Benyacoub, S., & Giraudoux, P. (2010). Spatial distribution of heavy metal concentrations in urban, suburban and agricultural soils in a Mediterranean city of Algeria. Environmental Pollution, 158, 2294–2301. https://doi.org/10.1016/j.envpol.2010.02.001

Marinho, A. P. F. D., Nascimento, C. W. A. D., & Cunha, K. P. V. D. (2022). Soil degradation and Cu, Cr, Ni, Pb and Zn contamination in dumpsites of humid and semiarid tropical regions in northeastern Brazil. Environmental Monitoring and Assessment, 194(7), 459. https://doi.org/10.1007/s10661-022-10122-8

Mekonnen, B., Haddis, A. & Zeine, W. (2020). Assessment of the effect of solid waste dump site on surrounding soil and river water quality in Tepi town, Southwest Ethiopia. Journal of environmental and public health, 2020(1), p.5157046. https://doi.org/10.1155/2020/5157046

Mouhoun-Chouaki, S., Derridj, A., Tazdaït, D. & Salah-Tazdaït, R. (2019). A study of the impact of municipal solid waste on some soil physicochemical properties: The case of the landfill of Ain‐El‐Hammam municipality, Algeria. Applied and Environmental Soil Science, 2019(1), p.3560456. https://doi.org/10.1155/2019/3560456

Nta, S.A., Ayotamuno, M.J., Igoni, A.H. & Okparanma, R.H. (2020). Soil quality as affected by municipal solid waste dumping. Asian Soil Research Journal, 3(2), pp.1-11. https://doi.org/10.9734/asrj/2020/v3i230067

Nwaogu, C., Ogbuagu, H. D., Abrakasa, S., Olawoyin, M. A. & Pavlů, V. (2017). Assessment of the impacts of municipal solid waste dumps on soils and plants. Chemistry and ecology, 33(7), pp.589-606. https://doi.org/10.1080/02757540.2017.1337101

Obi, J. C., Ibia, T. O., & Ikott, I. E. (2020). Effects of land use on forms of phosphorus in coastal plain sands of Nigeria. Nigerian Journal of Soil Science, 30(3), 31–41.

Obianefo, F. U., Agbagwa, I.O. & Tanee, F. B. G., (2017). Physicochemical characteristics of soil from selected solid waste dump sites in Port Harcourt, Rivers State, Nigeria. Journal of Applied Sciences and Environmental Management, 21(6), pp.1153-1156. https://doi.org/10.4314/jasem.v21i6.27

Ogundiran, O. O. and Afolabi, T. A. (2008). Assessment of the physicochemical parameters and heavy metals toxicity of leachates from municipal solid waste open dumpsite. International Journal of Environmental Science & Technology, 5(2), pp.243-250. https://doi.org/10.1007/bf03326018

Ojegu, J. (2016). Concentrations of selected heavy metals in soils around refuse dumpsites in some towns in delta state. MSc Thesis Dissertation, Delta State University, Abraka. Nigeria.

Oketola, A.A. & Akpotu, S.O. (2015). Assessment of solid waste and dumpsite leachate and topsoil. Chemistry and ecology, 31(2), pp.134-146.

Okoro , A. C., Chukwuma, G. O.,Chukwuma, E. C., Nwachukwu , P. C & Ezeh, K. A. (2013).

Investigating the effects of selected heavy metals on topsoil at the vicinities of two automobile mechanic villages, Owerri Municipal, Nigeria.  The International Journal of Engineering and Science, 2(11): 19-26.

Olagunju, E., Badmus, O., Ogunlana, F. & Babalola, M. (2018). Environmental impact assessment of waste dumpsite using integrated geochemical and physico-chemical approach: A case study of Ilokun waste dumpsite, Ado-Ekiti, Southern Nigeria. Civil Engineering Research Journal, 4(2), p.555631.

Oluyinka, O. A., Pedavoah, M. M., Abugri, J., Oyelude, E. O., Mosobil, R., Amos, K., Asamannaba, D. A., Issahaku, A. W., Isshak, A. K. K. & Aberinga, N. A., (2024). Soil quality and heavy metal contamination in an open dumpsite in Navrongo, Ghana. Environmental Monitoring and Assessment, 196(9), p.781. https://doi.org/10.1007/s10661-024-12930-6

Omada, J. I., Ogoko, E. C., Kelle, H. I., & Gideon, Y. B. (2024). Heavy metal pollution indices in soil and plants within the vicinity of the Gosa Dumpsite in Abuja, Nigeria. Environmental Geochemistry and Health, 46(7), 223. https://doi.org/10.1007/s10653-024-01979-4

Omeiza, A. J., Lawal, H. A., & Nur, M. (2023). Investigation of groundwater vulnerability to open dumpsites and its potential risk using electrical resistivity and water analysis. Heliyon, 8, e09855. https://doi.org/10.1016/j.heliyon.2023.e13265

Omeiza, J. A., Abdulwahab, O. O., Nur, M. S., et al. (2022). Effect of an active open dumpsite on the earth’s subsurface and groundwater resource. Asian Journal of Physical and Chemical Sciences, 10(2), 15–24. https://doi.org/10.9734/ajopacs/2022/v10i230152

Opara, M. N., Ukpong, U. M., & Okoli, I. C. (2005). Quantitative analysis of abattoir slaughtering of animals in Akwa Ibom State, Nigeria. Journal of Agriculture and Social Research, 5(1), 34–40. https://doi.org/10.4314/jasr.v5i1.2837

Orodu, V.E. &  Leizou, K. E. (2017). Determination of Heavy Metals (Cd and Ni) in Soils of Auto Mechanic Village, Suburb of the Yenagoa City, Bayelsa State, Nigeria. International Journal of Science and Technology, 6 (8): 875-880.

Osim, S.E., Etukudo, M.M., Eze, U.P., Udo, J.I. & Akwe, L.S. (2020). Characterization of physical and chemical properties of soil from three municipal solid waste dumpsites in Bayelsa State, Nigeria. International Journal of Scientific and Research Publications, 10(10), pp.401-404. https://doi.org/10.29322/ijsrp.10.10.2020.p10655

Osu, C. I. & Okereke, V. C. (2015).  Heavy metal Accumulation from Abattoir Wastes on Soils and Some Edible Vegetables in Selected Areas in Umuahia Metropolis. International Journal of Current Microbiology and Applied Sciences, 4(6): 1127-1132

Prechthai, T., Parkpian, P. & Visvanathan, C. (2008). Assessment of heavy metal contamination and its mobilization from municipal solid waste open dumping site. Journal of Hazardous Materials, 156(1-3), pp.86-94. https://doi.org/10.1016/j.jhazmat.2007.11.119

Regmi, T., Ghimire, M., & Shrestha, S. M. (2022). Impact evaluation with potential ecological risk of dumping sites on soil in Baglung Municipality, Nepal. Environmental Challenges, 8, 100564. https://doi.org/10.1016/j.envc.2022.100564

Sanga, V.F., Fabian, C. & Kimbokota, F. (2023). Heavy metal pollution in leachates and its impacts on the quality of groundwater resources around Iringa municipal solid waste dumpsite. Environmental Science and Pollution Research, 30(3), pp.8110-8122. https://doi.org/10.1007/s11356-022-22760-z

Sanga, V. F., & Pius, C. F. (2024). Heavy metal contamination in soil and food crops and associated human health risks in the vicinity of Iringa Municipal dumpsite, Tanzania. Discover Environment, 2(1), 104. https://doi.org/10.1007/s44274-024-00137-y

Sarma Bora, M., Devi, U., Bharadwaj, N., Sharma, P., Kalita, S., Baruah, S., Das, A.K. & Sarma, K.P. (2023). Pollution and health risk assessment of toxic metals in solid waste dumping site soil and its impact on groundwater: a case study. International Journal of Environmental Analytical Chemistry, 103(3), pp.612-632. https://doi.org/10.1080/03067319.2020.1862813

Sharma, A., Gupta, A.K. & Ganguly, R., (2018). Impact of open dumping of municipal solid waste on soil properties in mountainous region. Journal of Rock Mechanics and Geotechnical Engineering, 10(4), pp.725-739. https://doi.org/10.1016/j.jrmge.2017.12.009

Sharma, K. D., & Jain, S. (2020). Municipal solid waste generation, composition, and management: The global scenario. Social Responsibility Journal, 16(6), 917–948.  https://doi.org/10.1108/srj-06-2019-0210

Soubra, G., Massoud, M.A., Alameddine, I., Al Hindi, M. & Sukhn, C. (2021). Assessing the environmental risk and pollution status of soil and water resources in the vicinity of municipal solid waste dumpsites. Environmental Monitoring and Assessment, 193(12), p.857. https://doi.org/10.1007/s10661-021-09640-8

Udo, B. U., Ibia, T. O., & Akpan, U. S., (2025a). Analysis and characteristics of soils underlying open dumpsites in Akwa Ibom State, Nigeria: Their agricultural and environmental implications. Asian Soil Research Journal, 9(3), 90–106. https://doi.org/10.9734/asrj/2025/v9i3193

Udo, B. U., Ibia, T. O., & Udoh, B. T. (2009a). Evaluation of inland depression and floodplain (wetland) soils for rice and maize cultivation in a humid tropical climate of South-eastern Nigeria. International Journal of Agriculture, 1(2), 13–22.

Udo, B. U., Utip, K. E., Inyang, M. T., & Idungafa, M. A. (2009b). Fertility assessment of some inland depression and floodplain (wetland) soils in Akwa Ibom State. Agro-Science, 8(1), 14–19. https://doi.org/10.4314/as.v8i1.44108

Udo, B.U., Ibia, T.O., Akpan, U.S., and Udosen, N I. (2025b.) Waste fingerprints in soils: a study of the influence of wastes on the concentrations of selected heavy metals in soils underlying open dumpsites. Asian Soil Research Journal 9 (3):200-217. https://doi.org/10.9734/asrj/2025/v9i3201

Udoh, B. T., & Ibia, T. O. (2022). Fertility capability classification for agricultural land use planning in the beach sand area of Akwa Ibom State, Nigeria. Agro-Science, 21(2), 74–78. https://doi.org/10.4314/as.v21i2.8

Udosen, N. I. (2022). Geo-electrical modeling of leachate contamination at a major waste disposal site in south-eastern Nigeria. Modeling Earth Systems and Environment, 8(1), 847-856.

https://doi.org/10.1007/s40808-021-01120-9

Udosen, N.I., Ekanem, A.M. & George, N.J. (2024a). Geophysical exploration to assess

leachate percolation and aquifer protectivity within hydrogeological units at a major open dump in Eket, Nigeria. Results in Earth Sciences, p.100022.

https://doi.org/10.1016/j.rines.2024.100022

Udosen, N.I., Ekanem, A.M., George, N.J. and Thomas, J.E. (2024b). Geo-electrostratigraphic assessment of aquifer potential, protectivity, and pliable level of vulnerability within a coastal milieu. Water Practice & Technology,  19(5), pp.2010-2031.

https://doi.org/10.2166/wpt.2024.125

Udosen, N.I., Ekanem, K.R, and George. N. J. (2025). Hydrostratigraphic classification using geoelectrostratigraphic information and hydraulic flow unit analysis for high-resolution aquifer mapping in heterogeneous systems. Geosystems and Geoenvironment, 100434.  https://doi.org/10.1016/j.geogeo.2025.100434

Varier, J., Navya, K. N., & Sujatha C. H. (2025). Assessment of Heavy Metal Contamination and Health Risks in Municipal Solid Waste Dumpsite Soils: A Geostatistical and Multivariate Approach. Environmental Forensics, 1–15. https://doi.org/10.1080/15275922.2025.2539685

Winegardner, D. L. (2019). An Introduction of Soils for Environmental Professionals. CRC Press. https://doi.org/10.1201/9781315137322

World Health Organization (WHO). (2011). Children's health and the environment: WHO training package for the health sector (WHO/HSE/PHE/EPE/11.01.07). www.who.int/ceh  (Retrieved on 08/01/2026).

Wuana, R. A., & Okiemen, F. E. (2011). Heavy metals in contaminated soils: A review of sources, chemistry, risks and best available strategies for remediation. Ecology, 2011, Article ID 402647, 1– 20. https://doi.org/10.5402/2011/402647

Zhao, X., Jiang, G., Wu, L., & Wang, L. (2016). Economic analysis of waste-to-energy industry in China. Waste Management, 48, 604–618. https://doi.org/10.1016/j.wasman.2015.10.014