Impact of Food Waste-Derived Compost with Varying Biochar Contents on Soil Carbon Dioxide (CO₂) Emissions

ABSTRACT. Rising atmospheric carbon dioxide (CO₂) levels contribute significantly to global warming, with agricultural soils acting as both major sources and potential sinks for greenhouse gases (GHG). This study investigated the impact of food waste-derived compost amended with varying concentrations of coconut husk biochar on CO₂ emissions from highly weathered tropical soil. Biochar was produced via pyrolysis at 550 – 620°C and co-composted with domestic organic waste (cassava, plantain, and pineapple peels, and poultry manure) at ratios of 0%, 2%, 5%, and 10%. Six treatments, including an absolute control and inorganic NPK fertilizer, were applied to soil containers and incubated for 58 days. CO₂ emissions were quantified using sodium hydroxide (NaOH) trapping followed by hydrochloric acid (HCl) titration at intervals ranging from day 0 to day 58. Results indicated a significant reduction in emissions across all treatments after the initial incubation day. The Compost + 10% Biochar treatment consistently demonstrated the lowest CO₂ emissions (0.88 mg CO₂/day/kg on day 0), significantly outperforming the control (1.21 mg CO₂/day/kg), inorganic fertilizer, and lower biochar concentrations. By day 58, emissions across all treatments approached zero. The study concludes that amending food waste compost with 10% coconut husk biochar effectively suppresses soil CO₂ fluxes, acting as a carbon sink while enhancing soil fertility. This integrated organic management strategy offers a sustainable, climate-smart solution for reducing greenhouse gas emissions and promoting soil health in tropical agricultural systems.

Keywords: biochar, biochar co- compost, NPK fertilizer, carbon dioxide emission, coconut husk, organic

## INTRODUCTION

Greenhouse gas (GHG) emissions particularly carbon dioxide (CO₂) have risen significantly over recent decades contributing to approximately 60% of global warming (Minx et al., 2021). Soils play a dual role in the global carbon cycle acting both as a major source and a potential sink for atmospheric CO₂. Soil respiration alone accounts for about 20% of total CO₂ emissions (Kudeyarov, 2023) .Agricultural activities contribute 20–35% of global GHG emissions second only to the energy sector (Chataut et al., 2023) making sustainable soil management critical for climate change mitigation.

Organic amendments especially those derived from food waste offer a promising strategy to improve soil health while reducing reliance on synthetic inputs (Yadav et al., 2024). Globally an estimated 931 million tons of food were wasted in 2019 about 17% of total food available to consumers accounting for 8–10% of global GHG emissions (FOOD WASTE INDEX REPORT 2021, 2021) . In Ghana rapid population growth and urbanization exacerbate solid waste challenges with municipal systems often overwhelmed leading to indiscriminate dumping (Kwakye et al., 2024). Converting food waste into compost presents a waste-to-resource opportunity that can enhance soil fertility reduce landfill emissions and support circular agriculture (Kumar et al., 2025).

However the application of organic amendments can influence GHG dynamics in complex ways (Matisic et al., 2024). While compost improves soil structure water retention and microbial activity (Wang et al., 2022)  it may also stimulate microbial respiration and root activity potentially increasing CO₂ emissions (Panchal et al., 2022). The impact depends on the composition of the organic material particularly its carbon-to-nitrogen (C/N) ratio which affects decomposition rates and gas fluxes (Shakoor et al., 2021). Moreover organic fertilizers often contain variable nutrient concentrations posing challenges for precise nutrient management (Rajanna, 2023).

Biochar a carbon-rich stable product of biomass pyrolysis has emerged as a promising soil amendment for climate-smart agriculture (Bhattacharyya et al., 2024). When incorporated into soil biochar can enhance carbon sequestration improve nutrient retention and suppress CO₂ emissions by stabilizing organic matter and reducing microbial decomposition rates (Elkhlifi et al., 2023). Coconut husks abundant in tropical regions like Ghana represent an underutilized feedstock for biochar production (Fadavi et al., 2025). Combining biochar with compost may synergistically improve soil properties while mitigating GHG emissions yet systematic research on such blends particularly those using food waste and varying biochar ratios is limited (Mia & Zzaman, 2025).

The overuse of inorganic fertilizers further complicates this landscape contributing to soil acidification nutrient runoff reduced microbial diversity and elevated GHG emissions (Ali et al., 2025). Integrating organic and inorganic inputs in balanced proportions can optimize nutrient availability support soil health and reduce environmental impacts (Rajanna, 2023). However, access to organic fertilizers remains constrained in Ghana due to high costs logistical barriers and limited awareness (Daadi & Latacz-Lohmann, 2021).

The purpose of this study is to assess the impact of organic amendments prepared from food waste with varying biochar contents on CO₂ emissions. To achieve this objective, the study specifically evaluates the effect of biochar incorporation on CO₂ emissions, assesses the impact of compost derived from food waste on CO₂ emissions, and compares these organic treatments with inorganic fertilizer amendments to determine their relative influence on soil CO₂ fluxes.

## MATERIALS AND METHODS

## Experimental site

The experiments were conducted from 2022 to 2023 at A.G. Carson Technology Village at the School of Agriculture, University of Cape Coast, Ghana. The study area is in Cape Coast in the Central Region of Ghana, which is situated between latitudes 5° and 5.11° north of the equator and between longitudes 1.20° and 1.25° west of Greenwich. It is bounded on the east by Abura-Asebu-Kwamankese District, west by Komenda-Edina-Eguafo-Abrem (KEEA) District and south by Gulf of Guinea and north Twifo-Heman lower Denkyria District. The total land area of Cape Coast municipality is approximately 122sq. km.

## Climatic Conditions of the region

The area has two seasons of rainfall, with the highest rainfall occurring from March to July and the minor season occurring from September to November. According to (Owusu-Sekyere et al., 2011), the mean annual rainfall in the Cape Coast metropolis ranges between 900 mm and 1,600 mm, and the annual mean is 940.1mm. The mean or dry maximum and night temperatures are 32.7˚C and 23.5˚C respectively. Temperature variation is greatest in February and March the relative humidity is uniformly, high at about 70% in the morning and 55% in the afternoon.

## Biochar Production

Biochar used in the experiment was made from coconut husk. The feedstock for the pyrolysis was chosen as a result of ready availability, ease of accessibility and relatively cheaper. Dried coconut husk was collected from roadside sale locations in Ekumfi Abor, a farming village in Ghana's Central Region where the majority of farmers sell fresh coconut water along the highway. The feedstock was sun-dried  before pyrolysis began  (Ghysels et al., 2020). ELSA biomass pyrolytic barrels were loaded with dried coconut husk. The barrel is built from metal and has a combustion chamber, a ventilation exit, and a lid. The barrel was filled with dry coconut husk feedstock and dried rice straw was placed on the surface of the feedstock in the chamber as fuel material, and a match was ignited. The charring occurred between 550 and 620 °C, with a residence time of 30 to 45 minutes and a 46% char yield. The generated coconut husk biochar was milled into powdered form using a mill to obtain a uniform size before incorporation after two days of sun drying.

Figure 1. Coconut husk being dried in the sun before pyrolysis.          Figure 2.  Barrel filled with coconut husk for the charring process

## 3.3 Co-Composted Biochar preparation

Compost used for the study was prepared from three basic materials which include domestic waste (cassava peels, plantain peels, pineapple peels and solid food leftover), poultry manure and Coconut husk biochar using the windrow method. Coconut husk biochar was obtained through pyrolysis, poultry manure with decomposed bedding material collected from deep litter system layer pen in the University of Cape Coast Teaching and Research Farms, cassava, plantain and pineapple peels collected from nearby restaurants and solid food leftover collected from nearby Senior High Schools such as Adisadel College, Ghana National College and University Practice S.H.S. The food waste was air dried, separated, and grating was used to decrease the particle size. The remaining feedstocks except the biochar, was then measured, divided into four equal portions, and made into four compost heaps, each containing 100 kg of the feedstocks. The biochar co-composts were prepared separately by adding varying amounts (0%,2%, 5% and 10%) of the coconut husk biochar. In the four biochar co-compost rates, all feedstocks, including biochar, was thoroughly mixed, moistened, and heaped. Pure compost without biochar was Heap 1, 2 kg of biochar was added in Heap 2, 5 kg of biochar was added in Heap 3, and 10 kg of biochar was also added in Heap 4. For the first two weeks, the heaps were turned on every three days, then once a week until they reached maturity. The compost took two months to mature and for three weeks to cure and dry.

Figure 3. Domestic waste, poultry manure and Coconut husk biochar being mixed for compost using the windrow method

3.4 Table 1: Treatments details

| TREATMENTS | SOIL | N | P | K | COMPOST |
| --- | --- | --- | --- | --- | --- |
| Grams (g) |  |  |  |  |  |
| CONTROL | 100 | - | - | - | - |
| INORGANIC | 100 | 0.033 | 0.020 | 0.015 |  |
| COMPOST | 100 | - | 0.018 | 0.012 | 1.24 |
| COMPOST+2% BIOCHAR | 100 | - | 0.018 | 0.012 | 1.06 |
| COMPOST+5% BIOCHAR | 100 | - | 0.018 | 0.012 | 0.81 |
| COMPOST+10% BIOCHAR | 100 | - | 0.018 | 0.012 | 0.73 |

A characterized, extremely weathered tropical soil was chosen, sampled, air dried and placed in containers. 100g of the air-dried soil was weighed into each of the containers. For each container, various co-composted biochar was weighed based on plant nutrient requirements and compost nitrogen contents. Straight fertilizers’ (Urea, Muriate of Potash & Triple Superphosphate) top-up was used to supplement nutrient limitations in the composts. Treatments was six with four replications summing up to twenty-four (24) containers in total. Treatment 1 - absolute control, Treatment 2 -- amended control-NPK, Treatment 3 – 0% biochar (pure compost), Treatment 4 -- 2% biochar co-compost with P&K top up, Treatment 5 -- 5% biochar co-compost with P&K top up Treatment 6 -- 10% biochar co-compost with P&K top up. For each treatment and its replications; the composition 100g (0.1kg) of soil, compost and straight fertilizers). The 24 containers were incubated in a dark environment  (cardboard) in the soil science laboratory. 17mL of water was sprayed on each treatment.

Figure 4. Incubation of treatment

## 3.5 Laboratory analysis and carbon dioxide emission measurement

About 20 mL of 0.1M of NaOH was poured into small containers which was used for the trapping of the CO2 and was placed in the containers filled with the treatments. The containers were closed tightly and incubated in a cardboard. After 6 hours of incubation, the containers, containing the NaOH was removed. The various treatments containing the incubated NaOH was poured into a conical flask, 5mL of 1M barium chloride (BaCl2) was added to each incubated NaOH and there was a milky precipitate formed. 2-3 drops of phenolphthalein indicator were dropped and there was a color change from milky precipitate to pinkish. The NaOH was titrated against 1M hydrochloric acid solution (Nag et al., 2025) and the titer value was recorded. The study was conducted for 58 days of which after 6 hours of incubation, the titration done was noted as day 0, the analysis continued after every 3 days for 3 times, then weekly for 2 times, then after the 14th day and then the 21st day. The amount of CO2 evolved from the soil during the incubation was calculated using the formula;

Milligrams of C or CO2=Vx Nx E

Where V is the volume (mL) of HCl needed to titrate the NaOH in the containers incubated, N is the Normality of the HCl and E is the equivalent weight.

To express the data in terms of carbon, E = 6 to express it as CO2, E = 22.

Figure 5.  Incubated NaOH was poured into a conical flask, 5mL of 1M BaCl2 was added to each incubated NaOH and there was a milky precipitate formed, 2-3 drops of phenolphthalein indicator were dropped and there was a color change from milky precipitate to pinkish.

Data analysis

## The statistical analysis involved conducting four independent one-way ANOVA, one for each sampling day (Days 0, 3, 6, 9, 16, 23, 37 and 58), to assess the effect of soil amendment treatments on CO₂ emissions. When the ANOVA indicated significant differences (p ≤ 0.05), a post-hoc mean separation test (Duncan’s Multiple Range Test) was used to assign homogeneous letter groups, enabling clear visual identification of which treatments differed significantly from one another on each day.

## 4.0 RESULTS

The daily carbon dioxide emissions from the various treatments are shown in table 2 and 3 below.

4.1 Table 2: Soil amendments effects on CO2 emission on daily basis.

| TREATMENTS | DAY 0 | DAY 3 | DAY 6 | DAY 9 |
| --- | --- | --- | --- | --- |
| CO2 /day/kg |  |  |  |  |
| CONTROL | 1.21± 0.11a | 0.07± 0.00a | 0.03± 0.00a | 0.05±0.02a |
| INORGANIC | 1.18± 0.06a | 0.02± 0.01b | 0.03± 0.01a | 0.04±0.00a |
| COMPOST | 1.03± 0.05b | 0.01± 0.01bc | 0.00± 0.00b | 0.01±0.00b |
| COMPOST+ 2% BIOCHAR | 1.04± 0.12b | 0.01± 0.01bc | 0.01± 0.00b | 0.02±0.01b |
| COMPOST+ 5% BIOCHAR | 0.96±0.10bc | 0.01± 0.01bc | 0.01± 0.01bc | 0.02±0.01b |
| COMPOST+ 10% BIOCHAR | 0.88± 0.08c | 0.01± 0.01c | 0.01± 0.01c | 0.02±0.00b |
| ANOVA p-value | < 0.001 | < 0.001 | 0.003 | < 0.001 |

Table 3: Soil amendments effects on CO2 emission on daily basis continued

| TREATMENTS | DAY 16 | DAY 23 | DAY 37 | DAY 58 |
| --- | --- | --- | --- | --- |
| CO2 /day/kg |  |  |  |  |
| CONTROL | 0.02± 0.00a | 0.01± 0.00a | 0.00± 0.00a | 0.00± 0.00a |
| INORGANIC | 0.02± 0.00a | 0.01± 0.00b | 0.00± 0.00ab | 0.00± 0.00a |
| COMPOST | 0.01± 0.00b | 0.00± 0.00c | 0.00± 0.00bc | 0.00± 0.00a |
| COMPOST+ 2% BIOCHAR | 0.01± 0.00c | 0.00± 0.00c | 0.00± 0.00bc | 0.00± 0.00a |
| COMPOST+ 5% BIOCHAR | 0.00± 0.00c | 0.00± 0.00c | 0.00± 0.00c | 0.00± 0.00a |
| COMPOST+ 10% BIOCHAR | 0.00± 0.00c | 0.00± 0.00c | 0.00± 0.00c | 0.00± 0.00a |
| ANOVA p-value | < 0.001 | < 0.001 | 0.01 | >0.05 |

Based on the CO2 emission trend, it was noted that there was a significant reduction in emissions after the first day of incubation across all types of treatments. The specific level of CO2 emissions varied depending on the treatment. Aligning with the respective sampling dates. The maximum emission was recorded on the first day of incubation. After 3 days of incubation, CO2 emission came down sharply in all types of treatments. The lowest amount of CO2 emission was recorded in the treatment with compost+10% biochar (Table 2), while the highest amount of CO2 emission was recorded in the control where only soil was used. At the first 6 hours of incubation CO2 emission was about 1.21 CO2/day/kg soil in the control, 1.181 CO2/day/kg soil when inorganic amendments (NPK) were added, while it was about 1.03 CO2/day/kg soil when pure compost was added. Co-composted biochar (2%) mixed with soil released 1.043 CO2/day/kg soil, co-composted biochar (5%) mixed with soil released 0.955 CO2/day/kg soil while co-composted biochar (10%) mixed with soil also released 0.876 CO2/day/kg soil.

From (Table 3), the lowest emissions were observed in the co-composted biochar (10%) treatment. This can be attributed to the fact that this treatment initially had the lowest emission of carbon compared to the control. The reason behind this phenomenon is that the co-composted biochar (10%) has a high carbon content but is relatively stable and less prone to rapid decomposition. When 10% biochar is co-composted with other organic materials, it can act as a carbon sink, effectively sequestering carbon and reducing the overall emissions of CO2.The control emitted significantly higher CO2 in day 0, day 6, day 9, day 16 and day 58 than any other treatment except inorganic while compost with 10% biochar emitted significantly lower CO2. For day 3, day 23 and day 37, only the control emitted significantly higher CO2 than any other treatment while the compost with 10% biochar still emitted the lowest CO2.

After 6 hours of incubation the average CO2 emission was recorded as 1.049 CO2/day/kg soil. On day 3, the average CO2 emission was recorded as 0.022 CO2/day/kg soil, while the average CO2 emission was about 0.016 CO2/day/kg soil on 6th day of incubation. On the 9th day of incubation, the average CO2 emission recorded as 0.027 CO2/day/kg soil, also on the 16th day of incubation, the average CO2 emission recorded 0.01 CO2/day/kg soil. The average emission recorded 0.006 CO2/day/kg soil on the 23rd day while it also recorded 0.003 CO2/day/kg soil and 0.001 CO2/day/kg soil on the 37th day and 58th day.

The daily carbon emissions from the various treatments are shown in table 4 and 5 below.

Table 4: Soil amendments effects on CO2-C emission on daily basis

| TREATMENTS | DAY 0 | DAY 3 | DAY 6 | DAY 9 |
| --- | --- | --- | --- | --- |
| mg_CO2_C/day/kg |  |  |  |  |
| CONTROL | 0.33± 0.03a | 0.02± 0.00a | 0.01± 0.00a | 0.01± 0.00a |
| INORGANIC | 0.32± 0.02a | 0.01± 0.00b | 0.01± 0.00a | 0.01± 0.00a |
| COMPOST | 0.28± 0.01b | 0.00± 0.00bc | 0.00± 0.00b | 0.00± 0.00b |
| COMPOST+2% BIOCHAR | 0.28± 0.03b | 0.00± 0.00bc | 0.00± 0.00b | 0.01± 0.00b |
| COMPOST+5% BIOCHAR | 0.26±0.03bc | 0.00± 0.00bc | 0.00±0.00bc | 0.01± 0.00b |
| COMPOST+10% BIOCHAR | 0.23± 0.02c | 0.00± 0.00c | 0.00± 0.00c | 0.01± 0.00b |
| ANOVA p-value | < 0.001 | < 0.001 | 0.004 | 0.02 |

Table 5: Soil amendments effects on CO2-C emission on daily basis continued

| TREATMENTS | DAY 16 | DAY 23 | DAY 37 | DAY 58 |
| --- | --- | --- | --- | --- |
| mg_CO2_C/day/kg |  |  |  |  |
| CONTROL | 0.01± 0.00a | 0.00± 0.00a | 0.00± 0.00a | 0.00± 0.00a |
| INORGANIC | 0.00± 0.00a | 0.00± 0.00b | 0.00± 0.00ab | 0.00± 0.00a |
| COMPOST | 0.00± 0.00b | 0.00± 0.00c | 0.00± 0.00bc | 0.00± 0.00a |
| COMPOST+ 2% BIOCHAR | 0.00± 0.00c | 0.00± 0.00c | 0.00± 0.00bc | 0.00± 0.00a |
| COMPOST+ 5% BIOCHAR | 0.00± 0.00c | 0.00± 0.00c | 0.00± 0.00c | 0.00± 0.00a |
| COMPOST+ 10% BIOCHAR | 0.00± 0.00c | 0.00± 0.00c | 0.00± 0.00c | 0.00± 0.00a |
| ANOVA p-value | < 0.001 | < 0.001 | 0.01 | >0.05 |

Based on the CO2-C emission trend, it was noted that there was a significant reduction in emissions after the first day of incubation across all types of treatments. The specific level of CO2-C emissions varied depending on the treatment. Aligning with the respective sampling dates. The maximum emission was recorded on the first day of incubation. After 3 days of incubation, CO2-C emission came down sharply in all types of treatments. The lowest amount of CO2-C emission was recorded in the treatment with compost+10% biochar (Table 4 and 5), while the highest amount of CO2-C emission was recorded in the absolute control where only soil was used. At the first 6 hours of incubation CO2-C emission was about 0.33 mg CO2/day/kg soil in the absolute control, 0.3222 mg CO2/day/kg soil when inorganic amendments (NPK) were added, while it was about 0.2808 mg CO2/day/kg soil when pure compost was added. Co-composted biochar (2%) mixed with soil released 0.2844 mg CO2/day/kg soil, co-composted biochar (5%) mixed with soil released 0.2604 mg CO2/day/kg soil while co-composted biochar (10%) mixed with soil also released 0.2388 mg CO2/day/kg soil. The control emitted significantly higher CO2-C in day 0, day 6, day 9, day 16 and day 58 than any other treatment except inorganic while compost with 10% biochar emitted significantly lower CO2-C. For day 3, day 23 and day 37, only the control emitted significantly higher CO2-C than any other treatment while the compost with 10% biochar still emitted the lowest CO2-C.

After 6 hours of incubation, the average CO2-C emission was 0.286 mg CO2/day/kg soil. On day 3, the average CO2-C emission decreased to 0.006 mg CO2/day/kg soil. By the 6th  day of incubation, the average CO2-C emission further decreased to approximately 0.004 mg CO2/day/kg soil. On the 9th day of incubation, the average CO2-C emission slightly increased to 0.007 mg CO2/day/kg soil. Subsequently, on the 16th day of incubation, the average CO2-C emission decreased to 0.002 mg CO2/day/kg soil. Further monitoring showed a continued decrease, with the average emission reaching 0.001 mg CO2/day/kg soil on the 23rd day. As time progressed, the emissions declined even more, recording 0.0007 mg CO2/day/kg soil on the 37th day and 0.00028 mg CO2/day/kg soil on the 58th day.

This data illustrates the dynamic nature of CO2-C emissions from the soil over the course of the incubation period, with an initial increase followed by a gradual decline. It was reported that if organic matter added to soil containing high amount of labile carbon potentially enhances CO2 emission and thus restricts carbon accumulation in soil (Moore et al., 2008). The trend of CO2 and CO2-C emission from different treatments where emission dropped down after weeks of incubation compelled a reconsideration of the direct use of these biochar co-compost in agriculture. Biochar combined with compost could be the alternative environment-friendly practice, which reduces GHGs in the atmosphere and can enhance soil structure and fertility, leading to healthier and more productive soils. Healthy soils support plant growth and sequester carbon through plant biomass, reducing the need for fossil fuel-intensive agricultural practices.

DISCUSSION

The rate at which CO2 emissions change in response to biochar application is correlated with the amount of biochar added (Lyu et al., 2022). In particular, it was observed that cumulative CO2 production significantly increased compared to the control when biochar was applied at rates of 1% and 2% (Olaniyan et al., 2020). However, no discernible trends in CO2 emissions occurred when soil was amended with 5% and 10% biochar application rates. Moreover, the impact of biochar on CO2 emissions was contingent upon the initial carbon content of the soil it was applied to. The introduction of biochar to soil with a high carbon content did not yield any additional shifts in CO2 emissions (Lehmann et al., 2021).

From this study, the combination of biochar and compost had a considerable effect on the reduction of CO2 emissions after the initial days of incubation. This indicates that biochar contributed to the mitigation of initial emissions. The lowest emissions were seen in the treatment containing 10% of biochar, which is attributed to the high carbon content of the biochar and its stability, which reduces its tendency to decompose quickly. Furthermore, when the 10% biochar was added in the soil, it acted as a carbon sink, absorbing carbon and reducing CO2 emissions overall. These results suggest that biochar, if properly integrated into soil, not only has the potential to reduce CO2 emissions, but it also has the potential to sequester carbon, which is important for mitigating climate change. In our study, we observed that the effects of adding biochar with compost differ from the findings mentioned in the literature. This disparity can be attributed to the specific feedstock used during the biochar and compost preparation processes. The type and source of feedstock used in the production of both biochar and compost can significantly influence their properties and, subsequently, their impact on soil and CO2 emissions. Variations in feedstock characteristics, such as carbon content, nutrient composition, and physical structure, can lead to distinct outcomes when biochar and compost are combined and applied to soil. It is important to acknowledge that biochar and compost can be tailored to suit particular objectives. Therefore, the choice of feedstock and the preparation methods play a crucial role in determining how they interact with soil and influence CO2 emissions.

Composting can increase the content of soil organic matter, improve soil physiology and microbial characteristics, promote microbial fixation of nutrients, reduce nutrient loss, and reduce CH4 emissions (Manea et al., 2024).  However, at the same time, it can lead to an increase in N2O emissions, and promote the recycling of natural resources (Filonchyk et al., 2024). Therefore, transformation from inorganic agriculture to ecological and green agriculture is necessary (Lou et al., 2024). Use of compost can promote resource utilization of agricultural waste, reduce the use of chemical fertilizers, and improve crop yield and quality, indicating that it has a high practical application value (Mengqi et al., 2023).

From this study, the results indicate that when pure compost is added to incubated soil, there is an initial rise in CO2 emissions within the first 6 hours. This is to be expected, as compost provides organic matter that encourages microbial activity, resulting in the release of carbon dioxide. However, over time, the emissions decrease, suggesting that the pure compost may have contributed to improved soil structure and microbial community, which can help to maintain carbon sequestration and reduce emissions. This suggests that pure compost may be a viable option for soil management practices, as it may provide a balance between short-term emissions increases and potentially long-term benefits to soil health and sequestration of carbon.

The addition of inorganic amendments (NPK) resulted in a slight reduction in CO2 emissions compared to the control, in the long run. The initial rise can be explained by the presence of nitrogen-containing inorganic fertilizers in the soil. These nitrogen-containing fertilizers provide readily accessible nutrients to soil microbes, which in turn stimulate microbial activity. This microbial activity accelerates the rate at which organic matter decomposes in the soil. As a result of this microbial activity, CO2 is produced as a by-product. As a result, there is an initial increase in emissions. Inorganic amendments such as NPK are mainly responsible for the provision of essential nutrients to plants, such as nitrogen, phosphorus, and potassium. However, their effects are not limited to plant nutrition alone. The added nutrients stimulate an increase in microbial activity, especially among nitrogen fixation and nitrification bacteria. As nitrogen is a fundamental component of amino acid and protein synthesis, it is essential for microbial development. Consequently, the rapid microbial degradation of organic matter may result in an increase in CO2 emissions.

CONCLUSION

The incorporation of food waste-derived compost amended with coconut husk biochar (0%, 2%, 5%, and 10%) into weathered tropical soil significantly reduced CO₂ emissions after an initial peak on day 0, fulfilling the study's objectives to evaluate biochar's role, compost effects from food waste, and comparisons with inorganic NPK fertilizers. The 10% biochar co-compost treatment consistently exhibited the lowest cumulative CO₂ emissions (e.g., 0.88 mg CO₂ /day/kg soil on day 0, declining to near zero by day 58), outperforming pure compost, NPK, and control. This suppression is attributed to biochar's high stability and carbon sink capacity, which limits microbial decomposition of labile carbon.​ These findings underscore the potential of 10% biochar co-compost as a climate-smart amendment for tropical soils, promoting carbon sequestration, enhancing soil structure, and minimizing GHG contributions from agriculture critical in Ghana where, food waste and weathered soils challenge sustainability. While NPK provided minor early reductions, its stimulation of microbial activity warrants cautious use. Therefore, a balanced soil management strategy integrating compost and 10% biochar is recommended to harness short-term microbial benefits and long-term carbon storage, while inorganic fertilizer use should be carefully calibrated or supplemented with slow-release or organic alternatives to minimize CO₂ release. Future field trials should assess long-term effects on crop yields, nutrient dynamics, and other GHGs (e.g., CH₄, N₂O) under varying climatic conditions to validate scalability.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

## REFERENCES

1.	Minx JC, Lamb WF, Andrew RM, Canadell JG, Crippa M, Döbbeling N, et al. A comprehensive and synthetic dataset for global, regional, and national greenhouse gas emissions by sector 1970–2018 with an extension to 2019. Earth Syst Sci Data. 2021;13(11):5213–52.

2.	Kudeyarov VN. Soil Respiration and Carbon Sequestration : A Review. 2023;56(9):1191–200.

3.	Chataut G, Bhatta B, Joshi D, Subedi K, Kafle K. Greenhouse gases emission from agricultural soil : A review. J Agric Food Res [Internet]. 2023;11(January):100533. Available from: https://doi.org/10.1016/j.jafr.2023.100533

4.	Yadav H, Neeraj A, Vats P. Improvement of soil health and crop production through utilization of organic wastes : A sustainable approach. 2024;13(1):1–15.

5.	FOOD WASTE INDEX REPORT 2021. 2021.

6.	Kwakye SO, Ebo E, Amuah Y, Ankoma KA, Agyemang EB, Owusu B george. Understanding the performance and challenges of solid waste management in an emerging megacity : Insights from the developing world. Environ Challenges [Internet]. 2024;14(December 2023):100805. Available from: https://doi.org/10.1016/j.envc.2023.100805

7.	Kumar K, Khatri A, Thakur IS. Agricultural Waste Management for Food Security and Sustainability. 2025;

8.	Matisic M, Dugan I, Bogunovic I. Challenges in Sustainable Agriculture — The Role of Organic Amendments. 2024;

9.	Wang D, Lin JY, Sayre JM, Schmidt R, Fonte SJ, Rodrigues JLM, et al. Geoderma Compost amendment maintains soil structure and carbon storage by increasing available carbon and microbial biomass in agricultural soil – A six-year field study. 2022;427(August).

10.	Panchal P, Preece C, Peñuelas J, Giri J. Soil carbon sequestration by root exudates. Trends Plant Sci [Internet]. 2022;27(8):749–57. Available from: https://doi.org/10.1016/j.tplants.2022.04.009

11.	Shakoor A, Shahzad SM, Chatterjee N, Arif MS, Farooq TH, Altaf MM, et al. Nitrous oxide emission from agricultural soils: Application of animal manure or biochar? A global meta-analysis. J Environ Manage. 2021;285:112170.

12.	Rajanna GA. Integrated nutrient management for improving crop yields , soil properties , and reducing greenhouse gas emissions. 2023;(June):1–13.

13.	Bhattacharyya PN, Protim S, Bhaskar S, Abhay S. Biochar as Soil Amendment in Climate ‑ Smart Agriculture : Opportunities , Future Prospects , and Challenges. J Soil Sci Plant Nutr [Internet]. 2024;24(1):135–58. Available from: https://doi.org/10.1007/s42729-024-01629-9

14.	Elkhlifi Z, Iftikhar J, Sarraf M, Ali B, Saleem MH, Ibranshahib I, et al. Potential Role of Biochar on Capturing Soil Nutrients , Carbon Sequestration and Managing Environmental Challenges : A Review. 2023;

15.	Fadavi C, Felicia T, Daniel A. Biochar as a Soil Amendment in Rice Production : Current Status , Debates , and Opportunities for Enhancing Rice Farming in Ghana License : This is an open access article under the CC BY 4 . 0 license : https://creativecommons.org/licenses/by/4.0/ Corresp. 2025;148–84.

16.	Mia S, Zzaman W. Review Article Food Waste – Derived Organic Fertilizers : Critical Insights , Agronomic Impacts , and Pathways for Sustainable Adoption. 2025;2025.

17.	Ali A, Jabeen N, Chachar Z, Chachar S, Ahmed S, Ahmed N, et al. The role of biochar in enhancing soil health & interactions with rhizosphere properties and enzyme activities in organic fertilizer substitution. 2025;(June):1–23.

18.	Daadi BE, Latacz-Lohmann U. Organic fertilizer use by smallholder farmers: Typology of management approaches in northern Ghana. Renew Agric Food Syst. 2021;36(2):192–206.

19.	Owusu-Sekyere JD, Alhassan M, Nyarko BK. Assessment of climate shift and crop yields in the Cape Coast area in the Central Region of Ghana. ARPN J Agric Biol Sci. 2011;6(2):49–54.

20.	Ghysels S, Acosta N, Estrada A, Pala M, De Vrieze J, Ronsse F, et al. Integrating anaerobic digestion and slow pyrolysis improves the product portfolio of a cocoa waste biorefinery. Sustain Energy Fuels. 2020;4(7):3712–25.

21.	Nag SK, Ghosh B Das, Das BK, Sarkar UK. Wetlands function as carbon sink: Evaluation of few floodplains of middle Assam, northeast India in the perspective of climate change. J Environ Manage [Internet]. 2025;373(December 2024):123841. Available from: https://doi.org/10.1016/j.jenvman.2024.123841

22.	Moore DJP, Gonzalez-Meler MA, Taneva L, Pippen JS, Kim HS, DeLucia EH. The effect of carbon dioxide enrichment on apparent stem respiration from Pinus taeda L. is confounded by high levels of soil carbon dioxide. Oecologia. 2008;158:1–10.

23.	Lyu H, Zhang H, Chu M, Zhang C, Tang J, Chang SX, et al. Biochar affects greenhouse gas emissions in various environments: A critical review. L Degrad Dev. 2022;33(17):3327–42.

24.	Olaniyan JO, Isimikalu TO, Raji BA, Affinnih KO, Alasinrin SY, Ajala ON. An investigation of the effect of biochar application rates on CO2 emissions in soils under upland rice production in southern Guinea Savannah of Nigeria. Heliyon. 2020;6(11).

25.	Lehmann J, Cowie A, Masiello CA, Kammann C, Woolf D, Amonette JE, et al. Biochar in climate change mitigation. 2021;14(December).

26.	Manea EE, Bumbac C, Dinu LR, Bumbac M, Nicolescu CM. Composting as a Sustainable Solution for Organic Solid Waste Management: Current Practices and Potential Improvements. Sustain. 2024;16(15):1–25.

27.	Filonchyk M, Peterson MP, Zhang L, Hurynovich V, He Y. Greenhouse gases emissions and global climate change: Examining the influence of CO2, CH4, and N2O. Sci Total Environ. 2024;935(January).

28.	Lou R, Li Y, Liu Y, Jiang Q, Wu Q, He Y, et al. Is green manure a viable substitute for inorganic fertilizer to improve grain yields and advance carbon neutrality in paddy agriculture? Agric Ecosyst Environ [Internet]. 2024;359(September 2023):108762. Available from: https://doi.org/10.1016/j.agee.2023.108762

29.	Mengqi Z, Shi A, Ajmal M, Ye L, Awais M. Comprehensive review on agricultural waste utilization and high-temperature fermentation and composting. Biomass Convers Biorefinery. 2023;13(7):5445–68.