# Original Research Article

Occurrence and Distribution of Multi-Class Pesticide Residues in Municipal Solid Waste Compost from Bengaluru, India

.

ABSTRACT

The present study embodies results on the occurrence and distribution of pesticide residues in MSW compost collected from six operational composting facilities in Bengaluru, India. Pesticide residues were analysed using gas chromatography–tandem mass spectrometry (GC–MS/MS) employing a multi-residue screening approach targeting 185 pesticide residues. Among these, 172 were detected in at least one compost sample, indicating widespread occurrence of residues in MSW compost. Organo-Nitrogen residues (ONP) constituted the largest proportion, followed by Organo-phosphates (0PP), Organo-chlorines (OCP), and synthetic Pyrethroids (SPP). Frequency analysis showed that 21 pesticides were present in all compost samples, while others exhibited variable occurrence across composting units. Concentration analysis revealed that 21 pesticides exceeded 1 mg kg⁻¹, with cypermethrin (17 mg kg⁻¹) and cyfluthrin isomers (13 mg kg⁻¹) among the most abundant residues. The findings emphasize the importance of routine monitoring to ensure environmental safety and sustainable agricultural use.

Keywords: Frequency analysis; GC–MS/MS; MSW compost; Multi-residue screening; Pesticide residues

1. INTRODUCTION

Managing the biodegradable fraction of municipal solid waste (MSW) remains a major environmental challenge, particularly in rapidly urbanizing regions. Composting is widely recognized as an effective and sustainable tool for recycling organic waste, converting biodegradable materials into nutrient-rich compost that can improve soil fertility and support sustainable agricultural practices (Manea et al. 2024; Sayara et al. 2020). MSW compost is increasingly promoted as an organic soil amendment because of its ability to enhance soil structure, increase organic carbon content, and improve microbial activity. The application of compost derived from municipal waste can also contribute to nutrient recycling and reduce reliance on synthetic fertilizers (Hargreaves et al.,2008).

Despite several benefits, MSW compost made from mixed urban waste streams may contain many contaminants, including heavy metals, pharmaceuticals, and pesticide residues. Pesticides can enter municipal waste streams through discarded fruits and vegetables, market waste, household kitchen waste, and horticultural residues (Prasopsuk and Boonthai 2011). It is also observed that contaminant levels in urban areas exceed those in rural areas because of a generally more diverse urban ecosystem than agricultural ecosystems, which are limited to specific crops(Wittmer et al. 2010). The residues that enter municipal waste streams may persist during the composting process, depending on their chemical stability and composting conditions (Büyüksönmez et al. 1999; Lourencetti et al. 2007).

It is a known fact that composting can promote degradation of certain pesticide compounds through microbial activity and thermophilic conditions, some persistent pesticides may remain detectable in the final compost produce (Kupper et al. 2008; Lourencetti et al. 2007; Brown, et al. 1997). Long term application of such compost to agricultural soils may therefore introduce trace pesticide residues into soil ecosystems (Vorkamp et al. 2003). Hence, monitoring pesticide concentrations in compost is essential to track their transfer into soils and subsequent entry into trophic chains. Moreover, the potential contamination of soil, subsoil, and groundwater through leachates from pesticide-laden compost must be carefully considered. Unlike heavy metals, which are strictly regulated in compost under national ordinances and EU directives, there are currently no binding environmental policies worldwide that set tolerable levels for organic contaminants such as pesticides. Worker safety considerations further necessitate monitoring pesticide residues in composted agricultural wastes, as their use as soil amendments may expose workers to residual contaminants (Frenich et al. 2005).

Bengaluru, one of India’s rapidly expanding metropolitan cities, generates around 5–6 thousand tonnes per day (TPD) of MSW, including bulk waste, with a per capita generation of about 400 g/day.  The city follows a source-segregated collection system, where the wet wastes are directed to Wet-Waste Processing Plants (WPPs) for composting (BBMP 2022). While the quality of MSW compost has been evaluated in terms of nutrient content and heavy metals, limited information is available on the occurrence of pesticide residues in compost derived from municipal waste in the region. Therefore, the present study aimed to investigate the occurrence and distribution of pesticide residues in municipal solid waste compost collected from composting facilities in Bengaluru, India, using a multi-residue analytical approach based on GC–MS/MS with emphasis on the occurrence and class distribution of, detection frequency across composting units, and concentration patterns of dominant pesticide residues.

2. methodology

Sample Collection and Preparation

Compost samples were collected from six operational municipal solid waste (MSW) composting facilities located within the Bengaluru Urban District (Fig. 1). To ensure representativeness, composite samples were prepared by combining multiple grab samples obtained from different sections of the finished compost piles, following standardized sampling protocols (IS 11380: 1985). The collected compost samples were air-dried, homogenized, sieved, and stored under appropriate conditions before pesticide analysis.

Fig. 1: MSW Compost sampling locations in Bengaluru.

Pesticide Extraction and Clean-up

Pesticide residues were extracted using the QuEChERS (Quick, Easy, Cheap, Effective, Rugged and Safe) extraction method, according to the official AOAC method (2007.01), which is widely used for multi-residue pesticide analysis in complex matrices. To prepare the samples, 5 g of a previously homogenized compost sample was weighed into a clean 50 ml centrifuge tube with 10 ml deionized water, 10 ml of 1% acetic acid in acetonitrile, and vortexed for 5 minutes.  A mixture of extraction salts, including magnesium sulfate (MgSO4) and sodium acetate (NaOAc), was added to promote phase separation and improve extraction efficiency. The sample was vigorously shaken and subsequently centrifuged to separate the organic phase.

The supernatant was subjected to dispersive solid-phase extraction (d-SPE) cleanup using sorbents such as primary secondary amine (PSA), C18, GCB, and MgSO4 to remove interfering organic compounds, pigments, and residual moisture. 2 ml of the extract was evaporated and again reconstituted using ethyl acetate, filtered into a GC vial before instrumental analysis.

INSTRUMENTAL ANALYSIS

Quantitative screening of pesticide residues was performed using GC–MS/MS (Shimadzu-TQ8050) operating in multiple reaction monitoring (MRM) mode. The analytical method targeted 185 pesticide compounds, including 75 organo-nitrogen pesticides (ONP), 44 organophosphate pesticides (OPP), 40 organochlorine pesticides (OCP), and 26 synthetic pyrethroids (SPP). Chromatographic separation was performed on an SH-I-5Sil MS capillary column (30 m × 0.25 mm ID, 0.25 µm film thickness) with helium as the carrier gas. Aliquot (1 µl) of the sample was injected in splitless mode with the injector temperature set at 280 °C. The GC oven temperature program and MS/MS detection parameters were optimized for multi-residue pesticide analysis. Identification of pesticide residues was based on retention time and characteristic mass transitions, while quantification was performed using calibration curves prepared from analytical pesticide standards.

DATA ANALYSIS

Detection frequency of pesticide residues was calculated to evaluate the distribution of compounds across the composting facilities using the following equation:

Detection Frequency=Number of samples in which the pesticide was detected​Total number of samples ×100

Pesticides were categorized based on their detection frequency across the six compost samples. Concentration values were expressed as milligrams per kilogram (mg kg⁻¹) on a dry weight basis. Descriptive statistical analysis was used to summarize the occurrence patterns and concentration ranges of pesticide residues.

3. RESULTS  AND Discussion

Out of 185 residues scanned, 172 compounds (93%) were detected in at least one sample, indicating widespread occurrence of pesticide residues in the municipal organic waste stream, while the remaining 13 compounds (7%) were not detected. ONP constituted the largest proportion of detected compounds (38%), followed by organophosphates (25%), organochlorines (23%), and synthetic pyrethroids (14%) (see Supplementary Table S1).

The relative distribution of pesticide classes in MSW compost samples indicate that pesticide contamination in compost originates from diverse chemical classes commonly used in agricultural and domestic pest control applications (Fig.2).

Fig. 2 Class-wise distribution of pesticide residues detected in MSW compost samples

Detection frequency analysis revealed substantial variability in pesticide occurrence across composting units. Twenty-one pesticides (11.4%) were detected in all six compost samples (100% detection frequency), suggesting ubiquitous presence across the facilities. An additional 67 compounds were detected in four to five units (67–83% frequency), while 38 pesticides occurred in three units (50% frequency). Moderate occurrence was observed for 23 compounds each at 33% and 17% detection frequencies, whereas 13 pesticides (7%) were not detected in any sample. The distribution of detection frequencies is summarized in Table 2 and illustrated in Figure 3.

Fig. 3 Detection frequency distribution of pesticide residues across composting units

Among the quantified residues, 21 pesticides showed concentrations exceeding 1 mg kg⁻¹, indicating relatively high residue levels in the compost samples. Notable compounds included cypermethrin (17.4 mg kg⁻¹), followed by cyfluthrin isomers, including cyfluthrin-1 (13.3 mg kg⁻¹) and cyfluthrin-2 (9.8 mg kg⁻¹). The other prominent residues included profenofos (7.5 mg kg⁻¹), cyfluthrin-3 and cyfluthrin-4 (5.7 and 5.3 mg kg⁻¹ each), and tetrahydro phthalimide (THPI) (3.2 mg kg⁻¹). Besides, several additional pesticides were detected at concentrations around 2 mg kg⁻¹, while others ranged between 1 and 2 mg kg⁻¹ (Table 1).

Table 1: Concentration of dominant pesticide residues (>1 mg kg⁻¹) in MSW compost

| Pesticide | WWP1 | WWP2 | WWP3 | WWP4 | WWP5 | WWP6 |
| --- | --- | --- | --- | --- | --- | --- |
| Cypermethrin-1 | 0.03 | 0.09 | 0.02 | 17.38 | 2.73 | 3.22 |
| Cyfluthrin-1 | ND | ND | ND | 13.28 | 4.02 | 8.33 |
| Cyfluthrin-2 | ND | ND | ND | 9.77 | 2.97 | 6.13 |
| Profenofos | ND | ND | ND | 7.51 | 0.03 | 2.03 |
| Cyfluthrin-3 | ND | ND | ND | 2.91 | 1.71 | 5.65 |
| Cyfluthrin-4 | ND | ND | ND | 2.74 | 1.62 | 5.29 |
| THPI (Tetrahydropthalimide) | ND | ND | 1.05 | 3.22 | 0.13 | 1.85 |
| Acrinathrin-1 | ND | ND | ND | 0.33 | 2.61 | 1.84 |
| lambda-Cyhalothrin | ND | ND | ND | 0.32 | 2.51 | 1.76 |
| Chlorpyrifos | ND | ND | 1.63 | 1.78 | 2.34 | 2.43 |
| Cyhalothrin-1 | 0.01 | 0.02 | 0.02 | 0.29 | 2.21 | 1.71 |
| Cyhalothrin-2 | 0.01 | 0.02 | 0.03 | 0.31 | 2.20 | 1.69 |
| Terbuthylazine | 2.12 | 1.21 | 1.87 | 0.01 | ND | 0.00 |
| Disulfoton | ND | ND | ND | 0.71 | 0.09 | 2.08 |
| Endosulfan-beta | ND | ND | ND | 2.04 | ND | 0.22 |
| 2,4'-Methoxychlor | 1.44 | 1.73 | 1.61 | ND | ND | ND |
| Endosulfan alpha | ND | ND | 1.71 | ND | 0.02 | ND |
| Quinalphos | ND | 0.00 | 0.09 | 0.32 | 0.02 | 1.62 |
| Malathion | ND | 0.17 | 1.38 | 0.02 | 0.01 | 0.01 |
| Flutolanil | 0.14 | 0.12 | 0.27 | 1.09 | ND | 0.25 |
| Terbacil | 0.97 | 1.01 | 1.05 | ND | 0.002 | 0.01 |

WWP – Wet Waste Processing Plant; Values in mg kg⁻1

The detection of multiple pesticide residues in compost reflects the heterogeneous nature of municipal waste inputs, which may include food waste, vegetable residues, household waste, and garden litter that could contain pesticide residues originating from agricultural and domestic pest control practices. Several studies have reported the occurrence of multiple pesticide residues in environmental matrices such as soil, compost, and agricultural products, highlighting their widespread distribution (Riedo et al. 2021; Silva et al. 2019).

Class-wise distribution pattern reflects the widespread use of nitrogen-containing pesticides and modern synthetic pesticides in agricultural and urban pest control applications. Previous investigations have shown that pesticide residues may persist in organic waste materials and can be transferred to compost products during waste processing (Kupper et al. 2008). Although composting involves microbial degradation and elevated temperatures that may promote the breakdown of certain pesticides, several compounds can survive the composting process depending on their physic-chemical properties and stability.

Detection frequency analysis indicated the occurrence of 21 pesticides exceeding 1 mg kg⁻¹, with pyrethroid insecticides such as cypermethrin and cyfluthrin showing the highest concentrations in all compost samples. Repeated applications and strong sorption to organic substrates (Riedo et al. 2021) also helps. Pyrethroids are widely used insecticides known for their strong adsorption to organic matter and low water solubility, which can result in their accumulation in organic matrices such as soil and compost (Liu et al. 2024). The strong affinity of pyrethroids for organic carbon may reduce their mobility but enhances their persistence in compost substrates, as observed (Briceño et al., 2007).

The presence of multiple pesticide residues in finished MSW compost raises concerns regarding the potential transfer of contaminants to agricultural soils when applied as an organic amendment. Although the practice of compost application for improving soil fertility and recycling of organic waste, as it is likely to influence soil microbial communities and ecological processes (Lee-Ann Ataikiru and Ajuzieogu 2023), the chances of their impact on soil organisms and long-term environmental contamination (Silva et al. 2019) cannot be ruled out.

4. CONCLUSION

The present study provides important insights into the occurrence and distribution of pesticide residues in municipal solid waste compost; however, certain limitations should be acknowledged. The analysis was based on six compost samples collected from different composting facilities, and each facility was sampled once. Although the sampling strategy ensured representation of major composting units within the study area, temporal variations in pesticide residues during different composting cycles were not evaluated. Additionally, the heterogeneous nature of municipal solid waste feedstock may contribute to variability in pesticide occurrence depending on seasonal waste composition and source materials. Furthermore, while the study focused on the detection and concentration of pesticide residues, detailed degradation pathways and transformation products formed during the composting process were not investigated. Future studies involving larger sample sizes, repeated sampling across seasons, and assessment of pesticide degradation dynamics would provide a more comprehensive understanding of pesticide behavior in MSW composting systems. Despite these limitations, the present investigation offers a comprehensive multi-residue screening of 185 pesticides, providing valuable baseline information on pesticide contamination in municipal compost used for agricultural applications.

Competing interests

The authors declare no conflicts of interests exist.

References

Anonymous, BBMP. 2022. “Solid Waste Management.” 2022. https://site.bbmp.gov.in.

Briceño, Gabriela, Graciela Palma, and Nelson Durán. 2007. Influence of Organic Amendment on the Biodegradation and Movement of Pesticides. Critical Reviews in Environmental Science and Technology. Vol. 37. https://doi.org/10.1080/10643380600987406.

Brown, K. W., J. C. Thomas, and F. Whitney. 1997. “Fate of Volatile Organic Compounds and Pesticides in Composted Municipal Solid Waste.” Compost Science and Utilization 5 (4): 6–14. https://doi.org/10.1080/1065657X.1997.10701892.

Büyüksönmez, Fatih, Robert Rynk, Thomas F. Hess, and Edward Bechinski. 1999. “Occurrence, Degradation and Fate of Pesticides during Composting: Part i: Composting, Pesticides, and Pesticide Degradation.” Compost Science and Utilization 7 (4): 66–82. https://doi.org/10.1080/1065657X.1999.10701986.

Frenich, A. Garrido, Manuel J. González Rodríguez, José L. Martínez Vidal, Francisco J. Arrebola, and María E. Hernández Torres. 2005. “A Study of the Disappearance of Pesticides during Composting Using a Gas Chromatography-Tandem Mass Spectrometry Technique.” Pest Management Science 61 (5): 458–66. https://doi.org/10.1002/ps.984.

Hargreaves, J. C., M. S. Adl, and P. R. Warman. 2008. “A Review of the Use of Composted Municipal Solid Waste in Agriculture.” Agriculture, Ecosystems and Environment 123 (1–3): 1–14. https://doi.org/10.1016/j.agee.2007.07.004.

Kupper, Thomas, Thomas D. Bucheli, Rahel C. Brändli, Didier Ortelli, and Patrick Edder. 2008. “Dissipation of Pesticides during Composting and Anaerobic Digestion of Source-Separated Organic Waste at Full-Scale Plants.” Bioresource Technology 99 (17): 7988–94. https://doi.org/10.1016/j.biortech.2008.03.052.

Lee-Ann Ataikiru, Tega, and Chinyere Augusta Ajuzieogu. 2023. “Enhanced Bioremediation of Pesticides Contaminated Soil Using Organic (Compost) and Inorganic (NPK) Fertilizers.” Heliyon 9 (12): e23133. https://doi.org/10.1016/j.heliyon.2023.e23133.

Liu, Shan, Jisha Yan, Bowen Xu, Xinyi Huang, Haixiong Qin, Jiayuan Zhao, Chen Xia, Shen Yan, and Gang Liu. 2024. “Fates and Models for Exposure Pathways of Pyrethroid Pesticide Residues: A Review.” Ecotoxicology and Environmental Safety 277 (April): 116342. https://doi.org/10.1016/j.ecoenv.2024.116342.

Lourencetti, Carolina, Rodrigo Favoreto, Mary R.R. Marchi, and Maria L. Ribeiro. 2007. “Residues of Organochlorine Pesticides and PCBs in Some Brazilian Municipal Solid Waste Compost.” Journal of Environmental Science and Health - Part B Pesticides, Food Contaminants, and Agricultural Wastes 42 (6): 697–705. https://doi.org/10.1080/03601230701465882.

Manea, Elena Elisabeta, Costel Bumbac, Laurentiu Razvan Dinu, Marius Bumbac, and Cristina Mihaela Nicolescu. 2024. “Composting as a Sustainable Solution for Organic Solid Waste Management : Current Practices and Potential Improvements,” 1–25.

Prasopsuk, Jarupong, and Chuleemas Boonthai Iwai. 2011. “Risk Assessment of Pesticide Residues in Organic Waste in Northeast Thailand.” IJERD-International Journal of Environmental and Rural Development, 2–3.

Riedo, Judith, Felix E. Wettstein, Andrea Rosch, Chantal Herzog, Samiran Banerjee, Lucie Buchi, Raphael Charles, et al. 2021. “Widespread Occurrence of Pesticides in Organically Managed Agricultural Soils-The Ghost of a Conventional Agricultural Past?” Environmental Science and Technology 55 (5): 2919–28. https://doi.org/10.1021/acs.est.0c06405.

Sayara, Tahseen, Rezq Basheer-Salimia, Fatina Hawamde, and Antoni Sánchez. 2020. “Recycling of Organic Wastes through Composting: Process Performance and Compost Application in Agriculture.” Agronomy 10 (11). https://doi.org/10.3390/agronomy10111838.

Silva, Vera, Hans G.J. Mol, Paul Zomer, Marc Tienstra, Coen J. Ritsema, and Violette Geissen. 2019. “Pesticide Residues in European Agricultural Soils – A Hidden Reality Unfolded.” Science of the Total Environment 653: 1532–45. https://doi.org/10.1016/j.scitotenv.2018.10.441.

Vorkamp, Katrin, Jörg Taube, Jörg Dilling, Esther Kellner, and Reimer Herrmann. 2003. “Fate of the Fungicide Dodemorph during Anaerobic Digestion of Biological Waste.” Chemosphere 53 (5): 505–14. https://doi.org/10.1016/S0045-6535(03)00544-7.

Wittmer, I. K., H. P. Bader, R. Scheidegger, H. Singer, A. Lück, I. Hanke, C. Carlsson, and C. Stamm. 2010. “Significance of Urban and Agricultural Land Use for Biocide and Pesticide Dynamics in Surface Waters.” Water Research 44 (9): 2850–62. https://doi.org/10.1016/j.watres.2010.01.030.

APPENDIX

Table S1: Pesticide residues in MSWC samples from six composting facilities in Bengaluru

| Pesticides | KCDC | CHICK | DODB | SEEGE | SUBR | LI0GD | Type |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acetochlor | 1 | 1 | 1 | 1 | 0 | 1 | ONP |
| Acrinathrin-1 | 0 | 0 | 0 | 1 | 1 | 1 | SPP |
| Alachlor | 1 | 1 | 1 | 0 | 0 | 1 | ONP |
| Aldrin | 1 | 1 | 1 | 0 | 1 | 1 | OCP |
| Allidochlor | 0 | 0 | 0 | 0 | 0 | 0 | ONP |
| Atrazine | 0 | 0 | 0 | 1 | 1 | 1 | ONP |
| Azinphos-ethyl | 1 | 1 | 1 | 1 | 1 | 1 | OPP |
| Azinphos-methyl | 0 | 1 | 0 | 0 | 1 | 1 | OPP |
| Benfluralin | 0 | 0 | 1 | 1 | 0 | 0 | ONP |
| BHC-alpha | 0 | 0 | 1 | 0 | 0 | 0 | OCP |
| BHC-beta | 1 | 1 | 0 | 1 | 1 | 1 | OCP |
| BHC-delta | 1 | 1 | 1 | 1 | 0 | 1 | OCP |
| BHC-gamma-(Lindane) | 0 | 0 | 0 | 1 | 1 | 1 | OCP |
| Bifenthrin | 1 | 1 | 1 | 1 | 0 | 0 | SPP |
| Bromfenvinphos | 0 | 0 | 1 | 0 | 1 | 1 | OPP |
| Bromophos | 0 | 0 | 1 | 1 | 1 | 0 | OPP |
| Bupirimate | 1 | 0 | 1 | 0 | 0 | 0 | ONP |
| Carbophenothion | 1 | 1 | 1 | 0 | 1 | 0 | OPP |
| Chlorbenside | 1 | 1 | 1 | 1 | 1 | 1 | OCP |
| Chlordane-cis | 1 | 0 | 1 | 0 | 0 | 0 | OCP |
| Chlordane-trans | 1 | 1 | 1 | 1 | 0 | 1 | OCP |
| Chlorfenapyr | 1 | 1 | 1 | 0 | 0 | 0 | ONP |
| Chlorfenson | 0 | 0 | 0 | 1 | 1 | 0 | OCP |
| Chlorfenvinphos | 1 | 1 | 1 | 0 | 0 | 0 | OPP |
| Chlorobenzilate | 0 | 1 | 1 | 0 | 0 | 0 | OCP |
| Chlorothalonil | 1 | 0 | 1 | 1 | 0 | 1 | ONP |
| Chlorpyrifos | 0 | 0 | 1 | 1 | 1 | 1 | OPP |
| Chlorpyrifos-methyl | 1 | 1 | 1 | 1 | 1 | 1 | OPP |
| Chlorthiophos-1 | 0 | 1 | 1 | 1 | 0 | 0 | OPP |
| Chlorthiophos-2 | 0 | 1 | 1 | 1 | 0 | 1 | OPP |
| Chlorthiophos-3 | 1 | 0 | 1 | 0 | 0 | 0 | OPP |
| Clomazone | 0 | 1 | 0 | 0 | 0 | 0 | ONP |
| Cycloate | 0 | 0 | 0 | 0 | 0 | 0 | ONP |
| Cyfluthrin-1 | 0 | 0 | 0 | 1 | 1 | 1 | SPP |
| Cyfluthrin-2 | 0 | 0 | 0 | 1 | 1 | 1 | SPP |
| Cyfluthrin-3 | 0 | 0 | 0 | 1 | 1 | 1 | SPP |
| Cyfluthrin-4 | 0 | 0 | 0 | 1 | 1 | 1 | SPP |
| Cyhalothrin-1 | 1 | 1 | 1 | 1 | 1 | 1 | SPP |
| Cyhalothrin-2 | 1 | 1 | 1 | 1 | 1 | 1 | SPP |
| Cypermethrin-1 | 1 | 1 | 1 | 1 | 1 | 1 | SPP |
| Cyprodinil | 1 | 1 | 1 | 0 | 1 | 0 | ONP |
| DDD-o,p' | 0 | 0 | 1 | 0 | 0 | 0 | OCP |
| DDD-p,p' | 0 | 0 | 1 | 0 | 0 | 0 | OCP |
| DDE-o,p' | 0 | 0 | 1 | 0 | 0 | 0 | OCP |
| DDE-p,p' | 1 | 1 | 1 | 1 | 1 | 1 | OCP |
| DDT-o,p' | 0 | 0 | 1 | 0 | 0 | 0 | OCP |
| DDT-p,p' | 1 | 1 | 1 | 1 | 1 | 1 | OCP |
| Deltamethrin-1 | 1 | 1 | 1 | 1 | 1 | 1 | SPP |
| Deltamethrin-2 | 0 | 0 | 0 | 0 | 1 | 0 | SPP |
| Di-allate-2 | 1 | 1 | 1 | 1 | 1 | 1 | ONP |
| Diazinon | 0 | 1 | 1 | 0 | 0 | 0 | OPP |
| Dichlobenil | 1 | 0 | 1 | 0 | 0 | 0 | ONP |
| Dichlofluanid | 1 | 1 | 1 | 1 | 1 | 1 | ONP |
| Dichloroaniline-3,4 | 1 | 1 | 1 | 1 | 0 | 1 | ONP |
| Dichlorobenzophenone-4,4' | 0 | 1 | 1 | 1 | 1 | 0 | OCP |
| Dieldrin | 1 | 0 | 1 | 0 | 0 | 1 | OCP |
| Dimethachlor | 0 | 0 | 1 | 0 | 0 | 0 | ONP |
| Diphenamid | 1 | 1 | 1 | 0 | 1 | 1 | ONP |
| Disulfoton | 0 | 0 | 0 | 1 | 1 | 1 | OPP |
| Edifenphos | 0 | 1 | 1 | 1 | 1 | 1 | OPP |
| Endosulfan ether | 0 | 1 | 1 | 1 | 1 | 1 | OCP |
| Endosulfan sulfate | 1 | 1 | 1 | 0 | 1 | 0 | OCP |
| Endosulfanalpha | 0 | 0 | 1 | 0 | 1 | 0 | OCP |
| Endosulfan-beta | 0 | 0 | 0 | 1 | 0 | 1 | OCP |
| Endrin | 0 | 0 | 1 | 1 | 0 | 0 | OCP |
| Endrin aldehyde | 0 | 1 | 1 | 1 | 0 | 1 | OCP |
| Endrin ketone | 0 | 1 | 1 | 1 | 0 | 1 | OCP |
| EPN | 0 | 0 | 0 | 1 | 1 | 0 | OPP |
| Ethalfluralin | 1 | 0 | 0 | 0 | 0 | 0 | ONP |
| Ethion | 0 | 0 | 1 | 1 | 1 | 0 | OPP |
| Etofenprox | 0 | 0 | 0 | 0 | 0 | 0 | SPP |
| Etridiazole | 0 | 0 | 0 | 0 | 0 | 0 | ONP |
| Fenamiphos | 1 | 0 | 0 | 0 | 0 | 0 | OPP |
| Fenarimol | 0 | 0 | 0 | 0 | 0 | 0 | ONP |
| Fenchlorphos | 0 | 0 | 1 | 0 | 0 | 0 | OPP |
| Fenitrothion | 0 | 0 | 1 | 0 | 0 | 0 | OPP |
| Fenpropathrin | 1 | 1 | 1 | 1 | 0 | 0 | SPP |
| Fenson | 0 | 0 | 1 | 0 | 0 | 0 | OCP |
| Fenvalerate-1 | 1 | 1 | 1 | 1 | 0 | 1 | SPP |
| Fenvalerate-2 | 0 | 0 | 0 | 0 | 0 | 0 | SPP |
| Fethion | 1 | 0 | 1 | 0 | 0 | 1 | OPP |
| Fipronil | 0 | 0 | 0 | 1 | 1 | 1 | ONP |
| Fluchloralin | 0 | 0 | 1 | 1 | 0 | 0 | ONP |
| Flucythrinate-1 | 1 | 0 | 1 | 1 | 0 | 1 | SPP |
| Flucythrinate-2 | 1 | 0 | 1 | 1 | 1 | 1 | SPP |
| Fludioxonil | 1 | 1 | 1 | 1 | 1 | 1 | ONP |
| Fluquinconazole | 1 | 1 | 1 | 1 | 1 | 1 | ONP |
| Fluridone | 1 | 1 | 1 | 0 | 0 | 0 | ONP |
| Flusilazole | 0 | 0 | 0 | 0 | 0 | 1 | ONP |
| Flutolanil | 1 | 1 | 1 | 1 | 0 | 1 | ONP |
| Flutriafol | 0 | 0 | 1 | 1 | 1 | 1 | ONP |
| Fluvalinate-1 | 1 | 1 | 1 | 0 | 1 | 1 | SPP |
| Fonofos | 0 | 0 | 0 | 0 | 0 | 0 | OPP |
| Heptachlor | 0 | 0 | 0 | 0 | 0 | 0 | OCP |
| Heptachlor-exo-epoxide | 0 | 0 | 1 | 0 | 0 | 0 | OCP |
| Hexachlorobenzene | 1 | 1 | 1 | 1 | 1 | 1 | OCP |
| Hexazinone | 1 | 1 | 1 | 1 | 0 | 1 | ONP |
| Iprodione | 1 | 0 | 1 | 0 | 0 | 0 | ONP |
| Isazofos | 1 | 1 | 1 | 0 | 1 | 1 | OPP |
| Isodrin | 1 | 0 | 1 | 1 | 1 | 0 | OCP |
| Isopropalin | 1 | 1 | 1 | 1 | 1 | 1 | ONP |
| lambda-Cyhalothrin | 0 | 0 | 0 | 1 | 1 | 1 | SPP |
| Lenacil | 1 | 1 | 0 | 0 | 0 | 0 | ONP |
| Leptophos | 0 | 1 | 1 | 1 | 1 | 1 | OPP |
| Malathion (INS) | 0 | 1 | 1 | 1 | 1 | 1 | OPP |
| Metazachlor | 1 | 1 | 1 | 0 | 0 | 1 | ONP |
| Methacrifos | 0 | 0 | 1 | 0 | 0 | 0 | OPP |
| Methoxychlor | 1 | 1 | 0 | 1 | 1 | 1 | OCP |
| methoxychlor olefin-4,4' | 1 | 1 | 1 | 0 | 1 | 1 | OCP |
| Methoxychlor-2,4' | 1 | 1 | 1 | 0 | 0 | 0 | OCP |
| Metolachlor | 0 | 0 | 1 | 0 | 0 | 0 | ONP |
| Mevinphos-1 | 0 | 0 | 1 | 1 | 1 | 1 | OPP |
| Mirex | 1 | 1 | 1 | 0 | 1 | 0 | OCP |
| Myclobutanil | 0 | 1 | 1 | 0 | 1 | 1 | ONP |
| N-(2,4-dimethylphenyl) formamide | 0 | 0 | 0 | 1 | 1 | 1 | ONP |
| Nitralin | 1 | 1 | 1 | 1 | 1 | 1 | ONP |
| Nitrofen | 0 | 0 | 1 | 1 | 1 | 0 | ONP |
| Nonachlor-cis | 1 | 1 | 1 | 0 | 0 | 1 | OCP |
| Nonachlor-trans | 0 | 1 | 0 | 0 | 0 | 1 | OCP |
| Norflurazon | 0 | 1 | 1 | 0 | 0 | 0 | ONP |
| Oxadiazon | 1 | 0 | 1 | 1 | 0 | 1 | ONP |
| Oxyfluorfen | 1 | 1 | 1 | 1 | 0 | 0 | ONP |
| Paclobutrazol | 1 | 0 | 1 | 0 | 1 | 1 | ONP |
| Parathion | 1 | 1 | 1 | 1 | 0 | 0 | OPP |
| Parathion-methyl | 1 | 1 | 1 | 1 | 1 | 1 | OPP |
| Pebulate | 0 | 1 | 1 | 1 | 0 | 0 | ONP |
| Penconazole | 0 | 0 | 1 | 0 | 1 | 1 | ONP |
| Pendimethalin | 1 | 0 | 1 | 1 | 1 | 1 | ONP |
| Pentachloroaniline | 1 | 1 | 1 | 0 | 0 | 0 | ONP |
| Pentachloroanisole | 1 | 1 | 1 | 1 | 0 | 1 | OCP |
| Pentachlorobenzene | 1 | 1 | 1 | 0 | 0 | 0 | OCP |
| Pentachlorobenzonitrile | 0 | 1 | 0 | 0 | 0 | 0 | ONP |
| Pentachlorothioanisole | 1 | 1 | 1 | 1 | 0 | 1 | OCP |
| Permethrine-cis | 0 | 0 | 0 | 1 | 1 | 1 | SPP |
| Permethrine-trans | 0 | 0 | 0 | 1 | 1 | 1 | SPP |
| Phorate | 1 | 1 | 0 | 0 | 0 | 0 | OPP |
| Phosalo0e | 0 | 0 | 0 | 1 | 0 | 1 | OPP |
| Phosmet | 1 | 0 | 1 | 1 | 0 | 1 | OPP |
| Piperonyl butoxide | 1 | 1 | 1 | 1 | 0 | 1 | OPP |
| Pirimiphos ethyl | 1 | 0 | 0 | 0 | 0 | 0 | OPP |
| Pirimiphos-methyl | 1 | 1 | 1 | 0 | 0 | 0 | OPP |
| Pretilachlor | 1 | 1 | 1 | 1 | 1 | 1 | ONP |
| Prochloraz | 1 | 0 | 1 | 1 | 1 | 1 | ONP |
| Procymidone | 1 | 0 | 1 | 1 | 0 | 1 | ONP |
| Prodiamine | 0 | 0 | 1 | 0 | 0 | 0 | ONP |
| Profenofos | 0 | 0 | 0 | 1 | 1 | 1 | OPP |
| Profluralin | 0 | 0 | 0 | 0 | 0 | 0 | ONP |
| Propachlor | 1 | 1 | 0 | 0 | 0 | 0 | ONP |
| Propanil | 0 | 0 | 0 | 0 | 0 | 0 | ONP |
| Propargite-1 | 1 | 1 | 1 | 1 | 0 | 1 | ONP |
| Propisochlor | 1 | 1 | 1 | 0 | 1 | 1 | ONP |
| Propyzamide | 1 | 1 | 1 | 1 | 1 | 1 | ONP |
| Pyraclofos | 0 | 1 | 1 | 0 | 1 | 0 | OPP |
| Pyrazophos | 1 | 1 | 1 | 1 | 1 | 1 | OPP |
| Pyridaben | 0 | 0 | 1 | 1 | 1 | 1 | ONP |
| Pyridaphenthion | 0 | 0 | 1 | 0 | 0 | 0 | OPP |
| Pyrimethanil | 1 | 1 | 1 | 1 | 0 | 0 | ONP |
| Pyriproxyfen | 1 | 0 | 1 | 1 | 0 | 1 | ONP |
| Quinalphos | 0 | 1 | 1 | 1 | 1 | 1 | OPP |
| Quintozene | 1 | 0 | 1 | 1 | 0 | 1 | ONP |
| Resmethrin-1 | 0 | 0 | 1 | 0 | 0 | 0 | SPP |
| Sulfotep | 1 | 1 | 1 | 0 | 1 | 0 | OPP |
| Sulprofos | 0 | 0 | 0 | 1 | 1 | 1 | OPP |
| Tebuconazole | 0 | 0 | 0 | 1 | 1 | 1 | ONP |
| Tebufenpyrad | 1 | 1 | 1 | 1 | 0 | 0 | ONP |
| Tecnazene | 1 | 1 | 1 | 0 | 1 | 1 | ONP |
| Tefluthrin | 0 | 1 | 1 | 0 | 0 | 0 | SPP |
| Terbacil | 1 | 1 | 1 | 0 | 1 | 1 | ONP |
| Terbuthylazine | 1 | 1 | 1 | 1 | 0 | 1 | ONP |
| Tetrachloroaniline- 2,3,5,6 | 1 | 1 | 1 | 0 | 0 | 0 | ONP |
| Tetrachlorvinphos | 0 | 1 | 1 | 0 | 1 | 1 | OPP |
| Tetradifon | 0 | 1 | 1 | 0 | 0 | 1 | OCP |
| Tetramethrin-1 | 0 | 0 | 1 | 0 | 0 | 0 | SPP |
| Tetramethrin-2 | 0 | 0 | 1 | 1 | 0 | 1 | SPP |
| THPI * | 0 | 0 | 1 | 1 | 1 | 1 | ONP |
| Tolclofos-methyl | 0 | 1 | 1 | 0 | 0 | 0 | OPP |
| Transfluthrin | 1 | 1 | 1 | 1 | 1 | 1 | SPP |
| Triadimefon | 0 | 0 | 0 | 0 | 0 | 0 | ONP |
| Triadimenol-1 | 1 | 1 | 1 | 1 | 0 | 1 | ONP |
| Tri-allate | 0 | 0 | 0 | 0 | 0 | 0 | ONP |
| Triazophos | 0 | 1 | 1 | 0 | 1 | 1 | OPP |
| Tricyclazole | 0 | 1 | 1 | 1 | 1 | 1 | ONP |
| Triflumizole | 0 | 0 | 1 | 0 | 0 | 0 | ONP |
| Trifluralin | 0 | 0 | 0 | 0 | 0 | 1 | ONP |
| Vinclozolin | 0 | 0 | 1 | 1 | 0 | 1 | ONP |

- 1 = detected; 0 = not detected; *THPI-Tetrahydropthalimide