Efficiency of different chemical extractants in evaluating heavy metal bioavailability and ecological risk in waste-impacted soils: a comparative study

Abstract

This study evaluated extraction efficiencies and investigated how choice of extractant influenced contamination assessments. Soil samples were taken from areas impacted by auto mechanic, paint processing, and abattoir wastes and analyzed for Pb, Cd, Cr, Ni, and V using three extractants: aqua regia (AR), nitric–perchloric acid (NP), and ethylenediamine tetraacetic acid (EDTA). Overall mean concentrations were computed from all samples collected at all waste disposal sites and control sites across depth intervals of 0–20, 20–60, and 60–80 cm. Mean Pb concentrations extracted with aqua regia (AR), nitric–perchloric acids (NP), and EDTA were 1534.0, 1419.0, and 1192.0 mg/kg, respectively. Similar trends were observed for Cd, Cr, Ni, and V, where aqua regia consistently yielded higher concentrations than nitric–perchloric acids and EDTA, reflecting the stronger digestion capability of aqua regia in releasing metals bound to resistant mineral phases. Results were compared with regulatory limits set by the Food and Agriculture Organization (FAO). Pb and Cd exceeded FAO limits in all methods (indicating soil degradation), Ni slightly exceeded in aqua regia, but was below limits in NP and EDTA, while V and Cr remained below permissible limits in all extractions. Heavy metal concentration decreased with soil depth, indicating surface enrichment that heightens exposure risk for plants, humans, and soil organisms. The results indicate that reliance on strong-acid digestion extractants may overestimate health and ecological risks, while dependence on milder extractants may underestimate contamination levels and associated hazards. Using multiple extractants provides a more accurate assessment of soil contamination.

Keywords: Chemical extractants; Digestion methods; Extractant efficiency; Heavy metals; Open dumpsites; Soil contamination

INTRODUCTION

The contamination of soils by heavy metals resulting from urbanization, industrial activities, and indiscriminate waste disposal has become a major environmental concern (Zang et al., 2010; Kaparwan et al., 2020; Wang et al., 2021; Udo et al., 2025a). Contaminated soils typically receive wastes that introduce potentially toxic elements into the environment (Gibson and Farmer, 1986; Umoh and Etim, 2013; Ojiego et al., 2022; Udo et al., 2026a). Soils contaminated by heavy metals have posed a serious ecological and human health risk as a result of the persistent, toxic, and bio-accumulative properties of the metals (Wuana and Okeimen, 2011; Sharma and Jain, 2020; Udo et al., 2026b). In the FAO’s report of 2004, mercury, cadmium, lead, zinc, nickel, chromium, copper, and cobalt were listed among the top twenty dangerous substances. These elements are some of those released into the soils through various waste materials.

Heavy metals are toxic even at low levels (Wuana and Okeimen, 2011). Since life on earth owes its sustenance to certain elements derived from the earth's crust (Winegardner, 2019), soils contaminated by heavy metals cause unquantifiable damage to the entire ecosystem (Tóth et al., 2016; Tyagi and Kumar, 2021; Munir et al., 2021). For instance, their toxicities can affect environmentally friendly soil microbial populations that enhance soil fertility and health (Jyothi, 2020). In addition, the toxic nature of these metals can damage human health (Mahurpawar, 2015), affect water quality (Zhang et al., 2022), degrade natural food quality (Munir et al., 2021), and destabilize the overall functioning of the ecosystem (Igwe and Nwachukwu, 2016; Adagunodo et al., 2018; Ebong et al., 2020). Consequently, it is always recommended that contaminated soils be remediated for environmental sustainability (Maturi et al., 2008; Kaparwan et al., 2020). Such soils should first be analyzed to ascertain the concentrations and forms of heavy metals before designing remedial strategies.

To effectively determine heavy metal levels in contaminated soils, appropriate chemical extractants should be employed (Lee and Kao, 2004; Santoro et al., 2017; Li, 2024), as extractants differ in their extraction efficiencies and metal affinity. The extractant type, therefore, significantly influences the reliability and accuracy of results (Hlavay et al., 2004).  Improper choice of extractant can lead to partial dissolution of heavy metals in the soil sample, reducing metal concentrations.

Five different classes of extractants: acids, nonionic, chelating agents, surfactants, and cosolvents, were identified by Maturi et al. (2008). Strong single acids, e.g., ultra-pure HNO3 or double acids, e.g., aqua regia (HCl and HNO3), have been used to determine pseudo-total concentrations of metallic elements (Ivezić et al., 2013). Aqua regia is widely used for the determination of total forms of Pb, Cr, Cd, Ni, and Zn due to its simple and adaptable nature. Aqua regia can oxidize organic matter and metal oxide, just like hydrogen peroxide (Akannam and Kakulu, 2022). On the other hand, synthetic acids (chelating agents), e.g., ethylenediaminetetraacetic acid (EDTA), have been used to determine reactive metal concentrations. Water can also be used to extract metal concentrations from soil solution (Ivezić et al., 2013).

Owing to different properties of each element in the soil, no particular digestion method may be efficient for all metals (Idera et al., 2014). Factors influencing the choice of extractant for analytical processes include the nature of the targeted heavy metal, the chemical form of the metal within the matrix, and the analytical method employed (Manouchehri and Bermond, 2009).  All extractant types do not have the same affinities for all heavy metals; if the wrongly selected extractant type is employed, the accuracy of the results will be adversely affected.  Extractants vary in their reaction; for example, H2SO4 has been found inappropriate for digesting samples containing Barium (Ba), and HCl has been found unsuitable for digesting samples containing silver (Ag) and lead (Pb) (Abegunde et al., 2018). Similarly, arsenic (As) compounds have been found unsuitable to act as solvents for alkaline earth metals because they react with volatile compounds and HCl and H2SO4 to form insoluble sulphate salts (Twyman, 2005).

To improve metal extraction efficiency, acid mixtures have been advocated (Wilson et al., 2005; Szakova et al., 2010). Combining acids such as HF, HNO3, HClO4, and H2SO4 can enhance the simultaneous analysis of Cu, Cd, Ni, and Zn in soils (Idera et al., 2014). This combination can be modified by substituting H2SO4 for HCl acids since Pb can be precipitated with H2SO4 in solution. Although these combinations cannot completely dissolve silicates, they have been found capable of extracting heavy metals from the soil matrix. Aqua regia (concentrated HCl and HNO3 acids in 4:1 or 3:1 v/v), for example, has been found effective in extracting trace metals in soils (Baker and Amacher, 1982; Tukura et al., 2013; Idera et al., 2014; Rocky et al., 2024).

This study therefore aimed to evaluate the heavy-metal removal efficiency of various chemical extractants in soils impacted by three waste sources: automechanic, paint-processing, and abattoir dumpsites in Akwa Ibom State, Nigeria. In this research, it was hypothesized that different extractants would significantly influence the concentrations of heavy metals extracted from soils. Three chemical extractants: aqua regia (HCl and HNO3 in a 3:1 ratio), nitric-perchloric acids, and EDTA were used to assess the concentrations of five heavy metals (lead, cadmium, chromium, nickel, and vanadium) within the waste-impacted soils. Using multiple digestion methods to analyze heavy metal concentrations across soils contaminated by these waste types will help identify the optimal method with the highest accuracy and analytical efficiency.

LOCATION AND GEOLOGY OF THE STUDY SITES

This study was conducted in Akwa Ibom State, in Southern Nigeria. The State has a humid tropical climate and is situated between latitudes 4°32′ and 5°33′ N and longitudes 7°25′ and 8°25′ E. Cross River State borders it to the east, Rivers State to the west, Abia State to the north, and the Atlantic Ocean to the south. About 70 % soils of Akwa Ibom State are from the Benin Formation (coastal plain sands) (Udoh et al., 2008; Ekanem and Udosen, 2023a, b; Udosen et al., 2024a, b). Other parent materials include sandstones, beach ridge sands, shales, and alluvial deposits (Udoh and Ibia, 2022; Udosen and George, 2024). A typical soil originating from coastal plain sands is predominantly coarse-textured, making the soil generally well-drained (Udo et al., 2009a, b; Akpan-Idiok, 2012).

Three local government areas (LGAs) in the State: Uyo, Ikot Ekpene, and Etinan were selected for the study (Figure 1). Uyo is the State’s capital, and as such, the center of all developmental projects in the State, with the highest pollution (Opara et al., 2005). The location of Ikot Ekpene along the major federal highway linking the State to the southeastern part of the country has given it an advantage for rapid development, characterized by widespread automobile repair workshops and metal works, as well as food-processing industries. Etinan had also experienced some degree of heavy metal contamination from heavy metal-laden waste discharges. For example, one of Nigeria's leading paint-processing plants, the defunct Peacock Paint Limited, was located in Etinan.

Three dumpsites receiving waste from automechanic (A), paint-processing (P), and abattoir (AB) operations were selected in each LGA to represent multiple contamination sources. One control site from each LGA with no known history of waste disposal or industrial activities was selected, ensuring that the soil type was similar to that of the waste-impacted soils. The soils were sampled at depths: 0–20, 20 - 40, and 40-60 cm.

Figure 1: Map indicating locations of study areas.

MATERIALS AND METHODS

Extractions of the heavy metals from the soils

(i) Extraction with aqua regia (concentrated hydrochloric and nitric acids (HCl/HNO3) in the ratio of 3:1).

Extraction was performed using the method of Abegunde et al. (2018). One gram of the soil sample was weighed into a digestion flask, and 20 ml of aqua regia was added to the flask. The digestion was carried out using a heating mantle in the fume cupboard. The temperature was gradually increased with occasional agitation of the mixture until the flask contained approximately 5 ml of heated liquid. The flask was then covered with a watch glass to prevent excessive evaporation. The remaining solution was filtered, washed with deionized and double-distilled water, and then transferred to a 50 ml volumetric flask. Distilled water was added to make up to the 50 ml mark, and the resulting solution was analyzed using an Atomic Absorption Spectrophotometer (AAS) (Model AA500, PG Instruments.

(ii) Digestion with nitric and perchloric acids (HNO3/HClO4)

Extraction was done as in the method of Abegunde et al. (2018). The soil sample (1 g) was measured into a 250 ml digestion flask, and 10 ml of concentrated HNO3 acid was added. The mixture was gently boiled for approximately 40 minutes to remove all oxidizable materials. It was allowed to cool, and 5 ml of perchloric acid (70% w/w) was added. The mixture was reheated until whitish fumes evolved, then allowed to cool, after which 20 ml of distilled water was added. It was boiled again to remove fumes, cooled, and filtered through Whatman No. 42 filter paper and a <0.451 Millipore filter before being transferred to a 50 ml volumetric flask. Distilled water was added to make up to the 50 ml mark, and the resulting solution was analyzed using an Atomic Absorption Spectrophotometer (AAS) (Model AA500, PG Instruments.

(iii) Digestion with ethylenediamine tetraacetic (EDTA):

Extraction was performed using the method of Lindsay and Norvell (1978), as described by Omaka et al. (2015). One gram of the soil sample was weighed into extraction bottles, to which 50 ml of 1 M EDTA (pH 7.0) was added. The mixture was agitated for 60 minutes on a rotary shaker. The resulting suspension was then filtered with Whatman filter paper No 42, and concentrations of heavy metals in the extract were determined using AAS.

Statistical analyses

Data obtained from the analyses were subjected to analysis of variance (ANOVA). Where a significant difference was noticed among means, they were separated using the least significant difference (LSD) at 95 % confidence interval.

RESULTS AND DISCUSSION

Extracting capacities of the extractants

Table 1 presents the overall mean concentrations of the investigated heavy metals as extracted using the three chemical extractants. The overall mean values represent the average concentrations obtained from all samples collected across the nine dumpsites and the thee control site, considering all investigated depths (0–20, 20–60, and 60–80 cm). The mean concentration of Pb extracted using aqua regia (AR), nitric–perchloric acids (NP), and ethylenediamine tetraacetic acid (EDTA) were 1534.0, 1419.0, and 1192.0 mg/kg, respectively. These values exceeded the FAO permissible limit of 300 mg/kg for Pb in residential soils by approximately 411%, 373%, and 267%, respectively, indicating severe lead contamination in the study area. The Pb concentrations obtained using the three extractants differed significantly from one another, as confirmed by the LSD value shown in Table 1, and followed the decreasing order: AR > NP > EDTA. The relative extraction efficiency ratio for Pb was approximately 1.29 : 1.19 : 1.00 for AR, NP, and EDTA, respectively.

Table 1: Mean concentrations of the heavy metals extracted with different extractants

| Extractant | Lead | Cadmium | Chromium | Nickel | Vanadium |
| --- | --- | --- | --- | --- | --- |
|  | mg/kg |  |  |  |  |
| Aqua regia | 1534.0 | 82.9 | 294.8 | 158.0 | 142.3 |
| Nitric-perchloric acids | 1419.0 | 56.9 | 254.5 | 116.8 | 105.0 |
| EDTA | 1192.0 | 31.7 | 197.9 | 75.4 | 63.5 |
| LSD | 31.0 | 3.3 | 3.8 | 5.1 | 4.9 |

Keys: EDTA = ethylenediamine tetraacetic acid, e = ecological risk, t = health risk

The values extracted by AR and NP were pseudo-total trace metal concentrations, while those extracted by EDTA were exchangeable metal concentrations (Ivezić et al., 2013). AR values reflected the total recoverable metals, including those bound in silicates and metal oxides, providing a conservative estimate of overall contamination.  These results agree with previous research (Abegunde et al., 2018) which indicated that aqua regia extracted the highest concentration of Pb from dumpsite soils of Ado-Ekiti, Nigeria, in comparison to HNO3/HClO4 and HNO3.  These results also corroborated previous research (Elless and Blaylock, 2000), which indicated that EDTA-extracted forms of Pb were linked to the exchangeable and carbonate fractions of the metal, while reduced solubility with EDTA was obtained in organic, oxides, and residual forms

Mean concentrations of Cadmium obtained with aqua regia (AR), nitric-perchloric acids (NP), and ethylenediamine tetraacetic acid (EDTA) were 82.3, 56.9, and 31.7 mg/kg, respectively (Table 1). These values were higher than the FAO regulatory limit for residential soils (5 mg/kg) by 1546%, 1038%, and 534%, respectively. Cadmium concentrations from the 3 extractants were significantly different from each other, with the ratio: 3.6: 1.8: 1.0 for AR: NP: EDTA, respectively. EDTA, although it had the lowest values, represented the reactive fraction of the total metal concentration (Ivezić et al., 2013). This is important in environmental management, as it informs the most chemically active and bioavailable fraction of the total metal concentration in soils.

Mean concentration of Chromium obtained using AR, NP, and EDTA extractants were 294.8, 254.5, and 197.9 mg/kg, respectively (Table 1). These values were less than FAO’s regulatory limit (300 mg/kg) for residential zones by 1.73, 15.17, and 34.03 %, respectively.  Chromium concentrations from the 3 extractants were significantly different from each other, with the ratio: 1.48: 1.29: 1 for AR: NP: EDTA, respectively. These results validated previous research reporting higher aqua regia-extracted Cr values than those extracted with EDTA (Tukura et al., 2013).

Mean concentration of Nickel obtained using AR, NP, and EDTA extractants were 158.0, 116.8, and 75.4 mg/kg, respectively (Table 1). These values were higher than FAO’s regulatory limit (150 mg/kg) by 5.87, 22.13, and 49.73%, respectively. Nickel concentrations from the 3 extractants were significantly different, with the ratios AR:NP: EDTA being 2.11:1.55:1, respectively. These results confirmed previous research (Tukura et al., 2013; Akannam and Kakulu, 2022), which reported higher aqua regia-extracted Ni than EDTA.

Mean concentrations of vanadium obtained using AR, NP, and EDTA extractants were 142.3, 105.0, and 53.5 mg/kg, respectively. These were less than FAO’s regulatory limit (300 mg/kg) by 52.57, 65, and 82.17 %, respectively.  Vanadium concentrations from the 3 extractants were significantly different from each other and in the ratio: 2.66: 1.96: 1 for AR: NP: EDTA, respectively. EDTA is an effective complexing agent that forms mixed solutions with dilute acids and inorganic salts, thereby improving the extractability of metals from soil. Still, it may not yield consistent results across metals (Yang et al., 2022). EDTA forms more stable complexes with Zn, Pb, Cd, Cu, and Ni than with V, which explains the comparatively lower concentrations of vanadium extracted using EDTA.

The results, therefore, showed that the three chemical extractants (AR, NP, and EDTA) have different metal-extraction capacities. Concentrations of the extracted metals were generally significantly different from each other and followed the order: AR>NP>EDTA. Aqua regia and nitric-perchloric acids performed better than EDTA (Abegunde et al., 2018) due to the combination of two strong acids present in these two reagents.  Work done by Bozym (2017), however, found that 0.05 M EDTA could extract up to 37.7 mg/kg of Pb from foundry sand waste, and this was the highest concentration compared to other concentrations extracted by H2O, HCl, MgCl2, CH3COOH, and NaCOOH. In another research conducted by Maturi et al. (2008), different EDTA concentrations (0.01, 0.05, 0.1, and 0.2 M) were used; the results indicated that the amount of zinc extracted from soils was directly proportional to the extractant concentration.

In this study, a 1.0 M EDTA concentration was used, whereas Maturi et al. (2008) used 0.01, 0.05, 0.1, and 0.2 M EDTA. Despite the high concentrations employed in this study, EDTA could only extract the lowest concentrations of these metals (Pb, Cd, Cr, Ni, and V) when compared to those of aqua regia and nitric-perchloric acids. This has highlighted the limited effectiveness of EDTA in extracting certain metals from soil (Tukura et al., 2013). Similar results by Maturi et al. (2008) indicated that two chelating agents used (EDTA and DPTA) were ineffective at extracting Zn and Pb from soil.  This was due to the presence of other metals, which affected the stability of zinc-chelate and lead chelate complexes. Another plausible reason was that the chelating agents formed complexes with other non-target metals.

In other research (Elless and Blaylock, 2000; Lee and Kao, 2004), EDTA was found to be more suitable for extracting the exchangeable forms of Pb than immobile fractions, i.e., those attached to Fe-Mn oxides and soil matrix (residual form). The soils in this study have been reported to have high Fe contents (Udo et al., 2008; Udoh et al., 2008; Ebong et al., 2020), probably contributing to high concentrations of Fe-Mn oxides bound to Pb, making Pb less available for EDTA extraction.

Comparison of extracted heavy-metal concentrations with international soil quality guideline values (MEF, 2007)

Table 2 compared the concentrations of the extracted heavy metals with the threshold values established by the 2007 Ministry of the Environment, Finland (MEF), for assessing soil contamination and remediation needs.

Table 2: Regulatory guideline values of heavy metals compared with those obtained with different extractants.

| Heavy metal | NC (a) (mg/kg) | TV (mg/kg) (a) | L (a) | H (a) | AR |  | NP |  | EDTA |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | D (mg/kg) | AR - H (%) | D (mg/kg) | NP - H (%) | D (mg/kg) | EDTA - H (%) |
| Lead | 1-5 | 60.0 | 200(t) | 750.0 (e) | 1534.0 | 667 | 1419.0 | 609.5 | 1192.0 | 496 |
| Cadmium | 0.01-0.15 | 1.0 | 10 | 10.0 (e) | 82.9 | 729 | 56.9 | 469 | 31.7 | 217 |
| Chromium | 6-120 | 100.0 | 200 | 300.0 (e) | 294.8 | 47.4 | 254.5 | 27.27 | 197.9 | -1.05 |
| Nickel | 3-100 | 50 | 100 | 250 (e) | 158.0 | 216 | 116.8 | 16.8 | 75.4 | -24.6 |
| Vanadium | 10-115 | 10 | 150 | 150 (e) | 142.3 | -5.13 | 105.0. | -30 | 63.5 | -57.67 |

Keys: NC = natural concentration in soils, TV = threshold value, L = lower guideline value, H = higher guideline value, a = MEF (2007), D = field data, AR = aqua regia, NP = nitric-perchloric acids, EDTA = ethylenediamine tetraacetic acid, e = ecological risk, t = health risk

Concentrations of lead at 1534.0, 1419.0, and 1192.0 mg/kg obtained using AR, NP, and EDTA extractants were higher than the 2007 MEF lower guideline value of 60 mg/kg by 667, 609.5, and 496 %, respectively.  Cadmium values were also higher than the threshold values.  Chromium extracted with AR and NP exceeded the lower guideline value by 47.4% and 27.27%, respectively, while the concentration obtained with EDTA was below the lower guideline value. Similar observations were noted for nickel. For vanadium, the AR-, NP-, and EDTA-extracted values were below the lower guidelines by 5.13, 30, and 57.67%, respectively. These variations were due to differences in the extractants' extraction capacities (Idera et al., 2014; Abegunde et al., 2018).

Comparison of heavy metal concentrations at topsoil depth across different dumpsites using three extractants

According to Table 3, the mean Pb concentrations at topsoil depth (0-20 cm) increased from control soils (8.2 mg/kg) to abattoir soils (68.2 mg/kg), paint soils (1465.3 mg/kg), and automechanic soils (3877.0 mg/kg), giving the order: control < abattoir < paint < automechanic. The increase reflected increasing anthropogenic influence. The control soil represents background levels, while abattoir soil may receive Pb from waste disposal and minor mechanical activities. Elevated Pb in paint soils are linked to lead-based pigments and additives, whereas the highest concentration in automechanic soil was attributed to automobile wastes such as used oil, batteries, metal scraps, and vehicular emissions, which are common sources of Pb contamination.

Lead concentration obtained using aqua regia digestion (1534.2 mg/kg) was higher than those extracted with nitric–perchloric acid (1341.1 mg/kg) and EDTA (1188.0 mg/kg). The extraction efficiency followed the order EDTA < nitric–perchloric acid < aqua regia, indicating the stronger digestion capacity of aqua regia compared with nitric–perchloric acid and the chelating extractant EDTA.

Table 3: Concentrations of lead extracted with the three extractants from the top soil (0-20 cm) of different dumpsite soils.

| Extractant | Dumpsite soils |  |  |  | Mean |
| --- | --- | --- | --- | --- | --- |
|  | Abattoir | Automechanic | Control | Paint |  |
|  | mg/kg |  |  |  |  |
| Aqua regia (AR) | 73.0 | 4310.1 | 10.2 | 1745.0 | 1534.2 |
| Nitric-perchloric acids (NP) | 67.1 | 3824.2 | 8.4 | 1464.1 | 1341.1 |
| EDTA | 63.1 | 3498.3 | 6.1 | 1187.3 | 1188.0 |
| Mean | 68.2 | 3877.0 | 8.2 | 1465.3 |  |
| LSD | Extractants (336.1); Dumpsite (388.1); Extractants/Dumpsite (672.2) |  |  |  |  |

Keys: AR = Aqua regia; EDTA = Ethylenediamine tetraacetic acid; NP = Nitric-perchloric acids; LSD = Least Significant Difference

The mean Cd concentrations in the soils followed the order automechanic > paint > abattoir > control (Table 4), indicating increasing contamination with the intensity of waste-related activities at the sites. The low Cd level in the control soils reflected background conditions, while higher values in the abattoir soils were associated with waste disposal and organic residues. Elevated Cd in the paint soils were linked to pigments, stabilizers, and additives, whereas the highest concentration in the automechanic soil were due to automobile wastes such as used oil, batteries, metal scraps, and lubricants, which are known sources of Cd contamination. The concentration extracted with aqua regia (82.3 mg/kg) was higher than those obtained with nitric–perchloric acid (78.2 mg/kg) and EDTA (71.7 mg/kg), giving the order EDTA < NP < AR.

Table 4: Concentrations of cadmium extracted with the three extractants from the top soil (0-20 cm) of different dumpsite soils.

| Extractant | Dumpsite soils |  |  |  | Mean |
| --- | --- | --- | --- | --- | --- |
|  | Abattoir | Automechanic | Control | Paint |  |
|  | mg/kg |  |  |  |  |
| Aqua regia (AR) | 26.5 | 179.0 | 0.8 | 122.9 | 82.3 |
| Nitric-perchloric acids (NP) | 24.1 | 171.5 | 0.7 | 116.6 | 78.2 |
| EDTA | 22.1 | 158.2 | 0.5 | 106.1 | 71.7 |
| Mean | 24.2 | 169.6 | 0.7 | 115.2 |  |
| LSD | Extractants (10.51); Dumpsite (12.59); Extractants/Dumpsite (21.81) |  |  |  |  |

Keys: AR = Aqua regia; EDTA = Ethylenediamine tetraacetic acid; NP = Nitric-perchloric acids; LSD = Least Significant Difference.

The mean chromium concentrations at the abattoir, automechanic, control, and paint dumpsites were 104.6, 430.0, 1.3, and 552.2 mg/kg, respectively, showing significant differences in the order paint > automechanic > abattoir > control (Table 5). The high Cr levels at the paint and automechanic sites were consistent with inputs from pigments, metal scraps, and mechanical wastes, whereas the control site represented background conditions with minimal anthropogenic influence. Extractant efficiency followed the sequence EDTA < NP < AR, reflecting the stronger digestion ability of the acid extractants relative to EDTA.

Table 5: Concentrations of chromium extracted with the three extractants from the top soil (0-20 cm) of different dumpsite soils.

| Extractant | Dumpsites |  |  |  | Mean |
| --- | --- | --- | --- | --- | --- |
|  | Abattoir | Automechanic | Control | Paint |  |
|  | mg/kg |  |  |  |  |
| Aqua regia (AR) | 109.3 | 467.5 | 1.5 | 600.7 | 294.8 |
| Nitric-perchloric acids (NP) | 105.9 | 434.6 | 1.3 | 546.2 | 272.0 |
| EDTA | 98.6 | 387.7 | 1.1 | 509.7 | 249.3 |
| Mean | 104.6 | 430.0 | 1.3 | 552.2 |  |
| LSD | Extractants (22.73); Dumpsite (26.48); Extractants/Dumpsite (45.87) |  |  |  |  |

Keys: AR = Aqua regia; EDTA = Ethylenediamine tetraacetic acid; NP = Nitric-perchloric acids; LSD = Least Significant Difference

Table 6 shows that mean nickel concentrations at the abattoir, automechanic, control, and paint dumpsites were 118.1, 271.7, 4.9, and 161.0 mg/kg, respectively, differing significantly in the order automechanic > paint > abattoir > control. The elevated Ni levels at the automechanic and paint dumpsites were linked to the disposal of metal scraps, lubricants, and industrial materials, whereas the control site reflected background concentration. Extraction efficiency followed the sequence AR > NP > EDTA (158.0, 139.1, and 119.8 mg/kg), indicating stronger recovery of nickel by the acid digestants compared to EDTA.

Table 6: Concentrations of nickel extracted with the three extractants from the top soil (0-20 cm) of different dumpsite soils.

| Extractant | Dumpsites |  |  |  | Mean |
| --- | --- | --- | --- | --- | --- |
|  | Abattoir | Automechanic | Control | Paint |  |
|  | mg/kg |  |  |  |  |
| Aqua regia (AR) | 128.5 | 302.5 | 6.3 | 194.6 | 158.0 |
| Nitric-perchloric acid (NP) | 119.9 | 271.8 | 5.0 | 159.6 | 139.1 |
| EDTA | 105.9 | 240.8 | 3.5 | 128.8 | 119.8 |
| Mean | 118.1 | 271.7 | 4.9 | 161.0 |  |
| LSD | Extractants (30.55); Dumpsite (35.27); Extractants/Dumpsite (61.10) |  |  |  |  |

Keys: AR = Aqua regia; EDTA = Ethylenediamine tetraacetic acid; NP = Nitric-perchloric acids; LSD = Least Significant Difference

Vanadium concentrations at the control, abattoir, paint, and automechanic dumpsites were 1.8, 94.8, 201.4, and 215.7 mg/kg, respectively, increasing in the order control < abattoir < paint < automechanic (Table 7). The low concentration at the control site reflected background levels, while higher values at the waste dumpsites indicated contamination from anthropogenic activities. Extraction efficiency followed the order AR (142.3 mg/kg) > NP (127.4 mg/kg) > EDTA (115.5 mg/kg), showing that the acid digestants recovered more vanadium than the chelating extractant.

Table 7: Concentrations of vanadium extracted with the three extractants from the top soil (0-20 cm) of different dumpsite soils.

| Extractant | Dumpsites |  |  |  | Mean |
| --- | --- | --- | --- | --- | --- |
|  | Abattoir | Automechanic | Control | Paint |  |
|  | mg/kg |  |  |  |  |
| Aqua regia (AR) | 99.5 | 227.6 | 2.2 | 240.0 | 142.3 |
| Nitric-perchloric acid (NP) | 95.2 | 197.9 | 1.9 | 214.6 | 127.4 |
| EDTA | 89.6 | 178.8 | 1.3 | 192.5 | 115.5 |
| Mean | 94.8 | 201.4 | 1.8 | 215.7 |  |
| LSD | Extractants (25.45); Dumpsite (29.39); Extractants/Dumpsite (50.90) |  |  |  |  |

Keys: AR = Aqua regia; EDTA = Ethylenediamine tetraacetic acid; NP = Nitric-perchloric acids; LSD = Least Significant Difference

Depth-Dependent Distribution of Heavy Metals

The concentration of heavy metals across soil depths (0–20, 20–40, 40–60 cm) indicated a decrease with depth (Table 8; Figures 2 - 6), showing that surface soils were more heavily contaminated than subsurface soils.

Table 8: Heavy metal concentrations (mg/kg) at different soil depths extracted with different reagents

| Soil Depth (cm) | Pb (AR) | Pb (NP) | Pb (EDTA) | Cr (AR) | Cr (NP) | Cr (EDTA) | Ni (AR) | Ni (NP) | Ni (EDTA) | V (AR) | V (NP) | V (EDTA) | Cd (AR) | Cd (NP) | Cd (EDTA) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0–20 | 1534 | 1341 | 1188 | 294.8 | 272 | 249.3 | 158 | 139.1 | 119.8 | 142.3 | 127.4 | 115.5 | 82.3 | 78.2 | 71.7 |
| 20–40 | 1419 | 1253 | 1037 | 254.5 | 224.8 | 207.4 | 116.8 | 98.3 | 82 | 105 | 87.2 | 74.7 | 56.9 | 51.8 | 52.8 |
| 40–60 | 1192 | 1014 | 830 | 197.9 | 176 | 156.1 | 75.4 | 64.1 | 54.2 | 63.5 | 53.5 | 47.3 | 31.7 | 28.2 | 24.4 |

Notes:  Units = mg/kg; AR = Aqua Regia; NP = Nitric–Perchloric acids; EDTA = Ethylenediamine tetraacetic acid

Figure 2: Comparison of lead (Pb) concentrations (mg/kg) in soils at three different depths (0-20cm, 20-40cm, and 40-60cm) using three extraction methods: aqua regia, nitric–perchloric acid digestion, and EDTA extraction.

Figure 3: Comparison of cadmium (Cd) concentrations (mg/kg) in soils at three different depths (0-20cm, 20-40cm, and 40-60cm) using three extraction methods: aqua regia, nitric–perchloric acid digestion, and EDTA extraction.

Figure 4: Comparison of chromium (Cr) concentrations (mg/kg) in soils at three different depths (0-20cm, 20-40cm, and 40-60cm) using three extraction methods: aqua regia, nitric–perchloric acid digestion, and EDTA extraction.

Figure 5: Comparison of nickel (Ni) concentrations (mg/kg) in soils at three different depths (0-20cm, 20-40cm, and 40-60cm) using three extraction methods: aqua regia, nitric–perchloric acid digestion, and EDTA extraction.

Figure 6: Comparison of vanadium (V) concentrations (mg/kg) in soils at three different depths (0-20cm, 20-40cm, and 40-60cm) using three extraction methods: aqua regia, nitric–perchloric acid digestion, and EDTA extraction.

Lead (Pb) concentrations extracted with aqua Regia (AR) decreased from 1534 mg/kg at 0–20 cm to 1192 mg/kg at 40–60 cm, while cadmium (Cd) decreased from 82.3 at 0–20 cm to 31.7 mg/kg at 40–60 cm. Chromium (Cr), nickel (Ni), and vanadium (V) exhibited similar downward trends, suggesting limited vertical migration, except for Cd, which maintained relatively high concentrations even at 40–60 cm; this is consistent with its high mobility and solubility in water (Vaněk et al., 2005; Kubier et al., 2019; Pikuła and Stępień, 2021). The surface enrichment of these metals highlighted the risk of exposure to plants, humans, and soil organisms, as the topsoil is the most interactive layer for biological and agricultural processes. This is particularly concerning for Pb and Cd, which showed significant concentrations even in the bioavailable fraction (EDTA-extractable), indicating high potential for bioaccumulation.

In summary, this study indicated that aqua regia (AR) was the most effective extractant, suggesting that the mixture of nitric and hydrochloric acids was particularly efficient. High AR-extracted values were due to the component acids' ability to dissolve and mobilize metals from their soil matrices. HNO3 worked as a strong oxidizer to convert metals into their oxidized state, and HCl donated chlorine ions to stabilize the metallic ions in the solution via the formation of metal-chlorine complexes. These two mechanisms enabled aqua regia to dissolve metals effectively.

The mixture of nitric acid (HNO3) and perchloric acid (HClO4) was also effective; however, it was less aggressive than aqua regia. The HNO3/HClO4 combination was a "wet digestion" method that extracted metals by breaking down complex organic matter and dissolving mineral components. The mechanism relied on a two-stage process in which HNO3 served as the initial, less aggressive oxidizer. This was followed by HClO4, which acted as a powerful, high-temperature oxidizer to finalize the decomposition.

EDTA, which worked by chelating metals, extracted the lowest values, though its effectiveness might be further reduced in soils with tightly-bound metals or less available metal forms. EDTA was effective as a chelating agent due to its metal-ion exchange capacity and redox reactions (Manouchehri and Bermond, 2009). It has a hexadentate ligand structure with four carboxyl and two amine groups; all these combined to enhance its ability to form chelates with different metallic ions. It therefore serves as an effective reagent for remediation due to its high solubility in aqueous solutions and its ability to mobilize metal ions.

CONCLUSION

This study demonstrates that the metal extraction efficiencies of aqua regia (AR), nitric-perchloric acids (NP), and ethylenediamine tetraacetic acid (EDTA) vary significantly. The combination of two acids in the cases of AR and NP gave them more strength to attack and destroy any soil component to release heavy metals that were tightly held. EDTA, on the other hand, had weaker strength and could not remove metals highly bound to the soil matrix. However, reliance on strong-acid digestion procedures may overestimate health and ecological risks, whereas reliance on mild extractants may underestimate contamination levels and associated risks. This highlights the need to employ multiple extraction approaches for realistic contamination assessment.

It is recommended that environmental policymakers and managers adopt integrated assessment approaches that account for the total and exchangeable (mobile and bioavailable) concentrations of metals when developing remediation strategies and land-use plans. Environmental monitoring and regulatory programs should require explicit specification of extraction procedures and guidelines for data interpretation to ensure accuracy and consistency. Stricter waste management protocols and regular environmental monitoring should be implemented to safeguard public health, particularly in urban and peri-urban areas.

DATA AVAILABILITY

All relevant data are included in the paper or its Supplementary Information.

COMPETING INTERESTS STATEMENTS

The authors have no competing interests to declare that are relevant to the content of this article.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCES

Abegunde, S.M., Oyebanji, A.O. and Osibanjo, O., 2018. Evaluation of digestion procedures on heavy metals in soil of a dumpsite in Ibadan, South-western Nigeria. Suan Sunandha Science and Technology Journal, 5(2), pp.1-5.

Adagunodo, T.A., Sunmonu, L.A. and Emetere, M.E., 2018. Heavy metals' data in soils for agricultural activities. Data in brief, 18, pp.1847-1855.https://doi.org/10.1016/j.dib.2018.04.115

Akannam, P.O. and Kakulu, S. E. 2022. Efficacy of methods for heavy metal extraction in some soils in Nnewi-North, Anambra State, Southeastern Nigeria. Int’l Journal of Agric. and Rural Dev. 25(2): 6365-6373

Akpan-Idiok, A. U. 2012. Physicochemical properties, degradation rate and rate of vulnerability potential of soil formed in coastal plain sands in Southeast Nigeria. International Journal of Agricultural Research, 7(7), 358–366. https://doi.org/10.3923/ijar.2012.358. 366

Baker, D.E. and Amacher, M.C., 1982. Nickel, copper, zinc, and cadmium. Methods of soil analysis: Part 2 chemical and microbiological properties, 9, pp.323-336. https://doi.org/10.2134/agronmonogr9.2.2ed.c19

Bożym, M., 2017. The study of heavy metals leaching from waste foundry sands using a one-step extraction. In E3S Web of Conferences (Vol. 19, p. 02018). EDP Sciences. https://doi.org/10.1051/e3sconf/20171902018

Ebong, G.A., Ettesam, E.S. and Dan, E.U., 2020. Impact of abattoir wastes on trace metal accumulation, speciation, and human health–related problems in soils within Southern Nigeria. Air, Soil and Water Research, 13, p.1178622119898430. https://doi.org/10.1177/1178622119898430

Ekanem, A.M. and Udosen, N.I., 2023. Hydrogeochemical–geophysical investigations of

groundwater quality and susceptibility potential in Ikot Ekpene–Obot Akara Local Government Areas, Southern Nigeria. Water Practice and Technology, 18(11), pp.2675-2704. https://doi.org/10.2166/wpt.2023.187

Ekanem, A.M. and Udosen, N.I., 2023. Evaluation of groundwater potentiality and quality in Ikot Ekpene-Obot Akara Local Government Areas, Southern Nigeria. Environmental Contaminants Reviews, 6(1): 46-57. https://doi.org/10.26480/ecr.01.2023.46.57

Elless, M.P. and Blaylock, M.J., 2000. Amendment optimization to enhance lead extractability from contaminated soils for phytoremediation. International Journal of Phytoremediation, 2(1), pp.75-89. https://doi.org/10.1080/15226510008500031

Food and Agriculture Organization of the United Nations. (2004). Maximum limits for dangerous substances in soil and groundwater. https://faolex.fao.org/docs/pdf/est97999E.pdf

Gibson, M.J. and Farmer, J.G., 1986. Multi-step sequential chemical extraction of heavy metals from urban soils. Environmental Pollution Series B, Chemical and Physical, 11(2), pp.117-135. https://doi.org/10.1016/0143-148x(86)90039-x

Hlavay, J., Prohaska, T., Weisz, M., Wenzel, W.W. and Stingeder, G.J., 2004. Determination of trace elements bound to soils and sediment fractions. Pure Appl. Chem, 76(2), pp.415-442. https://doi.org/10.4314/as.v21i2.8

Idera, F., Omotola, O., Paul, U.J. and Adedayo, A., 2014. Evaluation of the effectiveness of different acid digestion on sediments. IOSR Journal of Applied Chemistry, 7, 39-47. https://doi.org/10.9790/5736-071213947

Igwe, A.C. and Nwachukwu, O.I., 2016. Soil Health, our health: Effect of paint effluent contaminated soil on the heavy metal content of okra (Abelmoschus esculentus Moench). Nigerian Journal of Soil Science, 26, pp.68-77.

Ivezić, V., Lončarić, Z., Engler, M., Kerovec, D. and Singh, B. R. 2013. Comparison of different extraction methods representing available and total concentrations of Cd, Cu, Fe, Mn, and Zn in soil. Poljoprivreda 19 (1): 53-58.

Jyothi, N.R., 2020. Heavy metal sources and their effects on human health. In Heavy metals-their environmental impacts and mitigation. IntechOpen. http://dx.doi.org/10.5772/intechopen.95370.

Kabata-Pendias, A., 2000. Trace elements in soils and plants. CRC Press. https://doi.org/10.1201/9781420039900

Kaparwan, Dimple, N. S. Rana, and B. P. Dhyani. Heavy metals toxicity in agricultural soils–critical review of possible sources, influence on soil health and remedial measures to remove, reduce and stabilize contaminants in soil. International Journal of Current Microbiology and Applied Sciences 9, no. 6 (2020): 1467-1482. https://doi.org/10.20546/ijcmas.2020.906.182

Kubier, A., Wilkin, R.T. and Pichler, T., 2019. Cadmium in soils and groundwater: A review. Applied geochemistry, 108, p.104388. https://doi.org/10.1016/j.apgeochem.2019.104388

Lee, C.S. and Kao, M.M., 2004. Effects of extracting reagents and metal speciation on the removal of heavy metal contaminated soils by chemical extraction. Journal of Environmental Science and Health, Part A, 39(5), pp.1233-1249. https://doi.org/10.1081/ese-120030328

Li, X., 2024. A comparative study on the efficacy of conventional and green chelating agents for soil heavy metal remediation. Theoretical and Natural Science, 37(1), pp.206-215. https://doi.org/10.54254/2753-8818/37/20240193

Lindsay, W.L. and Norvell, W., 1978. Development of a DTPA soil test for zinc, iron, manganese, and copper. Soil Science Society of America Journal, 42(3), pp.421-428. https://doi.org/10.2136/sssaj1978.03615995004200030009x

Mahurpawar, M., 2015. Effects of heavy metals on human health. Int J Res Granthaalayah, 530(516), pp.1-7. https://doi.org/10.29121/granthaalayah.v3.i9se.2015.3282

Manouchehri, N. and Bermond, A., 2009. EDTA in soil science: a review of its application in soil trace metal studies. Terr Aquat Environ Toxicol, 3(1), pp.1-15.

Maturi, K., Khodadoust, A.P. and Reddy, K.R., 2008. Comparison of extractants for removal of lead, zinc, and phenanthrene from manufactured gas plant field soil. Practice Periodical of Hazardous, Toxic, and Radioactive Waste Management, 12(4), pp.230-238. https://doi.org/10.1061/(asce)1090-025x(2008)12:4(230)

Ministry of Environment, Finland (MEF). 2007. Government Decree on the Assessment of Soil Contamination and Remediation Needs. 214/2007. https://www.finlex.fi/api/media/statute-foreign-language-translation/687954/mainPdf/main.pdf?timestamp=2014-06-04T21%3A00%3A00.000Z [Retrieved on 16/02/2026]

Munir, N., Jahangeer, M., Bouyahya, A., El Omari, N., Ghchime, R., Balahbib, A., Aboulaghras, S., Mahmood, Z., Akram, M., Ali Shah, S.M. and Mikolaychik, I.N., 2021. Heavy metal contamination of natural foods is a serious health issue: A review. Sustainability, 14(1), p.161. https://doi.org/10.3390/su14010161

Obi, J.C., Ibia, T.O. and Ikott, I.E., 2020. Effects of land use on forms of phosphorus in coastal plain sands of Nigeria. Nigerian Journal of Soil Science. 30(3): 31-41.

Ojiego, B.O., Ilo, O.P., Okolo, J.C., Igborgbor, J.C., Ishaku, T., Abdullahi, S.A., Gadzama, I.M.K. and Bolorunduro, P., 2022. Concentrations of heavy metals in soil samples from dumpsites located at Kuje and Kwali area councils, Abuja, Nigeria. Journal of Materials and Environmental Sciences, 13(9), pp.1037-1046.

Omaka, N.O., Offor, I.F., Chukwu, E R. and Ewuzie, U. (2015). Comparison of the Extraction Efficiencies of Four Different Solvents used in Trace metal Digestion of Selected Soils within Abakaliki, Nigeria.  J. Appl. Sci. Environ. Mgt, 19(2): 225-232

Opara, M.N., Ukpong, U.M. and Okoli, I.C., 2005. Quantitative analysis of abattoir slaughtering of animals in Akwa Ibom State, Nigeria. Journal of Agriculture and Social Research (JASR), 5(1), pp.118-126. https://doi.org/10.4314/jasr.v5i1.2837

Pikuła, D. and Stępień, W., 2021. Effect of the degree of soil contamination with heavy metals on their mobility in the soil profile in a microplot experiment. Agronomy, 11(5), p.878. https://doi.org/10.3390/agronomy11050878

Rocky, M.M.H., Rahman, I.M., Sakai, Y., Biswas, F.B., Rahman, S., Endo, M., Wong, K.H., Mashio, A.S. and Hasegawa, H., 2024. Enhanced recovery of gold from aqua regia leachate of electronic waste using dithiocarbamate-modified cellulose. Journal of Material Cycles and Waste Management, 26(2), pp.816-829. https://doi.org/10.1007/s10163-023-01824-3

Santoro, A., Held, A., Linsinger, T.P., Perez, A. and Ricci, M., 2017. Comparison of total and aqua regia extractability of heavy metals in sewage sludge: The case study of a certified reference material. TrAC Trends in Analytical Chemistry, 89, pp.34-40. https://doi.org/10.1016/j.trac.2017.01.010

Sharma, K.D. and Jain, S., 2020. Municipal solid waste generation, composition, and management: the global scenario. Social Responsibility Journal, 16(6), pp.917-948. https://doi.org/10.1108/srj-06-2019-0210

Száková, J., Miholová, D., Tlustoš, P., Šestáková, I. and Frková, Z., 2010. Effect of soil properties and sample preparation on extractable and soluble Pb and Cd fractions in soils. https://doi.org/10.4236/as.2010.13015

Tóth, G., Hermann, T., Da Silva, M.R. and Montanarella, L.J.E.I., 2016. Heavy metals in agricultural soils of the European Union with implications for food safety. Environment international, 88, pp.299-309. https://doi.org/10.1016/j.envint.2015.12.017

Tyagi, B. and Kumar, N., 2021. Bioremediation: Principles and applications in environmental management. In Bioremediation for Environmental Sustainability (pp. 3-28). Elsevier. https://doi.org/10.1016/b978-0-12-820524-2.00001-8

Tukura, B.W., Usman, N.L. and Mohammed, H.B., 2013. Aqua regia and ethylenediaminetetracetic acid (EDTA) trace metal levels in agricultural soil. Journal of Environmental Chemistry and Ecotoxicology, 5(11), pp.284-291.

Udo, B. U. (2025). Evaluation of concentration and geochemical speciation of heavy metals in dumpsite soils of coastal plain origin of Akwa Ibom State, Nigeria (PhD dissertation). University of Uyo, Uyo, Akwa Ibom State, Nigeria.

Udo, B. U. Edem, S. O., Udom, G. N. and Ndaeyo, N. U. (2008). Chemical Characteristics of Wetland Soils in Akwa Ibom State, Nigerian Journal of Agricultural Technology, Vol. 13: 53 – 64.

Udo, B. U., Ibia, T. O. and Udoh, B. T., 2009b. Evaluation of inland depression and floodplain (wetland) soils for rice and maize cultivation in a humid tropical climate of South-Eastern Nigeria. International Journal of Agriculture. 1(2): 13-22.

Udo, B.U., Ibia,, T.O. and.Akpan, S.A., 2025a. Analysis and characteristics of soils underlying open dumpsites in Akwa Ibom State, Nigeria: Their agricultural and environmental implications. Asian Soil Research Journal, 9(3), pp.90-106. https://doi.org/10.9734/asrj/2025/v9i3193

Udo, B.U., Ibia, T.O., Akpan, U.S. and Udosen, N.I., 2025b. Waste fingerprints in soils: A study of the influence of wastes on the concentrations of selected heavy metals in soils underlying open dumpsites. Asian Soil Research Journal, 9(3), pp.200-217. https://doi.org/10.9734/asrj/2025/v9i3201

Udo, B.U., Udosen, N.I. and Dominic, P.B., 2026a. Geochemical Signatures and Waste-Soil Interactions of Heavy Metals in Open Dumpsite Soils: Implications for Contaminant Dynamics and Soil Health. Journal of Global Ecology and Environment, 22(1), pp.89-104. https://doi.org/10.56557/jogee/2026/v22i110233.

Udo, B.U., Udosen, N.I. and Ibia, T.O., 2026b. Comprehensive geochemical characterization

and multi-index ecotoxicological appraisal of heavy metal contamination in soils impacted

by open waste disposal practices. Discover Soil. 3:17:1-35. https://doi.org/10.1007/s44378-026-00177-1

Udo, B.U., Utip, K.E., Inyang, M.T. and Idungafa, M.A., 2009a. Fertility assessment of some inland depression and floodplain (wetland) soils in Akwa Ibom State. Agro-Science, 8(1). https://doi.org/10.4314/as.v8i1.44108

Udoh, B.T. and Ibia, T.O., 2022. Fertility capability classification for agricultural land use planning in the beach sands area of Akwa Ibom State, Nigeria. Agro-Science, 21(2), pp.74-78. https://doi.org/10.4314/as.v21i2.8

Udoh, B.T., Ibia, T.O., Udo, B.U. and Edem, S.O., 2008. Assessment of micronutrient status of inland depression and floodplain (wetland) soils in Akwa Ibom State, South-Eastern Nigeria. Journal of Tropical Agriculture, Food, Environment and Extension. 7(2): 156 – 161

Udosen, N.I. and George, N.J., 2024. Evaluation of specific retention, specific yield, and storage-dependent drainability efficiency in a coastal milieu via geo-electrical technology. Water Practice and Technology, 19(9), pp.3654-3675. https://doi.org/10.2166/wpt.2024.208

Udosen, N.I., Ekanem, A.M., George, N.J. and Thomas, J.E., 2024a. Geo-electrostratigraphic assessment of aquifer potential, protectivity, and pliable level of vulnerability within a coastal milieu. Water Practice and Technology. 19(5), pp.2010-2031. https://doi.org/10.2166/wpt.2024.125

Udosen, N.I., Ekanem, A.M. and George, N.J. 2024b. Geo-electrical prognosis of aquifer

protectivity, corrosivity, and vulnerability via index-based models within a major coastal

milieu. Discover Geoscience, 2, 18. https://doi.org/10.1007/s44288-024-00020-6

Umoh, S.D. and Etim, E.E., 2013. Determination of heavy metal contents from dumpsites within Ikot Ekpene, Akwa Ibom State, Nigeria using atomic absorption spectrophotometer. The International Journal of Engineering and Science (IJES), 2(2), pp.123-129.

Vaněk, A., Borůvka, L., Drábek, O., Mihaljevič, M. and Komárek, M., 2005. Mobility of lead, zinc and cadmium in alluvial soils heavily polluted by smelting industry. Plant, Soil and Environment, 51(7), pp.316-321. https://doi.org/10.17221/3592-pse

Wang, J.X., Xu, D.M., Fu, R.B. and Chen, J.P., 2021. Bioavailability assessment of heavy metals using various multi-element extractants in an indigenous zinc smelting contaminated site, southwestern China. International Journal of Environmental Research and Public Health, 18(16), p.8560. https://doi.org/10.3390/ijerph18168560

Wilson, B., Braithwaite, A. and Brian Pyatt, F., 2005. An evaluation of procedures for the digestion of soils and vegetation from areas with metalliferous pollution. Toxicological and Environmental Chemistry, 87(3), pp.335-344. https://doi.org/10.1080/02772240500165570

Winegardner, D.L., 2019. An introduction to soils for environmental professionals. Routledge. https://doi.org/10.1201/9781315137322

Wuana, R.A. and Okieimen, F.E., 2011. Heavy metals in contaminated soils: a review of sources, chemistry, risks and best available strategies for remediation. International Scholarly Research Notices, 2011(1), p.402647. https://doi.org/10.5402/2011/402647

Yang, J., Wang, Y., Gao, X., Zuo, R., Song, L., Jin, C., Wang, J. and Teng, Y., 2022. Vanadium: a review of different extraction methods to evaluate bioavailability and speciation. Minerals, 12(5), p.642. https://doi.org/10.3390/min12050642

Twyman, R.M., 2005. Sample dissolution for elemental analysis| Wet Digestion. Encyclopedia of analytical science, pp.146-153. London: Elsevier Science https://doi.org/10.1016/b0-12-369397-7/00539-2

Zhang, P., Yang, M., Lan, J., Huang, Y., Zhang, J., Huang, S., Yang, Y. and Ru, J., 2023. Water quality degradation due to heavy metal contamination: health impacts and eco-friendly approaches for heavy metal remediation. Toxics, 11(10), p.828. https://doi.org/10.3390/toxics11100828