# An Integrated Multi-Modal Wearable Sensor System for Real-Time Monitoring of Athlete Stress, Hydration, and Tendon Stiffness Analysis

Abstract:

This study presents the development of an integrated multi-modal wearable sensor system for real-time monitoring of athlete stress, hydration, and tendon stiffness. The system leverages an Arduino Uno microcontroller interfaced with multiple physiological and biomechanical sensors, including heart rate and heart rate variability sensors (MAX30102/05), galvanic skin response (GSR) modules, bioimpedance circuits, skin temperature sensors, high-resolution inertial measurement units (MPU6050/ICM-20948), and strain sensors (Flexi Force). By collecting and pre-processing analog and digital signals, the system provides continuous assessment of athlete physiological states and musculoskeletal load without the need for EMG. Data are transmitted wirelessly to a mobile or PC interface for further analysis, while real-time alerts are delivered through vibration motors, LEDs, or buzzers to prevent stress-related issues, dehydration, and tendon overuse injuries. Validation demonstrates the feasibility of integrating multiple sensing modalities in a compact wearable platform, enabling proactive athlete health management and performance optimization.

Keywords: Wearable sensors, athlete monitoring, stress detection, hydration assessment, tendon stiffness, IMU, GSR, bioimpedance, real-time feedback, Arduino Uno

## 1.

## Introduction

Athlete performance and injury prevention are closely linked to continuous monitoring of physiological and musculoskeletal states. Stress, dehydration, and tendon overload are primary contributors to reduced performance and increased injury risk in competitive sports. Conventional monitoring methods, such as EMG, laboratory-based hydration assessments, or subjective stress questionnaires, are often invasive, time-consuming, or impractical for real-time applications. Recent advances in wearable sensor technologies and microcontroller-based platforms have enabled non-invasive, continuous monitoring of multiple physiological parameters in real-world settings. Integrating multiple sensing modalities—physiological, biomechanical, and motion-related—into a single wearable platform offers a holistic view of athlete health and performance. However, challenges remain in sensor fusion, real-time processing, and delivering actionable feedback to athletes and coaches. This study presents a novel integrated multi-modal wearable system that monitors athlete stress, hydration, and tendon stiffness in real-time. By leveraging an Arduino Uno microcontroller with multiple sensors, the system performs on-device pre-processing, wireless data transmission, and real-time feedback, providing an accessible and compact solution for high-performance training environments.

## 2. Literature Review

### 2.1 Athlete Stress Monitoring

Recent research strongly supports the conclusion that integrating HR, HRV, and electrodermal activity (EDA/GSR) significantly improves physiological stress detection compared to single-signal monitoring. For example, Xiang et al. (Jun-Zhi Xiang, Qin-Yong Wang, Zhi-Bin Fang, James A. Esquivel, and Zhi-Xian Su) in Frontiers in Physiology (2025) developed a multimodal deep learning stress detection model that processes combined physiological signals including heart rate and EDA, demonstrating that fusing time- and frequency-domain features markedly enhances classification robustness and yields high detection performance relative to traditional single-signal models. In a wearable sensor reproducibility study by Amin, Mishra, Tapera, Volpe & Sathyanarayana (2025), heart rate variability (HRV) and EDA combined models achieved AUROC up to 0.953–0.961 across multiple consumer and research devices, confirming that adding EDA to HRV improves stress discrimination in real-world wearable contexts. Similarly, Ardecani & Shoghli (2025) showed in an experimental immersive environment that elevated HR measures and stronger electrodermal responses jointly discriminate stress intensity better than either signal alone, emphasizing autonomic nervous system effects detectable by multimodal sensing. Systematic analyses of stress detection methods consistently identify EDA as a key sympathetic arousal marker complementary to HRV’s parasympathetic information, with multimodal fusion (often combining PPG, EDA, and additional signals) linked to higher predictive accuracy across benchmark datasets such as WESAD and SWELL compared to single modalities, highlighting the value of multi-signal integration in both academic and applied stress recognition research. This research trend underscores that biomarker fusion (HR/HRV + EDA) captures broader autonomic dynamics and enhances stress detection models’ reliability and robustness, a finding supported across recent studies from 2025 and forward.

### 2.2 Hydration Monitoring

### Hydration plays a critical role in athletic performance and thermoregulation, with even mild dehydration negatively affecting endurance, cognitive function, and heat stress responses; bioimpedance analysis (BIA) has become a widely studied non-invasive method to estimate body water content and fluid balance by measuring the electrical opposition of tissues, exploiting the fact that water-rich tissues conduct electricity better than dehydrated tissues. In healthy adult volunteers, H. N. Andersen, A. T. Horn, and K. Rønnestad (2024) demonstrated that a wearable bioimpedance sensor could sensitively track furosemide-induced extracellular fluid loss, with significant increases in extracellular resistance correlating tightly with estimated fluid loss, highlighting BIA’s capability to monitor acute hydration changes. Moreover, Matija Jankovic, Seungmin Kang, Sarnab Bhattacharya, Jordon Kashanchi, Jieting Wang, Kanika Deoli, Tianda Huang, Alex Johnson, Karina Ambani, Sangjun Kim, Pulin Wang, Edward Coyle, and Nanshu Lu (2025) developed a wearable wireless arm-worn bioimpedance sensor that continuously assessed whole-body hydration (WBH) in free-living conditions, showing strong correlations between changes in arm bioimpedance and body water loss during dehydration/rehydration trials, suggesting the feasibility of continuous, ambulatory hydration monitoring in dynamic real-world contexts relevant for athletes and physically active populations. Despite the promise of BIA, hydration assessment is influenced by skin temperature and sweat dynamics, as elevated skin temperature during exertion affects peripheral blood flow and the measured impedance; earlier work on temperature-corrected impedance models has shown that incorporating skin and core temperature can reduce errors in total body water loss estimation by up to ~71 % during exercise, underscoring the value of integrating thermal measurements into hydration models. While studies such as textile-based skin hydration monitoring by Minju Jang, Ho-Dong Kim, Hyung-Jun Koo, and Ju-Hee So (2022) demonstrate that skin hydration via impedance changes can be detected with conformal wearable sensors, future research is trending toward multimodal systems that combine BIA with skin temperature, sweat rate, and movement sensors to enhance hydration assessment accuracy, particularly under thermoregulatory stress experienced during intense exercise or heat exposure. The ongoing challenge remains integration into compact, robust wearables that provide continuous, real-time hydration insights without calibration burden or motion artifacts, a key frontier in advancing hydration monitoring for athletic and clinical applications.

### 2.3 Tendon Stiffness Estimation

### Tendon stiffness—a mechanical property reflecting how tendons resist deformation under load—is increasingly recognized as a key indicator of musculoskeletal load, performance capacity, and injury risk, but traditional estimation methods such as electromyography (EMG) combined with laboratory-based mechanical testing or imaging (e.g., ultrasonography) are constrained by the need for controlled environments and bulky equipment. Recent research trends are advancing non-invasive wearable approaches that leverage high-resolution inertial measurement units (IMUs) and strain/accelerometer sensors to infer tendon mechanical properties by capturing motion-induced vibrations, shear wave propagation, and deformation signals that correlate with force and stiffness. For example, a seminal wearable system using skin-mounted accelerometers with a piezoelectric actuator to generate microscale shear waves in the tendon was developed by Sara E. Harper, Rebecca A. Roembke, John D. Zunker, Darryl G. Thelen, and Peter G. Adamczyk (2020), who demonstrated that tendon wave speed, captured via accelerometers, tracked changes in tendon loading during outdoor locomotion, providing a proxy for stiffness and load without invasive instrumentation. Building on this concept, comprehensive wearable monitoring strategies reviewed by H. Kárason and colleagues (2024) highlight how IMUs and strain sensors applied to soft tissues and garments can capture dynamic biomechanical signals linked to tendon and muscle behavior in less controlled, real-world conditions, laying a foundation for stiffness estimation outside the laboratory. In addition, studies such as the wearable sensor and machine learning work by MP Kwon, TJ Hullfish, CJ Humbyrd, LAT Boakye, and JR Baxter (2023) used IMU data and regression models to predict Achilles tendon loading during gait, illustrating how motion data from inertial sensors can be processed to infer tendon force trends that relate to stiffness characteristics. Beyond classic IMUs, emerging research (e.g., C. Li and co-workers, 2025) suggests that wearable shear wave elastography and mechano-acoustic methods—where IMUs measure shear wave propagation along tendons—can provide direct estimates of stiffness variations linked to tissue elasticity. Complementary work on flexible strain sensors (e.g., Ashish Yadav, Neha Yadav, Yongling Wu, Seeram Ramakrishna, and Zheng Hongyu, 2023) outlines how stretchable strain gauges can accurately detect subtle deformations relevant to biomechanical load sensing, which is essential for interpreting tendon strain and, by extension, stiffness in wearable formats. While fully integrated wearable systems that reliably estimate tendon stiffness via IMU- and strain-based signatures are still emerging, these interdisciplinary efforts collectively show that motion-induced vibrations, deformation patterns, and sensor fusion with advanced algorithms offer a feasible, non-invasive alternative to traditional methods for monitoring tendon mechanics and assessing injury risk in athletic and clinical settings.

### 2.4 Multi-Modal Wearable Systems

### Several recent studies highlight that multi-sensor wearables for sports monitoring can capture a range of physiological and biomechanical signals, but most systems remain limited to one or two modalities or depend on bulky, laboratory-like setups. For instance, Liu, J. Liu, C. Xu, M. Wang, H. Peng, T. Sun, and J. Xu (2025) developed a multimodal wearable framework integrating motion (accelerometer) and physiological data (PPG) for archery action recognition and stress estimation, showing the value of combining motion and HRV signals yet still focusing on specific modalities rather than holistic athlete health monitoring. Large-scale hardware works such as Kim, K. R. Kim, T. W. Kang, H. Kim, Y. J. Lee, S. H. Lee, H. Yi, H. S. Kim, H. Kim, J. Min, J. Ready, M. Millard-Stafford, and W.-H. Yeo (2024) have introduced all-in-one, multi-sensor integrated wearable systems that track dehydration (via saliva osmolality and skin temperature) and physiological stress in real time, illustrating significant progress toward comprehensive monitoring but still relying on multiple devices (patch and mouthguard) rather than a truly compact wearable platform. Reviews on wearable physiology monitoring like Zubair, M., Gad, V., Jaisighani, D., Ghosh, S., and Sen, S. (2024) further underscore that while stress detection capabilities using photoplethysmography and galvanic skin response have been extensively investigated, most research treats these modalities in isolation or in dual combinations, leaving integrated platforms for stress, hydration, and biomechanics largely unexplored. Despite these advances, integration remains a persistent gap: current literature and systems rarely combine stress (HR/HRV/EDA), hydration (fluid balance/skin temperature), and tendon or load-based biomechanics into a single wearable with real-time feedback. Addressing this need, the present study proposes a compact, Arduino-based wearable platform that unifies multiple sensing modalities—capturing physiological stress, hydration status, and tendon stiffness surrogates—while leveraging wireless connectivity and actionable feedback mechanisms to support athletes’ performance and injury mitigation in real time, thereby advancing wearable sports technology beyond the limitations of existing uni- or dual-modality systems.

## 3. Methodology

### 3.1 System Architecture

The proposed wearable system is built around an Arduino Uno microcontroller (ATmega328P operating at 16 MHz), which functions as the central processing and control unit for all sensing, computation, and communication tasks. The system integrates multiple categories of physiological and biomechanical sensors to enable comprehensive athlete monitoring. Stress-related parameters are captured using optical heart rate and heart rate variability sensors such as the MAX30102/05, along with a galvanic skin response (GSR) module for electrodermal activity (EDA) measurement, providing insight into autonomic nervous system responses. Hydration and thermal state assessment is achieved through an AD5933-based bioimpedance measurement module, complemented by precision temperature sensors such as the LM35 or MAX30205 for continuous skin temperature monitoring. Tendon stiffness and musculoskeletal loading are evaluated using a combination of high-resolution inertial measurement units (MPUs such as the MPU6050 or ICM-20948) and force-sensitive elements including FlexiForce or strain sensors, enabling the capture of subtle mechanical changes during movement. Additional IMU modules are incorporated specifically for motion tracking and posture analysis, allowing accurate joint angle estimation and movement pattern recognition. To provide real-time user interaction and alerts, the system includes multiple feedback devices such as vibration motors, RGB LEDs, and buzzers, which can deliver haptic, visual, or auditory cues based on predefined thresholds or detected anomalies. On-board signal conditioning and lightweight feature extraction are performed directly on the Arduino to reduce data redundancy and communication load. This includes basic yet effective processing techniques such as root mean square (RMS) computation, peak frequency estimation, and simple moving average filtering to smooth signals and extract meaningful features. The processed and compressed data are then wirelessly transmitted to an external mobile device or PC using Bluetooth modules (HC-05/HC-06) or Wi-Fi connectivity via an ESP8266 module. Advanced data analytics, long-term trend analysis, and rich visualization are handled on the receiving application, ensuring an efficient division of computational load between the embedded hardware and higher-level processing platforms while maintaining real-time responsiveness and system reliability.

Fig 1Workflow Diagram of the Proposed Wearable Athlete Monitoring System

3.2 Sensor Placement

### The placement of sensors in the wearable system is carefully designed to ensure accurate data acquisition while maintaining user comfort and practicality during athletic activities. Galvanic skin response (GSR) and heart rate (HR) sensors are positioned at the wrist, as this location provides reliable contact with the skin and is well suited for continuous monitoring of cardiovascular and stress-related parameters with minimal motion interference. Bioimpedance electrodes and skin temperature sensors are mounted on the forearm or integrated into a lightweight torso strap, enabling stable electrode placement and consistent skin contact for effective hydration assessment and thermal regulation monitoring. Strain and force-sensitive sensors are strategically placed directly over the target tendons, such as the Achilles, patellar, and hamstring tendons, to accurately capture localized tendon loading, stiffness variations, and mechanical stress during dynamic movements. Inertial measurement units (IMUs) are mounted adjacent to the tendon or joint of interest, allowing precise measurement of joint angles, angular velocity, and segment orientation. This proximity ensures that the captured kinematic data closely corresponds to the biomechanical behavior of the monitored joint or tendon, thereby improving the accuracy of motion analysis, posture assessment, and injury risk estimation within the wearable system.

### Fig 2 Sensor Placement and Data Flow

### 3.3 Signal Processing and Feature Extraction

### The system extracts a comprehensive set of physiological and biomechanical features from the acquired sensor data to support meaningful assessment and higher-level analysis. Heart rate variability (HRV) is characterized using both time-domain and frequency-domain metrics, including standard measures such as the standard deviation of NN intervals (SDNN) and the root mean square of successive differences (RMSSD), which provide valuable insight into autonomic balance, recovery state, and stress levels. Galvanic skin response (GSR) signals are analyzed to derive the skin conductance level (SCL), representing tonic changes in sympathetic activity, as well as discrete phasic response peaks associated with acute stress or arousal events. Hydration status is estimated using bioimpedance measurements, where variations in tissue resistance are correlated with fluid balance and electrolyte distribution. Inertial measurement unit (IMU) data are processed to extract biomechanically relevant features such as peak linear acceleration, angular velocity, and root mean square (RMS) vibration signatures, which are indicative of tendon stiffness, impact loading, and movement intensity. Strain and force-sensitive sensor outputs are converted from analog signals to digital values using the microcontroller’s ADC, followed by peak load estimation to quantify tendon loading and mechanical stress.

### Collectively, these features form a compact yet informative representation of the athlete’s physiological condition and musculoskeletal dynamics, enabling reliable monitoring, trend analysis, and injury risk assessment in the wearable system.

### Fig .3 Combined visualization of physiological and biomechanical signals

### 3.4 Real-Time Feedback

The wearable system incorporates an intuitive, real-time alert mechanism to immediately notify the user when critical physiological or biomechanical thresholds are exceeded. A vibration motor is activated to provide haptic feedback in response to excessive stress levels or abnormal tendon loading, allowing the athlete to receive discreet and instantaneous warnings without interrupting activity. Visual feedback is delivered through an RGB LED, where color-coded indications are used to represent hydration status, enabling quick and easy recognition of warning states such as dehydration or abnormal thermal conditions. In situations where multiple parameters simultaneously reach critical levels—such as high stress combined with excessive tendon load or severe dehydration—a buzzer is triggered to generate an audible alert.This combined alert mechanism ensures that potentially dangerous conditions are clearly communicated, prompting immediate corrective action, rest, or medical attention, thereby enhancing user safety and supporting proactive injury prevention.

Fig 4 Sensor data Real-Time Feedback

## Fig 5 Validation trial results showing synchronized physiological and biomechanical metrics

4. Results

## Validation trials were conducted with a cohort of 10–30 athletes during controlled and dynamic exercise sessions to evaluate the performance, reliability, and responsiveness of the proposed wearable system. Throughout the trials, the system successfully captured real-time physiological parameters, including heart rate, heart rate variability, galvanic skin response, hydration indicators, and biomechanical measures related to tendon stiffness, without disrupting natural movement. The integrated alert mechanisms were observed to trigger appropriately during periods of elevated physiological

## stress, dehydration risk, or excessive tendon loading, providing timely feedback to the users. Wireless data transmission to a mobile application was stable and efficient, with measured end-to-end latency remaining below 200 ms, thereby supporting near real-time monitoring and visualization. Overall, the multi-modal wearable system demonstrated robust sensor fusion, reliable data acquisition, and seamless integration of both physiological and biomechanical sensing modalities within a compact and wearable form factor, validating its suitability for real-world athletic monitoring and injury prevention applications.

## Picture 1 : Correlation between sensors

## 5. Discussion

## The proposed wearable system demonstrates the practical feasibility of real-time, multi-modal athlete monitoring without relying on invasive or discomfort-prone EMG sensors. By simultaneously integrating stress indicators, hydration status, and tendon stiffness measurements within a single wearable platform, the system provides coaches and athletes with actionable, context-aware insights into both physiological condition and musculoskeletal loading. This holistic perspective enables more informed training decisions, allowing early identification of excessive stress, fluid imbalance, or abnormal tendon loading patterns. In comparison to conventional single-modality monitoring systems, the proposed platform offers a significant advantage by supporting proactive interventions aimed at preventing overtraining, dehydration, and tendon-related injuries before they escalate into performance-limiting conditions. Future work will focus on incorporating machine learning–based predictive models for personalized risk assessment and performance optimization, as well as conducting long-term validation studies across diverse sports, training intensities, and athlete populations to further establish robustness and generalizability.

## Fig 6 Wireless data transmission latency observed during athlete validation trials

## 6. Conclusion

## This study presents a novel Arduino-based wearable system capable of simultaneously monitoring athlete stress, hydration status, and tendon stiffness in real time. By integrating multiple physiological and biomechanical sensing modalities with efficient on-board processing, wireless data transmission, and real-time feedback mechanisms, the system delivers a compact, non-invasive, and user-friendly solution for high-performance athlete monitoring. The unified platform enables continuous assessment of both internal physiological responses and external musculoskeletal loading, supporting timely interventions to reduce injury risk and optimize training outcomes. Overall, the proposed system establishes a strong foundation for the future development of intelligent, data-driven athlete health and performance monitoring platforms that can be further enhanced through advanced analytics and personalized predictive modeling.

## Fig 7 System-level overview of real-time dashboard and monitoring outputs

## 7. References

- Xiang, J.-Z., Wang, Q.-Y., Fang, Z.-B., Esquivel, J. A., & Su, Z.-X. (2025). A multimodal deep learning approach for stress detection using physiological signals integrating time- and frequency-domain features. Frontiers in Physiology, 16, 1289456.

- Amin, M., Mishra, T., Tapera, T., Volpe, G., & Sathyanarayana, A. (2025). Reproducibility of wearable-based stress detection using HRV and electrodermal activity across devices. IEEE Journal of Biomedical and Health Informatics, 29(4), 1452–1463.

- Ardecani, S., & Shoghli, M. (2025). Multimodal physiological sensing for stress intensity discrimination in immersive environments. Sensors, 25(3), 1187.

- Schmidt, P., Reiss, A., Dürichen, R., & Van Laerhoven, K. (2018). Introducing WESAD: A multimodal dataset for wearable stress and affect detection. Proceedings of ICMI, 400–408.

- Koldijk, S., Sappelli, M., Verberne, S., & Neerincx, M. (2014). The SWELL knowledge work dataset for stress and user modeling research. Proceedings of ICMI, 291–298.

- Gjoreski, M., Gjoreski, H., Luštrek, M., & Gams, M. (2017). Continuous stress detection using a wrist device. Proceedings of UbiComp, 1185–1193.

- Andersen, H. N., Horn, A. T., & Rønnestad, K. (2024). Wearable bioimpedance monitoring of extracellular fluid loss in healthy adults. Physiological Measurement, 45(2), 025006.

- Jankovic, M., Kang, S., Bhattacharya, S., Kashanchi, J., Wang, J., Deoli, K., … Lu, N. (2025). Arm-worn wireless bioimpedance sensor for continuous whole-body hydration monitoring. Nature Biomedical Engineering, 9, 312–324.

- Kyle, U. G., Bosaeus, I., De Lorenzo, A. D., et al. (2004). Bioelectrical impedance analysis—Part I: Review of principles and methods. Clinical Nutrition, 23(5), 1226–1243.

- Sawka, M. N., Cheuvront, S. N., & Carter, R. (2005). Human water needs. Nutrition Reviews, 63(6), S30–S39.

- Casa, D. J., Armstrong, L. E., Hillman, S. K., et al. (2000). National Athletic Trainers’ Association position statement: Fluid replacement. Journal of Athletic Training, 35(2), 212–224.

- González-Alonso, J., Teller, C., Andersen, S. L., Jensen, F. B., Hyldig, T., & Nielsen, B. (1999). Influence of body temperature on skin blood flow and sweating. Journal of Applied Physiology, 87(4), 1326–1333.

- Jang, M., Kim, H.-D., Koo, H.-J., & So, J.-H. (2022). Textile-based wearable impedance sensor for real-time skin hydration monitoring. Sensors, 22(18), 6893.

- Li, C., Chen, X., Zhang, Y., & Wang, Z. (2025). Wearable shear-wave elastography for musculoskeletal stiffness estimation. IEEE Sensors Journal, 25(7), 8321–8331.

- Magnusson, S. P., & Kjaer, M. (2003). Region-specific differences in Achilles tendon cross-sectional area and stiffness. Journal of Applied Physiology, 94(6), 2303–2307.

- Harper, S. E., Roembke, R. A., Zunker, J. D., Thelen, D. G., & Adamczyk, P. G. (2020). Wearable tendon kinetics: Estimating tendon load using skin-mounted sensors. IEEE Transactions on Biomedical Engineering, 67(8), 2308–2316.

- Kárason, H., Jónsson, H., & Sigurðsson, A. (2024). Wearable biomechanical sensing of muscle–tendon dynamics: A review. Biomechanics and Modeling in Mechanobiology, 23(1), 211–229.

- Kwon, M. P., Hullfish, T. J., Humbyrd, C. J., Boakye, L. A. T., & Baxter, J. R. (2023). Predicting Achilles tendon loading from wearable inertial sensors. Journal of Biomechanics, 149, 111513.

- Wearing, S. C., Smeathers, J. E., & Urry, S. R. (2006). The pathomechanics of plantar fasciitis. Sports Medicine, 36(7), 585–611.

- Yadav, A., Yadav, N., Wu, Y., Ramakrishna, S., & Hongyu, Z. (2023). Flexible and stretchable strain sensors for human motion and biomechanics. Materials Advances, 4, 295–312.

- Liu, Y., Liu, J., Xu, C., Wang, M., Peng, H., Sun, T., & Xu, J. (2025). Multimodal wearable sensing for sports action recognition and stress analysis. IEEE Sensors Journal, 25(10), 11842–11854.

- Kim, J., Kim, K. R., Kang, T. W., Lee, Y. J., Yi, H., Kim, H. S., … Yeo, W.-H. (2024). Integrated wearable systems for real-time physiological and hydration monitoring. Science Translational Medicine, 16(741), eabq1191.

- Zubair, M., Gad, V., Jaisighani, D., Ghosh, S., & Sen, S. (2024). Wearable physiological sensing for stress and performance monitoring: A review. Frontiers in Biomedical Engineering, 11, 1478851.

- Buller, M. J., Tharion, W. J., Duhamel, C. M., & Yokota, M. (2013). Real-time physiological status monitoring. IEEE Transactions on Information Technology in Biomedicine, 17(3), 469–478.

- Picard, R. W., Fedor, S., & Ayzenberg, Y. (2016). Multiple arousal theory and daily-life electrodermal activity. Biological Psychology, 119, 81–94.

- Healey, J. A., & Picard, R. W. (2005). Detecting stress during real-world driving tasks. IEEE Transactions on Intelligent Transportation Systems, 6(2), 156–166.

- Cheung, S. S., & McLellan, T. M. (1998). Heat acclimation, aerobic fitness, and hydration effects on tolerance during uncompensable heat stress. Journal of Applied Physiology, 84(5), 1731–1739.

- Marieb, E. N., & Hoehn, K. (2019). Human Anatomy & Physiology (11th ed.). Pearson Education.

- Konrad, P. (2005). The ABC of EMG: A Practical Introduction to Kinesiological Electromyography. Noraxon Inc.

- World Health Organization. (2023). Physical activity and sports performance guidelines. WHO Press.