GIS Data Quality Challenges in the Transition to Next Generation 9-1-1:

A Critical Review of Standards, Implementation Gaps,

and Remediation Strategies for Rural Jurisdictions

(REVIEW ARTICLE)

# Abstract

The transition from Enhanced 9-1-1 (E9-1-1) to Next Generation 9-1-1 (NG9-1-1) replaces tabular database lookups with geospatial queries that depend on accurate GIS-based address point and road centerline data. This shift promises improvements in emergency response, but its success depends on the quality of local GIS datasets. Rural jurisdictions face steep challenges: limited staffing, aging address data, incomplete MSAG synchronization, and insufficient maintenance resources. This review synthesizes the academic and practitioner literature on GIS data quality in the NG9-1-1 context, examining four themes: (1) technical requirements of the NENA GIS Data Model Standard; (2) documented patterns of data quality deficiencies; (3) structural barriers in rural counties; and (4) remediation strategies. Five mathematical metrics are presented for quantifying data quality: Root Mean Square Error for positional accuracy, completeness ratio, attribute deficiency rate, MSAG match rate, and misrouting probability. A four-phase framework for systematic data quality assessment is proposed, combining automated validation, MSAG–GIS cross-validation, risk-prioritized remediation, and ongoing quality monitoring. The review finds that the rural data quality gap is fundamentally a staffing and resource problem rather than a technical one, and that state-level coordination programs offer the most promising path forward. Recommendations for policy, practice, and future research.

Keywords: Next Generation 9-1-1, GIS data quality, address point data, road centerline, NENA standards, rural jurisdictions, MSAG, public safety GIS

# 1. Introduction

When someone in a rural community calls 911, a chain of technical processes determines whether help arrives in time. Under legacy E9-1-1, call routing relied on tabular databases, the Master Street Address Guide (MSAG), and Automatic Location Identification (ALI) records tied to wireline infrastructure (Federal Communications Commission [FCC], 2016). That architecture was designed for a telecommunications environment that no longer exists. Roughly 80% of 911 calls now originate from wireless devices (National Emergency Number Association [NENA], 2021).

NG9-1-1 replaces this with a geospatial approach. Every call is located by querying GIS data against the caller’s coordinates and routed to the correct Public Safety Answering Point (PSAP) using spatial boundaries (NENA, 2022b). As shown in Figure 1, this architectural shift means that GIS data quality serves as the operational foundation of emergency response. The FCC adopted location-based routing requirements for wireless 911 calls in 2024 (FCC, 2024), and the Geospatial Data Act of 2018 formalized federal coordination of authoritative geospatial datasets (USDOT, 2023). However, the readiness of local GIS data remains deeply uneven.

Advances in geospatial technology have expanded GIS capabilities for public safety. The integration of machine learning with GIS and remote sensing has shown promise in flood prediction and disaster management, where spatial data quality is essential (Gyang et al., 2025). However, even as these capabilities grow, the quality of the underlying data determines whether they produce reliable results. Urban counties maintain dedicated GIS departments with larger staffs and frequent update cycles. Rural counties often have no dedicated GIS staff, and the absence of dedicated GIS personnel carries measurable costs: misrouted emergency calls, delayed response times, failed address validations, and noncompliance with federal NG9-1-1 mandates that can jeopardize funding eligibility. Without trained personnel to maintain and update address point and road centerline data, errors accumulate and go undetected, compounding over time. This review addresses these challenges directly by synthesizing the literature around four themes: (a) NENA’s GIS data model requirements; (b) documented deficiency patterns; (c) structural barriers in rural jurisdictions; and (d) remediation strategies. By identifying practical, low-cost quality assurance approaches and proposing a phased framework tailored to limited-resource settings, this review equips rural counties and state 911 coordinators with actionable guidance. It concludes with recommendations for policy and research.

Figure 1. Comparison of E9-1-1 and NG9-1-1 call routing architectures. Under E9-1-1 (a), calls are routed via tabular MSAG/ALI lookups. Under NG9-1-1 (b), calls are routed through the ESInet using GIS-based spatial queries via the ECRF and LVF—source: Adapted from NENA-STA-010.3a (2022).

# 2. Review Methodology

This review follows a structured narrative synthesis approach. Literature searches were conducted across Web of Science, Scopus, Google Scholar, and the NENA Knowledge Base using terms including “GIS data quality,” “address point accuracy,” “NG9-1-1,” “NENA standards,” and “rural addressing.” Searches covered English-language publications from 2000 to 2025. NENA standards, FCC orders, and National 911 Program reports were obtained from organizational websites. State-level documents from Missouri, Tennessee, Vermont, and Georgia provided illustrative examples. Table 1 summarizes the 72 sources included.

Table 1: Summary of Sources Included in the Review

| Source Type | Count | % |
| --- | --- | --- |
| Peer-reviewed journal articles | 34 | 47.2 |
| NENA standards/information documents | 12 | 16.7 |
| Government reports (federal/state) | 14 | 19.4 |
| Conference proceedings/practitioner | 8 | 11.1 |
| Books and book chapters | 4 | 5.6 |

# 3. NENA GIS Data Model Requirements

The technical foundation for NG9-1-1 readiness is NENA-STA-006.2-2022, which specifies required attribute fields for Site/Structure Address Points (SSAPs), Road Centerlines, PSAP Boundaries, and Emergency Service Boundaries (NENA, 2022b). Table 2 lists the key required attributes.

## 3.1 Required Attribute Schema

A single address point record may require over 20 populated fields for compliance. Road centerlines require additional attributes, as address ranges, parity, and other fields must be defined independently for both sides of each road segment. For a county with tens of thousands of address points, maintaining compliance requires sustained data governance.

Table 2: Key Required Attributes for NENA-Compliant Address Points and Road Centerlines

| Address Point Attributes | Road Centerline Attributes |
| --- | --- |
| Address Number (Add_Number) | Left/Right From Address |
| Street Name (St_Name) | Left/Right To Address |
| Pre-Directional / Post-Type | Left/Right Parity |
| Community / Municipality | Street Name Components (full set) |
| State, Postal Code, County | Left/Right Community, State |
| Placement Method | Valid Linear Geometry (Polyline) |

Note. Adapted from NENA-STA-006.2-2022. Shows required fields only.

## 3.2 Location Validation and Call Routing Functions

The Location Validation Function (LVF) checks whether a caller’s location falls within a known service boundary. If GIS data contains gaps, the LVF may fail to validate a precise location, producing a routing failure. The Emergency Call Routing Function (ECRF) determines which PSAP should receive a call based on spatial coordinates. Standardising address records so that they align with NG9-1-1 data model requirements is an essential prerequisite for this routing to succeed (1Spatial, 2025). A National 911 Program white paper warned that using substandard GIS data, such as Census TIGER files, for routing would likely cause a regression in location accuracy, noting that TIGER’s positional errors averaged 167 feet nationally and nearly 2 miles for rural geocoding (USDOT, 2020).

# 4. Documented Data Quality Deficiencies

This section examines positional accuracy and attribute completeness, the two quality dimensions most critical to NG9-1-1.

## 4.1 Positional Accuracy

The standard metric for positional accuracy is Root Mean Square Error:

RMSE =  1n∑ᵢ dᵢ2        #1

where dᵢ is the distance between the geocoded and true location for address i, and n is the total number of addresses evaluated (Zandbergen, 2008). Cayo and Talbot (2003) found a mean rural error of 614 m, compared with 58 m in urban areas. At the 95th percentile, rural addresses geocoded within 2,872 m, placing a responder in the wrong neighborhood. Ward et al. (2005) found that only 56% of rural addresses had errors of less than 100 m. Chow et al. (2016) compared eight platforms and found parcel-based geocoding to be the most accurate (mean 24.8 m), but the urban–rural gap persisted. Table 3 consolidates these metrics across urban and rural contexts; Figure 2 summarizes the study findings, and Figure 3 models the cumulative error distributions. The updated NG9-1-1 GIS data model standard reinforces that address-point-level accuracy is non-negotiable for location validation and call routing (NENA, 2026b), a requirement rural jurisdictions frequently struggle to meet given the documented positional error differentials.

Table 3: Comparison of Key GIS Data Quality Metrics: Urban vs. Rural Jurisdictions

| Metric | Urban | Rural |
| --- | --- | --- |
| Mean positional error (RMSE) | ~58 m | ~614 m |
| 95th percentile error | <200 m | ~2,872 m |
| Address completeness rate | >97% (typical) | 70–85% (documented gaps) |
| Dedicated GIS staffing | 2–5 FTEs (typical) | 0–1 FTE (often part-time) |
| Data update cycle | Continuous / monthly | Annual or ad hoc |

Note. FTE = full-time equivalent. RMSE values from Cayo and Talbot (2003). Completeness and staffing figures synthesized from state-level reports and USDOT (2023).

Figure 2. Mean positional errors by area type from four studies. The dashed line marks the approximate NG9-1-1 routing tolerance (~50 m). Source: Synthesized from Cayo and Talbot (2003), Bonner et al. (2003), Ward et al. (2005), and Chow et al. (2016).

For NG9-1-1, the misrouting probability near jurisdictional boundaries can be expressed as:

Pmisroute= Pdᵢ > Dₘₗₙ× Paddress near boundary         #2

where D min   is the distance to the nearest service boundary, Zandbergen (2009) showed that even 50 m errors can cause misrouting near boundaries. This compounding effect is particularly acute for wireless 911 calls: device-reported GPS coordinates carry their own uncertainty, which is then added to address-point positional error before routing decisions are made (Esri, 2025).

Figure 3. Modeled cumulative distribution of positional errors by area type. At 50 m, over 60% of urban but fewer than 20% of rural addresses fall within acceptable error. Source: Based on distributions from Cayo and Talbot (2003) and Bonner et al. (2003).

## 4.2 Attribute Accuracy and Completeness

Completeness is expressed as:

C = NactualNexpected× 100%          #3

Where Nactualis the GIS address count, and Nexpected is the total addressable locations. Rates below 95% are problematic for routing. Common deficiencies include missing postal codes, inconsistent directionals, null community names, and absent placement methods. The attribute deficiency rate is:

ADR = 1N∑ᵢ MᵢF× 100%          #4

where Mᵢ is the number of missing fields in record i and F is the total required fields. Figure 4 shows the most common deficiency types, and Figure 5 presents the six quality dimensions relevant to NG9-1-1. Table 4 summarizes the key studies.

Figure 4. Most commonly reported address point deficiencies in NG9-1-1 readiness assessments—source: Synthesized from state-level reports.

Figure 5. Dimensions of GIS data quality relevant to NG9-1-1—source: Adapted from Devillers and Jeansoulin (2006) and NENA-STA-006.2 (2022).

Table 4: Key Studies on Address Data Quality Relevant to NG9-1-1

| Study | Focus | Key Finding | NG9-1-1 Relevance |
| --- | --- | --- | --- |
| Zandbergen (2008) | Geocoding method comparison | Address points most accurate | Routing depends on point quality |
| Cayo & Talbot (2003) | Error by density | 614 m rural vs. 58 m urban | Rural errors exceed tolerance |
| Ward et al. (2005) | Rural geocoding | 56% rural < 100 m | Higher routing risk |
| Chow et al. (2016) | Multi-platform | Best: 24.8 m (parcel) | Platform choice matters |

Note. Studies listed chronologically. All findings are relevant to positional or attribute quality in the NG9-1-1 context.

# 5. Structural Barriers in Rural Jurisdictions

The challenges facing rural jurisdictions are structural, not merely technical. Solutions that assume urban levels of staffing and funding will not transfer. Rural counties must compete for limited budgets against immediate demands such as road maintenance and law enforcement, leaving GIS programs chronically underfunded. A national readiness analysis found that the vast majority of rural counties face significant data preparation gaps ahead of the FCC’s NG9-1-1 transition mandate (GeoComm, 2025). Many lack access to formal GIS training programs, and cloud-based collaboration tools that urban jurisdictions rely on for real-time data sharing. Broadband connectivity is an additional structural constraint: cloud-based GIS editing platforms and real-time data submission pipelines require reliable high-speed internet, which remains unavailable in many rural areas, complicating adoption of state-led collaborative programs (Federal Communications Commission, 2025). For instance, ArcGIS Enterprise solutions that create an enabling environment for GIS data editing, storing, analysis, and sharing.

## 5.1 Staffing and Institutional Capacity

The National 911 Program identified hiring and retaining qualified GIS staff as a widespread challenge, particularly acute in rural areas where salaries cannot compete with private sector or urban government pay (USDOT, 2023). When a single GIS employee departs, accumulated institutional knowledge may be lost. Severinsen et al. (2019) noted this knowledge dependency as one of the most underappreciated risk factors in local government GIS. Figure 6 illustrates the capacity gap.

Figure 6. GIS infrastructure capacity comparison between urban and rural counties. Source: Based on USDOT (2023).

## 5.2 Legacy Data and MSAG–GIS Synchronization

Many rural counties converted from route-and-box addressing relatively recently. Zimmerman and Li (2010) found that address points were frequently placed at road intersections rather than at structures. The MSAG–GIS synchronization challenge is critical: under E9-1-1, the MSAG was the routing authority; under NG9-1-1, the GIS replaces it. However, the two systems often evolve independently. The match rate provides a key metric:

Match Rate = NmatchNMSAG× 100%          #5

where Nmatchis the number of MSAG street name and range entries that correspond to valid GIS road centerline records, and NMSAG is the total number of MSAG entries. NENA recommends a minimum 98% match rate before transitioning to GIS-based routing (NENA, 2021).

Figure 7 illustrates this synchronization process.

Figure 7. MSAG–GIS data synchronization process. The cross-validation engine compares tabular MSAG records against spatial GIS road centerline data—source: Authors’ conceptualization.

# 6. Remediation Strategies

Approaches to improving GIS data quality vary in their applicability to rural NG9-1-1 readiness. Table 5 compares five strategies, and Figure 8 provides a visual assessment.

## 6.1 Automated Validation

Automated validation scripts represent the most accessible starting point for resource-constrained jurisdictions. NENA supports validation tools, including the GIS Data Model Validation Toolkit. Commercial platforms such as ArcGIS Pro include data reviewer extensions. Several states have deployed automated validation in statewide aggregation pipelines (Tennessee Emergency Communications Board, 2021). In Missouri, the 911 Service Board uses the DATAMARK Validation and Enhancement Platform (VEP) to validate GIS records on a monthly cycle; county GIS personnel submit their address point and road centerline data, and the platform flags discrepancies that are then rectified before the next submission. These tools efficiently check thousands of records but cannot assess positional accuracy.

## 6.2 Field Verification and Imagery-Based Review

Field verification with survey-grade GPS remains the gold standard for positional accuracy, but is resource-intensive. Imagery-based desktop review using NAIP orthophotography offers a middle ground: address points compared against aerial imagery without field visits, though accuracy depends on imagery currency.

## 6.3 Collaborative Editing and State Programs

Cloud-based GIS platforms enable multi-agency collaborative editing. Several states use hub-and-spoke models with central validation and county-level editors. The National 911 Program advocates creating GIS data supply chains that deliver accurate data into NG9-1-1 as quickly as possible (USDOT, 2023).

Figure 8. Comparative assessment of five remediation approaches. Automated validation and state QA/QC programs offer the best combination of effectiveness and applicability in rural areas. Source: Authors’ assessment based on reviewed literature.

Table 5: Comparison of Remediation Approaches

| Approach | Strengths | Limitations | Rural Fit |
| --- | --- | --- | --- |
| Automated validation | Fast, scalable | No positional check | High |
| Field verification | Gold standard | Labor intensive | Low–Mod |
| Imagery review | No field visits | Imagery currency | High |
| Cloud collaborative | Real-time | Internet, licensing | Moderate |
| State QA/QC | Economies of scale | State commitment | High |

Note. Rural Fit is assessed on a scale from Low to High based on staffing, cost, and infrastructure requirements. —source: Authors’ conceptualization based on reviewed literature.

# 7. Proposed Quality Assessment Framework

Drawing on the patterns identified above, this section proposes a four-phase framework designed for rural jurisdictions and state 911 coordinators (Figure 9, Table 6).

Phase 1 (Automated Baseline Assessment) runs validation checks against NENA-STA-006.2 and quantifies the attribute deficiency rate using Equation 4. Phase 2 (MSAG–GIS Cross-Validation) compares road centerlines against MSAG records using fuzzy string matching and range comparison, producing a categorized discrepancy report. Phase 3 (Risk-Prioritized Remediation) corrects errors near PSAP boundaries and critical facilities first, ensuring that limited staff time yields the maximum routing improvement. Phase 4 (Ongoing QA/QC) repeats validation quarterly, tracking metrics, completeness (Eq. 3), ADR (Eq. 4), MSAG match rate (Eq. 5), and RMSE (Eq. 1), on a quality dashboard.

Figure 9. Proposed a four-phase GIS data quality framework with a continuous feedback loop—source: Authors’ conceptualization based on reviewed literature.

Table 6: Summary of the Proposed Four-Phase Framework

| Phase | Activity | Output | Tools |
| --- | --- | --- | --- |
| 1 | Attribute validation vs. NENA-STA-006.2 | Baseline deficiency report | ArcGIS Pro, Python |
| 2 | MSAG–GIS cross-validation | Discrepancy report | Python fuzzy matching |
| 3 | Risk-prioritized correction | Corrected datasets | ArcGIS Pro/Online, GPS |
| 4 | Recurring QA/QC (quarterly) | Quality dashboard | Phase 1 tools + dashboard |

Note. Framework designed for scalability across jurisdictions of varying size and capacity. —source: Authors’ conceptualization based on reviewed literature.

## 7.1 Illustrative Application: Phase 1 in a Rural Tennessee County

To ground the four-phase framework in practice, consider a hypothetical rural county in Tennessee with a population of approximately 18,000 and a single part-time GIS technician who also manages IT infrastructure. The county has recently been directed by the Tennessee Emergency Communications Board to submit its address point and road centerline data for statewide NG9-1-1 integration (Tennessee Emergency Communications Board, 2021). This scenario is representative of conditions documented across the state’s rural counties and illustrates what Phase 1 output would look like in a resource-constrained setting.

In Phase 1, the technician runs the NENA GIS Data Model Validation Toolkit against the county’s address point shapefile, which contains 9,240 records. The tool reports the following: (a) 6.3% of records are missing a postal code (580 records); (b) 4.1% have null or inconsistent community names (379 records); (c) 2.8% lack a valid placement method attribute (259 records); and (d) 1.2% contain directional inconsistencies between the pre-directional and road name fields (111 records). Applying Equation 4, the county’s attribute deficiency rate is approximately 14.4%, well above the 5% threshold implied by NENA’s 95% completeness standard. Applying Equation 3 against parcel and permit records, address completeness stands at 82.7%, meaning roughly 1,900 known habitable structures are absent from the point layer entirely. The Phase 1 output report, completed in approximately three hours using free NENA-provided tools, gives the technician a prioritized list of 1,329 records requiring immediate correction before submission to the state aggregation pipeline. Without Phase 1, these errors would propagate into the statewide routing dataset undetected.

Phase 2 cross-validation against the county’s MSAG reveals a match rate of 91.2%, below NENA’s recommended 98% minimum. Analysis of the discrepancy log identifies 67 road segments where the MSAG street name differs from the GIS centerline name due to informal local usage (e.g., “Co Rd 14” in the MSAG versus “County Road 14” in the GIS). Phase 3 directs the technician to prioritize the 14 segments near the county PSAP and the two hospital access roads, where misrouting risk is highest. Phase 4 schedules a repeat validation in 90 days to confirm that corrections hold after the next scheduled address assignment cycle. This example demonstrates that the framework is executable with existing tools and limited staff time, requiring no new software investment.

# 8. Discussion

This review reveals a landscape where NG9-1-1 standards are well defined, but the capacity to meet them is deeply uneven. The NENA GIS Data Model provides a solid blueprint, and the literature offers reliable measurement methods. What remains missing is the bridge: practical approaches to achieving compliance at the local level.

First, the rural data quality gap is fundamentally a staffing and resource problem. The tools and standards exist; what is lacking is the human capacity to apply them. Any realistic strategy must either shift quality assurance to state-level programs or invest in training that maximizes the limited productivity of county staff.

Second, the absence of standardized county-level benchmarking is a significant gap. Without consistent measurement against NENA requirements, it is not possible to compare readiness across counties or track improvement. Comparable data-driven assessment approaches have proven effective in other public safety domains; for instance, statistical analysis of national surveillance data has been used to identify modifiable risk factors driving health disparities across U.S. communities (Korang et al., 2025). The framework proposed here offers one approach but requires empirical validation.

Third, MSAG–GIS synchronization deserves more attention. As NG9-1-1 expects GIS to assume the MSAG’s routing authority, this gap becomes a critical path issue. More granular measures that distinguish name mismatches, range gaps, and missing records would support targeted remediation.

Fourth, NG9-1-1 funding should explicitly allocate resources to GIS data quality, not only network infrastructure. As GIS data increasingly supports automated decision-making in public safety, data quality becomes inseparable from data governance. Parallel challenges have been documented in healthcare, where AI-driven supply chain optimization depends on accurate, standardized data (Dada et al., 2024), and inequitable access to supply chain resources disproportionately affects underserved communities (Adewumi et al., 2024). Utomi et al. (2024) documented how data protection compliance in the U.S. health sector imposes quality standards on data-driven systems, a parallel that applies to NG9-1-1, where datasets must meet defined accuracy thresholds before routing emergency calls.

## 8.1 Limitations

This review is bound by available published literature, which skews toward urban contexts and public health applications of geocoding; studies examining address data quality specifically in the NG9-1-1 context remain limited. The proposed four-phase framework is conceptual and has not been empirically tested in any jurisdiction: its phased sequence is grounded in best practices documented in the literature and practitioner guidance, but its effectiveness, time requirements, and quality gains have not been measured in a controlled or comparative study. Implementation cost constraints also deserve acknowledgment. Although the framework is designed to use free or low-cost tools, the staff time required for Phases 1 through 3 may be prohibitive in the smallest jurisdictions, where a single technician divides time across multiple responsibilities. Scalability is a further consideration: the framework assumes a county with at least part-time GIS capacity; counties with no dedicated GIS personnel may require state-level intervention before the framework can be applied at all. Future research should prioritize implementation studies in diverse rural settings, including counties with zero dedicated GIS staff, to test feasibility, document actual time investment, and measure improvements in MSAG match rates and address completeness following structured quality interventions.

# 9. Conclusion

The transition to NG9-1-1 is a generational opportunity to improve emergency response. By replacing tabular databases with geospatial queries, NG9-1-1 promises faster and more accurate location identification. However, that promise rests on GIS data that, in many rural jurisdictions, is not yet strong enough.

This review identified four connected challenges: the technical demands of the NENA data model, documented attribute and positional deficiencies, structural barriers in rural counties, and available remediation strategies. The metrics presented, RMSE, completeness ratio, attribute deficiency rate, and MSAG match rate, provide a quantitative vocabulary for tracking improvement. The proposed four-phase framework offers a practical starting point aligned with NENA standards. The people who call 911 in rural America deserve the same expectation of accurate location identification as those in any metropolitan area. Meeting that expectation starts with getting the data right. Future research should focus on three priorities: first, empirical validation of the proposed framework across jurisdictions of varying size and capacity; second, development of interstate GIS data harmonization protocols that reduce inconsistencies at county and state boundaries, where misrouting risk is highest; and third, longitudinal studies tracking MSAG match rates and address completeness before and after structured quality interventions, to build an evidence base that can inform federal and state NG9-1-1 funding decisions.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

# References

- Adewumi, G., Azai, J. S., Okonkwo, F. C., & Misszento, J. A. (2024). Equity in healthcare supply chain access: A narrative review of challenges and strategies in the U.S. International Medical Science Research Journal, 4(11), 996–1004. https://doi.org/10.51594/imsrj.v4i11.1738

- Bonner, M. R., Han, D., Nie, J., Rogerson, P., Vena, J. E., & Freudenheim, J. L. (2003). Positional accuracy of geocoded addresses in epidemiologic research. Epidemiology, 14(4), 408–412.

- Cayo, M. R., & Talbot, T. O. (2003). Positional error in automated geocoding of residential addresses. International Journal of Health Geographics, 2(1), Article 10.

- Chow, T. E., Dede-Bamfo, N., & Dahal, K. R. (2016). Geographic disparity of positional errors and matching rate of residential addresses among geocoding solutions. Annals of GIS, 22(1), 29–42. https://doi.org/10.1080/19475683.2015.1085437

- Coetzee, S., & Bishop, J. (2009). Address databases for national SDI: Comparing the novel data grid approach to data harvesting and federated databases. International Journal of Geographical Information Science, 23(9), 1179–1209. https://doi.org/10.1080/13658810802084806

- Dada, S. A., Korang, A., Umoren, J., & Donkor, A. A. (2024). The role of artificial intelligence and machine learning in optimizing U.S. healthcare supply chain management. World Journal of Advanced Research and Reviews, 24(02), 1996–2002. https://doi.org/10.30574/wjarr.2024.24.2.3343

- Devillers, R., & Jeansoulin, R. (Eds.). (2006). Fundamentals of spatial data quality. ISTE Publishing.

- Federal Communications Commission. (2016). Eighth annual report to Congress on state collection and distribution of 911 and enhanced 911 fees and charges. FCC. https://www.fcc.gov/document/fcc-releases-8th-annual-report-911-fees

- Federal Communications Commission. (2024). Report and order: Location-based routing for wireless 911 calls (FCC 24-4). Federal Register.

- Goldberg, D. W., Wilson, J. P., & Knoblock, C. A. (2007). From text to geographic coordinates: The current state of geocoding. URISA Journal, 19(1), 33–46.

- Guptill, S. C., & Morrison, J. L. (Eds.). (1995). Elements of spatial data quality. Elsevier.

- Gyang, P. A.-M., Akomolafe, O., Panful, B., & Yowetu, I. A. (2025). A review of integrated use of machine learning algorithms, GIS and remote sensing techniques in the prediction of rainfall patterns and floods in the U.S. World Journal of Advanced Engineering Technology and Sciences, 14(01), 159–167. https://doi.org/10.30574/wjaets.2025.14.1.0025

- Korang, A., Ugwu, D., Ajayi, A. A., & Yowetu, I. A. (2025). Investigating the role of modifiable risk factors in cardiovascular disease outcomes: A statistical analysis using BRFSS data. Magna Scientia Advanced Research and Reviews, 13, 170–178.

- National Emergency Number Association. (2021). NENA standard for the next generation 9-1-1 (NG9-1-1) i3 architecture (NENA-STA-010.3a-2021). NENA. https://www.nena.org/page/i3_Standard

- National Research Council. (2007). Successful response starts with a map. The National Academies Press.

- Severinsen, J., de Róiste, M., Reitsma, F., & Hartato, E. (2019). VGTrust: Measuring trust for volunteered geographic information. International Journal of Geographical Information Science, 33(8), 1683–1701. https://doi.org/10.1080/13658816.2019.1572893

- Tennessee Emergency Communications Board. (2021). Tennessee NG9-1-1 GIS data quality program: Annual report. TECB.

- Utomi, E., Osifowokan, A. S., Donkor, A. A., & Yowetu, I. A. (2024). Evaluating the impact of data protection compliance on AI development and deployment in the U.S. health sector. World Journal of Advanced Research and Reviews, 24(02), 1100–1110. https://doi.org/10.30574/wjarr.2024.24.2.3398

- National 911 Program. (2020). Don’t take shortcuts when developing GIS data for NG911 [White paper]. U.S. Department of Transportation. https://www.911.gov/documents-and-tools/dont-take-shortcuts-when-developing-gis-data-for-ng911/

- U.S. Department of Transportation. (2023). Current status of 911 GIS technologies: Final report. National 911 Program.

- Ward, M. H., et al. (2005). Positional accuracy of two methods of geocoding. Epidemiology, 16(4), 542–547.

- Zandbergen, P. A. (2008). A comparison of address point, parcel, and street geocoding techniques. Computers, Environment and Urban Systems, 32(3), 214–232.

- Zandbergen, P. A. (2009). Geocoding quality and implications for spatial analysis. Geography Compass, 3(2), 647–680.

- Zimmerman, D. L., & Li, J. (2010). Effects of local street network characteristics on the positional accuracy of automated geocoding for geographic health studies. International Journal of Health Geographics, 9, Article 10. https://doi.org/10.1186/1476-072X-9-10

- National Emergency Number Association. (2026). NENA standard for NG9-1-1 GIS data model (NENA-STA-006.3-2026). NENA. https://www.nena.org/news/725196/Publication-Announcement-of-NENAs-Standard-for-NG9-1-1-GIS-Data-Model.htm

- GeoComm. (2025). FCC NG9-1-1 order: Progress, challenges, and what’s next for public safety GIS. GeoComm Public Safety Solutions. https://www.geocomm.com/fcc-ng911-order/

- Federal Communications Commission. (2025). Facilitating implementation of next generation 911 services (NG911); location-based routing for wireless 911 calls [Notice]. Federal Register, 90(56), 13556–13557. https://www.federalregister.gov/documents/2025/03/25/2025-04833/

- Esri. (2025). FCC’s new rules propel next-generation 911 forward: What this means for GIS. Esri Industries Blog. https://www.esri.com/en-us/industries/blog/articles/fccs-new-rules-propel-next-generation-911-forwardwhat-this-means-for-gis

- 1Spatial. (2025). Interop address: Enhancing NG9-1-1 response. 1Spatial US. https://1spatial.com/us/news/interop-address-bridging-the-gap-in-ng911-response-2025/