Back-Extrusion Characterization of Fresh Kesar Mango Pulp for Thermal Processing and Equipment Design

Abstract

The structural behaviour of fruit pulp during processing is intrinsically associated with its firmness, consistency, and internal bonding characteristics. These rheological and textural properties play a critical role in determining the flow behaviour of the pulp within processing systems. Variations in such properties may alter the circulation patterns of the pulp inside processing equipment, thereby affecting fluid dynamics and mixing efficiency. Consequently, these changes can influence heat transfer performance during thermal processing operations, with potential implications for process efficiency, product quality, and thermal uniformity. The present study evaluated the rheo-textural characteristics of fresh Kesar mango pulp using a back-extrusion test to establish baseline structural data for process and equipment design applications. Texture measurements were performed using a TA-XT Plus texture analyser equipped with a back-extrusion rig probe under controlled compression-withdrawal conditions. The back-extrusion parameters obtained were firmness, consistency, cohesiveness and work of cohesion. Fresh Kesar mango pulp showed firmness of 775.59 ± 5.93 g, consistency of 784.95 ± 231.23 g·s, cohesiveness of −359.80 ± 23.67 g and work of cohesion of −173.48 ± 11.09 g·s. The results indicated that the pulp had moderate firmness, appreciable internal structural bonding and balanced adhesive and flow-related characteristics due to the combined effects of pectin network, soluble solids and suspended fibrous particles. Future work should include useful baseline rheo-textural information for optimisation of agitation, pumping and thermal processing operations involving mango pulp, particularly in agitation-assisted ohmic heating and related unit operations.

Keywords: Kesar mango pulp, Back-extrusion test, Texture analyser, Thermal processing operations

Introduction

Processing of mango fruit into diverse shelf-stable products makes the seasonal fruit conveniently available to consumers all year round. Some of the common processed products from mango fruit are derived from the pulp. Apart from the primary products from mango pulp, derivatives of mango pulp can be used to enrich or flavor secondary products such as yoghurt, ice cream, beverages, and soft drinks. Byproducts of mango processing such as the peel and kernel have been shown to be rich in bioactive compounds including carotenoids, polyphenols, and dietary fibers. The byproducts of mango processing can be used in food fortification and manufacture of feeds, thereby gaining greater value from the fruit while reducing wastage (Owino & Ambuko, 2021). Mango (Mangifera indica L.) is one of the most widely consumed tropical fruits and is extensively used for the preparation of pulp, puree, beverages, concentrates and other processed products due to its characteristic flavour, attractive colour and nutritional value (Ribeiro et al., 2024; Sharma et al., 2026). Among the Indian mango cultivars, Kesar is highly preferred for processing because of its rich aroma, bright saffron-coloured pulp and good pulp recovery (Yadav et al., 2025).

Texture and firmness are widely recognised as critical quality attributes of mango-based products, as they significantly influence processing performance, handling characteristics, and overall consumer acceptability. In semi-solid fruit systems, the resistance of the pulp to deformation is governed by the complex interactions among suspended particles, pectic substances, and soluble solids within the pulp matrix. These interactions contribute to the development and maintenance of the structural integrity of the pulp and determine its rheological response under applied forces. Such structural characteristics are particularly important during unit operations such as pumping, mixing, and thermal processing, where the pulp is continuously exposed to mechanical stresses that may alter its microstructure and flow behaviour (Jha et al., 2010). Understanding these properties is therefore essential for optimising processing conditions and ensuring consistent product quality.

The flow behaviour of mango pulp is an important consideration during industrial processing, as it directly affects pumping, agitation, heat transfer and product handling operations (Manohar et al., 1990; Hoshino, 2020). Since mango pulp behaves as a semi-solid and non-Newtonian material, its movement inside processing equipment depends largely on its resistance to deformation and flow under applied mechanical force. These characteristics are mainly influenced by the presence of suspended fibrous particles, soluble solids, pectic substances and other colloidal components in the pulp matrix (Chandel et al., 2022; Sharma et al., 2026). Variations in these components may alter the consistency, cohesiveness and resistance to flow of mango pulp during thermal and ohmic heating operations (Makroo et al., 2019).

The structural behaviour of fruit pulp during processing is closely related to its firmness, consistency and internal bonding characteristics. Variations in these properties may affect the circulation pattern of pulp inside processing equipment and can influence heat transfer efficiency during thermal operations. Therefore, understanding the mechanical response of mango pulp under compression and flow conditions is useful for improving process design and operational control (Jarimopas & Kitthawee, 2007).

Knowledge of the structural and flow-related properties of mango pulp is therefore essential for designing thermal processing units such as agitation-assisted ohmic heating systems in which controlled laminar movement and uniform mixing of pulp are required. Such information helps in selecting suitable agitator speed, vessel geometry and operational conditions for efficient pulp handling while minimising flow stagnation and uneven heating during processing (Hoshino, 2020; Lee et al., 2025). Changes in the structural behaviour of mango pulp may further influence heat transfer pattern, heating uniformity and energy utilisation efficiency during thermal processing operations involving semi-solid food systems (Makroo et al., 2019).

Back-extrusion test is widely used for evaluating the texture and flow-related behaviour of semi-solid food materials because it can effectively characterise heterogeneous and particulate-containing samples such as fruit pulp (Bourne, 2002; Hoshino, 2020). The force-time curve generated during back-extrusion provides useful parameters including firmness, consistency, cohesiveness and work of cohesion, which describe the resistance of the pulp to deformation, flow, internal structural bonding and adhesive behaviour, respectively (Bourne, 2002; Hoshino, 2020). Such parameters are useful for better interpreting the mechanical behaviour of pulp during agitation and bulk movement inside processing systems (Sahin & Sumnu, 2006; Lee et al., 2025).

Back-extrusion analysis has also been successfully applied for characterising highly viscous and semi-solid food products because the method provides rapid and reproducible information regarding flow resistance, consistency and structural stability under confined deformation conditions. The technique is particularly suitable for particulate-containing systems in which conventional flow measurements may become difficult (Nasaruddin et al., 2012; Chen et al., 2021).

During back-extrusion testing, the sample experiences simultaneous compression, shear and upward flow around the probe surface. Because of this combined deformation behaviour, the technique is considered suitable for evaluating semi-solid food materials containing fibrous and particulate matter. The generated force-time curve provides useful information regarding structural resistance, adhesive nature and flow-related behaviour of fruit pulp systems under confined deformation conditions (Jarimopas & Kitthawee, 2007).

Although several studies have reported rheological and firmness properties of mango and other fruit pulps, detailed back-extrusion characterisation of fresh Kesar mango pulp is still limited. Information related to firmness, consistency, cohesiveness and adhesive behaviour is particularly important for understanding pulp movement and structural stability during processing operations. Therefore, the present study was carried out to evaluate the back-extrusion texture properties of fresh Kesar mango pulp in order to establish baseline structural information useful for future thermal processing and equipment design applications.

Material and Methods

Texture Analysis of Fresh Kesar Mango Pulp

The texture properties of fresh Kesar mango pulp were measured using a texture analyser (TA-XT plus model, Microsystems, US). The back-extrusion rig with 35 mm disc probe (A/BE-d35) was used to analyse the sample. Fresh pulp was obtained from ripe Kesar mangoes as suggested by Yadav et al., (2025). The sample was filled into the back-extrusion rig carefully to avoid air incorporation.

During testing, the disc probe moved downward into the pulp sample and forced the material to flow upward through the narrow gap between the probe and container wall. This created a combined compression and extrusion effect which simulated the type of mechanical deformation generally experienced by semi-solid food materials during industrial handling and agitation processes. The confined annular flow generated between the probe and container wall allowed the pulp to undergo controlled deformation during testing, which helped in obtaining reproducible force-time responses for evaluating texture-related parameters (Chen et al., 2021).

The test was conducted in compression mode at a pre-test speed of 1.0 mm/s, test speed of 2.0 mm/s and post-test speed of 10.0 mm/s. The trigger force was maintained at 5 g under auto-force detection, and data were collected at 500 peaks per second. The texture parameter was obtained directly by Exponent Connect software associated with the texture analyser. All measurements were carried out in triplicate and reported as mean ± standard deviation.

The force-time curve generated during the test consisted of a positive compression region and a negative withdrawal region. The positive region represented the resistance offered by the pulp against probe movement, whereas the negative region reflected the adhesive and cohesive behaviour of the sample during probe withdrawal.

Firmness (g): Maximum positive force recorded during compression, representing the resistance of pulp to probe penetration.

Consistency (g·s): Positive area under the force–time curve during compression, indicating resistance to flow.

Cohesiveness (g): Maximum negative force during probe withdrawal, representing internal structural bonding of the pulp matrix.

Work of Cohesion (g·s): Negative area under the withdrawal curve, indicating adhesive energy required to separate the probe from the sample.

Results and Discussion

The back-extrusion texture parameters of fresh Kesar mango pulp are presented in Table 1. The back-extrusion test provided detailed information related to the resistance, adhesive nature, internal bonding and flow characteristics of mango pulp, which collectively reflected the overall structure of fresh Kesar mango pulp.

The obtained texture profile indicated that fresh Kesar mango pulp behaved as a structured semi-solid system with noticeable resistance against deformation and flow (Jarimopas & Kitthawee, 2007). The observed behaviour might be possible due to the combined effect of pectin substances, suspended fibrous particles, soluble sugars and colloidal interactions within the pulp matrix (Schouten et al., 2018). Similar structural characteristics have been reported in fruit-based semi-solid food systems where the internal network contributes significantly toward texture and flow behaviour (Bourne, 2002; Hoshino, 2020).

| Table 1. Back-extrusion texture parameters of fresh Kesar mango pulp |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | R1 | R2 | R3 | Mean | SD |
| Firmness | g | 781.41 | 769.55 | 775.82 | 775.59 | 5.93 |
| Consistency | g·s | 1016.17 | 553.72 | 784.95 | 784.95 | 231.23 |
| Cohesiveness | g | -383.66 | -336.34 | -359.40 | -359.80 | 23.67 |
| Work of Cohesion | g·s | -162.23 | -184.37 | -173.85 | -173.48 | 11.09 |

The firmness of fresh Kesar mango pulp was recorded as 775.59 ± 5.93 g. It indicates the maximum force required for probe penetration during compression. This parameter reflects the resistance of the pulp matrix to deformation and is governed by the structural integrity of suspended insoluble solids, cell wall fragments and pectic substances dispersed throughout the pulp. The observed moderate firmness suggests that fresh Kesar mango pulp possessed a sufficiently structured semi-solid matrix with appreciable resistance to compressive deformation. Similar findings have shown that firmness in fruit-based semi-solid systems is strongly associated with the integrity of suspended particulates and hydrocolloidal interactions within the matrix (Bourne, 2002; Hoshino, 2020).

The firmness behaviour may also be associated with the maturity stage of mango used for pulp preparation (Jarimopas & Kitthawee, 2007). During ripening, enzymatic degradation of cell wall materials and gradual solubilization of pectin substances reduce tissue rigidity and contribute toward softening of fruit pulp (Schouten et al., 2018). However, the presence of sufficient suspended solids and partially intact structural components may still provide appreciable resistance against compression in fresh mango pulp (Jarimopas & Kitthawee, 2007).

The consistency of the pulp was found to be 784.95 ± 231.23 g·s. It represents the positive area under the force–time curve during compression and indicates resistance to flow under applied stress. Higher consistency values are generally associated with greater viscosity and stronger flow resistance in semi-solid systems. The relatively high consistency observed in the present study confirms the viscous and non-Newtonian behaviour of mango pulp. This may be due to the presence of soluble solids, pectin and fibrous suspended particles. Similar relationships between consistency and soluble hydrocolloidal content have been reported in fruit puree systems (Manohar et al. 1990). Similar observations have been reported for mango puree systems in which an increase in soluble solids and pectic interactions contributed toward higher consistency and stronger resistance to flow behaviour during processing operations (Makroo et al., 2019).

From a processing point of view, higher consistency generally indicates greater resistance to flow and may influence pumping energy requirement and mixing behaviour during industrial operations. At the same time, moderate consistency can help maintain better suspension stability of pulp particles during storage and thermal processing (Chandel et al., 2022).

Back-extrusion consistency measurements are often considered useful indicators of flowability and bulk movement behaviour in semi-solid food systems. The resistance generated during confined flow may therefore provide indirect information regarding the handling behaviour of mango pulp during pumping and agitation operations (Chen et al., 2021).

The cohesiveness of the pulp was measured as −359.80 ± 23.67 g. The magnitude of cohesiveness indicates the degree of attraction among suspended solids, colloidal particles and hydrocolloidal components. The appreciable cohesiveness observed suggests strong internal structural interactions within the mango pulp. It is likely due to the natural pectin network and dispersed fibrous matter. Such cohesive behaviour has also been reported in pectin-rich fruit-based semi-solid systems (Chandel et al., 2022). The cohesiveness of mango pulp is generally associated with the interaction among soluble sugars, pectic substances and suspended solids, which collectively contribute toward structural stability and viscoelastic behaviour of the pulp system during processing (Makroo et al., 2019).

The cohesive nature of the pulp may help to maintain uniform distribution of suspended particles during agitation and thermal treatment. Strong internal bonding within the pulp structure may also reduce phase separation during handling and storage operations (Chandel et al., 2022).

The work of cohesion was recorded as −173.48 ± 11.09 g·s. It represents the adhesive energy required to separate the probe from the pulp during withdrawal. This parameter reflects the stickiness and adhesive nature of the pulp and is influenced by soluble sugars, hydrated pectin molecules and serum phase viscosity. The observed value indicates moderate adhesive behaviour of fresh Kesar mango pulp. It suggests sufficient tackiness without excessive stickiness. Such balanced adhesive behaviour is desirable in semi-solid food products because it contributes toward better handling properties while avoiding excessive surface sticking during processing (Nasaruddin et al., 2012). Similar adhesive responses in fruit-based colloidal systems have been linked to sugar-pectin interactions and hydration properties (Hoshino, 2020).

The moderate adhesive behaviour observed in the present study may be beneficial during industrial processing because excessively sticky pulp can create operational problems such as wall deposition, sticking and difficulty during product discharge (Bourne, 2002; Hoshino, 2020). Therefore, balanced adhesive properties are important for smooth handling and processing of fruit pulp systems.

Conclusion

The back-extrusion texture profile showed that fresh Kesar mango pulp possesses a moderately firm, cohesive and structurally stable semi-solid pulp with balanced adhesive and flow properties. These textural characteristics are primarily governed by the natural pectin network, soluble solids, suspended fibrous particles and colloidal interactions within the pulp. The obtained texture parameters provide baseline structural information for future comparison during thermal processing, concentration, drying, enzymatic treatment, or storage studies. The generated rheo-textural information may also be useful for understanding the flow, heating and handling behaviour of mango pulp during advanced thermal processing techniques such as ohmic heating and related semi-solid food processing operations.

Conflict of Interest

The authors declare no competing interests for this article.

Data Availability Statement

The data supporting the findings of this study are available from corresponding author upon reasonable request.

Clinical Trial Registration

Not applicable.

Ethics approval and consent to participate

Not applicable.

Consent for publication

Not applicable.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

Bourne, M. C. (2002). Food texture and viscosity: Concept and measurement (2nd ed.). Academic Press.

Chandel, V., Kaur, G., Bhardwaj, A., Kaur, H., Kaur, A., & Singh, B. (2022). Current advancements in pectin: Extraction, properties and applications. Foods, 11(17), 2683.

Chen, W., Hadde, E. K., & Chen, J. (2021). Development of a ball back extrusion technique for texture analysis of fluid food. Journal of Texture Studies, 52(4), 461-469.

Hoshino, T. (2020). Analysis of viscosity measurements obtained using the short back extrusion method. Part 1: Theory of short back extrusion in viscometry. Journal of Texture Studies, 51(2), 201–213.

Jarimopas, B., & Kitthawee, U. (2007). Firmness properties of mangoes. International Journal of Food Properties, 10(4), 899-909.

Jha, S. K., Sethi, S., Srivastav, M., Dubey, A. K., Sharma, R. R., Samuel, D. V. K., & Singh, A. K. (2010). Firmness characteristics of mango hybrids under ambient storage. Journal of Food Engineering, 97(2), 208-212.

Lee, S. Y., Kim, H. J., Park, J. Y., & Choi, Y. M. (2025). Texture characterization of semi-solid foods for older adults: Sensory analysis and instrumental approaches. LWT, 205, 117582.

Makroo, H. A., Prabhakar, P. K., Rastogi, N. K., & Srivastava, B. (2019). Characterization of mango puree based on total soluble solids and acid content: Effect on physico-chemical, rheological, thermal and ohmic heating behavior. Lwt, 103, 316-324.

Manohar, B., Ramakrishna, P., & Ramteke, R. S. (1990). Effect of pectin content on flow properties of mango pulp concentrates. Journal of Texture Studies, 21(2), 179–190.

Nasaruddin, F., Chin, N. L., & Yusof, Y. A. (2012). Effect of processing on instrumental textural properties of traditional dodol using back extrusion. International Journal of Food Properties, 15(3), 495-506.

Ribeiro, S., Nascimento, A., Lima, J., Batista, L., Almeida, R., & Lisboa, H. (2024). Modulation of rheological and textural properties in avocado pulp: Impact of maltodextrin concentration and temperature. Food Materials Research, 4, e014.

Sahin, S., & Sumnu, S. G. (2006). Physical properties of foods. Springer.

Schouten, R. E., Fan, S., Verdonk, J. C., Wang, Y., Kasim, N. F. M., Woltering, E. J., & Tijskens, L. M. M. (2018). Mango firmness modeling as affected by transport and ethylene treatments. Frontiers in plant science, 9, 1647.

Sharma, S., Wani, K. M., Mujahid, S. M., Jayan, L. S., & Rajan, S. S. (2026). Review on pectin: sources, properties, health benefits and its applications in food industry. Journal of Future Foods. 6(2): 205–219

Yadav, S., Dhamsaniya, N. K., & Rathod, P. J. (2025). Physicochemical and Enzymatic Profile of Fresh Kesar Mango (Mangifera indica L.) Pulp. Journal of Biology and Nature, 17(2), 867-874.

Owino, W. O., & Ambuko, J. L. (2021). Mango Fruit Processing: Options for Small-Scale Processors in Developing Countries. Agriculture, 11(11), 1105. https://doi.org/10.3390/agriculture11111105