Impact of seasonal variation on physicochemical parameters of Chawandiya pond, Bhilwara district, Rajasthan, India

Abstract

Water quality is largely determined by its physicochemical properties, which significantly influence the biological diversity found in aquatic ecosystems such as ponds, lakes and rivers. Assessing these properties is crucial for evaluating the health of these ecosystems. This study was conducted from November 2024 to October 2025 to examine seasonal variations in the physicochemical parameters at various sampling sites (S1, S2, S3, S4 and S5) in Chawandia pond, Bhilwara, Rajasthan. Average pH was highest during monsoon season (6.33) followed by winter (6.31) and lowest average pH was observed in summer (6.27). Highest average temperature was recorded in summer (29.76°C) followed by monsoon (23.10°C) and lowest in winter (16.15°C). Electrical conductivity peaked in summer (242 mS/cm) followed by winter (226 mS/cm) and lowest electrical conductivity was observed during monsoon (206 mS/cm). Total dissolved solids (TDS) content was highest in summer (153.60 mg/l) followed by winter (142.60 mg/l) and lowest during monsoon (133.80 mg/l). Maximum hardness was observed in summer (79.20 mg/l) followed by winter (71 mg/l) and minimum hardness was observed during monsoon (68.60 mg/l). Highest average calcium (Ca2+) concentration occurred in summer (22.64 mg/l) followed by winter (21.76 mg/l) and lowest in monsoon (20.32 mg/l). Highest average magnesium (Mg2+) concentration was recorded in summer (5.18 mg/l) followed by monsoon (4.27 mg/l) and lowest in winter (3.98 mg/l). Alkalinity in Chawandia village pond ranged from 84 mg/l to 98 mg/l during winter, from 99.2 mg/l to 107.30 mg/l in summer and from 76 mg/l to 90 mg/l during monsoon across different sampling stations. Chloride (Cl-) ion concentrations ranged from 10.99 mg/l to 13.99 mg/l in winter, from 11.82 mg/l to 13.79 mg/l in summer and from 9.99 mg/l to 13.99 mg/l during monsoon. Fluoride (F-) concentrations ranged from 0.12 mg/l to 0.16 mg/l in winter, with a slight increase in summer (0.14 mg/l to 0.20 mg/l) and during monsoon, they ranged from 0.14 mg/l to 0.18 mg/l. Highest average sulphate (SO42-) concentration was observed in summer (5.75 mg/l) followed by winter (4.96 mg/l), with the lowest concentration observed during monsoon (4.92 mg/l). Nitrate (NO3-) concentrations remained below the detectable limit (<0.1 mg/l) across all five sampling stations in Chawandia pond throughout three seasons. Highest average dissolved oxygen concentration was observed during monsoon season (4.02 mg/l) followed by winter (3.68 mg/l) and lowest concentration was recorded in summer (3.24 mg/l). Physicochemical parameters of Chawandia pond exhibited distinct seasonal variations that influenced water quality. During summer, there was a marked increase in temperature, electrical conductivity, total dissolved solids, hardness, calcium, magnesium, alkalinity, chloride, fluoride and sulphate levels. Conversely, highest pH and dissolved oxygen levels were observed during monsoon season. These findings underscore the dynamic nature of pond water quality throughout the year, emphasizing the need for continuous monitoring to assess the ecological health and sustainability of aquatic ecosystems in the region.

Keywords: Water, Physicochemical, Chawandia pond, Season, Ecological health.

Introduction

Lakes and surface water are vital sources of freshwater that offer numerous benefits for humans and ecosystem. Freshwater is indispensable for human survival, agriculture, industrial activities and consumption. Ensuring a sufficient and clean supply of freshwater is essential for sustainable development (Mahananda et al., 2005). Industrial and municipal solid waste has emerged as significant contributors to the pollution of both surface and ground water. Presence of heavy metal contamination and various types of pollutants in water makes it unsuitable for human consumption (Masindi & Muedi, 2018; Rehman et al., 2018). Quality of water is largely determined by its physicochemical properties, which significantly influence the biological diversity found in aquatic ecosystems such as ponds, lakes and rivers. These physicochemical parameters are largely influenced by various natural processes and features, such as rainfall, soil composition, geology of the catchment area, sediment erosion and evaporation (Horne et al., 1994). In addition to these natural factors, human activities considerably affect the chemical composition of water bodies (Boyd, 2015). Factors such as temperature, turbidity, pH, dissolved oxygen and total suspended solids in water significantly influence the growth of organisms in aquatic environments (Qureshimatva et al., 2015). Introduction of substantial amounts of domestic or industrial waste into water bodies leads to a marked reduction in oxygen levels and an increase in ammonia and nitrogen concentrations downstream of effluent discharge (Nayar, 2020). Changes in physical, chemical or biological properties of water due to introduction of unwanted or foreign substances through the discharge of sewage, industrial, municipal waste and agricultural runoff are responsible for the degradation of water quality (Parashar et al., 2006; Patil et al., 2012). Freshwater reservoirs have been significantly contaminated and polluted owing to increasing various types of pollutants, industrial growth, extensive use of fertilizers in agriculture and other human activities. These factors have introduced numerous hazardous substances and compounds that affect the water quality of the particular water reservoir such as ponds, lakes and rivers. Therefore, regularly assessing quality of drinking water is crucial for preventing waterborne diseases resulting from contaminated water. Additionally, analyzing the physicochemical parameters of water can offer important insights into ecosystem health and clarify the overall hydro biological interactions (Basavarajappa et al., 2011). Therefore, present study was conducted from November 2024 to October 2025 to assess seasonal fluctuations in the physicochemical parameters of different sampling sites in Chawandia village pond in Bhilwara district, Rajasthan.

Material and methods

Chawandiya pond is situated in Chawandiya village within Kotri Tehsil of Bhilwara district, Rajasthan, at coordinates 25°19'53.38''N and 74°46'23.62''E. Chawandiya village is bordered by Bhilwara tehsil to west, Mandalgarh Tehsil to east, Shahpura tehsil to north and Suwana tehsil to west. Spanning approximately 48 hectares, this freshwater wetland encompasses a variety of habitats, including open water regions and adjacent buffer zones, which provide a home for numerous aquatic species, such as fish, insects and birds. Various physicochemical parameters, such as pH, temperature, electrical conductivity, total dissolved solids, total hardness, calcium, magnesium, alkalinity, chloride, fluoride, sulphate, nitrate and dissolved oxygen were measured using different procedures according to the APHA (2012). Period of water sample collection within the year was categorized into three distinct seasons: winter (November to February), summer (March to June) and monsoon (July to October) seasons.  Water temperature was measured using a digital thermometer. pH of  collected water samples was analyzed using a buffered electronic pH meter. Total dissolved solid (TDS) and electrical conductivity (EC) were estimated using a digital WTW meter (Wissenschaftich Technische Werkstatten). Titration method was employed to determine the total hardness; chlorides, alkalinity and dissolved oxygen, nitrate, phosphate, fluoride and sulphate levels were assessed using a Systronics digital spectrophotometer. Electrical conductivity measurements were performed using a digital conductivity meter and the results were measured in mS/cm at 25 °C.

Result and discussion

Water can be contaminated in various ways, such as through floating particles, suspended particles and microbiological and bacterial contaminants. Evaluating the physical properties of water, including its color, temperature, odor, turbidity, pH and total dissolved solids (TDS), is essential for assessing the health of a particular aquatic ecosystem, such as ponds, lakes and rivers. Chemical assessments are necessary to determine the biochemical oxygen demand, chemical oxygen demand, dissolved oxygen, hardness and alkalinity. Physicochemical characteristics of freshwater bodies are significantly influenced by climate, geochemistry, geomorphology and presence of pollutants (Sahni & Yadav, 2012). Physicochemical parameters are best indicators for evaluating water quality in aquatic environments. Even minor alterations in these physicochemical characteristics can affect ecosystem diversity. Biological productivity of any aquatic system is closely related to physicochemical properties of water in particular lakes, ponds and rivers (Sharma et al., 2013). Therefore, we planned a study carried out along the different sampling stations (S1, S2, S3, S4, and S5) in Chawandia village pond to assess seasonal fluctuations in physiochemical parameters from November 2024 to October 2025 (Figure 1).  During study, various physiochemical parameters of water, such as pH, temperature, electrical conductivity (EC), total dissolved solids (TDS), total hardness, calcium (Ca2+), magnesium (Mg2+), alkalinity, chloride (Cl-), fluoride (F-), sulphate (SO42-), nitrate (NO3-) and dissolved oxygen (DO) were analyzed during the three distinct seasons, namely winter, summer and monsoon (Table 1).

- pH:

pH is a crucial factor in identifying toxicity in water-based solutions. In the field of chemistry, pH values range from 0 to 14, indicating acidity or basicity of water and solutions. Ideally, the pH of drinking water should be 7. A pH value below 7, approaching 0, signifies increased acidity, whereas a pH above 7, reaching up to 14, indicates higher alkalinity levels in water (Alley, 2007; Islam et al., 2017). Acidic conditions arises when water has more free hydrogen ions, whereas increased alkalinity results from reduced photosynthetic activity, carbon dioxide uptake and bicarbonate levels (Kale, 2016). The pH of water influences amount of solute that can dissolve in it (Zhang YanHao et al., 2018). In winter season, maximum pH value was found at sampling station S4 (6.75) followed by S2 (6.34), S3 (6.25), S1 (6.24) and minimum value of pH found at sampling station S5 (6.00) (Table 1). Subsequently, during summer season maximum value of pH was observed at sampling station S2 (6.40) followed by S1 (6.36), S3 (6.30), S4 (6.20) and minimum value of pH found at sampling station S5 (6.10) (Table 1). In context to monsoon season, highest value of pH was observed from sampling station S1 (6.42) followed by S5 (6.40), S3 (6.37), S4 (6.25) and S2 (6.24) (Table 1).  In context of overall seasonal variation, average pH value was higher in monsoon season (6.33) followed by winter (6.31) and lowest average pH value was found during summer season (6.27) (Table 1).

Source of water and landscape features surrounding the water bodies influence pH levels. This factor is crucial for maintaining the physicochemical equilibrium of water and is influenced by the dissolution and precipitation of minerals (Belghiti et al., 2013; Benrabah et al., 2016). Similarly Beniwal & Jain (2024) assessed physiochemical parameters of Kot dam, Jhunjhunu, Rajasthan and value of pH obtain between 6.84 to 7.45 along the different sampling sites of Kot dam.  Mishra et al. (2019) analyzed physicochemical parameters of Pichola lake Udaipur and they found that the pH of Pichola lake was moderately alkaline and value ranged between 7.38 to 8.23 along different sampling stations.  Patel et al. (2018) studied physicochemical parameters of Mahi Bajaj Sagar Dam, Banswara and observed average pH ranged from 8.1 to 8.7 at all the five stations. They concluded that the pH of Mahi Bajaj Sagar was moderately alkaline, which is suitable for supporting fairly good aquatic productivity.

- Temperature (°C):

Temperature is a key factor that significantly affects aquatic ecosystems (Sedamkar & Angadi, 2003). According to Pandey et al. (2012), water temperature is a vital factor influencing the development of organisms and impacting overall quality and biodiversity of aquatic ecosystems. Water temperature is essential for controlling numerous abiotic and biotic elements within aquatic ecosystems (Radhika et al., 2004; Sharma & Sarang, 2004). Elevated temperatures, combined with high nutrient concentrations, facilitate the rapid proliferation of plankton and aquatic plants (Chishty & Choudhary, 2022b; Mei et al., 2022; Hao et al., 2024). Water temperature is a major factor that influences this process. Development of organisms is significantly influenced by water temperature, which in turn affects the overall quality of water (Pandey et al., 2012).Variations in water temperature are generally influenced by factors such as seasonal changes, geographical location, timing of sampling and temperature of effluents entering the waterway (Ahipathi & Puttaiah, 2006; Sahni & Yadav, 2012; Chishty & Choudhary, 2022a). In winter season, maximum temperature was found at sampling station S1 (16.21°C) followed by S3 (16.16°C), S2 (16.14°C), S5 (16.13°C) and minimum temperature was found at sampling station 4 (16.12°C) (Table 1). During summer season, maximum temperature was recorded at sampling station S1 (30.01°C) followed by S2 (29.91°C), S5 (29.86°C), S4 (29.84°C) and minimum temperature was recorded at sampling station 3 (29.21°C) (Table 1). In monsoon season, maximum value of temperature was observed at sampling station 5 (23.22°C) followed by S1 (23.21°C), S4 (23.18°C), S2 (23.02°C) and minimum value of temperature was found at sampling station 3 (22.90°C). In the context of overall seasonal variation, average temperature was highest in summer season (29.76°C) followed by monsoon (23.10°C) and lowest average temperature was recorded during winter season (16.15°C) (Table 1).

Soni & Thomas (2013) carried out study on physicochemical characteristics of the Dakor sacred wetland in Anand, Gujarat. Their results indicated that the water temperature was at its peak during summer season, in contrast to monsoon and winter period. Similarly Beniwal & Jain (2024) assessed physiochemical parameters of Kot dam, Jhunjhunu district Rajasthan and observed higher temperature during summer season followed by monsoon and lowest temperature observed during winter season. Patel et al. (2018) studied physicochemical parameters of Mahi Bajaj Sagar Dam, Banswara and observed water temperature between 24.3°C to 28.9°C at all five stations. Mishra et al. (2019) analyzed physicochemical parameters of Pichola lake Udaipur and they found that the water temperature of Pichola lake was ranged between 20.23°C to 30.33°C along the all four sampling stations.

Temperature is a crucial physical aspect of water quality assessment, as it affects aquatic life by changing the concentration of dissolved oxygen in water. This alteration reduces the availability of oxygen for respiration and metabolic processes in aquatic organisms (Jalal & Sanalkumar, 2012; Tank & Chippa, 2013). In conclusion, depth of water, along with climatic and topographic shifts in a specific area, influences its temperature (Chishty & Choudhary, 2022 a; Beniwal & Jain, 2024). Changes in atmospheric temperature lead to corresponding alterations in temperature of water bodies (Zeighami & Kurylyk, 2025).

- Electrical conductivity:

Electrical conductivity measures ability of an aqueous solution to conduct electric current. It is an effective method for evaluating water purity (Acharya et al., 2008). Presence of suspended impurities and concentration of ions in water influence its conductivity (Joshi et al., 2009). Inorganic fertilizers entering aquifers and sewage from nearby residential areas contribute to high electrical conductivity values. Total concentration of dissolved solids or ions in a water body is an important metric for determining chemical concentrations as an indicator of suitability (Jhingran, 1982).

Electrical conductivity was measured in mS/cm. In winter season, highest value of electrical conductivity was found at sampling station S5 (240 mS/cm) followed by S1 and S4 (230 mS/cm from each site), S2 (220 mS/cm) and lowest value of electrical conductivity was found at sampling station S3 (210 mS/cm) (Table 1). Subsequently, during summer season, highest value of electrical conductivity was observed at sampling station S5 (260 mS/cm) followed by S1 (250 mS/cm) and S2 (240 mS/cm) and lowest electrical conductivity was found at sampling stations S3 and S4 (230 mS/cm at each sampling site) (Table 1). In monsoon season, highest value of electrical conductivity was observed from sampling station S5 (220 mS/cm) followed by S1 and S4 (210 mS/cm at each sampling site), S2 (200 mS/cm), S2 (200 mS/cm) and lowest value of electrical conductivity was observed from S4 (190 mS/cm) (Table 1). In the context of seasonal variation, highest value of electrical conductivity was observed during summer season (242 mS/cm) followed by winter (226 mS/cm) and lowest value of electrical conductivity was observed during monsoon season (206 mS/cm) (Table 1).

According to Polat (1997), a lake is considered contaminated or polluted when its conductivity exceeds 1000 mS/cm. During summer, electrical conductivity values are higher because of elevated temperatures and reduced water flow, resulting in increased conductivity. Increase in temperature accelerates chemical reactions, which may enhance the dissolution of minerals and other conductive substances. Additionally, water evaporation intensifies in summer, concentrating ions and further increasing conductivity levels in particular water bodies (Dewangan et al., 2023; Hammoumi et al., 2024; Singh & Saxena, 2025).

- TDS (Total dissolve solid):

Total Dissolved Solids (TDS) refers to the total amount of inorganic and organic substances or salts present in water (Rubel et al., 2019). Typically, these inorganic substances include cations such as calcium (Ca2+), magnesium (Mg2+), potassium (K+) and sodium (Na+), while anions consist of carbonates (CO32-), nitrates (NO3-), bicarbonates (HCO3-), chlorides (Cl-) and sulphates (SO42-). TDS indicates that the water is not entirely pure because it contains additional materials.  Presence of dissolved solids can alter the taste of water (Rubel et al., 2019). In winter season, highest value of TDS content was observed from sampling station S5 (150 mg/l) followed by S1 (146 mg/l), S4 (145 mg/l), S2 (140 mg/l) and minimum value of TDS content was observed from S3 (132 mg/l) (Table 1). Subsequently, during summer season, highest value of TDS was observed from sampling station S5 (165 mg/l) followed by S1 (160 mg/l), S2 (152 mg/l), S3 (146 mg/l) and lowest TDS value was found from sampling station S4 (145 mg/l) (Table 1). In monsoon season, maximum amount of TDS was found from sampling station 5 (145 mg/l) followed by S4 (137 mg/l), S1 (135 mg/l), S2 (127 mg/l) and lowest value of TDS was found from sampling station S3 (125 mg/l) (Table 1). In context of seasonal fluctuations in total dissolved solids, average highest value of TDS was found during summer season (153.60 mg/l) followed by winter (142.60 mg/l) and lowest TDS was observed during monsoon season (133.80 mg/l) (Table 1).

Patel et al. (2018) studied physicochemical parameters of Mahi Bajaj Sagar Dam, Banswara and observed total dissolved solids (TDS) ranged from 236.8 to 313.6 mg/l. Mishra et al. (2019) analyzed physicochemical parameters of Pichola lake, Udaipur and they found that the total dissolved solid (TDS) content of Pichola lake was ranged between 257.63 to 327.95 mg/1 with an average value of 296.83 mg/l.

Similarly Beniwal & Jain (2024) assessed physiochemical parameters of Kot dam, Jhunjhunu, Rajasthan and observed higher TDS during summer season followed by winter and lowest TDS during monsoon season. During summer months, rise in water temperature led to a higher concentration of total dissolved solids owing to enhanced evaporation and solubility of various salts and minerals. Moreover, as dissolved minerals and salts evaporate, levels of total dissolved solids increase and elevated temperatures cause certain substances to dissolve in water, thereby raising the TDS value in water bodies (Dey et al., 2021; Gawle et al., 2021; Rahman et al., 2021).

- Total hardness:

Water hardness is defined by the presence of a significant amount of minerals in water (APHA, 2012). When minerals dissolve in water, they can cause problems such as scale buildup in hot water pipes and reduced soap lathering ability (Davis, 2010). Main ions responsible for hardness of natural water sources are calcium (Ca2+) and magnesium (Mg2+). Elevated carbonate levels may result from the dissolution of calcium carbonate in the lower layer of lake (Wetzel & Likens, 2000). In winter season, highest value of hardness was found from sampling station S5 (78 mg/l) followed by S1 (73 mg/l), S4 (72 mg/l), S2 (68 mg/l) and lowest value of hardness was observed from sampling station S3 (64 mg/l) (Table 1). During summer season, highest value of hardness was observed at sampling station S5 (84 mg/l) followed by S1 and S2 (80 mg/l at each site) and lowest value of hardness was found at sampling stations S3 and S4 (76 mg/l at each site) (Table 1). In monsoon season, highest value of hardness was observed from sampling station 5 (75 mg/l) followed by S4 (71 mg/l), S1 (70 mg/l), S2 (65 mg/l) and lowest value of hardness was found from sampling station S3 (62 mg/l) (Table 1). In terms of seasonal variation in hardness, maximum hardness was observed during summer (79.20 mg/l) followed by winter (71 mg/l) and minimum hardness was observed during monsoon (68.60 mg/l) (Table 1). Sharma et al. (2013) conducted a study on physicochemical analysis of surface and ground water in Abhanpur block of Raipur district, Chhattisgarh and found that water hardness ranged from 130 mg/l to 280 mg/l.  Similarly, Patel et al. (2018) studied physicochemical parameters of Mahi Bajaj Sagar Dam, Banswara and concluded that the total hardness ranged from 90 to 152 mg/l along the all five sampling stations. Similarly Beniwal & Jain (2024) conduct a study on physiochemical parameters of Kot dam, Jhunjhunu, Rajasthan and found that water hardness ranges between 117 to 172 mg/l along the different sampling stations throughout the various seasons.

- Calcium (Ca2+):

Natural water bodies generally have higher calcium levels, mainly because it is leached from the rocks. Calcium is a crucial micronutrient for most aquatic species. Nearly every water body contains calcium, which is a critical nutrient for aquatic organisms (Ansari & Prakash, 2000). Concentration of calcium ions ranged between 22.80 mg/l to 23.60 mg/l during winter, whereas in summer, it ranged between 22 mg/l to 24 mg/l (Table 1). During monsoon season, concentration of calcium ions ranged from 18.80 mg/l to 22 mg/l along the different sampling stations of Chawandia village pond (Table 1). In context of seasonal fluctuations in calcium ion concentrations, highest average calcium concentration was found during summer season (22.64 mg/l) followed by winter (21.76 mg/l) and lowest calcium concentration was found during the monsoon season (20.32 mg/l) (Table 1). Similarly Beniwal & Jain (2024) conduct a study on physiochemical parameters of Kot dam, Jhunjhunu, Rajasthan and observed calcium ions concentration between 27.93 mg/l to 38.25 mg/l along the different sampling stations throughout different seasons.

- Magnesium ion (Mg2+):

Magnesium (Mg2+) is often found alongside calcium in various water sources, although its levels are generally lower than those of calcium (Venkatasubramani & Meenambal, 2007). This element is vital for chlorophyll synthesis and acts as a limiting factor in the significant growth of phytoplankton (Dagaonkar & Saksena, 1992). Concentration of magnesium ions ranged from 3.60 mg/l to 4.56 mg/l during winter season, whereas summer season had magnesium ion concentrations between 4.8 mg/l to 5.76 mg/l (Table 1). Subsequently, during monsoon season, the value of magnesium ions ranged between 3.60 mg/l to 4.8 mg/l (Table 1). In the context of seasonal fluctuations in magnesium ion concentrations, highest average magnesium concentration was found during summer (5.18 mg/l) followed by monsoon (4.27 mg/l) and lowest magnesium concentration was found during winter (3.98 mg/l) (Table 1).

High magnesium ions concentrations can occur due to intrusion of human-made or household waste or through cation exchange with sodium ions (Thompson et al., 1992). Magnesium ions appear in aquatic environments due to the decomposition of minerals and soils that are rich in magnesium. Presence of magnesium ions in both surface and groundwater is attributed to minerals such as dolomite and magnesite (Haritash et al., 2016; Kumar et al., 2022). Additionally, the processes of rock weathering and erosion contribute to occurrence of magnesium ions in water bodies, such as lakes, pond and dams (Kovalenko & Sova, 2024). In various water types, magnesium is frequently linked to calcium; however, its levels are typically lower than those of calcium (Venkatasubramani & Meenambal, 2007; Krishna Kumar et al., 2017). Similarly, in this study, concentration of magnesium ions was lower than that of calcium ions at all five sampling stations throughout study period (Table 1).

- Alkalinity:

Alkalinity refers to the ability of water to neutralize acids, encompassing all titratable bases (APHA, 2012). Measuring the alkalinity of water is essential for calculating the necessary quantities of lime and soda for processes such as water softening and controlling corrosion in boiler feed water systems (Tchobanoglous et al., 2003). Water is considered alkaline when it contains ions such as hydroxide (OH-), bicarbonate (HCO3-), carbonate (CO32-), or a mixture of these ions. Alkalinity in water arises from dissolved substances such as carbonates, bicarbonates and hydroxides, which neutralize acids and stabilize the pH of water. Furthermore, borates (BO33-), silicates (SiO44-) and phosphates (PO43-) also contribute to alkalinity of water (Bhateria & Jain, 2016; Dhoke, 2023). Alkalinity of Chawandia village pond ranged from 84 mg/l to 98 mg/l during winter, whereas summer season alkalinity value ranged between 99.2 mg/l to 107.30 mg/l and monsoon season comprised alkalinity values between 76 mg/l to 90 mg/l at different sampling stations of Chawandia pond (Table 1). In the context of seasonal fluctuations in alkalinity of Chawandia pond water, highest average alkalinity was found during summer (104.20 mg/l) followed by winter (90.40 mg/l) and lowest alkalinity was found during monsoon season (82.40 mg/l) (Table 1).

Mishra et al. (2019) analyzed physicochemical parameters of Pichola lake Udaipur and they found that value of alkalinity content in Pichola lake was ranged between 141.25 to 172.25 mg/l along all four sampling stations. Similarly Beniwal & Jain (2024) conduct a study on physiochemical parameters of Kot dam, Jhunjhunu district Rajasthan and measure alkalinity of water along the three sampling stations. They observed alkalinity values between 126 to 128.75 mg/l for sampling station I, 112.65 to 124.25 mg/l for station II and 114 to 121.44 mg/l at station III.  Surface water alkalinity is mainly due to the presence of carbonates and bicarbonates (Muhammad et al., 2000).

- Chlorides (Cl-)

Presence of chloride (Cl-) is an indicator of sewage contamination. Individuals accustomed to elevated chloride levels in their drinking water may experience laxative effects (Dahiya & Kaur, 1999). Chloride is ionized form of chlorine, is one of the most prevalent inorganic ions in natural water and wastewater. Chloride is naturally present in all water types. They dissolve in water and move easily through soil and rock formations. When chloride concentrations exceed 100 mg/l, the water acquires a salty taste. During study, it was found that the value of chloride ions ranged between 10.99 mg/l to 13.99 mg/l during winter, whereas summer revealed chloride ion concentration between 11.82 mg/l to 13.79 and monsoon exhibited chloride ion concentration between 9.99 mg/l to 13.99 mg/l along the different sampling stations of Chawandia village pond (Table 1). In the context of seasonal fluctuations, highest average chloride ion concentration was found during summer (13.00 mg/l) followed by monsoon (12.19 mg/l) and lowest chloride ion concentration was found during winter season (11.99 mg/l) (Table 1).

Beniwal & Jain (2024) conduct a study on physiochemical parameters of Kot dam, Jhunjhunu, Rajasthan and measure chloride concentration of water along three sampling stations. They found that the chloride concentration of Kot dam ranged between 80.5 to 83.50 mg/l for sampling station I, 78.32 to 87.75 mg/l for sampling station II and 75.75 to 80.33 mg/l for station III. Chlorides, which are important inorganic anions, are present in varying concentrations in natural water bodies and act as pollution markers (Makhoukh et al., 2011). Presence of chlorides in lake or pond water can indicate human-induced pollution, as they are found in both urine and cleaning products (Matini et al., 2009).

- Fluoride

The World Health Organization (W.H.O.) advises maintaining a fluoride concentration of 1.5 mg/l in drinking water to avert dental fluorosis and cavities (Harrison, 2005; Jagtap et al., 2012; Ali et al., 2016). During study, it was observed that concentration of fluoride was ranged between 0.12 mg/l to 0.16 mg/l during winter and summer exhibited slightly increasing trends in fluoride concentration (0.14 mg/l to 0.20 mg/l). During monsoon season, fluoride ion concentrations were between 0.14 mg/l and 0.18 mg/l (Table 1). Similar to our observations, Beniwal & Jain (2024) conduct a study on physiochemical parameters of Kot dam, Jhunjhunu, Rajasthan and measure fluoride concentration of water along three sampling stations. They found that the fluoride level ranged between 0.1 mg/l to 0.11 mg/l at station I, from 0.1 mg/l to 0.9 mg/l at station II and between 0.1 mg/l to 0.11 mg/l at station III.

- Sulphate (SO42-):

Sulphate ions (SO42-) are present in both natural water and wastewater. Leaching of sodium sulphate or magnesium sulphate (from natural sources often leads to increased sulphate concentrations in natural water (Davis & David, 2008; Davis, 2010). During winter season sulphate concentration was maximum found at sampling station S5 (5.59 mg/l) followed by S4 (5.22 mg/l), S1 (5.17 mg/l), S2 (4.83 mg/l) and minimum value of sulphate ions was found at sampling station S3 (4.00 mg/l) (Table 1). In summer seasons, maximum concentration of sulphate was observed from sampling station S5 (8.142 mg/l) followed by S1 (6.29 mg/l), S3 (5.159 mg/l), S4 (4.764 mg/l) and minimum value of sulphate was observed from sampling station S2 (4.397 mg/l) (Table 1). In monsoon season, highest value of sulphate was observed from sampling station S5 (5.689 mg/l) followed by S1 (5.291 mg/l), S4 (4.98 mg/l), S2 (4.76 mg/l) and lowest value of sulphate was observed from sampling station S3 (3.91 mg/l) (Table 1). In context of seasonal fluctuations, highest average sulphate concentration was found during summer (5.75 mg/l) followed by winter (4.96 mg/l) and lowest sulphate concentration was found during monsoon season (4.926 mg/l) (Table 1).

Similar to our findings, Bhatnagar & Bharadwaj (2016) analyzed physiochemical parameters of Kota barrage, Kota and observed that the sulphate concentration was found to be higher during summer season. Nandal et al. (2020) assessed water quality parameters of pond water of Kalanpur Block, Haryana along the different sampling stations and they find that value of sulphate content was ranged between 53 mg/l to 454.9 mg/l. Our observations exhibited very low sulfate content in the Chawandia village pond. Subsequently, Kumar et al. (2026) assessed sulphate levels in Gambhiri River, Chittorgarh and found that its level ranged between 28.19±4.58 mg/l to 47.64±3.91 mg/l along the different seasons.

- Nitrate (NO3-)

Nitrate (NO3-) is a highly stable form of nitrogen that boosts plankton growth, density and primary productivity. Concentration of nitrate ions was below the detectable limit (<0.1 mg/l) at all five sampling stations of the Chawandia village pond in all three seasons, namely, winter, summer and monsoon (Table 1). Patel et al. (2018) studied physicochemical parameters of Mahi Bajaj Sagar Dam, Banswara and concluded that the value of nitrate varied from 0.07 to 0.33 mg/l along the all five sampling stations. Mishra et al. (2019) analyzed physicochemical parameters of Pichola lake, Udaipur and they found that the value of nitrate content in Pichola lake was ranged 0.44 to 0.71 mg/l, with an average of 0.60 mg/l along all four sampling stations.

- Dissolved oxygen (DO):

Dissolved oxygen (DO) is an important water quality parameter. It can determine the level of water contamination based on the lack of oxygen in water and can also indicate increased use of oxygen by organic substances in water (Cantor, 2009). Temperature plays an important role in determining the dissolved oxygen level in water. Amount of oxygen that water can hold depends on temperature, salinity and water flow pressure (Kale, 2016). Availability of dissolved oxygen affects nutrient accessibility, which in turn affect the productivity of entire aquatic ecosystem. A high level of pollution can significantly decrease the dissolved oxygen level in a particular water body (Yeole & Patil, 2005). Oxygen's solubility in water is low, and it is highly responsive to temperature variations. Concentration of dissolved oxygen ranged between 3.4 mg/l to 4.0 mg/l during winter seasons, whereas summer season exhibited decreasing trends in dissolved oxygen concentration and value ranged between 2.9 mg/l to 3.8 mg/l (Table 1). In monsoon season, dissolved oxygen concentration exhibited elevated trends and values ranged from 3.8 mg/l to 4.20 mg/l along the different sampling stations of Chawandia village pond (Table 1). In the context of seasonal fluctuations, highest average dissolved oxygen concentration was found during monsoon season (4.02 mg/l) followed by winter (3.68 mg/l) and lowest dissolved oxygen concentration was found during summer season (3.24 mg/l) (Table 1).

Sahni & Yadav (2012) conducted a study on dissolved oxygen (DO) levels in Bharawas pond, Rewari, Haryana and found that value of dissolved oxygen ranged from 4.73 to 5.34 mg/l. Mishra et al. (2019) analyzed physicochemical parameters of Pichola lake, Udaipur and they found that value of dissolved oxygen content in Pichola lake was ranged between 5.28 to 6.63 mg/1 with average value of 5.95 mg/l. Subsequently, Beniwal & Jain (2024) conduct a study on physiochemical parameters of Kot dam, Jhunjhunu, Rajasthan and measure dissolved oxygen concentration of water along the three sampling stations. They found that dissolved oxygen level ranged between 2.67 to 4.06 mg/l at sampling station I, from 2.67 to 4.72 mg/l at station II and from 2.40 to 3.06 mg/l at station III.

According to Chaurasia & Pandey (2007), factors such as water temperature and air partial pressure can have a direct or indirect impact on dissolved oxygen level in water. Low levels of dissolved oxygen are linked to self-purification ability of flowing water, photosynthetic capacity of aquatic plants, airflow and various other factors (Sharma et al., 2016; Shenoy et al., 2020). Dirican (2015) noted that deeper water bodies tend to have lower concentrations of dissolved oxygen than surface waters. During monsoon season, elevated dissolved oxygen levels in water are primarily attributed to increased aeration from rainfall, water movement and abundant sunlight (Naik et al., 2020; Rajesh & Rehana, 2022). During study, it was noted that the dissolved oxygen level was higher during monsoon season than in other seasons because of increased aeration from rainfall and water movement in pond.

Figure 1: Map showing different sampling station for the collection of water sample

Table 1: seasonal variation on physicochemical parameters of Chawandiya pond, Bhilwara district (S- Sampling site)

| Season | Parameters | Unit | Different sampling sites of Chawandia pond |  |  |  |  | Average of all sampling sites |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Winter 2024-25 |  |  | S1 | S2 | S3 | S4 | S5 |  |
|  | pH (at 25°C) | - | 6.24 | 6.34 | 6.25 | 6.75 | 6 | 6.31 |
|  | Temperature | °C | 16.21 | 16.14 | 16.16 | 16.12 | 16.13 | 16.15 |
|  | Electrical Conductivity | mS/cm | 230 | 220 | 210 | 230 | 240 | 226 |
|  | Total Dissolve Solid | mg/l | 146 | 140 | 132 | 145 | 150 | 142.6 |
|  | Total hardness as CaCo3 | mg/L | 73 | 68 | 64 | 72 | 78 | 71 |
|  | Calcium as Ca2+ | mg/L | 22.8 | 20.4 | 19.2 | 22.8 | 23.6 | 21.76 |
|  | Magnesium as Mg2+ | mg/L | 3.84 | 4.08 | 3.84 | 3.6 | 4.56 | 3.98 |
|  | Alkalinity | mg/L | 92 | 88 | 84 | 90 | 98 | 90.4 |
|  | Chlorides as Cl- | mg/L | 11.99 | 10.99 | 10.99 | 11.99 | 13.99 | 11.99 |
|  | Fluoride as F- | mg/L | 0.14 | 0.16 | 0.12 | 0.14 | 0.16 | 0.144 |
|  | Sulphate as SO42- | mg/L | 5.17 | 4.83 | 4 | 5.22 | 5.59 | 4.962 |
|  | Nitrate as NO3- | mg/L | BDL(<0.1) | BDL(<0.1) | BDL(<0.1) | BDL(<0.1) | BDL(<0.1) | BDL(<0.1) |
|  | Dissolve oxygen (D.O.) | mg/L | 4 | 3.6 | 3.8 | 3.4 | 3.6 | 3.68 |
| Summer 2025 | pH (at 25°C) | - | 6.36 | 6.4 | 6.3 | 6.2 | 6.1 | 6.272 |
|  | Temperature | °C | 30.01 | 29.91 | 29.21 | 29.84 | 29.86 | 29.76 |
|  | Electrical Conductivity | mS/cm | 250 | 240 | 230 | 230 | 260 | 242 |
|  | Total Dissolve Solid | mg/L | 160 | 152 | 146 | 145 | 165 | 153.6 |
|  | Total hardness as CaCo3 | mg/L | 80 | 80 | 76 | 76 | 84 | 79.2 |
|  | Calcium as Ca2+ | mg/L | 24 | 22.4 | 22.4 | 22.4 | 22 | 22.64 |
|  | Magnesium as Mg2+ | mg/L | 4.8 | 5.76 | 4.8 | 4.8 | 5.76 | 5.18 |
|  | Alkalinity | mg/L | 107.3 | 103.6 | 99.9 | 99.2 | 111 | 104.2 |
|  | Chlorides as Cl- | mg/L | 13.79 | 13.79 | 11.82 | 11.82 | 13.79 | 13 |
|  | Fluoride as F- | mg/L | 0.16 | 0.15 | 0.14 | 0.15 | 0.2 | 0.16 |
|  | Sulphate as SO42- | mg/L | 6.291 | 4.397 | 5.159 | 4.764 | 8.142 | 5.75 |
|  | Nitrate as NO3- | mg/L | BDL(<0.1) | BDL(<0.1) | BDL(<0.1) | BDL(<0.1) | BDL(<0.1) | BDL(<0.1) |
|  | Dissolve oxygen (D.O.) | mg/L | 3.8 | 3.5 | 2.9 | 3 | 3 | 3.24 |
| Monsoon 2025 | pH (at 25°C) | - | 6.42 | 6.24 | 6.37 | 6.25 | 6.4 | 6.33 |
|  | Temperature | °C | 23.21 | 23.02 | 22.9 | 23.18 | 23.22 | 23.1 |
|  | Electrical Conductivity | mS/cm | 210 | 200 | 190 | 210 | 220 | 206 |
|  | Total Dissolve Solid | mg/L | 135 | 127 | 125 | 137 | 145 | 133.8 |
|  | Total hardness as CaCo3 | mg/L | 70 | 65 | 62 | 71 | 75 | 68.6 |
|  | Calcium as Ca2+ | mg/L | 20.4 | 18.8 | 18.8 | 21.6 | 22 | 20.32 |
|  | Magnesium as Mg2+ | mg/L | 4.56 | 4.32 | 3.6 | 4.08 | 4.8 | 4.27 |
|  | Alkalinity | mg/L | 84 | 80 | 76 | 82 | 90 | 82.4 |
|  | Chlorides as Cl- | mg/L | 12.99 | 11.99 | 9.99 | 11.99 | 13.99 | 12.19 |
|  | Fluoride as F- | mg/L | 0.16 | 0.14 | 0.15 | 0.16 | 0.18 | 0.15 |
|  | Sulphate as SO42- | mg/L | 5.29 | 4.76 | 3.91 | 4.98 | 5.68 | 4.926 |
|  | Nitrate as NO3- | mg/L | BDL(<0.1) | BDL(<0.1) | BDL(<0.1) | BDL(<0.1) | BDL(<0.1) | BDL(<0.1) |
|  | Dissolve oxygen (D.O.) | mg/L | 4.2 | 3.9 | 3.8 | 4 | 4.2 | 4.02 |

## Conclusions

The physicochemical characteristics of Chawandiya Pond varied across winter, summer and monsoon. Summer was associated with the highest mean temperature, total dissolved solids, hardness, calcium, magnesium, alkalinity, chloride, fluoride and sulphate. In contrast, the highest mean pH and dissolved oxygen values were recorded during monsoon. Nitrate remained below the reported detection limit at all stations and in all seasons. These findings provide baseline information on spatial and seasonal patterns in the measured water-quality variables. However, the observations should be interpreted descriptively because statistical significance was not established. Continued monitoring using clearly replicated sampling, analytical quality control and biological indicators is needed to assess long-term water-quality trends and ecological condition.

## Limitations

The investigation covered one annual cycle and five sampling stations in a single pond. The number of monthly and laboratory replicates was not clearly stated, and measures of variability were not presented. Seasonal and spatial differences were not evaluated statistically. Biological, microbiological, heavy-metal, BOD and COD measurements were not included. Pond depth, water level, rainfall, inflow and surrounding land-use pressures were also not quantified. Therefore, the study describes seasonal physicochemical patterns but does not provide a complete assessment of ecological health or water suitability for a specified use.

Declaration of AI Use

This manuscript was prepared through the combined contributions of all author(s), including contributions to the study design, data, content development, results, interpretation, and related scholarly work. The author(s) acknowledge the use of Grammarly and ChatGPT to assist with grammar checking, language refinement, reference formatting. These AI-assisted tools were not used as authors and did not replace the intellectual contributions or scholarly judgment of the author(s). All AI-assisted outputs, including content, references, and interpretations, were carefully reviewed, revised, verified, and approved by the author(s). The author(s) accept full responsibility for the accuracy, integrity, and final content of the manuscript.

Top of Form

References

- Acharya, G.D., Hathi, M.V., Patel, A.D. & Parmar, K.C. (2008). Chemical properties of groundwater in Bhiloda Taluka Region, North Gujarat. India. E-Journal of Chemistry, 5(4), 792-796.

- Ahipathi, M.V. & Puttaiah, E.T. (2006). Ecological Characteristics of Vrishabhavathi River in Bangalore (India). Environmental Geology, 49, 1217-1222.

- Ali, S., Thakur, S.K., Sarkar, A. & Shekhar, S. (2016). Worldwide contamination of water by fluoride. Environmental Chemistry Letters, 14, 291-315.

- Alley, E.R. (2007). Water Quality Control Handbook. Volume- 2. New York: McGrawHill.

- Ansari, K.K. & Prakash, S. (2000). Limnological studies on Tulsidas Tal of Tarai region of Balrampur in relation to fisheries. Pollution Research, 19(4), 651-655.

- A.P.H.A. (2012). Standard Methods for the Examination of Water and Wastewater. 22nd Ed. American Public Health Association (APHA), American Water Works Association (AWWA) and Water Environment Federation (WEF), Washington D.C., U.S.A.

- Basavarajappa, S.H., Raju, N.S., Hosmani S.P. & Niranjana, S.R. (2011). Freshwater diatoms as indicators of water quality of some important lakes of Mysore. Indian Hydrobiology, 14(1), 42-52.

- Belghiti, M. L., Chahlaoui, A. & Bengoumi, D. (2013). Study of the Physicochemical and Bacteriological Quality of Groundwater from the Plio-Quaternary Aquifer in the Meknes Region (Morocco). Larhyss Journal, 14, 21-36.

- Beniwal V.D. & Jain S. (2024). Seasonal Variation in Physicochemical Parameters of Kot Dam, Jhunjhunu, Rajasthan, India. Bulletin of Pure and Applied Sciences-Zoology, 43A(2), 348-363.

- Benrabah, S., Attoui, B. & Hannouche, M. (2016). Characterization of groundwater quality destined for drinking water supply of Khenchela City (Eastern Algeria). Journal of Water and Land Development, 30, 13–20.

- Bhateria, R. & Jain, D. (2016). Water quality assessment of lake water: a review. Sustainable Water Resources Management, 2(2), 161-173.

- Bhatnagar, M. & Bharadwaj, N. (2016). Seasonal variations in physico-chemical parameters of Kota barrage, Kota Rajasthan. International Journal of Advanced Research in Arts, Science, Engineering & Management (IJARASEM), 3(3), 1035-1043.

- Boyd, C.E. (2015). Water Quality: An Introduction (Second ed.) Switzerland; Springer. ISBN 978-3-319-17445-7.

- Cantor, A. (2009). Water Quality Parameters. Water Distribution System Monitoring, 113–128.

- Chaurasia, M. & Pandey, G.C. (2007). Study of Physico-chemical characteristics of some water ponds of Ayodhya, Faizabad. Indian Journal of Environmental Protection, 27(11), 1019-1023.

- Chishty, N. & Choudhary, N.L. (2022a). Seasonal Dynamic in Primary Productivity and Phytoplankton Diversity in Berach River System of Udaipur District, Rajasthan. Asian Journal of Biological and Life Sciences, 11(1), 82-92.

- Chishty, N. & Choudhary, N.L. (2022b). Seasonal Dynamic of Physicochemical Parameters of Berach River system in Udaipur district, Rajasthan. Toxicology International, 29(3), 379-391.

- Dagaonkar, A. & Saksena, D.N. (1992). Physicochemical and biological characterization of a temple tank, Kaila Sagar, Gwalior, Madhya Pradesh. Hydrobiological Journal, 8(1), 11-19.

- Dahiya, S. & Kaur, A. (1999). Physicochemical characteristics of underground water in rural areas of Tosham subdivisions, Bhiwani district, Haryana. Journal of Environment Pollution, 6(4), 281.

- Davis, M.L. & David, A. (2008). Introduction to Environmental Engineering. 4th ed. New York: McGraw-Hill Companies.

- Davis, M.L. (2010). Water and Wastewater Engineering-Design Principles and Practice. New York: McGraw-Hill.

- Dewangan, S. K., Shrivastava, S., Kadri, M., Saruta, S., Yadav, S. & Minj, N. (2023). Temperature effect on electrical conductivity (EC) and total dissolved solids (TDS) of water: A review. International Journal of Research Analysis and Review, 10(2), 514-520.

- Dey, S., Botta, S., Kallam, R., Angadala, R. & Andugala, J. (2021). Seasonal variation in water quality parameters of Gudlavalleru Engineering College Pond. Current Research in Green and Sustainable Chemistry, 4, 100058.

- Dhoke, S. K. (2023). Determination of alkalinity in the water sample: a theoretical approach. Chemistry Teacher International, 5(3), 283-290.

- Dirican, S. (2015). Assessment of water quality using physico-chemical parameters of Çamlıgöze Dam Lake in Sivas, Turkey. Ecologia, 5(1), 1-7.

- Gawle, S., Pateria, K. & Mishra, R.P. (2021). Physico-chemical analysis of groundwater during monsoon and winter season of Dindori district, India. Groundwater for Sustainable Development, 12, 100550.

- Hammoumi, D., Al-Aizari, H. S., Alaraidh, I. A., Okla, M. K., Assal, M. E., Al-Aizari, A. R. & Bejjaji, Z. (2024). Seasonal variations and assessment of surface water quality using water quality index (WQI) and principal component analysis (PCA): a case study. Sustainability, 16(13), 5644.

- Hao, X., Shi, X., Zhao, S., Yu, H., Kang, R., Han, Y., ... & Wang, S. 2024. Impacts of temperature and nutrient dynamics on phytoplankton in a lake: A case study of Wuliangsuhai Lake, China. Sustainability, 16(24), 11195.

- Haritash, A. K., Gaur, S. & Garg, S. 2016. Assessment of water quality and suitability analysis of River Ganga in Rishikesh, India. Applied Water Science, 6, 383-392.

- Harrison, P.T. 2005. Fluoride in water: a UK perspective. Journal of Fluorine Chemistry, 126(11-12), 1448-1456.

- Horne, A.J., Goldman, & Charles, R. (1994). Limnology (second ed.). United States of America: McGraw-Hill.  ISBN 978-0-07-023673-8.

- Islam, R., Faysal, S. M., Amin, R., Juliana, F. M., Islam, M. J., Alam, J., ... & Asaduzzaman, M. (2017). Assessment of pH and total dissolved substances (TDS) in the commercially available bottled drinking water. IOSR Journal of Nursing and health Science, 6(5), 35-40.

- Jagtap, S., Yenkie, M.K., Labhsetwar, N. & Rayalu, S. (2012). Fluoride in drinking water and defluoridation of water. Chemical Reviews, 112(4), 2454-2466.

- Jalal, F.N. & Sanalkumar, M.G. (2012). Hydrology and water quality assessment of Achencovil River in relation to pilgrimage season. International Journal of Scientific and Research Publications, 2(12), 1-5.

- Jhingran, V.G. (1982). Fish and fisheries in India. Hindustan Publishing Corporation, Delhi.

- Joshi, D. M., Kumar, A. & Agrawal, N. (2009). Studies on physicochemical parameters to assess the water quality of river Ganga for drinking purpose in Haridwar district. Rasayan Journal of Chemistry, 2(1), 195-203.

- Kale, V.S. (2016). Consequence of temperature, pH, turbidity and dissolved oxygen water quality parameters. International Advanced Research Journal in Science, Engineering and Technology, 3(8), 186-190.

- Kovalenko, V.F. & Sova, A. M. (2024). Effects of Calcium and Magnesium Ion Ratios in Natural and Drinking Water on the Vitality of Test Organisms. Journal of Water Chemistry and Technology, 46(4), 414-418.

- Krishna Kumar, S., Hari Babu, S., Eswar Rao, P., Selvakumar, S., Thivya, C., Muralidharan, S. & Jeyabal, G. (2017). Evaluation of water quality and hydrogeochemistry of surface and groundwater, Tiruvallur District, Tamil Nadu, India. Applied Water Science, 7, 2533-2544.

- Kumar, A., Tripathi, V.K., Sachan, P., Rakshit, A., Singh, R.M., Shukla, S.K. & Panda, K.C. (2022). Sources of ions in the river ecosystem. In Ecological significance of river ecosystems Elsevier, 187-202.

- Kumar, S., Veerwal, B. & Garwa, H.  (2026). Assessment of water quality and zooplankton community structure in the Gambhiri River (Chittorgarh, Rajasthan, India). Limnology and Freshwater Biology, 3; 148-158. : https://doi.org/10.31951/2658-3518-2026-A-3-148

- Mahananda, H.B., Mahananda, M.R. & Mohanty, B.P. (2005). Physicochemical and biological parameters of a fresh water pond ecosystem as an indicator of water pollution. Ecology Environment and Conservation, 11(3-4), 537-541.

- Makhoukh, M., Sbaa, M., Berrahou, A. & Clooster, M.V. (2011). Contribution to the physico-chemical study of the surface water of oued Moulouya, oriental Morocco. Larhyss Journal, 9, 149-169.

- Masindi, V. & Muedi, K.L. (2018). Environmental contamination by heavy metals. Heavy metals, 10(4), 115-133.

- Matini, L., Moutou, J.M. & Kongo-Mantono, M.S. (2009). Hydro - chemical evaluation of groundwater in urban areas south-west of Brazzaville, Congo. Afrique Science, 5(1), 82–98.

- Mei, X., Gao, S., Liu, Y., Hu, J., Razlustkij, V., Rudstam, L. G., ... & Zhang, X. (2022). Effects of elevated temperature on resources competition of nutrient and light between benthic and planktonic algae. Frontiers in Environmental Science, 10, 908088.

- Mishra, V., Sharma, S.K., Sharma, B.K., Shukla, A. & Shukla, B.N. (2019). Zooplanktonic fauna in relation to physico-chemical characteristics in Lake Pichhola, Udaipur, Rajasthan, India. Journal of Pharmacognosy and Phytochemistry, 8(1), 155-158.

- Muhammad, A., Salam, A., Azeem, A., Shafiq, M. & Khan, B.A. (2000). Studies on the effect of seasonal variations on physical and chemical characteristics of mixed water from rivers Ravi and Chenab at union site in Pakistan. Journal of Research Science, 11(1), 11-17.

- Naik, S., Mishra, R.K., Sahu, K.C., Lotliker, A.A., Panda, U.S. & Mishra, P. (2020). Monsoonal influence and variability of water quality, phytoplankton biomass in the tropical coastal waters–a multivariate statistical approach. Frontiers in Marine Science, 7, 648.

- Nandal, A., Kaushik, N., Yadav, S. S., Rao, A. S., Singh, N. & Gulia, S. S. (2020). Water quality assessment of pond water of Kalanaur block, Rohtak, Haryana. Indian Journal of Ecology, 47(1), 1-6.

- Nayar, R. (2020). Assessment of Water Quality Index and Monitoring of Pollutants by Physico-Chemical Analysis in Water Bodies: A Review. International Journal of Engineering Research and Technology (IJERT), 9(1), 178–185.

- Pandey, S.C., Singh, S.M., Pani, S. & Malhosia, A. (2012). Limnology: A Case Study Of Highly Polluted Laharpur Reservoir, Bhopal, (M.P.) India. Journal Of Chemical, Biological and Physical Sciences, 2(3), 1560-1566

- Parashar, C., Dixit, S. & Shrivastava, R. (2006). Seasonal variations in physico-chemical characteristics in upper lake of Bhopal. Asian Journal of Experimental Sciences, 20(2), 297-302.

- Patel, R.K., Sharma, B.K., Sharma, S.K., Uppadhyay, B. & Yadav, J.P. (2018). Study on certain physico-chemical parameters of Mahi Bajaj Sagar dam, Banswara, Rajasthan. Journal of Entomology and Zoology Studies, 6(5), 296-303.

- Patil, P.N., Sawant, D.V. & Deshmukh, R.N. (2012). Physico-chemical parameters for testing of water – A review. International Journal of Environmental Sciences, 3(3), 1194- 1207.

- Polat, M. (1997(. Physical and chemical parameters monitored rivers and lakes. Proceedings of the Water Quality Management Seminar, May 15, 1997, Ankara, Turkey, Pp: 45-57.

- Qureshimatva, U. M., Maurya, R. R., Gamit, S. B., Patel, R. D. & Solanki, H. A. (2015). Determination of physico-chemical parameters and water quality index (Wqi) of Chandlodia Lake, Ahmedabad, Gujarat, India. Journal of Environmental & Analytical Toxicology, 5(288), 2161-0525.

- Radhika, C.G., Mini, I. & Gangadevi, T. (2004). Studies on abiotic parameters of a tropical fresh water lake-Vellayani Lake, Trivandrum, Kerala. Pollution Research, 23(1), 49- 63.

- Rahman, A., Jahanara, I. & Jolly, Y.N. (2021). Assessment of physico- chemical properties of water and their seasonal variation in an urban river in Bangladesh. Water Science and Engineering, 14(2), 139-148.

- Rajesh, M. & Rehana, S. (2022). Impact of climate change on river water temperature and dissolved oxygen: Indian riverine thermal regimes. Scientific Reports, 12(1), 9222.

- Rehman, K., Fatima, F., Waheed, I. & Akash, M. S. H. (2018). Prevalence of exposure of heavy metals and their impact on health consequences. Journal of Cellular Biochemistry, 119(1), 157-184.

- Rubel, M.D., Islam, M.S., Akij, S.M. & Uddin, M. H. (2019). An assessment on different solids, dissolved oxygen in industrial effluents and its impact on public health. American Journal of Biomedical Science & Research, 5(5), 382-390.

- Sahni, K. & Yadav, S. 2012. Seasonal Variations in Physico-Chemical Parameters of Bharawas Pond, Rewari, Haryana. South Asian Journal of Experimental Biology, 26 (1), 61-64.

- Sedamkar, E. & Anagadi, S.D. (2003). Physico-chemical parameters of freshwater bodies of Gulbarga, India, with special reference to phytoplankton. Pollution Research, 22(3), 411-422.

- Sharma, C., Jindal, R., Singh, U.B., Ahluwalia, A.S. & Thakur, R.K. (2013). Population dynamics and species diversity of plankton in relation to hydrobiological characteristics of river Sutlej, Punjab India. Ecology Environment and Conservation, 19(3), 717–724.

- Sharma, L.L. & Sarang, N. (2004). Physicochemical limnology and productivity of Jaisamand Lake, Udaipur, Rajasthan. Pollution Research, 23(1), 87-92.

- Sharma, R.C., Singh, N. & Chauhan, A. (2016). The influence of physico-chemical parameters on phytoplankton distribution in a head water stream of Garhwal Himalayas: a case study. Egyptian Journal of Aquatic Research, 42(1), 11-21.

- Shenoy, D.M., Suresh, I., Uskaikar, H., Kurian, S., Vidya, P. J., Shirodkar, G. & Naqvi, S.W.A. (2020). Variability of dissolved oxygen in the Arabian Sea Oxygen Minimum Zone and its driving mechanisms. Journal of Marine Systems, 204, 103310.

- Singh, K.P. & Saxena, A. (2025). Seasonal variations and correlation analysis of water quality parameters in the Chambal River, Rajasthan, India. Water Environment Research, 97(2), e70035.

- Soni, H.B. & Thomas, S. (2013). Preliminary assessment of surface water quality of tropical pilgrimage wetland of central Gujarat, India. International Journal of Environment, 2(1), 202-223.

- Tank, S.K. & Chippa, R. C. (2013). Analysis of water quality of Halena block in Bharatpur area. International Journal of Scientific and Research Publications, 3(3), 1-6.

- Tchobanoglous, G., Burton, F.L. & Stensel, H.D. (2003). Metcalf and Eddy Wastewater Engineering: Treatment and Reuse. 4th ed. New Delhi: Tata McGraw-Hill Limited.

- Thompson, P. A., Guo, M. X. & Harrison, P. J. (1992). Effects of variation in temperature. I. On the biochemical composition of eight species of marine phytoplankton. Journal of Phycology, 28(4), 481-488.

- Venkatasubramani, R. & Meenambal, T. (2007). Study on subsurface water quality in Mettupalayam Taluka of Coimbatore District, Tamil Nadu. Nature Environment and Pollution Technology, 6(2), 307–310.

- Wetzel, R. & Likens, G.E. (2000). Limnological Analysis, 3rd ed., New York: Springer Verlag.

- Yeole, S.M. & Patil, G.P. (2005). Physico-chemical status of Yedshi lake in relation to water pollution. Journal of Aquatic Biology, 20(1), 41-44.

- Zeighami, A. &  Kurylyk, B. L. (2025). Modelled Water Temperature Patterns and Energy Balance of a Threatened Coastal Lagoon Ecosystem. Hydrological Processes, 39(2), e70068. https://doi.org/10.1002/hyp.70068

- Zhang YanHao, Z. Y., Zhang HaoHan, Z. H., Zhang ZhiBin, Z. Z., Liu ChengYing, L. C., Sun CuiZhen, S. C., Zhang Wen, Z. W. & Marhaba, T. (2018). pH effect on heavy metal release from a polluted sediment. Journal of Chemistry, 1, 1-7.