Comparative Evaluation of Selected Indoor Ornamental Plants for Improving Indoor Air Quality under Controlled Conditions

ABSTRACT

Indoor air quality significantly affects human health and well-being, particularly in enclosed urban environments where ventilation is limited and pollutants may accumulate. Indoor plants have been reported to improve indoor air quality through pollutant absorption by leaf surfaces and root-associated microbial activity. This study investigated the influence of selected indoor plants, namely areca palm (Dypsis lutescens), spider plant (Chlorophytum comosum), and money plant (Epipremnum aureum), on indoor air quality in a controlled room environment. Unlike many previous studies that evaluated individual plant species, the present study comparatively assessed single-species and multi-species ornamental plant combinations under identical controlled indoor conditions using multiple indoor air quality parameters. The study included both single-species and multi-species combinations monitored over a three-day experimental period. Key air quality parameters, including particulate matter (PM2.5 and PM10), carbon dioxide (CO2), formaldehyde (HCHO), total volatile organic compounds (TVOCs), and the overall air quality index (AQI), were measured before human occupancy, after human occupancy, and following overnight plant exposure. The results suggest that combined plant treatments may provide greater improvements than single-species treatments. The study also highlights fundamental limitations associated with small-scale indoor environments, including a limited leaf-area-to-volume ratio and a short exposure duration. The results indicated that all treatments improved indoor air quality, whereas multi-species combinations demonstrated greater pollutant removal efficiency. One-way analysis of variance (ANOVA) revealed significant differences among treatments for PM2.5 and TVOCs (P < 0.05) and PM10 (P < 0.001), with multi-species plant combinations generally showing greater reductions. The greatest reductions in PM2.5 (34.9%), PM10 (36.4%), and TVOCs (50.0%) were observed in multi-species plant treatments, indicating their superior performance compared with individual plant species. These findings suggest that strategically selected multi-species indoor ornamental plant combinations may serve as an environmentally sustainable, low-cost supplementary approach for improving indoor air quality in residential, educational, and office environments.

KEYWORDS

Indoor air quality; Indoor ornamental plants; Phytoremediation; PM2.5; Carbon dioxide (CO2); Total volatile organic compounds (TVOCs); Multi species plant combinations

Introduction

In modern urban settings, people spend a considerable proportion of their time in enclosed indoor environments such as homes, offices, educational institutions, and commercial buildings. Consequently, indoor air quality (IAQ) has become an important determinant of human health, comfort, well-being, and productivity. Poor indoor air quality has been associated with a wide range of adverse health effects, including respiratory disorders, allergic reactions, headaches, fatigue, and reduced cognitive performance (Deng & Deng, 2018; Han & Ruan, 2020; Kumar et al., 2023; Ravindra & Mor, 2022).

According to the World Health Organization (WHO, 2010), indoor air pollution remains a major environmental health risk worldwide, particularly in poorly ventilated buildings.

Indoor air pollution originates from multiple sources, including inadequate ventilation, human occupancy, building materials, household products, cooking activities, and outdoor pollutant infiltration.

In densely occupied indoor environments, particulate matter and gaseous pollutants often accumulate at concentrations that may exceed outdoor levels. Carbon dioxide generated through human respiration is commonly used as an indicator of ventilation efficiency, while particulate matter (PM2.5 and PM10), formaldehyde (HCHO), and total volatile organic compounds (TVOCs) are among the most important indoor pollutants affecting human health.

Conventional approaches to improving indoor air quality primarily rely on mechanical ventilation systems and air purification devices. Although these technologies are generally effective, they require continuous energy consumption, regular maintenance, and relatively high operational costs. Therefore, increasing attention has been directed towards sustainable and nature-based alternatives for improving indoor environmental quality (Bandehali et al., 2021; Brilli et al., 2018; Irga et al., 2018).

Among these alternatives, indoor ornamental plants have attracted considerable attention because they require minimal energy input while simultaneously providing environmental, aesthetic, and psychological benefits. Indoor plants can improve air quality through biological processes collectively referred to as phytoremediation. Pollutants may be removed through adsorption onto leaf surfaces, absorption through stomata, metabolism within plant tissues, and degradation by microorganisms associated with the rhizosphere (Bandehali et al., 2021; Brilli et al., 2018; Dela Cruz et al., 2014; Irga et al., 2018; Kim et al., 2018).

Several studies have demonstrated the potential of indoor plants to reduce airborne pollutants. Wolverton et al. (1989) reported the removal of volatile organic compounds such as benzene and formaldehyde by indoor plants under controlled conditions (Aydogan & Montoya, 2011; Liu et al., 2007; Orwell et al., 2004; Treesubsuntorn & Thiravetyan, 2012). Dela Cruz et al. (2014) further highlighted the ability of potted ornamental plants to reduce indoor VOC concentrations (Irga et al., 2013; Kim et al., 2010; Su & Liang, 2015; Tani & Hewitt, 2009). Yang et al. (2009) suggested that combinations of different plant species may improve pollutant removal efficiency because of complementary physiological characteristics and differences in pollutant uptake mechanisms.

A controlled study that assessed PM2.5, PM10, CO2, formaldehyde, and VOCs across several indoor plant treatments reported pollutant-specific differences among species, reinforcing the value of comparative evaluation using multiple air-quality indicators (Sarker et al., 2022). More recent investigations conducted in occupied or operational indoor settings have also reported that the magnitude of pollutant reduction varies with plant selection and environmental context, indicating that plant-based interventions should be evaluated under clearly defined experimental conditions (Elhadad et al., 2025; Madhusudanan et al., 2026).

Recent studies published in the Indian Journal of Ecology have further emphasised the role of indoor ornamental plants in sustainable indoor environmental management. Gautam et al. (2023) evaluated ornamental plant species suitable for indoor vertical gardens under different nutrient formulations, whereas Kumar et al. (2024) assessed the Air Pollution Tolerance Index (APTI) of indoor ornamental plants and recommended several species, including Epipremnum aureum, for interior landscaping and air quality improvement. These studies further demonstrate the increasing importance of indoor ornamental plants in sustainable indoor environmental management and provide a scientific basis for the comparative evaluation of different plant combinations.

Despite these advances, relatively few studies have systematically compared single-species and multi-species ornamental plant combinations under identical controlled indoor conditions while evaluating multiple indoor air quality parameters. Furthermore, quantitative information on the comparative performance of commonly available indoor ornamental plant combinations under controlled indoor conditions remains limited.

Therefore, the present study was undertaken to compare the effectiveness of selected indoor ornamental plant species and their combinations in improving indoor air quality under controlled indoor conditions. Air quality parameters, including PM2.5, PM10, CO2, HCHO, TVOCs, and AQI, were monitored to assess pollutant reduction efficiency. It was hypothesised that multi-species plant combinations would exhibit greater pollutant reduction efficiency than individual plant species because of their complementary pollutant uptake characteristics. The findings of this study are intended to provide practical scientific evidence for selecting effective indoor ornamental plant combinations as sustainable, low-cost solutions for improving indoor air quality in residential, educational, and office environments.

Materials and Methods

Study Area and Experimental Setup

The experiment was conducted in the Department of Biotechnology, Dronacharya Government College, Gurugram, Haryana, India, in a closed indoor room measuring approximately 8.0 m × 6.0 m × 3.5 m (volume: 168 m³). The room was maintained under normal indoor conditions, with the doors and windows kept closed during the experimental period except for routine entry and exit. No mechanical ventilation or air purification system was used throughout the study.

Plant Selection

Three commonly used indoor plant species were selected based on their availability, adaptability to indoor conditions, and frequent use in previous indoor air-quality studies.

1. Areca palm (Dypsis lutescens)

2. Spider plant (Chlorophytum comosum)

3. Money plant (Epipremnum aureum)

All plants were healthy, medium-sized, and grown in standard pots containing garden soil. The plants were acclimatised to indoor conditions for several days before the experiment to minimise stress. All selected plants were maintained in similarly sized pots containing comparable amounts of garden soil to ensure uniform plant health and growth conditions before the commencement of the experiment.

Air Quality Measurement Instrument

Indoor air quality parameters were measured using a portable air quality monitoring device (DeWire, MT26 NEW). The instrument was calibrated according to the manufacturer's instructions before the commencement of the experiment. It was used to record particulate matter (PM2.5 and PM10), carbon dioxide (CO2), formaldehyde (HCHO), total volatile organic compounds (TVOCs), and the overall air quality index. The same device was used consistently throughout the study to ensure comparability of the results. All measurements were recorded at the same fixed location within the room and at approximately breathing height to ensure consistency and minimise measurement variability throughout the experimental period.

Experimental Design

The study was conducted over multiple three-day experimental cycles, involving both single-plant treatments and species combinations. The experimental conditions were organised as follows:

- Areca palm alone

- Spider plant alone

- Money plant alone

- Areca palm and Money plant

- Areca palm and Spider plant

- Spider plant and Money plant

- Combination of all three plants

During each cycle, the plants were placed uniformly within the room to ensure equal exposure to indoor air. Each experimental condition was performed in triplicate, with each replicate conducted independently under similar indoor environmental conditions to improve the reliability and reproducibility of the observations. All experimental treatments were conducted under comparable conditions to minimise variability among experimental cycles.

Human Occupancy and Exposure Duration

Approximately 30 students occupied the room daily from 9:00 AM to 2:00 PM under normal classroom conditions. This period allowed indoor pollutants, such as carbon dioxide and particulate matter, to be generated through human occupancy. Students performed their routine classroom activities during the occupancy period, while the doors and windows remained closed except for normal entry and exit.

Air Quality Monitoring Procedure

Measurements of indoor air quality were conducted at three different time points during each experimental cycle:

- Baseline reading at 9:00 AM before human occupancy.

- Post-occupancy reading at 2:00 PM after continuous human occupancy.

- Overnight exposure reading at 9:00 AM on the following day after the plants had remained in the room overnight.

This protocol enabled the assessment of immediate and prolonged effects of indoor plants on air quality.

The 9:00 AM reading obtained after overnight exposure was treated as both the overnight value for the preceding day and the baseline value for the corresponding experimental day, as the room remained undisturbed during the night period. To maintain consistency, all measurements were recorded at the same position within the room using the same monitoring instrument throughout the study.

Data Recording and Analysis

All air quality readings were systematically recorded in tabular form. Comparative analysis was performed to evaluate changes in air quality parameters across different plant treatments. The effects of single plants were compared with those of plant combinations to determine their relative effectiveness. Statistical analysis was performed using one-way analysis of variance (ANOVA) to compare the effects of different plant treatments on indoor air quality parameters. Data are expressed as the mean ± standard deviation (SD) of three independent experimental replicates (n = 3). Differences were considered statistically significant at P < 0.05.

Ethical and Safety Considerations

This study was conducted under standard indoor conditions without the use of hazardous substances.

The indoor ornamental plants used in this study included Areca Palm (Dypsis lutescens), Spider Plant (Chlorophytum comosum), and Money Plant (Epipremnum aureum). These cultivated ornamental plants were obtained from local commercial plant nurseries in Gurugram, Haryana, India.

No wild plant species were collected, and no endangered or protected plant species were used in this study. The handling and use of plant materials complied with relevant institutional, local, and national guidelines and regulations.

No field collection permits or special licences were required for the use of these commercially cultivated ornamental plants.

The study protocol was reviewed and approved by the Department of Biotechnology, Dronacharya Government College, Gurugram, Haryana, India.

All methods were performed in accordance with the relevant institutional guidelines and regulations of Dronacharya Government College.

No human biological samples or invasive procedures were involved in the study. The study involved only non-invasive environmental monitoring under normal classroom conditions.

Results

The experimental investigation evaluated changes in indoor air quality parameters under controlled indoor conditions following exposure to different indoor ornamental plant treatments. Variations in particulate matter (PM2.5 and PM10), carbon dioxide (CO2), formaldehyde (HCHO), total volatile organic compounds (TVOCs), and the overall air quality index (AQI) were monitored before human occupancy, after occupancy, and following overnight plant exposure.

The results presented below represent the mean values obtained from three independent experimental replicates (n = 3). One-way analysis of variance (ANOVA) indicated significant differences among treatments for PM2.5 (P < 0.05), PM10 (P < 0.001), and TVOCs (P < 0.05), whereas the differences in CO2 and HCHO were not statistically significant (P > 0.05).

The mean values of indoor air quality parameters recorded under different plant treatments are presented in Table 1.

Table 1 shows that pollutant concentrations increased after human occupancy but decreased following overnight exposure to indoor plants. Multi-species plant combinations generally produced greater reductions in pollutant concentrations than individual plant species.

[Insert Table 1 here]

The percentage reduction in indoor air quality parameters after overnight plant exposure is presented in Table 2.

Among the evaluated treatments, multi-species plant combinations demonstrated greater reductions in PM2.5, PM10, and TVOCs than single-species treatments, indicating improved pollutant removal efficiency.

[Insert Table 2 here]

The percentage reduction in PM2.5 under different indoor plant treatments following overnight exposure is illustrated in Fig. 1. The Areca palm–Spider plant combination exhibited the highest reduction in PM2.5, whereas individual plant treatments showed comparatively lower reductions.

[Insert Fig. 1 here]

The percentage reduction in PM10 under different indoor plant treatments following overnight exposure is presented in Fig. 2. The areca palm–spider plant combination produced the greatest reduction in PM10 (36.4%), followed by the other multi-species combinations.

[Insert Fig. 2 here]

The variation in CO2 concentrations under different indoor plant treatments following overnight exposure is presented in Fig. 3. Although all treatments reduced indoor CO2 concentrations to some extent, the differences among treatments were relatively small.

[Insert Fig. 3 here]

The percentage reduction in HCHO under different indoor plant treatments following overnight exposure is shown in Fig. 4. HCHO concentrations decreased under selected treatments, although the reductions were relatively small because the baseline HCHO concentrations were already low. Differences among treatments were not statistically significant (P > 0.05).

[Insert Fig. 4 here]

The percentage reduction in TVOCs under different indoor plant treatments following overnight exposure is presented in Fig. 5. The three-plant combination recorded the highest TVOC reduction (50.0%), indicating its enhanced capacity to remove volatile organic compounds from indoor air.

[Insert Fig. 5 here]

Table 1. Mean (± SD) values of indoor air quality parameters recorded under different indoor plant treatments.

| Plant Treatment | Exposure Condition | PM2.5 (µg/m³) | PM10 (µg/m³) | CO2 (ppm) | HCHO (mg/m3) | TVOCs (mg/m3) |
| --- | --- | --- | --- | --- | --- | --- |
| Areca Palm (Dypsis lutescens) | Baseline | 16.6 | 23.3 | 430.3 | 0.001 | 0.007 |
|  | Overnight | 14.0 | 19.3 | 425.0 | 0.001 | 0.007 |
| Spider Plant (Chlorophytum comosum) | Baseline | 16.6 | 23.6 | 434.3 | 0.002 | 0.008 |
|  | Overnight | 14.0 | 19.6 | 427.6 | 0.001 | 0.006 |
| Money Plant (Epipremnum aureum) | Baseline | 21.3 | 26.3 | 439.0 | 0.003 | 0.009 |
|  | Overnight | 17.3 | 22.6 | 431.3 | 0.002 | 0.007 |
| Areca Palm and Money Plant | Baseline | 14.0 | 19.6 | 426.0 | 0.001 | 0.006 |
|  | Overnight | 10.0 | 14.0 | 417.6 | 0.001 | 0.004 |
| Areca Palm and Spider Plant | Baseline | 12.3 | 17.3 | 426.6 | 0.001 | 0.005 |
|  | Overnight | 8.0 | 11.0 | 420.0 | 0.001 | 0.003 |
| Spider Plant and Money Plant | Baseline | 13.3 | 17.3 | 425.6 | 0.001 | 0.005 |
|  | Overnight | 9.3 | 11.6 | 419.3 | 0.001 | 0.003 |
| Three-plant combination | Baseline | 14.6 | 18.0 | 416.3 | 0.001 | 0.004 |
|  | Overnight | 10.6 | 12.0 | 405.0 | 0.001 | 0.002 |

Values represent the mean of three independent experimental replicates. Baseline values were recorded before human occupancy (9:00 AM), whereas overnight values were recorded after overnight plant exposure (9:00 AM on the following day). PM2.5 = particulate matter ≤2.5 µm; PM10 = particulate matter ≤10 µm; CO2 = carbon dioxide; HCHO = formaldehyde; TVOCs = total volatile organic compounds.

Table 1 presents the mean baseline and overnight values of indoor air quality parameters recorded under different plant treatments. Reductions in particulate matter (PM2.5 and PM10), carbon dioxide (CO2), formaldehyde (HCHO), and total volatile organic compounds (TVOCs) were observed following overnight plant exposure. Multi-species plant combinations generally demonstrated greater improvements in indoor air quality than individual plant treatments. Among all treatments, the three-plant combination showed the most consistent reductions across the measured air quality parameters.

The baseline measurements indicated an increase in indoor pollutants following daily classroom occupancy, while overnight exposure to indoor plants resulted in reductions across most measured parameters. Among the individual species, the money plant showed greater reductions in PM2.5 and CO2 than the areca palm and spider plant. However, multi-species treatments produced greater overall improvements. The combination of areca palm and spider plant recorded the highest reductions in PM2.5 (34.9%) and PM10 (36.4%), whereas the three-plant combination achieved the lowest overnight CO2 concentration (405 ppm) and TVOC level (0.002 mg m⁻³), indicating broader pollutant removal efficiency.

Table 2. Percentage Reduction in Indoor Air Quality Parameters following Overnight Exposure under different Indoor Plant Treatments

| Plant Treatment | PM2.5 (%) | PM10 (%) | CO2 (%) | HCHO (%) | TVOCs (%) |
| --- | --- | --- | --- | --- | --- |
| Areca Palm (Dypsis lutescens) | 15.7% | 17.2% | 1.2% | 0.0% | 0.0% |
| Spider Plant (Chlorophytum comosum) | 15.7% | 16.9% | 1.5% | 50.0% | 25.0% |
| Money Plant (Epipremnum aureum) | 18.8% | 14.1% | 1.8% | 33.3% | 22.2% |
| Areca Palm and Money Plant | 28.6% | 28.6% | 2.0% | 0.0% | 33.3% |
| Areca Palm and Spider Plant | 34.9% | 36.4% | 1.5% | 0.0% | 40.0% |
| Spider Plant and Money Plant | 30.1% | 32.9% | 1.5% | 0.0% | 40.0% |
| Spider Plant, Money Plant and Areca Palm | 27.4% | 33.3% | 2.7% | 0.0% | 50.0% |

Percentage reduction was calculated by comparing baseline and overnight values for each treatment. Values represent the mean percentage reduction obtained from three independent experimental replicates. TVOCs = total volatile organic compounds.

Table 2 summarises the percentage reduction in indoor air quality parameters following overnight exposure under different plant treatments. Overall, multi-species plant combinations exhibited greater pollutant reduction efficiencies than individual plant species. The three-plant combination showed broad improvement across the measured parameters and achieved the greatest reduction in total volatile organic compounds, indicating the potential advantage of combining multiple indoor ornamental plant species to improve indoor air quality.

Percentage reduction analysis further confirmed the superior performance of multi-species plant combinations over individual plant treatments. Although reductions in CO2 were relatively small (1.2–2.7%), larger decreases were observed for particulate matter and VOCs. The three-plant combination achieved the highest VOC reduction (50%), while the areca palm–spider plant combination exhibited the maximum reductions in PM2.5 and PM10. These observations suggest that combining different ornamental plant species enhances overall indoor air purification efficiency.

Figure 1. Percentage reduction in PM2.5 following overnight exposure under different indoor plant treatments. The greatest reduction was observed in the Areca Palm + Spider Plant treatment (34.9%), whereas single-plant treatments showed comparatively lower reductions.

Figure 2. Percentage reduction in PM10 following overnight exposure under different indoor plant treatments. Multi-species combinations exhibited greater PM10 removal than individual plant treatments, with the Areca Palm + Spider Plant combination showing the highest reduction (36.4%).

Figure 3. Percentage reduction in CO2 following overnight exposure under different indoor plant treatments. The three-plant combination showed the highest reduction (2.7%), while single-plant treatments produced relatively smaller decreases.

Figure 4. Percentage reduction in HCHO following overnight exposure under different indoor plant treatments. Detectable reductions were observed only for Spider Plant (50.0%) and Money Plant (33.3%), whereas other treatments showed no measurable change.

Figure 5. Percentage reduction in TVOCs following overnight exposure under different indoor plant treatments. The three-plant combination achieved the greatest reduction (50.0%), indicating enhanced pollutant removal by mixed plant treatments.

Discussion

The results of this study demonstrate that indoor plants contribute to measurable improvements in indoor air quality under controlled conditions. Across all treatments, reductions in particulate matter (PM2.5 and PM10), carbon dioxide (CO2), formaldehyde (HCHO), and total volatile organic compounds (TVOCs) were observed after overnight plant exposure. However, the magnitude of improvement was moderate, indicating that indoor plants serve as a supplementary measure rather than a replacement for mechanical ventilation systems (Cummings & Waring, 2020; Persiani, 2021).

The observed improvements indicate that indoor ornamental plants can complement existing ventilation strategies by contributing to gradual reductions in indoor pollutant concentrations under occupied indoor conditions (Pettit et al., 2018; Pettit et al., 2019).

The results presented in Tables 1 and 2 indicate that all plant treatments improved indoor air quality after overnight exposure. The observed differences among plant treatments were further supported by one-way ANOVA, which revealed statistically significant differences for PM2.5, PM10, and TVOCs (P < 0.05), indicating that plant composition influenced pollutant removal efficiency under the experimental conditions. However, the magnitude of improvement varied among treatments. Multi-species plant combinations generally demonstrated greater reductions in PM2.5, PM10, CO2, and TVOCs than individual plant species. Among all treatments, the three-plant combination showed the most consistent overall improvement, suggesting that combining different ornamental plant species may enhance pollutant removal efficiency through complementary physiological characteristics.

Among the different treatments, the three-plant combination yielded the highest reductions in CO2 (2.7%) and TVOCs (50%), whereas the areca palm–spider plant combination showed the greatest reductions in PM2.5 (34.9%) and PM10 (36.4%). This may be attributed to increased leaf surface area and enhanced microbial activity in the root zone, which collectively improve pollutant absorption. Similar findings have been reported in previous studies, suggesting that plant diversity enhances the efficiency of indoor air purification (Irga et al., 2018; Pettit et al., 2019; Yang et al., 2009).

Recent studies published in the Indian Journal of Ecology have also highlighted the importance of indoor ornamental plants for sustainable indoor environmental management. Gautam et al. (2023) demonstrated the suitability of ornamental plant species for indoor vertical gardens, while a subsequent study on the Air Pollution Tolerance Index (APTI) emphasised the potential of indoor plants for interior landscaping and environmental quality improvement (Kumar et al., 2024). In addition to direct pollutant removal studies, the Air Pollution Tolerance Index (APTI) has been widely used to assess the tolerance of ornamental plants to air pollution. Kumar et al. (2024) reported that Epipremnum aureum and several other ornamental plant species exhibited comparatively higher APTI values, indicating their suitability for interior landscaping and pollution mitigation. Although APTI was not evaluated in the present investigation, integrating APTI with indoor air quality measurements in future studies may provide a more comprehensive understanding of the pollutant removal potential of indoor ornamental plants. Unlike previous Indian studies, which primarily focused on plant suitability and air pollution tolerance characteristics, the present study experimentally compared individual and multi-species ornamental plant combinations using multiple indoor air quality indicators under identical controlled indoor conditions, thereby providing practical information for indoor plant selection and deployment.

Among the measured parameters, particulate matter (PM2.5 and PM10) exhibited the highest percentage reductions, whereas CO2 showed smaller changes. Formaldehyde (HCHO) concentrations decreased only under selected treatments because the initial HCHO concentrations were already very low. Similarly, reductions in TVOCs were more pronounced in multi-species treatments, indicating their greater potential to remove volatile organic pollutants from indoor environments (Gawrońska & Bakera, 2015; Su & Liang, 2015; Tani & Hewitt, 2009; Torpy et al., 2014).

The present study has certain limitations. The experiment was conducted in a single controlled indoor room with approximately 30 occupants over a relatively short exposure period. Environmental factors such as temperature, humidity, and seasonal variation were not evaluated. Therefore, further studies involving larger indoor environments, different occupancy levels, and longer monitoring periods are required to validate the findings under practical conditions. In addition, seasonal variation and long-term plant performance were beyond the scope of the present investigation.

The absence of an unplanted control treatment should also be considered when interpreting the findings. Future studies should incorporate appropriate control treatments to strengthen experimental comparisons.

Overall, the findings indicate that indoor ornamental plants, particularly when used in multi-species combinations, may serve as a useful supplementary strategy for improving indoor air quality. Although they cannot replace mechanical ventilation systems, they provide an environmentally friendly, low-cost, and sustainable approach to enhancing indoor environmental quality in educational institutions, offices, and residential buildings.

Conclusion

The findings demonstrate that the selected indoor ornamental plants contributed to measurable reductions in several indoor air pollutants under the controlled experimental conditions. Multi-species treatments generally produced greater improvements than individual plant treatments. The combination of areca palm and spider plant achieved the highest reductions in PM₂.₅ (34.9%) and PM₁₀ (36.4%), while the three-plant combination produced the greatest reductions in TVOCs (50.0%) and CO₂ (2.7%). Differences among treatments were statistically significant for PM₂.₅, PM₁₀, and TVOCs, but not for CO₂ or HCHO. These results indicate that combining ornamental plant species with different characteristics may support broader pollutant reduction than using single species alone. However, the observed changes were moderate and were obtained within a single controlled indoor environment over a short monitoring period. Indoor ornamental plants should therefore be regarded as a supplementary component of indoor air-quality management rather than an alternative to mechanical ventilation or established air-purification measures. Further controlled studies are needed to evaluate their performance over longer periods and in larger occupied spaces.

## Limitation

This study was conducted in a single controlled room with approximately 30 occupants, a short monitoring period, and only three experimental replicates per treatment. Temperature, humidity, seasonal variation, and long-term plant performance were not evaluated. The findings should therefore be interpreted within the specific experimental conditions used and require confirmation in larger indoor spaces with longer monitoring periods and appropriate control treatments.

Declaration of AI Use

This manuscript was prepared through the combined contributions of all author(s), including contributions to the study design, data, content development, results, interpretation, and related scholarly work. The author(s) acknowledge the use of Grammarly and ChatGPT to assist with grammar checking, language refinement, reference formatting. These AI-assisted tools were not used as authors and did not replace the intellectual contributions or scholarly judgment of the author(s). All AI-assisted outputs, including content, references, and interpretations, were carefully reviewed, revised, verified, and approved by the author(s). The author(s) accept full responsibility for the accuracy, integrity, and final content of the manuscript.

Funding Declaration

The authors received no funding for this study.

Clinical trial number: Not applicable.

Data availability statement: The datasets generated and analysed during the current study are available as supplementary material.

Ethics approval: The study protocol was reviewed and approved by the Department of Biotechnology, Dronacharya Government College, Gurugram, Haryana, India.

The ornamental indoor plants used in this study were commercially purchased from local plant nurseries in Gurugram, Haryana, India. Therefore, no field collection permits or special licences were required.

References

Aydogan, A., & Montoya, L. D. (2011). Formaldehyde removal by common indoor plant species and various growing media. Atmospheric Environment, 45(16), 2675–2682. https://doi.org/10.1016/j.atmosenv.2011.02.062

Bandehali, S., Miri, T., Onyeaka, H., & Kumar, P. (2021). Current state of indoor air phytoremediation using potted plants and green walls. Atmosphere, 12(4), Article 473. https://doi.org/10.3390/atmos12040473

Brilli, F., Fares, S., Ghirardo, A., de Visser, P., Calatayud, V., Muñoz, A., Annesi-Maesano, I., Sebastiani, F., Alivernini, A., Varriale, V., & Menghini, F. (2018). Plants for sustainable improvement of indoor air quality. Trends in Plant Science, 23(6), 507–512. https://doi.org/10.1016/j.tplants.2018.03.004

Cummings, B. E., & Waring, M. S. (2020). Potted plants do not improve indoor air quality: A review and analysis of reported VOC removal efficiencies. Journal of Exposure Science & Environmental Epidemiology, 30(2), 253–261. https://doi.org/10.1038/s41370-019-0175-9

Dela Cruz, M., Christensen, J. H., Thomsen, J. D., & Müller, R. (2014). Can ornamental potted plants remove volatile organic compounds from indoor air? A review. Environmental Science and Pollution Research, 21(24), 13909–13928. https://doi.org/10.1007/s11356-014-3240-x

Deng, L., & Deng, Q. (2018). The basic roles of indoor plants in human health and comfort. Environmental Science and Pollution Research, 25, 36087–36101. https://doi.org/10.1007/s11356-018-3554-1

Elhadad, S. M., Shalaby, E. A., Saleh, I. H., & Omar, M. Y. (2025). Comparative evaluation of air phytoremediation potential of four ornamental potted plants for ecofriendly biofilter applications. Scientific Reports, 15, Article 45659. https://doi.org/10.1038/s41598-025-30145-8

Gautam, B., Dubey, R. K., Deepika, R., Dubey, M., & Singh, S. (2023). Evaluation of indoor ornamental plants suitable for indoor vertical garden in response to different nutrient formulations. Indian Journal of Ecology, 50(3), 690–694. https://doi.org/10.55362/IJE/2023/3954

Gawrońska, H., & Bakera, B. (2015). Phytoremediation of particulate matter from indoor air by Chlorophytum comosum L. plants. Air Quality, Atmosphere & Health, 8(3), 265–272. https://doi.org/10.1007/s11869-014-0285-4

Han, K.-T., & Ruan, L.-W. (2020). Effects of indoor plants on air quality: A systematic review. Environmental Science and Pollution Research, 27(14), 16019–16051. https://doi.org/10.1007/s11356-020-08174-9

Irga, P. J., Torpy, F. R., & Burchett, M. D. (2013). Can hydroculture be used to enhance the performance of indoor plants for the removal of air pollutants? Atmospheric Environment, 77, 267–271. https://doi.org/10.1016/j.atmosenv.2013.04.078

Irga, P. J., Pettit, T. J., & Torpy, F. R. (2018). The phytoremediation of indoor air pollution: A review on the technology development from the potted plant through to functional green wall biofilters. Reviews in Environmental Science and Bio/Technology, 17(2), 395–415. https://doi.org/10.1007/s11157-018-9465-2

Kim, K. J., Jeong, M. I., Lee, D. W., Song, J. S., Kim, H. D., Yoo, E. H., Jeong, S. J., Han, S. W., Kays, S. J., Lim, Y.-W., & Kim, H.-H. (2010). Variation in formaldehyde removal efficiency among indoor plant species. HortScience, 45(10), 1489–1495. https://doi.org/10.21273/HORTSCI.45.10.1489

Kim, K. J., Khalekuzzaman, M., Suh, J. N., Kim, H. J., Shagol, C., Kim, H.-H., & Kim, H. J. (2018). Phytoremediation of volatile organic compounds by indoor plants: A review. Horticulture, Environment, and Biotechnology, 59(2), 143–157. https://doi.org/10.1007/s13580-018-0032-0

Kumar, D., Singh, A., Viyol, S. V., Shah, H. P., Bhanderi, D. R., & Shrivastava, A. (2024). Studies on air pollution tolerance index of indoor plants for interior landscaping. Indian Journal of Ecology, 51(6), 1498–1501. https://doi.org/10.55362/IJE/2024/4431

Kumar, R., Verma, V., Thakur, M., Singh, G., & Bhargava, B. (2023). A systematic review on mitigation of common indoor air pollutants using plant-based methods: A phytoremediation approach. Air Quality, Atmosphere & Health, 16(8), 1501–1527. https://doi.org/10.1007/s11869-023-01326-z

Liu, Y.-J., Mu, Y.-J., Zhu, Y.-G., Ding, H., & Arens, N. C. (2007). Which ornamental plant species effectively remove benzene from indoor air? Atmospheric Environment, 41(3), 650–654. https://doi.org/10.1016/j.atmosenv.2006.08.001

Madhusudanan, S., Venkatesan, R., Babu, S., Nallusamy, S., Sathiyanarayanan, M., Zurek Mortka, M., & Szymanski, J. (2026). Enhancing the indoor air quality in an urban apartment as in the case of Chennai using native plants. Environmental Science and Pollution Research, 33, 1652–1665. https://doi.org/10.1007/s11356-026-37410-x

Orwell, R. L., Wood, R. L., Tarran, J., Torpy, F., & Burchett, M. D. (2004). Removal of benzene by the indoor plant/substrate microcosm and implications for air quality. Water, Air, & Soil Pollution, 157(1), 193–207. https://doi.org/10.1023/B:WATE.0000038896.55713.5b

Persiani, S. G. L. (2021). Benefits of using plants in indoor environments: Exploring common research gaps. Architecture, 1(2), 83–98. https://doi.org/10.3390/architecture1020008

Pettit, T. J., Irga, P. J., & Torpy, F. R. (2018). Functional green wall development for increasing air pollutant phytoremediation: Substrate development with coconut coir and activated carbon. Journal of Hazardous Materials, 360, 594–603. https://doi.org/10.1016/j.jhazmat.2018.08.048

Pettit, T., Irga, P. J., & Torpy, F. R. (2019). The in situ pilot-scale phytoremediation of airborne VOCs and particulate matter with an active green wall. Air Quality, Atmosphere & Health, 12(1), 33–44. https://doi.org/10.1007/s11869-018-0628-7

Ravindra, K., & Mor, S. (2022). Phytoremediation potential of indoor plants in reducing air pollutants. Frontiers in Sustainable Cities, 4, Article 1039710. https://doi.org/10.3389/frsc.2022.1039710

Sarker, M. M. R., Sammy, H., Khan, M., & Akhtar, N. (2022). Modeling to reduce the indoor air pollution in Dhaka: An evidence from randomized experiment of NASA recommended plants. International Journal of Environment and Climate Change, 12(2), 84–92. https://doi.org/10.9734/ijecc/2022/v12i230631

Su, Y., & Liang, Y. (2015). Foliar uptake and translocation of formaldehyde with bracket plants (Chlorophytum comosum). Journal of Hazardous Materials, 291, 120–128. https://doi.org/10.1016/j.jhazmat.2015.03.001

Tani, A., & Hewitt, C. N. (2009). Uptake of aldehydes and ketones at typical indoor concentrations by houseplants. Environmental Science & Technology, 43(21), 8338–8343. https://doi.org/10.1021/es9020316

Torpy, F. R., Irga, P. J., & Burchett, M. D. (2014). Profiling indoor plants for the amelioration of high CO2 concentrations. Urban Forestry & Urban Greening, 13(2), 227–233. https://doi.org/10.1016/j.ufug.2013.12.004

Treesubsuntorn, C., & Thiravetyan, P. (2012). Removal of benzene from indoor air by Dracaena sanderiana: Effect of wax and stomata. Atmospheric Environment, 57, 317–321. https://doi.org/10.1016/j.atmosenv.2012.04.016

World Health Organization. (2010). WHO guidelines for indoor air quality: Selected pollutants. https://www.who.int/publications/i/item/9789289002134

Wolverton, B. C., Johnson, A., & Bounds, K. (1989). Interior landscape plants for indoor air pollution abatement (Final report). National Aeronautics and Space Administration, John C. Stennis Space Center. https://ntrs.nasa.gov/citations/19930073077

Yang, D. S., Pennisi, S. V., Son, K.-C., & Kays, S. J. (2009). Screening indoor plants for volatile organic pollutant removal efficiency. HortScience, 44(5), 1377–1381. https://doi.org/10.21273/HORTSCI.44.5.1377