# Original Research Article

Green Integrated Enzyme–Ultrasound Extraction of Astaxanthin into Soybean Oil from Dried Haematococcus pluvialis: Enhanced Cell Disruption and Response Surface Modeling

.

ABSTRACT

Aims: This study developed and statistically modelled an integrated enzyme–ultrasound process for recovering astaxanthin from dried Haematococcus pluvialis directly into food-grade soybean oil. Study design: Laboratory experimental study comprising treatment comparison, single-factor screening, and a three-factor, three-level Box–Behnken response surface design. Place and Duration of Study: Faculty of Pharmacy, Nguyen Tat Thanh University, Ho Chi Minh City, Vietnam, from December 2025 to June 2026. Methodology: Standardised dried H. pluvialis biomass (initial astaxanthin content: 21.94 mg/g dry weight) was subjected to enzymatic pretreatment followed by ultrasound-assisted extraction into edible oil. Soybean, sunflower, and rice-bran oils and five cellulase-to-pectinase ratios were screened. A Box–Behnken design comprising 15 randomised runs with three centre-point replicates was used to model total enzyme concentration (1.0–3.0%), sonication duration (15–55 min), and liquid-to-solid ratio (50–150 mL oil/g dry biomass). Astaxanthin was quantified by UV–Vis spectrophotometry at 475 nm using the specific absorption coefficient in acetone. Results: Under the preliminary comparison conditions (1% total enzyme, 15 min sonication, 1.0 mL oil), individual enzyme or ultrasound treatments produced <0.3% recovery, whereas sequential enzyme–ultrasound treatment yielded 20.822 ± 0.322%. Subsequent screening under intensified conditions (2% total enzyme, 35 min sonication, 1.0 mL oil) identified soybean oil and a 50:50 cellulase–pectinase blend as the best-performing conditions among those tested. The quadratic response-surface model was significant (F = 32.39, p = 0.000659), with R² = 0.9831, adjusted R² = 0.9528, and non-significant lack of fit (p = 0.4173). The liquid-to-solid ratio was the strongest linear factor. At the selected practical condition of 2.0% enzyme, 35 min sonication, and 100 mL/g, the model predicted 79.626% recovery and experimental validation yielded 79.708 ± 1.224% (95% CI; n = 6). Conclusion: Sequential enzymatic pretreatment and ultrasound markedly enhanced astaxanthin recovery compared with the individual treatments under the conditions tested. Response-surface modelling provided a statistically adequate description of the process and supported reproducible extraction into a food-compatible soybean-oil phase.

Keywords: Haematococcus pluvialis; astaxanthin; enzyme-assisted extraction; ultrasound-assisted extraction; soybean oil; response surface methodology

1. INTRODUCTION

Astaxanthin is a high-value ketocarotenoid with pronounced antioxidant activity and is widely investigated for applications in nutraceuticals, functional foods, cosmetics, and aquaculture. Among natural sources, the green microalga Haematococcus pluvialis is one of the most important biological sources of natural astaxanthin. During the encysted stage, H. pluvialis accumulates high levels of astaxanthin within a strong, multilayered cell wall, making intracellular pigment recovery a major downstream-processing challenge. The thick cell wall of H. pluvialis therefore represents an important technological barrier to efficient astaxanthin extraction and the development of economically viable algal biorefineries (Kim et al., 2022).

Recent work has also highlighted the antioxidant-related biochemical value of H. pluvialis, reinforcing continued interest in this microalga as a source of high-value bioactive compounds (Pham et al., 2025).

Conventional astaxanthin extraction commonly relies on organic solvents or energy-intensive cell-disruption technologies. Acid-assisted extraction followed by acetone treatment has been reported to recover astaxanthin efficiently from H. pluvialis. Other approaches include ultrasound-assisted extraction, supercritical carbon dioxide extraction, and direct extraction into vegetable oils. However, selecting an appropriate extraction technology involves trade-offs among cell-disruption efficiency, solvent use, energy consumption, product stability, and downstream processing requirements (Dong et al., 2014; Zou et al., 2013).

Recent studies have expanded green extraction routes for H. pluvialis astaxanthin to include integrated pulsed-electric-field/supercritical-CO₂ processing, natural deep eutectic solvents, bio-based ionic liquids, and aqueous biphasic systems (Al Fuhaid et al., 2024; Manikyam et al., 2026; Mussagy et al., 2025; Zhang et al., 2024).

Recent reviews of algal biotechnology have likewise emphasised eco-conscious extraction approaches for recovering high-value pigments and other bioactive compounds from microalgal biomass (Singh et al., 2024). Deep-eutectic-solvent-based aqueous two-phase pretreatment has also been shown to disrupt H. pluvialis cyst cells and facilitate subsequent astaxanthin separation under mild extraction conditions (Nemani et al., 2024).

Ultrasound-assisted extraction (UAE) is attractive because acoustic cavitation can promote mechanical disruption and enhance mass transfer. Nevertheless, H. pluvialis is particularly resistant to physical disruption because of its robust cell-wall structure, and ultrasound alone may therefore provide limited extraction efficiency under mild processing conditions. Previous studies have demonstrated the feasibility of ultrasound-assisted processes for astaxanthin recovery, including response-surface optimisation of liquid-to-solid ratio, extraction temperature, extraction time, and other operating variables (Khoo et al., 2020; Zou et al., 2013).

Response-surface methodology has also been used to optimise microalgal growth and carotenoid production, supporting its broader application to multivariable microalgal process optimisation (Afiya & Umar, 2022). More recent solvent-based optimisation of H. pluvialis extraction has further shown that extraction solvent, temperature, and time influence astaxanthin extraction performance, reinforcing the value of systematic process optimisation (Luo et al., 2024).

Enzyme-assisted extraction provides a complementary strategy by targeting structural components of the algal cell wall under relatively mild conditions. Researchers have investigated cellulase and pectinase for astaxanthin release from H. pluvialis, with enzyme type, pH, temperature, and hydrolysis time influencing extraction performance. In particular, pectinase has been reported to provide greater astaxanthin release than cellulase under the conditions investigated, supporting further investigation of combined enzymatic systems (Zhao et al., 2019).

Direct extraction into edible oils represents another attractive strategy because the recovered astaxanthin can remain in a food-compatible lipid phase without requiring a subsequent solvent-removal step. Soybean oil has previously been evaluated as an extraction or co-solvent phase for astaxanthin recovery from H. pluvialis (Krichnavaruk et al., 2008). More recently, astaxanthin-containing soybean oil was produced from H. pluvialis biomass using an integrated cell-disruption and phase-separation process, demonstrating the practical potential of edible oils as carriers for astaxanthin-enriched products (Lee et al., 2024).

Despite these advances, the efficiency of direct oil-based extraction depends strongly on the accessibility of intracellular astaxanthin. Therefore, combining biochemical cell-wall weakening through enzymatic pretreatment with ultrasound-induced mechanical disruption provides a rational strategy to enhance pigment release and subsequent partitioning into the lipid phase.

However, a process-level gap remains in the context of this study: the performance of sequential cellulase–pectinase pretreatment and ultrasound during direct soybean-oil extraction, together with the effects of total enzyme concentration, sonication duration, and liquid-to-solid ratio, requires systematic evaluation within one integrated experimental framework.

Objective

In the present study, we integrated a binary cellulase–pectinase system with ultrasound-assisted extraction to enable direct recovery of astaxanthin into food-grade soybean oil without organic solvents in the oil-extraction step. The study compared individual and combined treatments, screened suitable carrier oils and cellulase–pectinase ratios, examined cellular disruption by microscopy, and evaluated the effects of total enzyme concentration, sonication duration, and liquid-to-solid ratio using a Box–Behnken response-surface design.

2. Materials and methods

2.1. Objective

The primary biological matrix used in this study was microalgal biomass of Haematococcus pluvialis strain UTEX 2505, cultivated at the Institute of High Technology, Nguyen Tat Thanh University. To preserve astaxanthin integrity, the harvested biomass was freeze-dried to a constant mass, with residual moisture strictly controlled at < 5%. The dried biomass was then mechanically disrupted using a ball mill or stone mortar and passed through standard test sieves to obtain a homogeneous particle size distribution of 0.1–0.25 mm. The standardised powder was vacuum-sealed in opaque aluminium pouches and stored at -20 °C to inhibit spontaneous auto-oxidation before experimental use.

2.2. Equipment

A laboratory optical microscope (Optica, Italy), pH meter (Mettler Toledo S220, Switzerland), Z306 centrifuge (Hermle, Germany), UV–Vis spectrophotometer (Thermo Scientific, USA), vortex mixer (Labnet, USA), and Elma Select 100 ultrasonic bath (Elma, Germany) were used. Sample preparation was performed using a freeze dryer, ball mill, standard sieves, and thermostatic orbital shaker. The microscope was used to evaluate morphological changes in H. pluvialis cells following the extraction treatments.

2.3. Chemicals

Hydrochloric acid (HCl, 37% fuming; EMSURE®, ACS, ISO, Reag. Ph Eur) and acetone (99.8%, Merck, Germany) were used for the reference extraction of total astaxanthin. We used citrate buffer (0.1 M, pH 4.8) to hydrate biomass and perform enzymatic pretreatment. Commercial cellulase (100,000 U/g) and pectinase (60,000 U/g) were obtained from Sunson Industry Group Co., Ltd. (China) and used for enzymatic cell-wall pretreatment. We used food-grade soybean oil, rice bran oil, and sunflower oil (Tường An, Vietnam) as edible lipophilic extraction phases for preliminary carrier-oil screening. All chemicals and reagents were used as received unless otherwise stated.

2.4. Research Methods

2.4.1. Astaxanthin Quantification and Determination of Initial Total Content

The initial total astaxanthin content of standardised dried H. pluvialis biomass was determined as the reference value (Mtotal) for subsequent recovery calculations. Briefly, 10 mg of dried biomass was treated with 1.0 mL of 4 M HCl at 70 °C for 5 min, cooled to room temperature, and repeatedly extracted with acetone. Each extraction consisted of vortex mixing, 10 min ultrasonication, and centrifugation until the residual biomass was visibly decolourised. The combined acetone extracts were collected for spectrophotometric analysis.

To determine astaxanthin in the reference acetone extract, 100 µL of the extract was diluted with 4.90 mL acetone to a final volume of 5.00 mL (dilution factor = 50). Acetone was used as the spectrophotometric blank.

For astaxanthin determination in the recovered soybean-oil phase, 100 µL of the collected oil extract was diluted with 4.90 mL acetone to a final volume of 5.00 mL (dilution factor = 50). A matrix-matched blank consisting of 100 µL of blank soybean oil and 4.90 mL acetone was prepared using the same dilution procedure to correct for background absorbance arising from the oil matrix. Absorption spectra were scanned before quantitative measurements, and absorbance was recorded at 475 nm.

Astaxanthin content was spectrophotometrically estimated using the specific absorption coefficient method with (A1cm1%=2100) for astaxanthin in acetone. The astaxanthin content of the dried biomass was calculated as:

Mtotal=A×V×D×10A1cm1%×m

where (A) is the absorbance at 475 nm, (V) is the final extract volume (mL), (D) is the dilution factor, (A1cm1%) is the specific absorption coefficient of astaxanthin in acetone (2100), and (m) is the dry biomass mass (mg). Eight independent determinations (n=8) were performed, and the mean value was used as (Mtotal). Astaxanthin recovery from the oil phase was subsequently calculated relative to (Mtotal).

2.4.2. Green Extraction Protocol

Six extraction treatments were evaluated: Control (no enzyme, no ultrasound), E1 (cellulase), E2 (pectinase), E3 (cellulase–pectinase), U (ultrasound alone), and EU (enzyme pretreatment followed by ultrasound). For the E3 and EU treatments, cellulase and pectinase were initially combined at a 50:50 (w/w) ratio as a balanced formulation for the preliminary treatment comparison; the effect of enzyme composition was subsequently evaluated systematically in the single-factor screening experiment. For each treatment, 10 mg of standardised dried H. pluvialis biomass was dispersed in 1.0 mL of 0.1 M citrate buffer (pH 4.8).

For the preliminary treatment-comparison experiment, the enzyme-containing groups were evaluated at a total enzyme concentration of 1% (w/w), calculated as the total mass of enzyme preparation relative to the dry biomass mass, and the ultrasound-containing groups were sonicated for 15 min. Enzyme-containing treatments were incubated at 40 °C for 30 min, whereas the Control and U treatments received no enzyme. After enzymatic pretreatment/hydration, the aqueous phase was removed, and 1.0 mL of food-grade soybean oil was added. Control, E1, E2, and E3 were extracted without ultrasound, whereas U and EU received ultrasound treatment.

Ultrasonication was performed using an Elmasonic Select 100 ultrasonic bath (Elma Schmidbauer GmbH, Germany) operating at a nominal frequency of 37 kHz and a manufacturer-rated effective ultrasonic power of 150 W (±10%; maximum peak power 600 W). These values describe the instrument rating rather than the acoustic power delivered directly to each sample. Following extraction, samples were centrifuged at 10,000 rpm for 3 min at room temperature. The upper oil phase was collected, protected from light, and analysed using the UV–Vis procedure described above. Each treatment was performed in triplicate (n = 3).

2.4.3. Single-Factor Experimental Screening

Two preliminary one-factor screening experiments were conducted before response-surface modelling. Unless otherwise specified, the screening experiments used 10 mg of dried biomass, 1.0 mL of 0.1 M citrate buffer (pH 4.8), enzymatic incubation at 40 °C for 30 min, a total enzyme concentration of 2% (w/w, relative to dry biomass), 35 min of sonication, and 1.0 mL of oil.

Carrier-oil screening. Soybean oil, sunflower oil, and rice-bran oil were compared under otherwise identical extraction conditions. Astaxanthin concentration and recovery were determined for each oil, and the best-performing oil among those tested was selected for subsequent experiments.

Binary enzyme-ratio screening. Five cellulase–pectinase formulations (0:100, 25:75, 50:50, 75:25, and 100:0, w/w) were evaluated while the total enzyme concentration was maintained at 2% (w/w, relative to dry biomass). The formulation providing the highest astaxanthin recovery among those tested was selected for the subsequent response-surface experiments.

2.4.4. Response Surface Modelling and Validation

The integrated enzyme-assisted ultrasound extraction process was modelled using a three-factor, three-level Box–Behnken design generated in JMP Student Edition 19.1.4 (JMP Statistical Discovery LLC, USA). The independent variables were total enzyme concentration (X₁: 1.0, 2.0, and 3.0% w/w, relative to dry biomass), sonication duration (X2: 15, 35, and 55 min), and liquid-to-solid ratio (X3: 50, 100, and 150 mL oil/g dry biomass). Astaxanthin recovery (%) was used as the response (Y).

The design comprised 15 randomised experimental runs, including three replicated centre points. A full second-order polynomial model was fitted by standard least squares: Y = b0 + b1X1 + b2X2 + b3X3 + b11X12 +b22X22 +b33X32 +b12(X1X2) + b13(X1X3) + b23(X2X3)

Model adequacy was assessed by analysis of variance (ANOVA), R², adjusted R², root mean square error (RMSE), lack-of-fit testing, residual diagnostics, and the significance of individual model terms. Statistical significance was defined as p < 0.05.

Table 2 presents the complete randomised design matrix and experimental responses. Because the unconstrained stationary point identified by the fitted surface included an L/S value slightly outside the investigated range, it was not interpreted as an experimentally validated optimum. Instead, the centre operating condition (2.0% total enzyme, 35 min sonication, and 100 mL/g) was selected as a practical validation condition within the experimental domain and was tested in six independent experiments.

Table 1. Experimental factors and coded levels used in the Box–Behnken design

| Factor | Symbol | −1 | 0 | +1 |
| --- | --- | --- | --- | --- |
| Total enzyme concentration (%) | X1 | 1.0 | 2.0 | 3.0 |
| Sonication duration (min) | X2 | 15 | 35 | 55 |
| L/S ratio (mL/g) | X3 | 50 | 100 | 150 |

Table 2. Box–Behnken experimental design matrix and observed astaxanthin recovery

| Run | X1 (%) | X2 (min) | X3 (mL/g) | Recovery (%) |
| --- | --- | --- | --- | --- |
| 1 | 3 | 15 | 100 | 46.863 |
| 2 | 2 | 15 | 150 | 33.354 |
| 3 | 2 | 55 | 150 | 40.194 |
| 4 | 3 | 35 | 150 | 43.971 |
| 5 | 3 | 55 | 100 | 51.654 |
| 6 | 3 | 35 | 50 | 73.946 |
| 7 | 1 | 55 | 100 | 54.262 |
| 8 | 1 | 35 | 150 | 35.517 |
| 9 | 2 | 55 | 50 | 69.108 |
| 10 | 2 | 35 | 100 | 82.737 |
| 11 | 2 | 35 | 100 | 76.064 |
| 12 | 1 | 35 | 50 | 79.456 |
| 13 | 1 | 15 | 100 | 61.71 |
| 14 | 2 | 15 | 50 | 78.971 |
| 15 | 2 | 35 | 100 | 80.077 |

3. results and discussion

3.1 Results

3.1.1. Initial Total Astaxanthin Content (Mtotal)

The initial total astaxanthin content of the standardised Haematococcus pluvialis biomass was determined by exhaustive acid-assisted extraction followed by acetone extraction and used as the reference value (Mtotal) for subsequent recovery calculations. The determined astaxanthin content ranged from 20.67 to 22.85 mg/g DW across eight independent replicates (n=8), with a mean value of 21.94 mg/g DW and an RSD of 2.83% (Table 3). The low RSD indicates good repeatability of the reference extraction and provides a consistent baseline for evaluating the efficiency of the subsequent enzyme-assisted ultrasound extraction process.

Table 3. Initial total astaxanthin content of standardised H. pluvialis biomass determined by exhaustive extraction (n=8)

| Parameter | Value |
| --- | --- |
| Number of replicates (n) | 8 |
| Minimum value (mg/g DW) | 20.67 |
| Maximum value (mg/g DW) | 22.85 |
| Mean concentration (Mtotal) mg/g DW | 21.94 |
| Relative Standard Deviation (RSD) | 2.83% |

3.1.2. Effect of Enzymatic and Ultrasound-Assisted Treatments on Astaxanthin Recovery

The effects of enzymatic and ultrasound-assisted treatments on astaxanthin extraction from standardised dried Haematococcus pluvialis biomass were evaluated using six treatment conditions: untreated control (Control), cellulase pretreatment (E1), pectinase pretreatment (E2), combined cellulase–pectinase pretreatment (E3), ultrasound-assisted extraction alone (U), and combined enzyme–ultrasound extraction (EU). Table 4 summarises the astaxanthin concentrations and recovery yields from the three independent replicates.

The Control and individual treatments resulted in very low astaxanthin recovery. The mean astaxanthin concentrations for Control, E1, E2, E3, and U were 0.061, 0.047, 0.062, 0.044, and 0.042 mg/g, respectively, corresponding to recovery yields of 0.277, 0.212, 0.284, 0.200, and 0.191%. These values represented less than 0.3% of the initial astaxanthin content (Mtotal = 21.94 mg/g DW), indicating that cellulase, pectinase, their combination, or ultrasound alone produced only limited astaxanthin recovery under the conditions investigated.

In marked contrast, the combined enzyme–ultrasound treatment (EU) substantially enhanced astaxanthin extraction. The mean astaxanthin concentration reached 4.568 mg/g, corresponding to a 20.822% recovery. Relative to the untreated Control, the EU treatment increased the recovered astaxanthin concentration by approximately 75-fold (4.568 vs. 0.061 mg/g) and increased recovery from 0.277 to 20.822%.

Table 4. Astaxanthin concentration and recovery obtained from different enzymatic and ultrasound-assisted extraction treatments (n = 3 independent replicates per treatment).

| Treatment | Astaxanthin concentration (mg/g) | Recovery (%) |
| --- | --- | --- |
| Control | 0.061 ± 0.001 | 0.277 ± 0.003 |
| E1 – Cellulase | 0.047 ± 0.024 | 0.212 ± 0.107 |
| E2 – Pectinase | 0.062 ± 0.022 | 0.284 ± 0.102 |
| E3 – Cellulase + Pectinase | 0.044 ± 0.026 | 0.200 ± 0.117 |
| U – Ultrasound | 0.042 ± 0.014 | 0.191 ± 0.062 |
| EU – Enzyme + Ultrasound | 4.568 ± 0.071 | 20.822 ± 0.322 |

One-way ANOVA demonstrated a highly significant effect of treatment on both astaxanthin concentration ((F5,12=8808.80), (p<0.001)) and recovery ((F5,12=8895.12), (p<0.001)). Post hoc analysis showed that the EU treatment differed significantly from the other extraction treatments, whereas the Control, E1, E2, E3, and U groups did not differ significantly from one another. These results showed that the substantial improvement in astaxanthin recovery was associated specifically with the combined application of enzymatic pretreatment and ultrasound rather than with either treatment applied independently.

3.1.2.1. Microscopic Evidence of Cellular Disruption

The microscopic observations were consistent with the quantitative extraction results (Figure 1). The Control cells exhibited predominantly intact spherical morphology with intense orange-red intracellular pigmentation. Similar cellular morphology was observed in the cellulase (E1), pectinase (E2), and cellulase–pectinase (E3) treatments, with most cells retaining their spherical shape and visible pigmentation.

Ultrasound treatment alone (U) produced some morphological alterations and cellular debris; however, many apparently intact pigmented cells remained visible. In contrast, the combined enzyme–ultrasound treatment (EU) caused pronounced morphological disruption, with fewer intact pigmented cells, increased cellular debris, and the appearance of apparently decolourised cellular remnants.

The marked difference between the EU treatment and the individual treatments is consistent with complementary effects of enzymatic pretreatment and ultrasound. Enzymatic hydrolysis may weaken the polysaccharide-rich cellular matrix, increasing cell susceptibility to subsequent ultrasonic disruption, while ultrasound-induced cavitation may enhance mechanical disruption and mass transfer. The substantially higher astaxanthin recovery observed for EU compared with the enzyme-only and ultrasound-only treatments further supports the complementary effects of enzymatic pretreatment and ultrasound-assisted extraction.

| Control | Cellulase (E1) | Pectinase (E2) | Cellulase–pectinase (E3) | Ultrasound (U) | Enzyme–ultrasound (EU) |
| --- | --- | --- | --- | --- | --- |

Fig.1. Microscopic evidence of cellular disruption in Haematococcus pluvialis following enzymatic and ultrasound-assisted treatments.

3.1.2.2 Overall Performance of the Integrated Extraction Process

The results showed that the integrated enzyme–ultrasound process was substantially more effective than the individual treatments for recovering astaxanthin from dried H. pluvialis biomass. Although the enzyme-only and ultrasound-only treatments yielded recoveries below 0.3%, their combination increased recovery to 20.822%. This pronounced improvement provides the experimental basis for applying the enzyme–ultrasound system as the core extraction platform for subsequent response-surface modelling.

The EU treatment was therefore selected for the subsequent screening and optimisation experiments, with the cellulase–pectinase composition, total enzyme concentration, sonication duration, and liquid-to-solid ratio considered process variables according to the experimental design.

3.1.3. Single-Factor Experimental Screening

Carrier oil screening. Carrier-oil type significantly affected astaxanthin recovery (one-way ANOVA, F2,6 = 55.97, p<0.001). Soybean oil provided the highest recovery (80.283 ± 1.050%), followed by sunflower oil (64.853 ± 4.725%) and rice-bran oil (52.703 ± 2.700%) (mean ± SD, n=3). Tukey’s HSD test showed significant differences among all three carrier oils (p<0.05). Soybean oil was therefore selected as the carrier oil for the subsequent experiments.

A

B

Cellulase/pectinase ratio screening. The relative proportion of cellulase and pectinase significantly affected astaxanthin recovery (one-way ANOVA, (F4,10 = 119.16), (p<0.001)). The 50:50 cellulase–pectinase formulation provided the highest recovery (80.283 ± 1.050%), which was significantly higher than those obtained with the 100:0 (43.039 ± 1.003%), 25:75 (42.913 ± 3.245%), 75:25 (49.176 ± 1.913%), and 0:100 (46.742 ± 3.886%) formulations (Tukey’s HSD, p < 0.05). No significant differences were observed among the other four formulations. Therefore, the 50:50 cellulase–pectinase formulation was selected for the subsequent response-surface experiments. Values are expressed as mean ± SD (n = 3).

Fig.2. Effects of carrier-oil type (A) and cellulase-to-pectinase ratio (B) on astaxanthin recovery. Values are expressed as mean ± SD (n = 3). Different lowercase letters indicate significant differences among treatments within each experiment according to Tukey’s HSD test (p < 0.05).

Overall, soybean oil and the 50:50 cellulase/pectinase ratio were selected based on their superior astaxanthin concentration and recovery and were subsequently used as fixed conditions for the response-surface experiments. The higher recovery observed during screening (approximately 80%) should not be directly compared with the 20.822% recovery from the preliminary EU treatment comparison because the experimental intensities differed: the preliminary comparison used 1% total enzyme and 15 min sonication, whereas screening used 2% total enzyme and 35 min sonication, with 1.0 mL of oil in both stages.

3.1.4. Response Surface Modelling and Validation

The full quadratic model adequately described astaxanthin recovery within the investigated experimental domain. The fitted coded-variable equation was:

Y = 79.626 - 1.814X1 - 0.710X2 - 18.556X3 + 3.060X1X2 + 3.491X1X3 + 4.176X2X3 - 11.594X12 - 14.410X22 - 9.810X32

ANOVA showed that the overall quadratic model was highly significant (F = 32.39, p = 0.000659). The model explained 98.31% of the observed variation in astaxanthin recovery (R2 = 0.9831), with an adjusted R2 of 0.9528 and an RMSE of 3.864. The lack of fit was not significant (F = 1.5383, p = 0.4173), indicating that the quadratic model adequately represented the experimental data within the investigated domain.

Among the individual model terms, the linear effect of the L/S ratio ((X3)) was highly significant (p < 0.0001). Significant quadratic effects were also observed for total enzyme concentration ((X12), (p = 0.0022)), sonication duration ((X22), (p = 0.0008)), and L/S ratio ((X32), (p = 0.0046)). In contrast, the linear effects of total enzyme concentration ((X1), (p = 0.2416)) and sonication duration ((X2), (p = 0.6254)), as well as the interaction terms (X1X2) ((p = 0.1741)), (X1X3) ((p = 0.1306)), and (X2X3) ((p = 0.0830)), were not statistically significant at (p < 0.05). These results identify the L/S ratio as the dominant linear factor affecting astaxanthin recovery, while the significant negative quadratic coefficients indicate pronounced curvature in the response surface.

Canonical analysis identified a stationary maximum at approximately 1.739% total enzyme concentration, 30.95 min sonication, and an L/S ratio of 48.24 mL/g, corresponding to a predicted recovery of 89.54%. However, the L/S ratio at this stationary point was below the lower experimental boundary of 50 mL/g, so JMP identified the critical point as outside the investigated experimental range. Accordingly, this extrapolated stationary point was not considered an experimentally validated optimum (Fig. 3).

At the selected practical validation condition of 2.0% total enzyme concentration, 35 min sonication, and an L/S ratio of 100 mL/g, the model predicted an astaxanthin recovery of 79.626%. Six independent validation experiments yielded a mean recovery of 79.708% (SD = 1.166; RSD = 1.463%). The 95% confidence interval for the experimental mean was 79.708 ± 1.224% (78.484–80.931%). A one-sample t-test comparing the experimental validation results with the model-predicted value of 79.626% showed no statistically significant difference (t = 0.172, df = 5, p = 0.870). The close agreement between the predicted and experimental responses supports the predictive adequacy of the fitted model within the investigated experimental domain (Table 5).

Fig. 3. Prediction profiler of astaxanthin recovery as a function of total enzyme concentration, sonication duration, and liquid-to-solid ratio.

Table 5. Experimental validation of the response-surface model using six independent validation runs (n = 6).

| n | Recovery yield | Results of statistical processing |
| --- | --- | --- |
| 1 | 79.307 | n = 6 |
| 2 | 81.394 | Mean = 79.708% |
| 3 | 80.149 | SD =1.166 |
| 4 | 80.077 | RSD = 1.463 % |
| 5 | 79.456 | Predicted recovery = 79.626% |
| 6 | 77.864 | One-sample t-test: t = 0.172, df = 5, p = 0.870 |
| Average | 79.708 | 95% CI: 79.708 ± 1.224% (78.484–80.931%) |

3.2 Discussion

3.2.1. Complementary Effects of Enzymatic Pretreatment and Ultrasound

The results showed a pronounced increase in astaxanthin recovery when enzymatic pretreatment was combined with ultrasound-assisted extraction. Cellulase, pectinase, combined cellulase–pectinase treatment, and ultrasound alone yielded recoveries below 0.3%, whereas the combined enzyme–ultrasound treatment (EU) achieved 20.822 ± 0.322% under the preliminary comparison conditions before response-surface modelling. The approximately 75-fold increase relative to the untreated control indicates that the sequential enzyme–ultrasound treatment was substantially more effective than either treatment applied independently.

The limited recovery obtained with the individual treatments is consistent with the well-recognised resistance of Haematococcus pluvialis cysts to downstream cell disruption. The robust cell-wall structure is a major technological barrier to efficient astaxanthin release (Cui et al., 2025; Jaryal et al., 2025). Ultrasound-assisted extraction can promote mechanical disruption and mass transfer through acoustic cavitation, whereas enzymatic pretreatment may weaken structural polysaccharides within the cellular matrix. Previous studies have similarly highlighted the potential benefits of combining biochemical and physical cell-disruption approaches (Khoo et al., 2020; Zou et al., 2013). In the present study, the markedly greater recovery achieved by the sequential enzyme–ultrasound treatment, together with the microscopic observations, is consistent with complementary effects of the two treatments. However, because the experimental design did not directly quantify an enzyme × ultrasound interaction, these findings should not be interpreted as definitive statistical evidence of synergism.

3.2.2. Influence of Carrier Oil and Enzyme Composition

Preliminary carrier-oil screening showed that the extraction phase strongly affected astaxanthin recovery. Among the soybean, sunflower, and rice-bran oils evaluated in the present study, soybean oil produced the highest preliminary recovery (80.283 ± 1.050%). We therefore selected it as the carrier phase for subsequent response-surface experiments. Previous research supports soybean oil as a suitable edible carrier for H. pluvialis-derived astaxanthin. Dong et al. (2014) reported the feasibility of soy-oil extraction of astaxanthin from H. pluvialis, whereas more recent work demonstrated the production of astaxanthin-containing soybean oil with a cumulative recovery of 66.41% and substantially enhanced antioxidant capacity (Lee et al., 2024).

Recent research further confirms the growing interest in vegetable oils as food-compatible extraction phases. Mansouri et al. (2026) compared several disruption strategies and investigated vegetable oils including argan, cannabis, and olive oils as non-polar extraction phases for H. pluvialis biomass. These studies indicate that edible oils can act not only as extraction phases but also as potential final carriers for astaxanthin-enriched lipid products. Accordingly, soybean oil's significance in the present study does not lie in its use as a novel solvent, but in its incorporation into an enzyme-assisted ultrasound extraction platform.

The enzyme-composition screening further showed that the 50:50 cellulase–pectinase formulation provided the highest extraction performance among the tested enzyme ratios. This finding suggests that combining enzymes with complementary substrate specificities may improve the accessibility of intracellular astaxanthin compared with treatment using either enzyme alone. Zhao et al. (2019) demonstrated that both cellulase and pectinase could improve astaxanthin release from H. pluvialis, with pectinase providing significantly greater release than cellulase under their investigated conditions. The present findings broaden this concept by showing that the relative composition of the two enzymes can also influence the performance of the integrated extraction process.

3.2.3. Response Surface Modelling, Model Validation, and Green Extraction Implications

The response-surface analysis suggested that astaxanthin recovery was governed by the combined effects of enzymatic cell-wall weakening, ultrasound-induced disruption, and pigment partitioning into the soybean-oil phase. Enzymatic pretreatment can hydrolyse structural components of the resistant H. pluvialis cell wall, thereby facilitating subsequent pigment release (Kim et al., 2022; Zhao et al., 2019). The significant quadratic effect of enzyme concentration observed in this study suggests that an intermediate enzyme level was more favourable than simply increasing enzyme dosage.

Sonication duration also showed a significant quadratic effect. Ultrasound-induced cavitation can promote cell disruption and mass transfer, facilitating the release of intracellular astaxanthin from enzyme-weakened biomass (Khoo et al., 2020; Kim et al., 2022). Among the investigated variables, the L/S ratio showed the strongest linear effect on recovery, indicating the importance of the available lipid phase for mass transfer and partitioning of released astaxanthin. However, the significant curvature of the L/S response indicates that increasing oil volume does not necessarily increase recovery. Vegetable oils have also been shown to be suitable food-compatible extraction phases for astaxanthin (Krichnavaruk et al., 2008; Lee et al., 2024).

Together with the microscopic observations, these results support a complementary mechanism in which enzymatic pretreatment weakens the cellular barrier, ultrasound promotes further physical disruption and mass transfer, and soybean oil acts as a lipophilic phase for the released astaxanthin. The selected validation condition (2.0% total enzyme, 35 min sonication, and 100 mL/g) produced an experimental recovery consistent with the model prediction, supporting the model's predictive adequacy within the studied domain. Although the sequential enzyme–ultrasound treatment markedly enhanced astaxanthin recovery, the present design did not directly quantify an enzyme × ultrasound interaction; therefore, the findings are interpreted as evidence of complementary effects rather than definitive statistical proof of synergism.

Although the experimental recovery of approximately 79.7% demonstrates substantial astaxanthin recovery into the soybean-oil phase, this value should be interpreted in light of several limitations. Recovery was calculated relative to the total astaxanthin content determined by the reference extraction and was quantified using UV–Vis spectrophotometry rather than a compound-specific chromatographic method. A matrix-matched soybean-oil blank was used to correct for background absorbance from the oil matrix; however, potential spectral interference from other carotenoids or co-extracted absorbing compounds cannot be completely excluded. In addition, validation was conducted on a laboratory scale using six independent runs at a single selected operating condition. Therefore, the reported recovery should be considered specific to the materials, operating conditions, and analytical approach used in the present study. Further validation using chromatographic quantification and scale-up experiments would strengthen the analytical confirmation and broader applicability of these findings.

4. Conclusion

This study developed an integrated enzyme-assisted ultrasound process to recover astaxanthin from dried H. pluvialis directly into edible soybean oil. Under the preliminary comparison conditions, sequential enzyme–ultrasound treatment increased recovery to 20.822 ± 0.322%, whereas individual enzyme or ultrasound treatments remained below 0.3%. Subsequent screening identified soybean oil and a 50:50 cellulase–pectinase formulation as the best-performing conditions among those tested. The 15-run Box–Behnken model was significant (p = 0.000659), explained 98.31% of the response variability, and showed a non-significant lack of fit (p = 0.4173). At the selected practical condition of 2.0% total enzyme (w/w, relative to dry biomass), 35 min sonication, and an L/S ratio of 100 mL/g, the experimental recovery (79.708 ± 1.224%, 95% CI; n = 6) closely matched the model prediction (79.626%). These findings support a reproducible, food-compatible extraction strategy while indicating that formal statistical synergy and the mathematical optimum predicted outside the investigated experimental domain require separate experimental confirmation.

## Limitation

The study was conducted with dried H. pluvialis strain UTEX 2505 under laboratory-scale conditions and evaluated only three edible oils and five cellulase-to-pectinase ratios. Astaxanthin was quantified spectrophotometrically, and the experimental design did not directly quantify an enzyme × ultrasound interaction. In addition, the stationary maximum predicted by canonical analysis included a liquid-to-solid ratio slightly outside the investigated range and was therefore not experimentally validated. These factors limit extrapolation beyond the tested materials, operating ranges, and equipment.

Competing interests

Authors have declared that no competing interests exist.

Declaration of AI Use

This manuscript was prepared through the intellectual contributions of the author(s) to the study design, data analysis, interpretation of results, and scholarly content. Grammarly and ChatGPT were used solely to assist with grammatical refinement and reference formatting during manuscript revision. The author(s) accept full responsibility for the accuracy, integrity, and final content of the manuscript.

# COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

References

Afiya, H., & Umar, K. M. (2022). Optimization studies for microalgal growth and carotenoid production (before stress). Journal of Global Ecology and Environment, 15(3), 41–51. https://doi.org/10.56557/jogee/2022/v15i37586

Al Fuhaid, L., Wellman, G. B., Kharbatia, N., Farinha, A. S. F., Vrouwenvelder, J. S., Verpoorte, R., Choi, Y. H., Witkamp, G.-J., Lauersen, K. J., & Fortunato, L. (2024). Green extraction of pigment from astaxanthin-producing algae using natural deep eutectic solvents. Algal Research, 82, 103668. https://doi.org/10.1016/j.algal.2024.103668

Cui, H., Zhu, X., Yu, X., Li, S., Wang, K., Wei, L., Li, R., & Qin, S. (2025). Advancements of astaxanthin production in Haematococcus pluvialis: Update insight and way forward. Biotechnology Advances, 79, 108519. https://doi.org/10.1016/j.biotechadv.2025.108519

Dong, S., Huang, Y., Zhang, R., Wang, S., & Liu, Y. (2014). Four different methods comparison for extraction of astaxanthin from green alga Haematococcus pluvialis. The Scientific World Journal, 2014, 694305. https://doi.org/10.1155/2014/694305

Jaryal, S., Sharma, A. K., Lamba, B. Y., & Patel, A. (2025). A comprehensive review on microalgae based astaxanthin: Bioprocess optimization, technological barriers, industrial applications and future roadmap. Frontiers in Marine Science, 12, 1644644. https://doi.org/10.3389/fmars.2025.1644644

Khoo, K. S., Chew, K. W., Yew, G. Y., Manickam, S., Ooi, C. W., & Show, P. L. (2020). Integrated ultrasound-assisted liquid biphasic flotation for efficient extraction of astaxanthin from Haematococcus pluvialis. Ultrasonics Sonochemistry, 67, 105052. https://doi.org/10.1016/j.ultsonch.2020.105052

Kim, B., Lee, S. Y., Narasimhan, A. L., Kim, S., & Oh, Y.-K. (2022). Cell disruption and astaxanthin extraction from Haematococcus pluvialis: Recent advances. Bioresource Technology, 343, 126124. https://doi.org/10.1016/j.biortech.2021.126124

Krichnavaruk, S., Shotipruk, A., Goto, M., & Pavasant, P. (2008). Supercritical carbon dioxide extraction of astaxanthin from Haematococcus pluvialis with vegetable oils as co-solvent. Bioresource Technology, 99(13), 5556–5560. https://doi.org/10.1016/j.biortech.2007.10.049

Lee, J. Y., Lee, J. S., & Sim, S. J. (2024). Enhanced toxicity-free astaxanthin extraction from Haematococcus pluvialis via concurrent cell disruption and demulsification. Bioresource Technology, 406, 130974. https://doi.org/10.1016/j.biortech.2024.130974

Luo, F., Wang, S., Zhang, X., Liu, Z., Zhu, R., & Xue, W. (2024). Extraction of astaxanthin from Haematococcus pluvialis and preparation of astaxanthin liposomes. Molecules, 29(14), 3320. https://doi.org/10.3390/molecules29143320

Manikyam, H. K., Joshi, S. K., & Suresh, P. V. (2026). Integrated PEF–supercritical CO₂ technology for high-purity astaxanthin extraction from Haematococcus pluvialis. Journal of Scientific Research and Reports, 32(5), 217–228. https://doi.org/10.9734/jsrr/2026/v32i54168

Mansouri, Z., Lijassi, I., Rhazi, L., & Wahby, I. (2026). Extraction of astaxanthin from fresh Haematococcus pluvialis biomass: A sustainable approach using vegetable oils. Archives of Microbiology, 208(8), 391. https://doi.org/10.1007/s00203-026-04943-1

Mussagy, C. U., Caicedo-Paz, A. V., Figueroa, D., Santander, C., González, F., Tropea, A., Mondello, L., Spadaro, D., Trocino, S., Piazza, R. D., Marques, R. F. C., Godoy, K., & Cornejo, P. (2025). Maximizing Haematococcus biorefineries: Ionic liquid-based astaxanthin recovery, biocosmetic formulation, solar cell applications, and biofertilizer valorization. Bioresource Technology, 426, 132347. https://doi.org/10.1016/j.biortech.2025.132347

Nemani, N., Dehnavi, S. M., & Pazuki, G. (2024). Extraction and separation of astaxanthin with the help of pre-treatment of Haematococcus pluvialis microalgae biomass using aqueous two-phase systems based on deep eutectic solvents. Scientific Reports, 14, 5420. https://doi.org/10.1038/s41598-024-55630-4

Pham, L., Dang, V. P., Cao, P., & Vo, T. (2025). Thermal stress enhances phenolic metabolism and antioxidant capacity in Hamatococcus pluvialis: A sustainable platform for microalgal antioxidants. Journal of Biology and Nature, 17(2), 121–130. https://doi.org/10.56557/joban/2025/v17i29503

Singh, A., Pandey, S. N., Khushbu, & Kumar, H. (2024). Advances in algal biotechnology: Sustainable production of nutrient-rich biomass and high-value bioproducts. European Journal of Nutrition & Food Safety, 16(7), 87–104. https://doi.org/10.9734/ejnfs/2024/v16i71458

Zhang, L., Li, Y., & Gao, J. (2024). Selectively extraction of astaxanthin from Haematococcus pluvialis by aqueous biphasic systems composed of ionic liquids and deep eutectic solutions. Food Chemistry, 434, 137399. https://doi.org/10.1016/j.foodchem.2023.137399

Zhao, X., Zhang, X., Liu, H., Zhu, H., & Zhu, Y. (2019). Enzyme-assisted extraction of astaxanthin from Haematococcus pluvialis and its stability and antioxidant activity. Food Science and Biotechnology, 28(6), 1637–1647. https://doi.org/10.1007/s10068-019-00608-6

Zou, T.-B., Jia, Q., Li, H.-W., Wang, C.-X., & Wu, H.-F. (2013). Response surface methodology for ultrasound-assisted extraction of astaxanthin from Haematococcus pluvialis. Marine Drugs, 11(5), 1644–1655. https://doi.org/10.3390/md11051644