Original Research Article

Bioremediation Potential of Spent Mushroom Substrate and Costus afer Bagasse for the Removal of Petroleum Hydrocarbon Xenobiotics (TPH, TPAH, and TMAH) from Contaminated Soil

ABSTRACT Spent mushroom substrate (SMS) and Costus afer bagasse (CAB) are abundantly available agro-wastes in the Niger Delta region of Nigeria and may serve as cost-effective amendments for the remediation of petroleum hydrocarbon-contaminated soils. This study evaluated the bioremediation potential of SMS and CAB applied at three amendment rates (0.5, 1.0, and 1.5 kg) to sandy-loam soil contaminated with Bonny Light crude oil at low, medium, and high contamination levels (250 g, 500 g, and 750 g per 10 kg soil, respectively) over a 120-day period. Treatment performance was assessed using total petroleum hydrocarbon (TPH), total polycyclic aromatic hydrocarbon (TPAH), and total monocyclic aromatic hydrocarbon (TMAH) concentrations. Statistical analyses, including analysis of variance (ANOVA) and multiple linear regression, were conducted using Minitab® v19 at a 95% confidence level. Results indicated significant reductions (p < 0.05) in hydrocarbon concentrations across all amended treatments relative to the control. The highest remediation efficiency was achieved with 1.5 kg SMS (T4), recording TPH reductions of 93.20%, 89.80%, and 87.16% under low, medium, and high contamination levels, respectively. Although treated soils met Nigerian regulatory intervention limits, residual concentrations remained above target values. Overall, SMS demonstrated greater remediation efficiency than Costus afer bagasse. These findings suggest that locally available agro-wastes, particularly SMS, can significantly reduce the cost and duration of bioremediation of crude oil-contaminated soils in the Niger Delta.

Keywords: Crude oil; Spent mushroom substrate; Costus afer bagasse; Bioremediation; Niger Delta)

1. INTRODUCTION

Contamination of the natural environment due to economic activities leads to an increase in the environmental content of toxic organic compounds (Elechi et al., 2018). This has particularly serious consequences because some of these compounds (e.g., chloro-organic pesticides, polychlorinated biphenyls, dioxins, or some polycyclic aromatic hydrocarbons) are highly persistent in the environment and accumulate in soils, bottom sediments, and organisms (Vidonish et al., 2016). Chemical contaminants in the environment can adversely affect living organisms at levels ranging from soil microbes to upper-trophic-level vertebrates, including humans. Soil contamination by organic or inorganic pollutants is caused by several industries, including the chemical, pharmaceutical, plastics, automobile, nuclear, biomedical, mining, and municipal solid waste management industries.

Land contamination arises from a range of natural and anthropogenic sources, primarily through the introduction or mobilisation of contaminants in the terrestrial environment. Naturally occurring minerals, such as asbestos and uranium, may contribute to contamination when disturbed or exposed; however, contamination associated with undisturbed natural mineral deposits is relatively uncommon. In contrast, anthropogenic activities, including industrial processes, agricultural practices and accidental releases, represent major sources of land contamination. The contaminants generated by these activities typically occur as commercial products, industrial by-products and waste materials. Within the region, petroleum-related soil contamination is a particularly prevalent form of pollution (Adams et al., 2016). Petroleum is a complex mixture comprising thousands of organic compounds, many of which are hydrophobic and potentially hazardous. Its elemental composition includes carbon (85–90%), nitrogen (10–14%), sulphur (0.2–3%), oxygen (1–1.5%), hydrogen (less than 0.1–2%) and trace metals (less than 1%) (Sayed et al., 2021). Depending on its source, petroleum hydrocarbons account for approximately 50–98% of crude oil and constitute one of its principal components (Al-Dhabaan, 2019). Crude oil is broadly classified into four major groups of compounds: saturates, aromatics, resins and asphaltenes (Al-Hawash et al., 2018). The presence of these constituents in soil can adversely affect its physical, chemical and biological properties. Furthermore, exposure to petroleum-derived contaminants may induce carcinogenic and immunotoxic effects, presenting substantial risks to human health and the environment (Sales da Silva et al., 2020).

The remediation of hydrocarbon-contaminated soils has long been a global environmental challenge, prompting the development and application of various treatment strategies. Conventional physicochemical approaches include incineration, landfill disposal, soil excavation, thermal desorption, soil flushing, soil washing, soil vapour extraction, and stabilisation/solidification (Ossai et al., 2020). Despite their widespread application, these techniques are often associated with high operational costs and adverse environmental impacts. In contrast, various biological remediation technologies have been demonstrated to be effective in treating hydrocarbon-contaminated soils (Wang et al., 2019). Bioremediation involves the utilisation of microorganisms to degrade environmental pollutants into less harmful or non-toxic substances (Guarino et al., 2017). Owing to its relative cost-effectiveness and environmental compatibility, bioremediation has gained considerable attention as an alternative to conventional chemical and physicochemical treatment methods (Muttaleb & Ali, 2022). Commonly employed biological remediation strategies include phytoremediation, bioaugmentation, and biostimulation, among others.

Biostimulation represents one of the established biological approaches for the remediation of contaminated soils (Valizadeh Rad et al., 2021; Ayotamuno et al., 2006). This technique enhances the activity and growth of indigenous microbial communities by supplying essential nutrients required to optimise the biodegradation of contaminants (Muttaleb & Ali, 2022). Biostimulation is considered a cost-effective, sustainable, and environmentally and aesthetically compatible remediation strategy. It facilitates the immobilisation or stabilisation of contaminants, including pesticides, metals, and chlorinated hydrocarbons, and promotes their transformation into less harmful or innocuous forms, thereby minimising potential risks to the environment (Nweke et al., 2024a).

In Nigeria, several studies on the implications, bioavailability, risk assessment, and remediation of contaminants in soil have been reported (e.g., Okparanma et al., 2022; Ayotamuno et al., 2010). As an oil-producing nation, Nigeria is affected by crude oil contamination due to incessant oil spillages and the mismanagement of oily wastes such as drill cuttings (Anifowose et al., 2008). The associated xenobiotic contaminants can persist in the environment, affect land use, and consequently influence socioeconomic conditions in the affected areas, given that residents of the Niger Delta region, who are among those most affected by oil contamination, are predominantly farmers and fishermen. Furthermore, the coastal location of the region implies a scarcity of arable land, which is worsened by soil pollution arising from oil production and illegal bunkering activities, thereby creating a need for effective and affordable soil remediation methods. Recent evidence indicates that waste-derived amendments can enhance petroleum hydrocarbon removal by supporting microbial activity and contaminant degradation in polluted soils (Zhang et al., 2020; Omenna et al., 2024; Ugochi et al., 2025). Spent mushroom substrate is particularly relevant because its application has been reported to enhance hydrocarbon removal from contaminated soil, while recent microbial analyses show that different spent mushroom substrates can promote degradation of both aliphatic and aromatic petroleum hydrocarbons (Ogbonna et al., 2020; Mayans et al., 2024).

Although there are multiple methods for remediating contaminated soil, including biological, physical, and chemical approaches, the biological method (bioremediation) appears to be preferred because of its eco-friendliness and cost-effectiveness, given that other methods are either too expensive or may have secondary adverse impacts on the environment due to the materials used. Spent mushroom substrate (SMS) and Costus afer, both abundant in the region, may otherwise be regarded as wastes, but they can serve as natural, organic, and locally sourced amendments for contaminated soils. Their potential usefulness in addressing a localised pollution problem could represent an important remediation option in Nigeria. However, although several studies have examined contaminant degradation in soil treated with spent mushroom substrate, studies on the use of Costus afer bagasse in soil are limited. Hence, this research investigated the effects of spent mushroom substrate and Costus afer bagasse on the xenobiotics total petroleum hydrocarbon (TPH), total polycyclic aromatic hydrocarbon (TPAH), and total monocyclic aromatic hydrocarbon (TMAH) at different levels of crude oil contamination in soil.

2.1 MATERIALS AND METHODS

The Niger Delta region of Nigeria is in the southern part of the country and is bordered to the south by the Atlantic Ocean and to the east by Cameroon. Based on political classification, it covers nine states in Nigeria, including Abia, Akwa Ibom, Cross River, Edo, Imo, and Ondo, in addition to the parent Niger Delta states of Rivers, Bayelsa, and Delta. Delta State is part of the core Niger Delta. Akujuru (2014), citing studies carried out by the Niger Delta Development Commission (NDDC), reported that the region occupies a surface area of about 112,110 square kilometres, representing about 12% of Nigeria's total surface area. Nweke et al. (2024b) stated that the region has about 450km2 of coastline and is the world's second-largest delta, terminating at the Imo River. The region, which has 20,000km2, is the largest wetland in Africa and the third largest globally (Nweke et al., 2024a).

The Niger Delta region has three main vegetation types, including mangrove forest, freshwater swamp forest, and the savanna and stunted rain forests of the sand ridges and mainland margin. Its ecological zones have been classified into four categories: lowland rain forest, freshwater, mangrove swamp, and coastal inland zones. Uyigue and Agho (2007) reported that the region is classified as tropical rainforest, with an ecosystem comprising myriad species of flora and fauna, both aquatic and terrestrial. The Niger Delta region accommodates about 23% of Nigeria's total population of over 140 million.

The region is a major source of timber and forest products and contains important areas of highly endangered wildlife, with very high fishery and agricultural potential. As an oil- and gas-producing nation, Nigeria largely depends on the Niger Delta, where most of its oil and gas industries are located (Udem & Ezeasor, 2010). Costus afer is usually an unbranched tropical plant, often described as a herb with a creeping rhizome. It is a relatively small monocot shrub commonly found in humid and monstrous forests and along riversides (Udem & Ezeasor, 2010). It is a perennial plant that can grow as tall as 4 m and bears white and yellow flowers. The plant is abundant within the region and is disposed of as waste.

Figure 1 Map of Niger Delta Region of Nigeria.

Source :( https:// jackjessi.wordpress.com, 2020)

2.2	Sources of Materials

The soil samples for this research were collected from the Rivers Institute of Agricultural Research and Training (RIART) Teaching and Research Farm, Rivers State University, Port Harcourt. Crude oil was collected from an oil and gas exploration and production company in Nigeria. Drill cuttings were obtained from a waste management company in the same region. The drill cuttings used did not undergo any kind of pre-treatment process by the waste management company. The crude oil and drill-cutting samples were managed in accordance with the guidelines of the Department of Petroleum Resources (DPR, 2018). The spent mushroom substrate was collected from Dilomat Farm at Rivers State University, Port Harcourt. In addition, the Costus afer bagasse was obtained from farmland in the region.

2.3	Sample Preparation

About 10kg of uncontaminated agricultural soil was mixed with varying quantities of crude oil. Spent mushroom substrate and Costus afer bagasse were used as amendment sources at rates of 0.5, 1.0, and 1.5 kg within contamination levels of 250 g, 500 g, and 750 g per 10 kg of soil, representing low, medium, and high contamination, respectively. All the samples were placed in separate reactors and allowed to incubate for 72 hours; thereafter, samples were taken immediately after incubation for baseline analysis, including a sample of uncontaminated agricultural soil. The samples were analysed in the laboratory for total petroleum hydrocarbon (TPH), total polycyclic aromatic hydrocarbons (TPAH), and total monocyclic aromatic hydrocarbon (TMAH).

2.4	Experimental Design

The 2 X 7 factorial experiment with a completely randomised design (CRD) was used for this study. For this experiment, the design consisted of seven reactors: T2 received 0.5kg of SMS, T3 received 1.0kg of SMS, T4 received 1.5kg of SMS, T5 received 0.5kg of Costus afer bagasse, T6 received 1.0kg of Costus afer bagasse, and T7 received 1.5kg of Costus afer bagasse; the control, T1, received no treatment. Each treatment had two replications. Randomisation was achieved using the draw-lot approach, as described in the literature. Each plastic reactor used for the study had a diameter of 30cm, a depth of 35cm, and contained 10kg of soil.

2.5 Statistical Analysis

Two-way analysis of variance (ANOVA) was used to determine the significance of differences among the mean values of each treatment for TPH, TPAH, and TMAH, respectively. The ANOVA for the completely randomised design was carried out using the ANOVA Data Analysis ToolPak in Microsoft Excel 2019 (Microsoft Inc., USA). Differences were considered significant if F Computed > FTabulated at 5% (p 0.05) significance level (Gomez & Gomez, 1984).

3.1 results and discussion

Table.1: Mean Results of TPH Concentration in Crude Oil Contaminated Soil at Different Levels of Contamination

| Period (days) | Mix Levels | Total Petroleum Hydrocarbon Concentration, TPH (mg/kg) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T6 | T7 |
| 3 | L | 14377 | 14586 | 14682 | 14564 | 14358 | 14433 | 15304 |
|  | M | 20229 | 20438 | 20534 | 20416 | 20210 | 20285 | 21156 |
|  | H | 24172 | 24381 | 24477 | 24359 | 24153 | 24228 | 25099 |
| 30 | L | 13449 | 7497.2 | 7032.68 | 6364.47 | 6630.52 | 6084.95 | 5862.96 |
|  | M | 19651.5 | 9687.61 | 8716.68 | 7670.29 | 8690.3 | 7972.01 | 7825.6 |
|  | H | 23862.5 | 11434.7 | 10275.4 | 9163.86 | 10236 | 9461.03 | 8912.65 |
| 60 | L | 12675.5 | 4010.72 | 3613.63 | 2967.09 | 4283.42 | 3769.99 | 3088.3 |
|  | M | 19121 | 7244.9 | 6502.74 | 4442.69 | 7595.95 | 6743.11 | 6448.35 |
|  | H | 23541.5 | 9581.73 | 8916.97 | 5791.64 | 10224 | 9584.6 | 9238.94 |
| 90 | L | 12143.5 | 2940.54 | 2519.43 | 2237.03 | 3323.88 | 2991.96 | 2656.77 |
|  | M | 18815 | 4937.82 | 3934.31 | 3135.9 | 5487.02 | 5016.48 | 4307.36 |
|  | H | 23753.5 | 6621.88 | 5668.87 | 4715.9 | 6316.01 | 5991.58 | 4859.17 |
| 120 | L | 11674 | 2438.78 | 1957.11 | 1590.39 | 2831.4 | 2442.06 | 2072.16 |
|  | M | 18558.5 | 2902.2 | 2505.15 | 2082.43 | 3274.02 | 2880.47 | 2581.03 |
|  | H | 23556 | 3103.7 | 2626.38 | 2126.54 | 3557.74 | 2987.31 | 2693.12 |

T1,2,3,4,5,6,7 = Treatments; L, M, H = Mix Level (Low, Medium, and High contamination level).

Table.2: Mean Results of PAH Concentration in Crude Oil Contaminated Soil at Different Levels of Contamination.

| Period (days} | Mix Levels | Polycyclic Aromatic Hydrocarbon Concentration, PAH (mg/kg) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  | T4 | T5 |  | T6 | T7 |  |
| 3 | L | 125.87 | 127.87 |  | 125.87 | 125.87 |  | 125.22 | 125.87 |  | 125.735 |  |
|  | M | 147.12 | 148.165 | 145.17 |  | 146.62 | 147.645 |  | 147.12 | 145.12 |  |
|  | H | 167.545 | 167.195 | 168.695 |  | 166.495 | 167.045 |  | 164.995 | 166.045 |  |
| 30 | L | 102.65 | 65.72518 | 60.29173 |  | 55.00519 | 57.8266 |  | 53.06679 | 48.16908 |  |
|  | M | 130.12 | 70.23021 | 61.62467 |  | 55.08513 | 63.48735 |  | 57.81816 | 53.67989 |  |
|  | H | 160.545 | 78.41446 | 70.81816 |  | 62.63542 | 70.79367 |  | 64.43055 | 58.96258 |  |
| 60 | L | 101.65 | 35.16825 | 30.97571 |  | 25.64676 | 37.35148 |  | 32.87704 | 25.37344 |  |
|  | M | 122.12 | 52.5238 | 45.97509 |  | 41.59512 | 55.50686 |  | 48.90391 | 44.23258 |  |
|  | H | 139.545 | 65.70764 | 61.45559 |  | 53.09526 | 70.71015 |  | 65.27202 | 61.12116 |  |
| 90 | L | 91.65 | 25.77859 | 21.59929 |  | 19.33363 | 28.98843 |  | 26.09285 | 21.8276 |  |
|  | M | 113.12 | 35.79666 | 27.81457 |  | 22.52083 | 40.08562 |  | 36.38278 | 29.54643 |  |
|  | H | 132.045 | 45.41016 | 39.06976 |  | 32.23343 | 48.69362 |  | 44.10316 | 37.12766 |  |
| 120 | L | 81.435 | 14.37986 | 9.77847 |  | 6.745 | 18.69338 |  | 14.2972 | 10.02452 |  |
|  | M | 103.405 | 21.03943 | 17.71074 |  | 14.95524 | 23.91849 |  | 20.89104 | 17.70464 |  |
|  | H | 134.11 | 21.28392 | 18.10097 |  | 14.53501 | 24.60573 |  | 20.34388 | 17.81663 |  |

Table 3: Mean Results of TMAH Concentration in Crude Oil Contaminated Soil at Different Levels of Contamination.

| Period (days) | Mix Levels | TMAH Concentration (mg/kg) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T6 | T7 |
| 3 | L | 1.28 | 1.25 | 1.25 | 1.23 | 1.29 | 1.31 | 1.21 |
|  | M | 1.46 | 1.49 | 1.41 | 1.42 | 1.59 | 1.43 | 1.5 |
|  | H | 1.63 | 1.65 | 1.62 | 1.63 | 1.64 | 1.59 | 1.63 |
| 30 | L | 1.18 | 0.64 | 0.59 | 0.54 | 0.59 | 0.55 | 0.46 |
|  | M | 1.29 | 0.71 | 0.60 | 0.53 | 0.68 | 0.56 | 0.55 |
|  | H | 1.44 | 0.77 | 0.69 | 0.61 | 0.70 | 0.62 | 0.58 |
| 60 | L | 1.09 | 0.34 | 0.30 | 0.25 | 0.38 | 0.34 | 0.24 |
|  | M | 1.22 | 0.52 | 0.45 | 0.40 | 0.60 | 0.48 | 0.457 |
|  | H | 1.39 | 0.65 | 0.59 | 0.52 | 0.69 | 0.63 | 0.60 |
| 90 | L | 1 | 0.25 | 0.21 | 0.19 | 0.30 | 0.27 | 0.21 |
|  | M | 1.11 | 0.36 | 0.276 | 0.222 | 0.43 | 0.35 | 0.31 |
|  | H | 1.255 | 0.45 | 0.38 | 0.32 | 0.43 | 0.39 | 0.32 |
| 120 | L | 0.92 | 0.21 | 0.16 | 0.13 | 0.25 | 0.22 | 0.16 |
|  | M | 1.01 | 0.21 | 0.17 | 0.14 | 0.26 | 0.20 | 0.183 |
|  | H | 1.17 | 0.215 | 0.17 | 0.149 | 0.242 | 0.20 | 0.17 |

Table 4: TPH Concentrations Percentage Reduction (%) in Crude Oil Contaminated Soil at Different Levels of Contamination

| TPH Percentage Reduction (%) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mix Level | Period (Days) | T1 | T2 | T3 | T4 | T5 | T6 | T7 |
| Low | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 30 | 6.45 | 48.60 | 52.10 | 56.30 | 53.82 | 57.84 | 61.69 |
|  | 60 | 11.83 | 72.5 | 75.39 | 79.63 | 70.17 | 73.88 | 79.82 |
|  | 90 | 15.54 | 79.84 | 82.84 | 84.64 | 76.85 | 79.27 | 82.64 |
|  | 120 | 18.25 | 89.45 | 90.08 | 93.2 | 85.78 | 88.53 | 86.46 |
| Medium | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 30 | 1.60 | 52.08 | 57.06 | 61.45 | 56.51 | 60.21 | 63.01 |
|  | 60 | 2.51 | 64.55 | 68.33 | 78.24 | 62.41 | 66.76 | 69.52 |
|  | 90 | 6.99 | 75.84 | 80.84 | 84.64 | 72.85 | 75.27 | 79.64 |
|  | 120 | 8.26 | 85.8 | 87.80 | 89.80 | 83.80 | 85.80 | 87.80 |
| High | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 30 | 1.28 | 53.1 | 58.02 | 62.38 | 57.62 | 60.95 | 64.49 |
|  | 60 | 2.19 | 60.70 | 63.57 | 76.22 | 57.67 | 60.44 | 75.14 |
|  | 90 | 2.97 | 72.84 | 76.84 | 80.64 | 73.85 | 75.27 | 80.64 |
|  | 120 | 3.38 | 83.17 | 85.18 | 87.16 | 81.13 | 83.54 | 89.27 |

Table 5: TPAH Concentrations Percentage Reduction (%) in Crude Oil Contaminated Soil at Different Levels of Contamination

| TPAH Percentage Reduction (%) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mix Level | Period (Days) | T1 | T2 | T3 | T4 | T5 | T6 | T7 |
| Low | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 30 | 18.45 | 48.60 | 52.10 | 56.30 | 53.82 | 57.84 | 61.69 |
|  | 60 | 19.245 | 72.5 | 75.39 | 79.62 | 70.17 | 73.88 | 79.82 |
|  | 90 | 27.19 | 79.84 | 82.84 | 84.64 | 76.85 | 79.27 | 82.64 |
|  | 120 | 35.30 | 88.75 | 92.23 | 94.642 | 85.07 | 88.641 | 92.03 |
| Medium | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 30 | 11.56 | 52.6 | 57.55 | 62.43 | 57 | 60.7 | 63.01 |
|  | 60 | 17.00 | 64.55 | 68.33 | 71.63 | 62.41 | 66.76 | 69.52 |
|  | 90 | 23.11 | 75.84 | 80.84 | 84.64 | 72.85 | 75.27 | 79.64 |
|  | 120 | 29.71 | 85.80 | 87.80 | 89.80 | 83.80 | 85.80 | 87.80 |
| High | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 30 | 4.18 | 53.10 | 58.02 | 62.38 | 57.62 | 60.95 | 64.49 |
|  | 60 | 16.71 | 60.70 | 63.57 | 68.11 | 57.67 | 60.44 | 63.19 |
|  | 90 | 21.19 | 72.84 | 76.84 | 80.64 | 70.85 | 73.27 | 77.64 |
|  | 120 | 22.94 | 81.89 | 83.93 | 85.86 | 79.88 | 82.22 | 83.85 |

Table 6: TMAH Concentrations Percentage Reduction (%) in Crude Oil Contaminated Soil at Different Levels of Contamination

| TMAH Percentage Reduction (%) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mix Level | Period (Days) | T1 | T2 | T3 | T4 | T5 | T6 | T7 |
| Low | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 30 | 7.81 | 48.8 | 52.42 | 56.1 | 54.26 | 58.02 | 61.98 |
|  | 60 | 14.84 | 72.80 | 75.81 | 79.67 | 70.54 | 74.05 | 80.17 |
|  | 90 | 18.75 | 80.00 | 83.06 | 84.55 | 76.74 | 79.39 | 82.65 |
|  | 120 | 20.31 | 86.40 | 90.32 | 92.68 | 87.60 | 88.55 | 90.09 |
| Medium | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 30 | 4.79 | 52.35 | 57.45 | 62.68 | 57.23 | 60.84 | 63.33 |
|  | 60 | 6.85 | 64.43 | 68.09 | 71.83 | 62.26 | 66.43 | 69.33 |
|  | 90 | 8.22 | 75.84 | 80.85 | 84.51 | 72.96 | 75.52 | 79.33 |
|  | 120 | 9.59 | 85.91 | 87.94 | 90.14 | 83.65 | 86.01 | 88.00 |
| High | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 30 | 2.45 | 53.33 | 58.02 | 62.58 | 57.32 | 61.01 | 64.42 |
|  | 60 | 3.68 | 60.61 | 63.58 | 68.10 | 57.93 | 60.38 | 63.19 |
|  | 90 | 4.29 | 72.73 | 76.54 | 80.37 | 73.78 | 75.47 | 80.37 |
|  | 120 | 4.91 | 81.21 | 83.33 | 85.28 | 79.27 | 81.13 | 83.44 |

3.2 Effects of SMS and Costus afer bagasse on TPH, TPAH, and TMAH in the remediation of soil contaminated with petroleum hydrocarbon at different levels

Figures 2-4 show the chemical characteristics of the petroleum hydrocarbon-contaminated soil. The crude oil-contaminated sandy-loam soil contained TPH, TPAH, and TMAH, as corroborated in the literature (Okparanma et al., 2014). It was further observed from Tables 1-3 that the TPH, TPAH, and TMAH contents of the untreated soils, including the control, ranged from 14,377-25, 099mg/kg, 125.22 – 168.69mg/kg and 1.21 - 1.65mg/kg, respectively, and were above the 5000mg/kg, 40mg/kg and 1mg/kg intervention values specified by the DPR (2018), suggesting that the soil was unsafe for agricultural purposes and human health. Variations were also observed in the TPH, TPAH, and TMAH concentrations at different levels of crude oil contamination, with the high contamination level resulting in the highest TPH concentration, followed by the medium and low contamination levels (Figures 2-4). The TPH, TPAH, and TMAH concentrations at all contamination levels exceeded the intervention values prescribed by the Nigerian regulatory framework (DPR, 2018).

This, therefore, justifies the need for remediation of petroleum hydrocarbon-contaminated soils before they can be utilised for agricultural purposes or reused for civil engineering works. The TPH, TPAH, and TMAH contents of the sandy-loam soil treated with SMS and Costus afer bagasse at different levels of crude oil contamination over a period of 120 days are shown in Figures 2-4. Tables 1-3 also show the variation in TPH, TPAH, and TMAH concentrations in petroleum hydrocarbon-contaminated sandy-loam soil at different soil/SMS and Costus afer bagasse mix ratios (5:0.5, 5:1, 5:1.5) across the various petroleum hydrocarbon contamination levels (low, medium, and high). The degradation of TPH, TPAH, and TMAH after thirty (30) days of remediation in sandy-loam soil treated with different amounts of SMS and Costus afer bagasse (Tables 1-3) at low (L), medium (M), and high (H) contamination levels is shown in Figures 2-4. The degradation of TPH, TPAH, and TMAH increased with time. Degradation was more rapid during the first 30 days but slowed thereafter until day 120. This pattern is consistent with the findings of Okparanma et al. (2021), who reported that most residual TPH degradation occurs during the initial period of remediation.

Figures 2-4 also show that TPH, TPAH, and TMAH concentrations decreased with increasing quantities of SMS and Costus afer bagasse and with increasing treatment duration. Moreover, lower contamination levels resulted in higher removal efficiencies of TPH, TPAH, and TMAH in the soil (Tables 4-6). This may be attributed to the depletion of oxygen in the soil following the release of petroleum hydrocarbons, which can adversely affect microorganisms involved in contaminant degradation.

After 30 days of treatment, crude oil-contaminated soil showed significant reductions in hydrocarbon concentrations. At the low contamination level, the TPH, TPAH, and TMAH contents decreased. The reduction was higher in the soil/SMS and Costus afer bagasse ratio of 5:1.5 (T4T4T4), as shown in Tables 1-3; next was the 5:1 soil/SMS ratio (T3T2T2), and the lowest was the soil/SMS and Costus afer bagasse ratio of 5:0 (T1T1T1). The same pattern was observed across the medium and high contamination levels. The trend continued until the end of the 120-day remediation period, as presented in Figures 2-4. However, remediation of crude oil-contaminated soil with SMS and Costus afer bagasse was effective in reducing TPH, TPAH, and TMAH to below the local regulatory intervention values of 5000mg/kg, 40mg/kg, and 1mg/kg, respectively, but the concentrations exceeded the target values of 50mg/kg, 10mg/kg, and 0.05mg/kg (DPR, 2018). The untreated crude oil-contaminated soil served as the control and remained above the intervention values of the Nigerian regulatory framework (DPR, 2018). This, therefore, shows that the soil/SMS and Costus afer bagasse ratio of 5:1.5 (T4) achieved the highest TPH, TPAH, and TMAH reduction in the remediation of soil contaminated with crude oil at the low contamination level (C1). However, it remained above the DPR (2018) target limit for soil and sediment. This might be due to the remediation potential of SMS and Costus afer bagasse (Emeka et al., 2023; Gbarakoro & Chukumati, 2019; Mayans et al., 2024). The ANOVA results for the effects of different soil/SMS and Costus afer bagasse mix ratios on TPH, TPAH, and TMAH concentrations support the acceptability of the experimental results. There was a significant difference among the treatment means at the 5% significance level, as F-calculated > Tabulated. This suggests that, with 95% confidence, the differences among treatment means were due to the different soil/SMS and Costus afer bagasse mix ratios applied.

Figure 2: Effects of SMS and Costus afer Bagasse on TPH Contaminated Soil at Different Contamination Levels. Error Bar (Percentage)

Figure 3: Effects of SMS and Costus afer Bagasse on TPAH Contamination Soil at Different Contamination Levels. Error Bar (Percentage)

Figure 4:	Effects of SMS and Costus afer Bagasse on TMAH Contamination Soil at Different Contamination Levels. Error Bar (Percentage)

Conclusions

This study demonstrated that spent mushroom substrate (SMS) and Costus afer bagasse can significantly enhance the bioremediation of petroleum hydrocarbon-contaminated sandy-loam soil. SMS exhibited superior performance, achieving maximum reductions of 93.20% (TPH), 94.64% (TPAH), and 92.68% (TMAH) after 120 days, while Costus afer bagasse achieved reductions up to 89.27%, 92.03%, and 90.09%, respectively.

Statistical analyses confirmed significant differences (p < 0.05) among amendment rates, indicating that remediation efficiency increased with amendment quantity. Although treatment reduced hydrocarbon concentrations below Nigerian intervention limits, residual levels remained above regulatory target values, suggesting that extended treatment periods or integrated remediation approaches may be required. Overall, the findings support the use of locally available agro-wastes, particularly SMS, as cost-effective and environmentally sustainable amendments for remediating petroleum hydrocarbon-contaminated soils in the Niger Delta region. Therefore, SMS and Costus afer bagasse show promising potential as low-cost biostimulants; however, additional treatment duration or combined remediation strategies may be required to meet regulatory target values.

Limitations

This study was conducted in controlled reactors using sandy-loam soil, Bonny Light crude oil, and amendment materials sourced within the Niger Delta; therefore, the findings are most directly applicable to comparable conditions. The 120-day assessment focused on TPH, TPAH, and TMAH as the principal remediation endpoints. Field-scale performance may vary with soil type, environmental conditions, contaminant composition, and treatment duration. Further evaluation under a broader range of field conditions and over longer treatment periods would help establish the transferability and persistence of the observed remediation effects.

Competing interests

There was no competing interest during my study.

Declaration of AI Use

This manuscript was prepared through the intellectual contributions of the author(s) to the study design, data analysis, interpretation of results, and scholarly content. Grammarly and ChatGPT were used solely to assist with grammatical refinement and reference formatting during manuscript revision. The author(s) accept full responsibility for the accuracy, integrity, and final content of the manuscript.

REFERENCES

Adams, F. V., Niyomugabo, A., & Sylvester, O. P. (2016). Bioremediation of crude oil contaminated soil using agricultural wastes. Procedia Manufacturing, 7, 459–464. https://doi.org/10.1016/j.promfg.2016.12.037

Akujuru, V. A., & Ruddock, L. (2014). Incorporation of socio-cultural values in damage assessment valuations of contaminated lands in the Niger Delta. Land, 3(3), 675–692. https://doi.org/10.3390/land3030675

Al-Dhabaan, F. A. (2019). Morphological, biochemical and molecular identification of petroleum hydrocarbons biodegradation bacteria isolated from oil polluted soil in Dhahran, Saud Arabia. Saudi Journal of Biological Sciences, 26(6), 1247–1252. https://doi.org/10.1016/j.sjbs.2018.05.029

Al-Hawash, A. B., Dragh, M. A., Li, S., Alhujaily, A., Abbood, H. A., Zhang, X., & Ma, F. (2018). Principles of microbial degradation of petroleum hydrocarbons in the environment. The Egyptian Journal of Aquatic Research, 44(2), 71–76. https://doi.org/10.1016/j.ejar.2018.06.001

Anifowose, B., Lawler, D., van der Horst, D., & Chapman, L. (2008). Transportation in Nigeria's oil and gas industry: An environmental challenge. In Proceedings of the Postgraduate Researchers' Conference: Meeting Environmental Challenges in the Coastal Region of Nigeria. https://pureportal.coventry.ac.uk/en/publications/transportation-in-nigerias-oil-and-gas-industry-an-environmental-/

Ayotamuno, M. J., Kogbara, R. B., & Taleat, M. O. (2006). Bioremediation of a petroleum-hydrocarbon polluted agricultural soil at different levels of water application in Port Harcourt, Nigeria. Journal of Food, Agriculture & Environment, 4(3–4), 214–217. https://www.wflpublisher.com/Abstract/947

Ayotamuno, M. J., Kogbara, R. B., Agele, E. A., & Agoro, O. S. (2010). Composting and phytoremediation treatment of petroleum sludge. Soil and Sediment Contamination: An International Journal, 19(6), 686–695. https://doi.org/10.1080/15320383.2010.515627

Department of Petroleum Resources. (2018). Environmental guidelines and standards for the petroleum industry in Nigeria (EGASPIN) (3rd ed.). https://www.nuprc.gov.ng/wp-content/uploads/2019/03/DPR-NEWS-JANUARY-2019.pdf

Zhang, C., Wu, D., & Ren, H. (2020). Bioremediation of oil contaminated soil using agricultural wastes via microbial consortium. Scientific Reports, 10, Article 9188. https://doi.org/10.1038/s41598-020-66169-5

Elechi, O., Yelebe, Z. R., Oruabena, B., & Nelson, S. E. (2018). Comparative study of degradation kinetics of crude oil contaminated mangrove and clay soils. International Journal of Advances in Scientific Research and Engineering, 4(11), 113–124. https://doi.org/10.31695/IJASRE.2018.32956

Emeka, C., Okparanma, R. N., & Okogbule-Wonodi, A. (2023). Bioremediation of total polycyclic aromatic hydrocarbon in crude oil-contaminated soil using Costus afer plant. Research Journal of Ecology and Environmental Sciences, 3(1), 26–36. https://doi.org/10.31586/rjees.2023.656

Gbarakoro, T. N., & Chukumati, J. (2019). Short-term remediative impact of spent mushroom substrate on soil nutrients in spent automobile lubricant habitat-types at University Farm, University of Port Harcourt, Nigeria. International Journal of Ecosystem, 9(1), 1–5. https://doi.org/10.5923/j.ije.20190901.01

Gomez, K. A., & Gomez, A. A. (1984). Statistical procedures for agricultural research (2nd ed.). John Wiley & Sons. https://uat.store.wiley.com/en-us/statistical-procedures-for-agricultural-research-2nd-edition-p-9780471870920

Guarino, C., Spada, V., & Sciarrillo, R. (2017). Assessment of three approaches of bioremediation (natural attenuation, landfarming and bioagumentation–assistited landfarming) for a petroleum hydrocarbons contaminated soil. Chemosphere, 170, 10–16. https://doi.org/10.1016/j.chemosphere.2016.11.165

Mayans, B., Antón-Herrero, R., García-Delgado, C., Delgado-Moreno, L., Guirado, M., Pérez-Esteban, J., Escolástico, C., & Eymar, E. (2024). Bioremediation of petroleum hydrocarbons polluted soil by spent mushroom substrates: Microbiological structure and functionality. Journal of Hazardous Materials, 473, Article 134650. https://doi.org/10.1016/j.jhazmat.2024.134650

Muttaleb, W. H., & Ali, Z. H. (2022). Bioremediation an eco-friendly method for administration of environmental contaminants. MINAR International Journal of Applied Sciences and Technology, 4(2), 22–32. https://doi.org/10.47832/2717-8234.11.3

Nweke, B., Okparanma, R. N., & Ayotamuno, J. M. (2024a). Effectiveness of African palm (Raffia farinifera) substrate in the bioremediation of oil-based drill cuttings. International Journal of Academic Engineering Research, 8(4), 28–34. https://ijeais.org/wp-content/uploads/2024/4/IJAER240407.pdf

Nweke, B., Okogbule-Wonodi, A., & Chikwue, I. M. (2024b). The influence of fresh raffia palm trunk on the bioremediation of oil-based drill cuttings. International Journal of Pollution: Prevention & Control, 1(2), 41–48. https://journals.stmjournals.com/ijppc/article=2024/view=130866/

Ogbonna, D. N., Ngah, S. A., Okparanma, R. N., Ule, O., & Nrior, R. R. (2020). Percentage bioremediation assessment of spent mushroom substrate (SMS) and Mucor racemosus in hydrocarbon contaminated soil. Journal of Advances in Microbiology, 20(12), 1–21. https://doi.org/10.9734/jamb/2020/v20i1230305

Okparanma, R. N., Coulon, F., & Mouazen, A. M. (2014). Analysis of petroleum-contaminated soils by diffuse reflectance spectroscopy and sequential ultrasonic solvent extraction–gas chromatography. Environmental Pollution, 184, 298–305. https://doi.org/10.1016/j.envpol.2013.08.039

Okparanma, R. N., Orisa, F. T., & Ukoha-Onuoha, E. (2021, October). Synthesis and application of water treatment residuals from fishpond effluent for remediation of hydrocarbon-contaminated soil [Conference paper]. NIEE Port Harcourt Branch Conference: Transforming Solid Waste to Renewable Energy, Port Harcourt, Nigeria.

Okparanma, R. N., Ukoha-Onuoha, E., & Ayotamuno, M. J. (2022). Predicting selected kinetic parameters of hydrocarbon interactions with alum-based water treatment residuals. Results in Engineering, 16, Article 100726. https://doi.org/10.1016/j.rineng.2022.100726

Omenna, E. C., Omage, K., Ezaka, E., & Azeke, M. A. (2024). Bio-augmentation and bio-stimulation with kenaf core enhanced bacterial enzyme activities during bio-degradation of petroleum hydrocarbon in polluted soil. Scientific Reports, 14, Article 8. https://doi.org/10.1038/s41598-023-50882-y

Ossai, I. C., Ahmed, A., Hassan, A., & Hamid, F. S. (2020). Remediation of soil and water contaminated with petroleum hydrocarbon: A review. Environmental Technology & Innovation, 17, Article 100526. https://doi.org/10.1016/j.eti.2019.100526

Sales da Silva, I. G., Gomes de Almeida, F. C., Padilha da Rocha e Silva, N. M., Casazza, A. A., Converti, A., & Asfora Sarubbo, L. (2020). Soil bioremediation: Overview of technologies and trends. Energies, 13(18), Article 4664. https://doi.org/10.3390/en13184664

Sayed, K., Baloo, L., & Sharma, N. K. (2021). Bioremediation of total petroleum hydrocarbons (TPH) by bioaugmentation and biostimulation in water with floating oil spill containment booms as bioreactor basin. International Journal of Environmental Research and Public Health, 18(5), Article 2226. https://doi.org/10.3390/ijerph18052226

Udem, S. C., & Ezeasor, C. K. (2010). The acute and subchronic toxicity studies of aqueous leaf and stem bark extract of Costus afer Ker (Zingiberaceae) in mice. Comparative Clinical Pathology, 19(1), 75–80. https://doi.org/10.1007/s00580-009-0906-8

Ugochi, E. I., Ogugbue, C. J., & Onojake, M. C. (2025). Effect of animal bone biochar on total petroleum hydrocarbon reduction and fertility restoration in crude oil-contaminated soil. Asian Journal of Current Research, 10(4), 280–289. https://doi.org/10.56557/ajocr/2025/v10i49933

Uyigue, E., & Agho, M. (2007). Coping with climate change and environmental degradation in the Niger Delta of southern Nigeria. Community Research and Development Centre.

Valizadeh Rad, K., Motesharezadeh, B., Alikhani, H. A., & Dadrasnia, A. (2021). The potential use of Cordia myxa in the remediation of crude oil pollution. International Journal of Phytoremediation, 23(5), 445–453. https://doi.org/10.1080/15226514.2020.1825326

Vidonish, J. E., Zygourakis, K., Masiello, C. A., Sabadell, G., & Alvarez, P. J. J. (2016). Thermal treatment of hydrocarbon-impacted soils: A review of technology innovation for sustainable remediation. Engineering, 2(4), 426–437. https://doi.org/10.1016/J.ENG.2016.04.005

Wang, C., Gu, L., Ge, S., Liu, X., Zhang, X., & Chen, X. (2019). Remediation potential of immobilized bacterial consortium with biochar as carrier in pyrene-Cr(VI) co-contaminated soil. Environmental Technology, 40(18), 2345–2353. https://doi.org/10.1080/09593330.2018.1441328